IEEE P802.22
Wireless RANs

| 802.22 Incumbent Database Interface |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave. Ottawa, Ontario Canada K2H 8S2 | 613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This document summarizes the discussions that took place on Monday afternoon, July 13th, during the 802.22 System Meeting on the identification of the possible interface to the Incumbent Database from the 802.22 perspective.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### 802.22 Incumbent Database Interface

802.22 system model

The system model that has been assumed all along in the development of the 802.22 Standard is a point-to-multipoint model for extending broadband access to less populated rural areas where more available TV channels can be found.  In this model, the base station (BS) is assumed to control all RF parameters of its associated consumer premise equipment (CPE) (frequency, EIRP, modulation, etc.)  in a ”master-slave” relationship so that the responsibility of protecting the TV broadcast incumbents is fully carried by the Wireless Regional Area (WRAN) operator.

Because of this model, it is assumed that the interface to the incumbent database proposed in the FCC R&O 08-260 will take place entirely between the BS and the Incumbent Database on behalf of its associated CPE unlike what has been described in the current version of the R&O.  The CPE will not be able to access the database directly but will be controlled by the BS based on information obtained from the Incumbent Database and sensing results gathered by the BS from its various associated CPEs.

There is no need to provide a list of available channels to each CPE in a WRAN cell because the BS will have to select the channel that is common to all associated CPEs to avoid interference to broadcast incumbents.  There is no need for a CPE to know that it could have used 5 channels at its location if he had been able to operate on its own while, as part of the WRAN vell, only 2 channels are available for all.  Queries to the incumbent database will be done by the BS on behalf of the CPE and the response will be kept and managed at the BS to find the ‘most common denominator’ to identify the operating channel and a prioritized list of backup channels that will then be sent to the CPEs for them to switch to if the operating channel becomes unavailable and also to be sensed during their idle time to make sure that these channels are still available, i.e., an incumbent has not appeared on one of these channels recently, in such case, this is reported to the BS so that the backup channel list is updated and sent to the CPEs for continuing monitoring.

##### Database access

First, the CPE has to provide the necessary information to the base station at the time of association otherwise such association will be rejected.  This information will be:

- Geolocation (latitude, longitude and height above average ground level)

- Device identification (Product identification (FCC ID) and the serial number (could be the MAC address)

- Type of device: fixed, portable mode 2 or portable mode 1

The operator of the BS will initially register with the Incumbent Database as a fixed device in providing all the information required as specified in the FCC R&O.  It will also register all its associated CPEs with their geolocation, device identification, etc. as obtained at association.

On an ongoing basis, the BS will then register with the Incumbent Database (at least once every 24 hours) and set up the dialog so that it can query the database as well as having the database send any update relevant to its operation through ‘push’ internet technology based on its location and the characteristics of the equipment used as well as the location and the characteristics of the associated CPEs. Such ‘push’technology would allow better reaction time than the 24 hours minimum access time currently specified while keeping the database traffic to a minimum.

To allow this ‘push’ technology, the connection from the database to the TVBD will need to be kept alive either by using a static IP address or the IP address of a proxy (e.g., one proxy per network operator using different ports for the different BSs) or other means.  In the point-to-multipoint topology assumed for 802.22, only the base station or even the proxy for a number of base stations from a same operator needs to be known to the database to implement this ‘push’ technology.

##### Registration and access primitives to the incumbent database

The following list defines what has currently been included in the 802.22 Draft standard to implement access to the incumbent database.  This list of primitives has been defined to exchange the necessary information between the Spectrum Manager which is the entity that is responsible for taking the decision about the choice of the operating channel for the WRAN cell at the BS and the Network Control Management Service (NCMS) which will interface at higher layer and through the network backhaul with the incumbent database.

M-DB-EXISTS.request: primitive that allows the SM to identify what database services exist through the Network Control Management Service (NCMS) in order to obtain channel availability. It will be generated by the Spectrum Manager (SM) and issued to its NCMS to identify the types of incumbent database services that exist and can be accessed.

M- EXISTS.confirm: primitive that allows the NCMS to inform the SM of the types of database services that exist and are accessible through the NCMS.

M-DB-AVAILABLE.request: primitive that allows the SM to identify what database services are accessible through the NCMS in order to obtain channel availability information.

M-DB-AVAILABLE.confirm: primitive that allows the NCMS to inform the SM of the types of database services that are accessible through the NCMS.

M-DB-REG.request: primitive that allows the SM to request the NCMS to register a geo-location, which could be either a geo-location of the BS itself, or a geo-location of an associated CPEs, or a geo-location of interest, in an incumbent database.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Database_type | Integer | 0-8 | The value identifies the type of database for which the request is directed. 0 = TV Incumbent Database 1 = Part 74 Incumbent Database |
| Device Type | Integer | 0-1 | The value identifies the type of device at the geo-location registering 0 = WRAN BS 1 = WRAN CPE |
| REG_TYPE | Integer | 0-1 | The value identifies if the present request is a registration request, or a query request for a already registered location. |
| Location Data String | Char | NMEA 0183 ASCII string | The value identifies the geo-location to be registered, |
| Antenna information |  | Variable | Antenna height, directionality information of a WRAN device at the said geo-location to facilitate the incumbent database calculation of the maximum allowed EIRP values at the registering location. |

M-DB-REG.confirm: primitive that is used to inform the SM whether its request to register a geolocation in an external incumbent database service was successfully generated by the NCMS.

M-DB-REG.response: primitive that is used to inform the SM whether its request to register a geolocation in an external database service was successfully received by the database.

M-DB-INDICATION.indication: primitive that is used to inform the SM when an indication to a previously registered geo-location in an external database service was successfully received by the NCMS.  When the SM receives the M-DB-INDICATION.indication, SM shall obtain the list of available channels and corresponding maximum EIRP at the registered geo-location.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Database_type | Integer | 0-8 | The value identifies the type of database for which the query request was directed. 0 = TV Incumbent Database 1 = Part 74 Incumbent Database |
| Location Data String | Char | NMEA 0183 ASCII string | The value identifies the registered geo-location. |
| For (i=1 to Number of Channels Avaliable, i++) { Channel_Number Max_Transmit_EIRP } | List of available channels and max EIRP allowed per channel |  | List of Available Channel Numbers and corresponding maximum transmit EIRP allowed. |

M-DB-DEREG.request: primitive that allows the SM to request the NCMS to deregister a previously registered geo-location in a database.

M-DB-DEREG.confirm: primitive that is used to inform the SM whether its request to deregister a previously registered geo-location in an external database service was successfully generated by the NCMS.

M-DB-DEREG.response: primitive that is used to inform the SM whether its request to deregister a previously registered geo-location in an external database service was successfully received by the database.

##### Actual parameters being considered

As can be seen from the above primitives, the registration to the incumbent database includes the location as well as the antenna height and directionality above average ground level of the TVBD which goes beyond the minimum information required by the FCC R&O.  This will allow for an extension of the database service to a larger number of device types because the location, antenna height and directivity are the key generic parameters in calculating potential interference, independent of how these devices are classified.

In the response parameters from the database, beyond the list of available channels in a specific location, the primitives call for the maximum EIRP allowed in each channel which gives much more capability in terms of expanding the database services from the engineering point of view since avoidance if interference is really based on limiting the amount of EIRP transmitted in a location.  Excluding the use of a channel is a crude way of doing the same thing.  The list of maximum EIRP per channel imbeds the notion of available/unavailable channel but can quantify it by comparing it to diffrent EIRP thresholds specified by the regulatory authority (e.g., 4 Watt for fixed, 100 mW for portable, 40 mW for portable in adjacent channels, and any other device types that can be identified in the future).  Note that the list of available channels will require 8-bit times the number of available channels at the specified location whereas the maximum allowed EIRP in each channel will require 8-bit times the total number of channels in a location which, sizewise, is not much bigger.

802.22 is of the opinion that besides the TVBD location, antenna height and directivity would be useful as input to the incumbent database for future extention of the service as well as a response in terms of maximum allowed EIRP per TV channel rather than a simple list of available channels, noting that the latter can be easily deduced from the former.

Security for these messages:

802.22 is of the opinion that security on the messages exchanged between the Base Station and the Incumbent Database will be critical for the proper operation of the systems to allow authentication of the database provider as well as the WRAN system querying for the service.  Security will also be necessary to avoid the message exchange being altered on the backhaul connection.  SSL was identified as an acceptable way to provide transport layer security.

### References:

[1]	P802.22/D2, Draft Standard for Wireless Regional Area Networks, “Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands”, May 2001

______________________________

Note that in its Petition for Reconsideration, the IEEE 802 has addressed the fact that the current interface proposes a direct interaction between the Incumbent Database and the CPEs which goes contrary to the assumption that the BS and its operator will be responsible for the protection of the broadcast incumbents.  It is hoped that the FCC, in response to the Petition for Reconsideration, will consider this “master-slave” relationship that the 802.22 has assumed in its work and allow the BS to be the proxy of its associatred CPEs..