IEEE P802.22
Wireless RANs

| 802.22 Database Service Interface |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-09-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave. Ottawa, Ontario Canada K2H 8S2 | 613-998-2500 | gerald.chouinard@crc.ca |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |
| Ranga Reddy | US Army | Bldg 2700, Fort Monmouth, NJ 07703 | 732-532-0085 | Ranga.reddy@us.army.mil |

Abstract

This document summarizes the discussions that took place on Monday afternoon, July 13th, during the 802.22 System Meeting and subsequent conference calls convened by Winston Caldwell between the San Francisco and the Hawaii 802.22 sessions on the identification of the possible interface to the Incumbent Database from the 802.22 perspective.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Wendong Hu> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### 802.22 Database Service Interface

802.22 system model

The system model that has been assumed all along in the development of the 802.22 Standard is a point-to-multipoint model for extending broadband access to less populated rural areas where more available TV channels can be found.  In this model, the base station (BS) is assumed to control all RF parameters of its associated customer premises equipment (CPE) (frequency, EIRP, modulation, etc.)  in a “master-slave” relationship so that the responsibility of protecting the TV broadcast incumbents is fully carried by the Wireless Regional Area Network (WRAN) operator.

##### Registration and access primitives to the database service

The following list of primitives defines the necessary messaging to support access to the database service by the BS.  The format described below shall be used for the messages sent directly to the database service as well as those received directly from the database service.

M-DB-AVAILABLE.request: primitive that allows the BS to verify that it is connected to the database service in order to receive channel availability and maximum allowed EIRP updates.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Base station-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Access Type | Integer | 1 byte | The value identifies the type of access being used for communication to the database. 0x00 = URL 0x01 = IPv4 IP address 0x02 = IPv6 IP address 0x03-0xFF = Reserved |
| Database Service Network Address { |  |  |  |
| If (Access Type = 0x00) |  |  |  |
| Database Service URL | ASCII String | Variable | A fully qualified URL starting with, http:// or https:// |
| Else If (Access Type = 0x01) |  |  |  |
| Database Service IPv4 Address | Integer | 4 bytes | IPv4 network address for database service |
| Else if (Access Type = 0x02) |  |  |  |
| Database Service IPv6 Address | Integer | 16 bytes | IPv6 network address for database service |
| Else [Access Type = 0x03 – 0xFF] |  |  |  |
| Reserved mechanism | ASCII String and/or Integer | Variable | Reserved to allow identification of other access mechanisms |
| } |  |  |  |
| Base station Network Address { |  |  |  |
| If (Access Type = 0x00) |  |  |  |
| Database Service URL | ASCII String | Variable | A fully qualified URL starting with, http:// or https:// |
| Else If (Access Type = 0x01) |  |  |  |
| Database Service IPv4 Address | Integer | 4 bytes | IPv4 network address for database service |
| Else if (Access Type = 0x02) |  |  |  |
| Database Service IPv6 Address | Integer | 16 bytes | IPv6 network address for database service |
| Else [Access Type = 0x03 – 0xFF] |  |  |  |
| Reserved mechanism | ASCII String and/or Integer | Variable | Reserved to allow identification of other access mechanisms |
| } |  |  |  |
| Timestamp | ASCII String | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string. |

M-DB-AVAILABLE.confirm: primitive that allows the database service to confirm that the BS is still connected to the database service.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Base station-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| } |  |  |  |
| Timestamp | ASCII String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE.request. |

M-DEVICE-REGISTRATION.request: primitive that allows the BS to register with the database service a device that has joined its WRAN network.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device obtained as part of its process to associate 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 1 0x03 = Personal/portable mode 2 0x04-0xFF = Reserved |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Location Data String | ASCII String | NMEA 0183 | The value identifies the location of the device (latitude, longitude). |
| Responsible Party Name { | ASCII String | Variable |  |
| If (Device Type = 0x00) |  |  |  |
| Contact Name | ASCII String | Variable |  |
| Contact Physical Address | ASCII String | Variable |  |
| Contact Email Address | ASCII String | Variable |  |
| Contact Telephone Number } { | ASCII String | Variable |  |
| If (Device Type = 0x00 or 0x01) |  |  |  |
| Antenna information |  | Variable | Antenna directionality information of the device (to facilitate the database calculation of the channel availability and the maximum allowed EIRP values at the registering location). |
| } |  |  |  |

M-DEVICE-REGISTRATION.confirm: primitive that allows the database service to confirm to the BS that the  new device has been successfully registered.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device being registered 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 1 0x03 = Personal/portable mode 2 0x04-0xFF = Reserved |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Location Data String | ASCII String | NMEA 0183 | The value identifies the the location of the device (latitude, longitude). |
| Responsible Party Name { | ASCII String | Variable |  |
| If (Device Type = 0x00) |  |  |  |
| Contact Name | ASCII String | Variable |  |
| Contact Physical Address | ASCII String | Variable |  |
| Contact Email Address | ASCII String | Variable |  |
| Contact Telephone Number } { | ASCII String | Variable |  |
| If (Device Type = 0x00 or 0x01) |  |  |  |
| Antenna information |  | Variable | Antenna directionality information of the device and the azimuth of its main beamn. |
| } |  |  |  |

M-DB-AVAILABLE-CHANNEL.request: primitive that allows the BS to request a list of available channels and maximum allowed EIRP per channel from the database service for the specified type of device at the particular location and height.

| Name | Type |  | Length |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Device Type | Integer |  | 1 byte |  |  | The value identifies the type of device at the geo-location registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 1 0x03 = Personal/portable mode 2 0x04-0x255 = Reserved |
| Device-ID | ASCII String |  | Variable |  |  | In US, this is FCC-ID |
| Serial Number | ASCII String |  | Variable |  |  |  |
| Location Data String | Char |  | NMEA 0183 ASCII string |  |  | The value identifies the location of the device (latitude, longitude). |
| Timestamp |  | ASCII String |  | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string. |  |

M-DB-AVAILABLE-CHANNEL.confirm: primitive that is used by the database service to inform the BS whether its request for available channels and maximum allowed EIRP was successfully received by the database service.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device at the geo-location registering 0 = Fixed 1 = Personal/portable mode 1 2 = Personal/portable mode 2 3-255 = Reserved |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Location Data String | Char | NMEA 0183 ASCII string | The value identifies the location of the device (latitude, longitude ). |
| Timestamp | ASCII String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE-CHANNEL.request. |

M-DB-AVAILABLE-CHANNEL.indication: primitive that is used to return to the BS the list of available channels as provided by the database service in the form of channel, maximum allowed EIRP, and availability schedule.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device- ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial number | ASCII String | Variable |  |
| Number of Channels Available | Integer | 1 byte |  |
| { If( Number of Channels Available > 0) |  |  | If the number of channels is equal to 0, this means that the device cannot operate. |
| For (i=1 to Number of Channels Available, i++) { Channel_Number Max_Allowed_EIRP (dBm) Availability schedule } | List of available channels and max EIRP (dBm) allowed per channel, and the schedule | 2 bytes and standard UTS date and time format | List of Available Channel Numbers and corresponding maximum allowed EIRP (dBm) as well as the availability schedule (start and stop date/time) for each channel in Universal date and time system. |
| } |  |  |  |
| Timestamp | ASCII String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE-CHANNEL.request. |

M-DB-DEREG.request: primitive that allows the BS to request the database service to remove the registration of a device that was associated with that base station.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Responsible Party Name | ASCII String | Variable |  |
| Location Data String | Char | NMEA 0183 ASCII string | The value identifies the location of the device (latitude, longitude). |

M-DB-DEREG.confirm: primitive that is used to inform the BS whether its request to deregister a previously registered device was successfully received and executed by the database service.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Location Data String | Char | NMEA 0183 ASCII string | The value identifies the location of the device (latitude, longitude). |

##### Actual parameters being considered

As can be seen from the above primitives, the registration to the database service includes the location of the TVBD as well as its antenna height above average ground level, its directionality and azimuthwhich goes beyond the minimum information required by the FCC R&O.  This will allow for an extension of the database service to a larger number of device types because the location, antenna height, directivity and azimuth are the key generic parameters in calculating potential interference, independent of how these devices are classified.

In the response from the database, beyond the list of available channels in a specific location, the primitives call for the maximum EIRP allowed in each channel which gives much more capability in terms of expanding the database services from the engineering point of view since interference avoidance is really based on limiting the amount of EIRP transmitted in a location to avoid interference to incumbents.  Excluding the use of a channel is a crude way of doing the same thing.  The list of maximum EIRP per channel imbeds the notion of available/unavailable channel but can quantify it by comparing it to different EIRP thresholds specified by the regulatory authority (e.g., 4 Watt for fixed, 100 mW for portable mode II, 50 mw for portable mode I, 40 mW for portable in adjacent channels, and any other device types that can be identified in the future).  In the case where the BS controls the EIRP of the associated TVBD, devices operating at lower EIRP because of advantageous propagation conditions would be allowed to operate in place where they would not normally be allowed if their maximume EIRP was considered.  Note that the list of available channels will require 8-bit times the number of available channels at the specified location whereas transmiting the maximum EIRP would simply required another 8-bit per available channel.  Another way to document it from the database service would be to transmit the maximum allowed EIRP in each channel which would require 8-bit times the total number of channels in a location which, sizewise, would not be much bigger.

The 802.22 WG is of the opinion that besides the TVBD location, antenna height, directivity and azimuth of the main antenna beam would be useful as input to the database service for an extention of the service. A response in terms of maximum allowed EIRP per TV channel rather than a simple list of available channels (noting that the latter can be easily deduced from the former) and time availability of the each channel would provide more useful information for TVBD operation..

Because of the point-to-multipoint nature of the 802.22 systems, batching requests to the database service should be allowed. The database service should also provide the ability to batch the responses for multiple devices.

When directly applying this model to the database service interface proposed in the FCC R&O 08-260, the initial assumptions made by the 802.22 working group were:

- This interface will have to take place entirely between the BS (e.g. master devices) and the database service

- This interface will not take place between BSs and their fixed, slave devices (know in 802.22 as CPEs)

The current network model that the 802.22 working group has defined is depicted in Figure 1. In this architecture, an entity called the Network Control and Management System (NCMS) is featured. The NCMS is an entity run by the WRAN service provider that provides applications and services to manage entities (e.g. BSs and CPEs) in the WRAN service provider network. The NCMS facilitates the management of various services, such as AAA, provisioning, radio resource management, QoS, routing, etc. In addition to these services, the NCMS will act as a proxy for database service queries that are to be made on behalf of BSs that are under its control.

This network model was adopted to address a few primary concerns with:

Cost: Obtaining certificates and maintaining those certificates over time will incur some cost to the WRAN service providers. Cost becomes an issue when considering the number of BSs that may (potentially) under the service provider’s purview. Use of the NCMS allows the WRAN service provider to maintain only one certificate/association with the database service and reduce costs. However, this requires the WRAN service provider to develop its’ own procedures, such as SSH or TLS with self-signed certificates, to secure the transmission of data received from the database to each of the BSs under its control.

Privacy: The database service request proxying that the NCMS will provide, also acts as an anonymizer for user data. Local, state, federal, and international regulations may require protection of personally identifiable information of WRAN’s customers. A NCMS is already in a position to provide for protection of user privacy as well as ensure accurate and timely information on the BSs and CPEs under its’ control, because the WRAN service providers will be required to provide these services natively as stipulated by existing laws and regulations.

Deployment Configurations: 802.22 BSs/CPEs will be deployed in rural areas and around the edge of suburban areas, where existing telecommunications infrastructure is sparse and may even be non-existent. Bringing in infrastructure to directly connect each 802.22 BS under the purview of a WRAN service provider to the internet in these areas will be a technical and logistical challenge. The NCMS in this case provides a central point-of-presence that allows the WRAN service provider to have flexibility in defining the backbone network that it uses to link BSs to the router(s) that will act as gateway(s) to the internet. This flexibility is needed to ensure that the WRAN service provider’s backbone network is redundant, and service interruption is minimized when outages occur.

Figure 1:  Structure of the 802.22 WRAN access to the database service

##### Database service access

First, the CPE has to provide the necessary information to the base station at the time of association otherwise such association will be rejected.  This information will include:

- Geolocation (latitude, longitude and height above average ground level)

- Device identification (Product identification (FCC ID) and the serial number (could be the MAC address)

- Type of device: fixed, portable mode 2 or portable mode 1

- Antenna directivity in terms of gain in the horizontal plane relative to the maximum gain in the main beam

The operator of the BS will initially register with the database service through the NCMS as a fixed device in providing all the information required as specified in the FCC R&O.  It will also register all its associated CPEs with their geographic location, device identification, etc. as obtained at association on a real time basis since its association may depend on the response from the database service.  One additional information will be added: the azimuth of the CPE antenna main beam which will be reckoned at the BS based on the geographic location of the BS and the CPE assuming that the CPE antenna will be aimend in the direction of the BS which is a safe assumption since the 802.22 CPE are to be professionally installed.

On an ongoing basis, the BS will then register with the database (at least once every 24 hours) and set up the dialog using the primitives defined above so that it can query the database service as well as having the database service send any update relevant to its operation through ‘push’ internet technology based on its location and the characteristics of the base station equipment used as well as the location and the characteristics of the associated CPEs. Such ‘push’technology would allow for a better reaction time than the 24 hours minimum access time currently specified while keeping the database traffic to a minimum.

To allow this ‘push’ technology, the connection from the database service to the BS will need to be kept alive.  In the point-to-multipoint topology assumed for 802.22, only the base station or even the proxy for a number of base stations (NCMS) from a same operator needs to be known to the database service to implement this ‘push’ technology.

Also, the BS should be allowed to query the database service with dummy CPE ID and location to verify the list of available channels and maximum allowed EIRP at potential CPE locations for planning of future coverage and verification of potential for interference before implementation.

Security for these messages:

802.22 is of the opinion that security on the messages exchanged between the Base Station and the database service will be critical for the proper operation of the systems to allow authentication of the database provider as well as the WRAN system querying the service.  Security will also be necessary to avoid the message exchange being altered on the backhaul connection.  SSL was identified as an acceptable way to provide transport layer security on the link  between the database service and the BS or the proxy NCMS.

802.22 recommends authentication based on Elliptic-Curve-Cryptography (ECC).  We note that the RSA is outdated while ECC provides more security with less overhead and faster processing.

### References:

[1]	P802.22/D2, Draft Standard for Wireless Regional Area Networks, “Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands”, May 2001

______________________________

Directionality will represent the antenna gain pattern in the horizontal plane in dB referred to the gain of its main lobe and it is assumed that the database service will use its knowledge of the geolocation of the base station and the device being registered to calculate the azimuth of the device antenna main lobe for interference calculations.

If the request is being done for a mode I personal/portable device, the Location Data String will correspond to the location of the mode II device to which it is associated.

If the request is being done for a mode I personal/portable device, the Location Data String will correspond to the location of the mode II device to which it is associated.

Note that in its Petition for Reconsideration, the IEEE 802 has addressed the fact that the current interface proposes a direct interaction between the Incumbent Database and the CPEs which goes contrary to the assumption that the BS and its operator will be responsible for the protection of the broadcast incumbents.  It is hoped that the FCC, in response to the Petition for