IEEE P802.22
Wireless RANs

| 802.22 Database Service Interface |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-09-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave. Ottawa, Ontario Canada K2H 8S2 | 613-998-2500 | gerald.chouinard@crc.ca |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90064 | 310-369-4367 | Winston.caldwell@fox.com |
| Ranga Reddy | US Army | Bldg 2700, Fort Monmouth, NJ 07703 | 732-532-0085 | Ranga.reddy@us.army.mil |

Abstract

This document summarizes the results of the 802.22 WG discussionson a proposed interface to the Incumbent Database consistent with the 802.22 Draft Standard.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Wendong Hu> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

### 802.22 Database Service Interface

802.22 system model

The system model that has been assumed all along in the development of the 802.22 Standard is a point-to-multipoint model for extending broadband access to less populated rural areas where more available TV channels can be found.  In this model, the base station (BS) is assumed to control all RF parameters of its associated customer premises equipment (CPE) (i.e., frequency, EIRP, modulation, etc.)  in a “master-slave” relationship so that the responsibility of protecting the TV broadcast incumbents is fully carried by the Wireless Regional Area Network (WRAN) operator.  When this model is applied to an interface to the database service proposed in the FCC R&O 08-260, the initial finding made by the 802.22 Working Group was that this interface is to take place entirely between the database service and the BS rather than with its individual CPEs.  The network model and interface to the database service that the 802.22 Working Group has developed is depicted in Figure 1.

Figure 1:  Structure of the 802.22 WRAN access to the database service

##### Enlistment and access primitives to the database service

The following list of primitives defines the necessary messaging to support access to the database service by the BS.  The format described below shall be used for the messages sent directly to the database service as well as those received directly from the database service.

M-DB-AVAILABLE.request: primitive that allows the BS to verify that it is connected to the database service in order to receive channel availability and maximum allowed EIRP updates.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Base station-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Access Type | Integer | 1 byte | The value identifies the type of access being used for communication to the database. 0x00 = URL 0x01 = IPv4 IP address 0x02 = IPv6 IP address 0x03-0xFF = Reserved |
| Database Service Network Address { |  |  |  |
| If (Access Type = 0x00) |  |  |  |
| Database Service URL | ASCII String | Variable | A fully qualified URL starting with, http:// or https:// |
| Else If (Access Type = 0x01) |  |  |  |
| Database Service IPv4 Address | Integer | 4 bytes | IPv4 network address for database service |
| Else if (Access Type = 0x02) |  |  |  |
| Database Service IPv6 Address | Integer | 16 bytes | IPv6 network address for database service |
| Else [Access Type = 0x03 – 0xFF] |  |  |  |
| Reserved mechanism | ASCII String and/or Integer | Variable | Reserved to allow identification of other access mechanisms |
| } |  |  |  |
| Base station Network Address { |  |  |  |
| If (Access Type = 0x00) |  |  |  |
| Database Service URL | ASCII String | Variable | A fully qualified URL starting with, http:// or https:// |
| Else If (Access Type = 0x01) |  |  |  |
| Database Service IPv4 Address | Integer | 4 bytes | IPv4 network address for database service |
| Else if (Access Type = 0x02) |  |  |  |
| Database Service IPv6 Address | Integer | 16 bytes | IPv6 network address for database service |
| Else [Access Type = 0x03 – 0xFF] |  |  |  |
| Reserved mechanism | ASCII String and/or Integer | Variable | Reserved to allow identification of other access mechanisms |
| } |  |  |  |
| Timestamp | ASCII String | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string. |

M-DB-AVAILABLE.confirm: primitive that allows the database service to confirm that the BS is still connected to the database service.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Base station-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| } |  |  |  |
| Timestamp | ASCII String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE.request. |

M-DEVICE-ENLISTMENT.request: primitive that allows the BS to enlist with the database service a device that has joined its WRAN network.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device obtained as part of its process to associate 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 1 0x03 = Personal/portable mode 2 0x04-0xFF = Reserved |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Location Data String | ASCII String | NMEA 0183 | The value identifies the location of the device (latitude, longitude). |
| Responsible Party Name { | ASCII String | Variable |  |
| If (Device Type = 0x00) |  |  |  |
| Contact Name | ASCII String | Variable |  |
| Contact Physical Address | ASCII String | Variable |  |
| Contact Email Address | ASCII String | Variable |  |
| Contact Telephone Number } { | ASCII String | Variable |  |
| If (Device Type = 0x00 or 0x01) |  |  |  |
| Antenna information |  | Variable | Antenna directionality information of the device (to facilitate the database calculation of the channel availability and the maximum allowed EIRP values at the registering location). |
| } |  |  |  |

M-DEVICE-ENLISTMENT.confirm: primitive that allows the database service to confirm to the BS that the new device has been successfully registered.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device being registered 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 1 0x03 = Personal/portable mode 2 0x04-0xFF = Reserved |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Location Data String | ASCII String | NMEA 0183 | The value identifies the the location of the device (latitude, longitude). |
| Responsible Party Name { | ASCII String | Variable |  |
| If (Device Type = 0x00) |  |  |  |
| Contact Name | ASCII String | Variable |  |
| Contact Physical Address | ASCII String | Variable |  |
| Contact Email Address | ASCII String | Variable |  |
| Contact Telephone Number } { | ASCII String | Variable |  |
| If (Device Type = 0x00 or 0x01) |  |  |  |
| Antenna information |  | Variable | Antenna directionality information of the device (to facilitate the database calculation of the channel availability and the maximum allowed EIRP values at the registering location). |
| } |  |  |  |

M-DB-AVAILABLE-CHANNEL.request: primitive that allows the BS to request a list of available channels and maximum allowed EIRP per channel from the database service for the specified type of device at the particular location.

| Name | Type |  | Length |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Device Type | Integer |  | 1 byte |  |  | The value identifies the type of device at the geo-location registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 1 0x03 = Personal/portable mode 2 0x04-0x255 = Reserved |
| Device-ID | ASCII String |  | Variable |  |  | In US, this is FCC-ID |
| Serial Number | ASCII String |  | Variable |  |  |  |
| Location Data String | Char |  | NMEA 0183 ASCII string |  |  | The value identifies the location of the device (latitude, longitude). |
| Timestamp |  | ASCII String |  | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string. |  |

M-DB-AVAILABLE-CHANNEL.confirm: primitive that is used by the database service to inform the BS whether its request for available channels and maximum allowed EIRP was successfully received by the database service.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device at the geo-location registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 1 0x03 = Personal/portable mode 2 0x04-255 = Reserved |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Location Data String | Char | NMEA 0183 ASCII string | The value identifies the location of the device (latitude, longitude ). |
| Timestamp | ASCII String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE-CHANNEL.request. |

M-DB-AVAILABLE-CHANNEL.indication: primitive that is used to return to the BS the list of available channels as provided by the database service in the form of channel, maximum allowed EIRP, and availability schedule.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device- ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial number | ASCII String | Variable |  |
| Number of Channels Available | Integer | 1 byte |  |
| { If( Number of Channels Available > 0) |  |  | If the number of channels is equal to 0, this means that the device cannot operate. |
| For (i=1 to Number of Channels Available, i++) { Channel_Number Max_Allowed_EIRP (dBm) Availability schedule } | List of available channels and max EIRP (dBm) allowed per channel, and the schedule | 2 bytes and standard UTS date and time format | List of Available Channel Numbers and corresponding maximum allowed EIRP (dBm) as well as the availability schedule (start and stop date/time) for each channel in Universal date and time system. |
| } |  |  |  |
| Timestamp | ASCII String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE-CHANNEL.request. |

M-DB-DELIST.request: primitive that allows the BS to request the database service to remove the enlistment of a device that was associated with that base station.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Responsible Party Name | ASCII String | Variable |  |
| Location Data String | Char | NMEA 0183 ASCII string | The value identifies the location of the device (latitude, longitude). |

M-DB-DELIST.confirm: primitive that is used to inform the BS whether its request to remove the enlistment of a device that was associated with that base station was successfully received and executed by the database service.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID | ASCII String | Variable | In US, this is FCC-ID |
| Serial Number | ASCII String | Variable |  |
| Responsible Party Name | ASCII String | Variable |  |
| Location Data String | Char | NMEA 0183 ASCII string | The value identifies the location of the device (latitude, longitude). |

##### Parameters being considered

The 802.22 WG is of the opinion that besides the TVBD location, antenna directivity would be useful as input to the database service.  This will allow for an extension of the capability of the database service to consider directional transmissions assuming that such devices are professionally installed such as in the case of the 802.22 devices.  The correct antenna azimuth would need to be calculated by the database service based on the location of the BS enlisting the device and the device location.

A response in terms of maximum allowed EIRP per TV channel rather than a simple list of available channels (noting that the latter can be easily deduced from the former), and time availability of each channel would provide more useful information for TVBD operation.  In the case where the BS controls the EIRP of its associated TVBD, devices operating at lower EIRP because of advantageous propagation conditions could operate in locations where they would not normally be allowed if only their maximum EIRP was considered.  The base station could also plan its channel usage based on the scheduled provided by the database service.

Figure 1:  Structure of the 802.22 WRAN access to the database service

##### Database service access

The BS will initially enlist with the database service as a fixed device.  It will also enlist all its associated CPEs with their geographic location, device identification, etc. as obtained at association on a real time basis since its association may depend on the response from the database service.

On an ongoing basis, the BS will then query the database (at least once every 24 hours) using the primitives defined above so that it can retrieve the channel information as well as having the database service send any update relevant to its operation through ‘push’ internet technologysince the network address of the base station isprovided as part of the primitives. Such ‘push’technology would allow for a better reaction time than the 24 hours minimum access time currently specified while keeping the database traffic to a minimum.

Security for these messages:

802.22 is of the opinion that security on the messages exchanged between the Base Station and the database service will be critical for the proper operation of the systems to allow authentication of the database provider as well as the WRAN system querying the service.  Security will also be necessary to avoid the message exchange being altered on the backhaul connection.  SSL was identified as an acceptable way to provide transport layer security on the link  between the database service and the BS .

802.22 recommends authentication based on Elliptic-Curve-Cryptography (ECC).  We note that the RSA is outdated while ECC provides more security with less overhead and faster processing.

### References:

[1]	P802.22/D2, Draft Standard for Wireless Regional Area Networks, “Part 22: Cognitive Wireless RAN Medium Access Control (MAC) and Physical Layer (PHY) specifications: Policies and procedures for operation in the TV Bands”, May 2001

______________________________

Note that this interface allows enlistment of TVBD devices (beyond 802.22 BS and CPE) that may not need to be formally  “registered” as required by the FCC R&O 08-260 for potential broader capability and applicability of the database service.

Antenna directionality will represent the antenna gain pattern in the horizontal plane in dB referred to the gain of its main lobe and it is assumed that the database service will use its knowledge of the geolocation of the base station and the device being registered to calculate the azimuth of the device antenna main lobe for interference calculations in the case of base station and CPE operation.  Omnidirectional antennas would be assumed for other device types.

If the request is being done for a mode I personal/portable device, the Location Data String will correspond to the location of the mode II device to which it is associated.

If the request is being done for a mode I personal/portable device, the Location Data String will correspond to the location of the mode II device to which it is associated.

If the request is being done for a mode I personal/portable device, the Location Data String will correspond to the location of the mode II device to which it is associated.

Note that in its Petition for Reconsideration, the IEEE 802 has addressed the fact that the current interface proposes a direct interaction between the Incumbent Database and the CPEs which goes contrary to the assumption that the BS and its operator will be responsible for the protection of the broadcast incumbents.  It is hoped that the FCC, in response to the Petition for Reconsideration, will consider this “master-slave” relationship that the 802.22 has assumed in its work and allow the BS to be the proxy of its associated CPEs. This the assumption used in this document.

For the purpose of this document, the base station is used as the contact point for the database service but, in reality, the exchange of information will likely take place with the Network Control and Management System (NCMS) which is the entity run by the WRAN service provider that will provide applications and services to manage the WRAN network (e.g., AAA, provisioning, radio resource management, QoS, routing, etc.)   In addition to these services, the NCMS will likely act as the proxy for the BS to access the database service.  The NCMS can also query the database service on behalf of a number of base stations that belong to the same operator.  This would allow simpler process for obtaining and maintaining certificates over time from the database service.  The WRAN operator would then have the responsibility to secure any communication between the NCMS and its base stations using SSH or TLS with self-signed certificates since he has the responsibility to avoid interference to TV incumbents from any device in his network.