IEEE P802.22
Wireless RANs

| Unification of sampling rates for the three TV bandwidths |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-06-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Robert S. Wu | Wilan | 11 Holland Avenue, Ottawa, Ontario, Canada K1Y 4S1 | 613-688-8959 | rwu@wilan.com |

Abstract

This contribution is to propose a unification of sampling rates for 6 MHz, 7 MHz and 8 MHz. We propose to use the parameters of 7 MHz as the base and to adjust the guard bands for 6 MHz and 8 MHz. 7 MHz has the merit that its base sampling rate is 8 MHz which results in rational parameters in both timing and frequencies. This unified sampling rate can simplify the radio frontend design for 3 bandwidths (one ADC/DAC for 3 bands); relax the phase noise/frequency offset limitation for 6 MHz radio; and simplify the hardware and software implementation for MAC and PHY.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

In 802.22/Draft Version 2.0, OFDM FFT size is fixed to 2048 for 3 potential channel Bandwidths, i.e. 6 MHz, 7 MHz and 8 MHz (refer to page 327 ~ 330). The following 4 tables summarize all the related OFDM parameters. We will copy and paste each original table then propose a new table to replace it.

- Table 224 --- Subcarrier spacing and FFT/IFFT period values for different bandwidth options based on sampling frequency equivalent to 8/7 channel bandwidth.

|  | 6 MHz based channels | 7 MHz based channels | 8 MHz based channels |
| --- | --- | --- | --- |
| Basic sampling frequency (MHz) | 6*8/7= 6.857143 | 7*8/7= 8 | 8*8/7= 9.144857 |
| Inter-carrier spacing, F (Hz) | (6x106*8/7)/2048 ≈ 3348.214 | (7x106*8/7)/2048 = 3906. 25 | (8x106*8/7)/2048 ≈ 4464.286 |
| FFT/IFFT period, TFFT (s)=1/F | ≈ 298.666… | = 256.000 | = 224.000 |
| Time Unit (ns) TU=TFFT/2048 | 1000/(6x106*8/7)= 145.833… | 1000/(7x106*8/7)= 125 | 1000/(8x106*8/7)= 109.375 |

- Remark: (a) For 6 MHz channel, the sampling clock is 48/7 MHz. Every 10 ms will result 68571 + 3/7 cycles.   (b) For 8 MHz channel, the sampling clock is 64/7 MHz. Every 10 ms will result 91428 + 4/7 cycles.

MAC Clock

PHY Clock

Start

Figure 1 Relative MAC and PHY clocks

Proposed New Table-224

- Table-224 — Subcarrier spacing and FFT/IFFT period values for different bandwidth are fixed basing on 8 MHz sampling frequency

|  | 6 MHz based channels | 7 MHz based channels | 8 MHz based channels |
| --- | --- | --- | --- |
| Basic sampling frequency (MHz) | 8 |  |  |
| Inter-carrier spacing, F (Hz) | (8x106)/2048 = 3906. 25 |  |  |
| FFT/IFFT period, TFFT (s)=1/F | 256 |  |  |
| Time Unit (ns) TU=TFFT/2048 | 1000/(8x106)= 125 |  |  |

The new table-224 uses 7 MHz bandwidth as the basis and keeps the fundamental OFDM parameters the same for the 6 MHz and 8 MHz. In this way, the OFDM parameters get unified for all three bandwidth options.

- Table 225 — Symbol duration for different cyclic prefixes and bandwidth options

|  |  | CP = TFFT/32 | CP = TFFT/16 | CP = TFFT/8 | CP = TFFT/4 |
| --- | --- | --- | --- | --- | --- |
| TSYM = TFFT + TCP (s) | 6 MHz | 308.000 | 317.333 | 336.000 | 373.333 |
|  | 7 MHz | 264.000 | 272.000 | 288.000 | 320.000 |
|  | 8 MHz | 231.000 | 238.000 | 252.000 | 280.000 |

As a consequence of the unification, symbol durations are unified as well for the 6 MHz, 7MHz and 8 MHz. The proposed new Table-225

- Table 225 — Symbol duration for different cyclic prefixes and bandwidth options

|  |  | CP = TFFT/32 | CP = TFFT/16 | CP = TFFT/8 | CP = TFFT/4 |
| --- | --- | --- | --- | --- | --- |
| TSYM = TFFT + TCP (s) | 6 MHz | 264 | 272 | 288 | 320 |
|  | 7 MHz |  |  |  |  |
|  | 8 MHz |  |  |  |  |

In order to deal with the spectrum mask requirements for different bandwidths, different guard bands will be applied to different bandwidth. 7 MHz will keep the original parameters while 6 MHz and 8 MHz need to be adjusted accordingly.

- Table-226— OFDM parameters for the 3 bandwidths of 2K FFT mandatory mode

| TV channel bandwidth (MHz) | 6 |  |  | 7 | 8 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Total no. of sub-carriers, NFFT |  | 2048 |  |  |  |  |
| No. of guard sub-carriers, NG (L, DC, R) |  | 368 (184, 1, 183) |  |  |  |  |
| No. of used sub-carriers, NT = ND+ NP |  | 1680 |  |  |  |  |
| No. of data sub-carriers, ND |  | 1440 |  |  |  |  |
| No. of pilot sub-carriers, NP |  | 240 |  |  |  |  |
| Signal bandwidth (MHz) |  | 5.625 | 6.566 |  |  | 7.504 |

For 6 MHz case, the transition bands at the edges of the channel to reach 55 dB rejections as requested in the FCC R&O 08-260 are 184.152026 kHz on each side.  This requires a 256-tap FIR filter before DAC (refer: 22-09-0023-02-0000-technical-complexity-related-to-meeting-the-rf-mask.ppt). In order to relax the frontend design, around 200 kHz guard band would seem to be necessary.

For 8 MHz situation, using 7 MHz parameters will waste 1 MHz spectrum therefore guard band can be reduced to allow more useful subcarriers.

The proposed new Table-226

- Table 226 — OFDM parameters for the 3 bandwidths of 2K FFT mandatory mode

| TV channel bandwidth (MHz) | 6 |  |  | 7 |  | 8 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Total no. of sub-carriers, NFFT |  | 2048 | 2048 |  |  |  | 2048 |
| No. of guard sub-carriers, NG (L, DC, R) |  | 648 (324,1,323) | 368 (184,1,183) |  |  |  | 144 (72, 1, 71) |
| No. of used sub-carriers, NT = ND+ NP |  | 1400 (= 50 sub ch) | 1680 (= 60 sub ch) |  |  |  | 1904 (= 68 sub ch) |
| No. of data sub-carriers, ND |  | 1200 | 1440 |  |  |  | 1636 |
| No. of pilot sub-carriers, NP |  | 200 | 240 |  |  |  | 272 |
| Signal bandwidth (MHz) |  | 5.46875 |  |  | 6.5625 |  | 7.4375 |

There maybe other options to increase one or two subchannels for 6 MHz and 8 MHz by reducing the guard bands. Here is the analysis.

Guard band width at edges of channels as a function of number of  subchannels

| Number of sub-channels | 6 MHz Guard band | 7 MHz Guard band | 8 MHz Guard band |
| --- | --- | --- | --- |
| 50 | 261.72 |  |  |
| 51 | 207.03 |  |  |
| 52 | 152.34 |  |  |
| 60 |  | 214.84 |  |
| 61 |  | 160.16 |  |
| 68 |  |  | 277.34 |
| 69 |  |  | 222.66 |
| 70 |  |  | 167.97 |

Therefore, alternative choices for table-226 are 51 and 69 subchannels, respectively, for 6 MHz and 8 MHz TV channels.  However, the 69 sub-channels in the 8 MHz case may need to modify the mapping of the data packets in the MAC section since only 6 bits were used for the subchannl mapping.  An additional bit maybe needed.

- Table 228 — WRAN frame parameters

| Cyclic Prefix | Number of symbols per frame 1 | Transmit-receive turnaround gap 2 (TTG) | Receive-transmit turnaround gap 3 (RTG) |
| --- | --- | --- | --- |
| BW | 6 MHz 7 MHz 8 MHz | 6 MHz 7 MHz 8 MHz | 6 MHz 7 MHz 8 MHz |
| 1/4 | 26 30 34 | 210 μs | 83.33 μs 190 μs 270 μs |
| 1/8 | 28 33 38 | 210 μs | 307 μs 174 μs 158 μs |
| 1/16 | 30 35 40 | 210 μs | 158 μs 174 μs 186 μs |
| 1/32 | 31 36 41 | 210 μs | 111 μs 174 μs 221 μs |

The proposed new Table-228

- Table 228 — WRAN frame parameters

| Cyclic Prefix | Number of symbols per frame (i.e. 10 ms) | Transmit-receive turnaround gap 2 (TTG) | Receive-transmit turnaround gap 3 (RTG) |
| --- | --- | --- | --- |
| BW | 6 MHz 7 MHz 8 MHz | 6 MHz 7 MHz 8 MHz | 6 MHz 7 MHz 8 MHz |
| 1/4 | 30 (Tsym = 320 s) | 210 μs | 190 μs |
| 1/8 | 33 (Tsym = 288 s) | 210 μs | 222 μs |
| 1/16 | 35 (Tsym = 272 s) | 210 μs | 174 μs |
| 1/32 | 36 (Tsym = 264 s) | 210 μs | 174 μs |

Note:  A TTG of 210 μs is set aside for all values of CP to allow the PHY layer to absorb the propagation time on the forward and return paths for up to 30 km distances between the BS and its CPEs, assuming a 10 μs turn-around time at the CPE.  The resulting RTG’s are amply sufficient for the turn-around time at the BS.]

The unification of the sampling rate will result in different numbers of data carriers and pilot carriers depending on the channel bandwidth.  Also, the corresponding subcarrier interleaving scheme parameters, preamble parameters and CBP parameters may need to be reviewed accordingly. Fortunately, the numbers of sub-channels are still integers therefore the change is minimal but the benefit for the system performance and implementation simplification would be significant. Particularly this unification will ease the implementation in silicon and worldwide deployment.

References:

[1] IEEE 802.22/Draft 2.0

[2] 22-06-0264-07-0000-ofdma-parameters.xls