IEEE P802.22
Wireless RANs

| Meeting Minutes of the Security Ad-Hoc Group in 802.22 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | P.O. Box 868, MER 15-2350, Nashua, NH 03061-0868 | 603-885-2621 404-819-0314 | apurva.mody@baesystems.com, apurva_mody@yahoo.com |
| Ranga Reddy | US Army (CERDEC) | Ft Monmouth, NJ | - | Ranga.reddy@us.army.mil |
| Tom Kiernan | US Army (CERDEC) | Ft Monmouth, NJ | - | Thomas.kiernan@us.army.mil |

Abstract

This document provides the minutes of the Security Ad-Hoc conference calls held on,

Year 2009 – August 3rd, August 10th, August 17th, August 24th, August 31st, Sept. 14th, October 5th , October 12th, October 26th, November 2nd, November 9th , November 30th 2009

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

1. August 3rd 2009 – Security Ad-Hoc Conference Call Meeting Minutes

Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Gerald Chouinard - CRC

Ranga Reddy – US Army

Winston Caldwell – Fox

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Comments 667, 668, 673, 674, 675, 676, 679 and 680 were resolved. The proposed resolutions can be found in the Document https://mentor.ieee.org/802.22/dcn/09/22-09-0120-03-0000-wran-draft-2-0-ballot-comment-database.xls

- Comment #662 – Gerald Chouinard. There were some discussions on this comment. Gerald’ s comment is to keep the complexity of the security suites in 802.22 low so that there is not a high barrier to the market

- Ranga Reddy suggested that when the security ad-hoc started exploring the architecture for the security in 802.22, we found the 802.16 model most appropriate. Many things from the 802.16 security architecture have been streamlined. There is also an option, that if a user does not have a capability to perform security related functions such as encryption / authentication, it may tell the BS and the BS may either choose to serve the user with no security features or it may decide to de-authorize the user from the network.

- Apurva Mody suggested that the security ad-hoc will consider providing some 802.11 type of mechanisms so that it may facilitate quicker adoption of the standard.

- Ranga Reddy suggested that security ad-hoc is already considering adding Extensivle Authentication Protocol (EAP) which will allow the service providers to automatically generate the certificates and keys.

- Ranga Reddy suggested that we wait until all the comments are resolved and then re-visti this comment to see if something can be done to reduce the complexity.

- Winston – We need to keep in mind the market where 802.22 is likely to be deployed more – not Africa but the US

- Apurva Mody suggested that looking at the trends in 802.16 / WiMAX, it looked like 802.22 may be adopted more quickly in other countries with lesser broadband penetration as compared to the US.

2. August 10th 2009 – Security Ad-Hoc Conference Call Meeting Minutes

Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Gerald Chouinard - CRC

Ranga Reddy – US Army

Winston Caldwell – Fox

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Discussions on security aspects with the external Whitespace database was carried out.

- Ranga Reddy brought to the attention of the group that the Whitespaces Databases Group has started talking about the security issues of the database with the Whitespace Device. This document includes Public Key Infrastructure (PKI), Transport Security and Device Security.

- PKI - For PKI, X.509 based use of certificates is suggested for key exchange. These X.509 certificates will hold an entity name in the SubjectAltName field, following the convention of the World Wide Web. A 2048-bit RSA key for initial deployment is suggested with forward compatibility for key size increases. Use key rollover procedures based on pre-published supersession to ensure long-term stability of the security architecture is proposed. The OSCP protocol (RFC2560) may be used by WSDs to verify that WSSP certificates have not been revoked by the Repository.

- Transport Security – Transport Layer Security or TLS (RFC4346, though it is sometimes better known by its old name, Secure Sockets Layer or SSL) is suggested. TLS provides authentication, integrity, and confidentiality properties at the transport layer below HTTP.

- Device Security – It is proposed that the device security be carried out using enrollment process with mutual authentication.

- Some discussions based on this proposed document were as follows:

- Winston – Database will be designed such that it will be able to shut down a particular device with a particular FCC ID

- Gerald – complexity needs to be taken into account

- Gerald – Path for 802.22 is clear since we go through a proxy server sucha as NCMS.

- Winston – Take a step back from complexity, implement it and then worry about 802.22 CPEs acting as the Whitespace access points for other Whitespaces network.

- Winston – we are inserting NCMS, Whitespaces Database group is inserting White Space Service Provider.

- Apurva Mody – Based on the proposed document it appears as if the Whitespace database group is thinking about an 802.11 type of a model which is not based on an Internet Service Provider concept. It is proposed that the White Space Device (WSD) will be able to access the Whitespace Service Provider / Repository directly.

- Apurva Mody – suggested that for an 802.11 type of a device, accessing the repository is relatively easy, since the user can easily access the 802.11 MIBs using internet browser application and configure the 802.11 hardware (provide it with the URL) such that the device will transparently access the repository and validate the channels.

- Apurva Mody – 802.22 model is not that simple. We chose to follow the Internet Service Provider route where typically, and ISP is likely to maintain an external server such as the NCMS to manage the network.

- Towards the end of the call, the ad-hoc tried to resolve some comments. However, due to lack of time, not much progress was made on comment resolution.

3. August 17th 2009 – Security Ad-Hoc Conference Call Meeting Minutes

Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Gerald Chouinard - CRC

Ranga Reddy – US Army

Winston Caldwell – Fox

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Ranga Reddy talked about some of the privacy concerns in current 802.22 security architecture - Multi-cast management messages are protected, however, multicast transport security is complex and it is left upto the service provider / higher layer to provide IPSec.

- Document 114 was reviewed.

- Ranga presented two approaches – pros and cons of both were discussed

- Gerald brought to the attention the fact that FCC requires MAC Address of the device to be sent in the clear.

- Gerald – The trick we found to reduce the overhead was that instead of sending MAC address in every US burst, 802.22 transmits it in the CBP burst every 2 minutes.

- Ranga Reddy - The main issue is the concern about privacy – malicious user should not be able to link MAC address with the CID.

- Winston Caldwell – FCC Report and Order Section 15.711 Part E – Fixed TVBD shall transmit identifying information. The ID information must confirm to a standard. ID should contain sufficient information to identify the device and its geolocation information.

- Apurva Mody - FCC says that the identification and geolocation information needs to be transmitted but does not say that it is in the clear – that is un-encrypted. MAC address may or may not qualify as the identifying information.

- Apurva Mody – Based on document 09-0114 it looks like Approach 1 is more suitable since the MAC address needs to be protected and the FCC does not require that the MAC address is required to be in the clear.

- Apurva Mody - Decision needs to be made in the coming weeks on whether Approach 1 or 2 should be adopted since the Privacy concerns are valid.

- Gerlad Chouinard – prefers Approach 2.

- Apurva Mody – prefers an approach that does not require major modifications to the standard and still overcomes the concerns.

- Winston Caldwell – Wants to provide utmost security that 802.22 can – leave complexity aside.

4. August 24th 2009 – Security Ad-Hoc Conference Call Meeting Minutes

Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Ranga Reddy – US Army

Winston Caldwell – Fox

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Comments 689, 693, 694, 695, 696 and 697 were discussed.

- Ranga Reddy  - is proposing to drop RSA – since supporting a certificate based methods are onerous, especially for small service providers

- Ranga Reddy – RSA is an older technology. Huge overhead since they are not strong and long key sizes are required. Going forward, longer and longer keys will be needed.

- Ranga Reddy – ECC is still present for CPE authentication. You can also use EAP. You can also do both. In case of both, you would first do ECC based authentication. Then EAP exchange happens. Information is used from both ECC and EAP for key generation.

- Ranga Reddy - Section 7.2.2.5 needs to be updated. Ranga will provide update to the second paragraph in this section with regards to how the cryptographic suits are selected. Relation between primary, secondary and GSA will be explained.

5. August 31st 2009 – Security Ad-Hoc Conference Call Meeting Minutes

Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Ranga Reddy – US Army

Winston Caldwell – Fox

Gerald Chouinard - CRC

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Comment 663, 664, 665, 666, 669, 670, 671, 672, 687, 688 were addressed. The proposed resolutions can be found in the updated versions of the comment database.

- Whitespaces database group has suggested that at the minimum, they require – AES – 128, SHA – 1, RSA.

- IEEE 802.22 currently supports AES 128 and RSA.

- Apurva - However there are currently two authentication / authorization mechanisms in 802.22. One is based on RSA and the other is based on Elliptic Curve Cryptography (ECC) based certificates.

- Ranga – RSA is outdated and can be easily replaced by ECC based certificates. However, if FCC ruling requires that all the Whitespace devices directly access the database, then RSA will have to be supported.

- ECC on the other hand is more powerful and is included in the IEEE 802.22.1 standard.

- IEEE 802.22 currently does not support SHA.

6. September 14th - 2009 – Security Ad-Hoc Conference Call Meeting Minutes

Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Ranga Reddy – US Army

Ivan Reede - Amerisys

Gerald Chouinard – CRC

Winton Caldwell - Fox

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Apurva Mody asked Ivan to briefly explain his comments and concerns related to Security in 802.22

- Ivan said that it was not possible to explain things in a few sentences.

- Ranga and Apurva said that they have read through Ivan’ s comments and they can explain what they have read. Ivan can respond if that is what he meant.

- Ranga – One of the concerns of Ivan is that he would like to have BS – CPE, CPE – BS mutual authentication.

- Ivan said that yes, this is a Major concern – if it was not bilateral then someone could spoof the BS and get the subscriber keys out of the BS

- Ranga suggested that mutual authentication is already part of 802.22 security since .22 borrows parts of PKMv1 and parts of PKMv2 from 802.16 where this issue has been taken care of.

- Ranga – Certificate based on EAP mechanism is of concern to Ivan.

- Database group is not defining if they need authentication down to the device. R&O has not come out

- Ivan – business decision concern – We are going to compete with 802.11, DOCSIS, 802.16. Some product allows you to enter your own keys. Does not prevent public / private keys

- Ivan – Operator will shun upon the use of a third party for certificate generation

- Ranga – agrees – previous language suggested manufacturer installed certificates – This is being changed to pre-installed – 802.22 will allow the operator to install its own certificates

- Ranga – we will make this even more clearer through the Draft v 2.0 comments.

- Ivan – Database group is planning to support SSH

- Ranga – SSH does not allow authentication

- Ranga – We have a comment to have EAP into the standard.

- Ivan – Problem with him requiring to pay someone on a continual basis for certificate generation and maintanence. This is cost prohibitive.

- Ranga – the current security architecture does not require a third party to generate certificates. This comment is well taken and things will be made abundantly clear.

- Ranga – Next major issue as we understand – Encryption. Ivan believes that 802.22 is providing too many complex mechanisms for encryption.

- Ranga and Apurva – This is not true - 802.22 is providing only one mechanism – Advanced Encryption Standard (AES) for encryption as well as integrity check – AES – GCM – will be used since it is a little faster. 802.11 also uses AES. All other encryption modes such as DES, AES-CCM etc. have been taken out.

- Ivan’ s other concern is that management messages are not protected.

- Ranga - All management messaes except for initial ranging, broadcast, are protected

- Authrization – Ivan feels that the complexity of the state machine needs to be controlled. We do not agree with that entirely. In order to make sure that every one implements the standard in the same way, we need to describe the key management process well.

- Apurva – We will take this comment into account and try to reduce the complexity in key management.

- Ranga – Security capabilities – We are essentially using only one protocol – AES – GCM mode – can be used in authentication only, encryption only or both. BS needs to select that when he selects the SA.

- Ranga – Section 7.5 defines the format of the certificate. Ivan has comments on that. Ranga – if you do not want to use certificates. Does this need to be manadatory in every BS and CPEs?

- Ranga – Certificates need to follow X.509 format – if you don’ want to use certificates, we can use EAP

- Ivan – Why don’ t we do that – no certificates

- Ranga – If 802.22 device needs to access the database and requires certificates then we need this format.

- Ivan – Why do we need to wait till new rules come out.

- Apurva – According to what he has heard the new rules could come out within next two to three months.

- Ranga – We will work on these issues and plan to bring back EAP and not remove RSA.

- Ranga – Next major issue is in Section 7.6 – Collaborative sensing

- Ivan – Are there any other mechanisms to avoid DoS

- Ivan – does not see sensing working since sensing at the levels required by the FCC automatically fails in presence of the computer since a local computer can generate much more noise.

- Ivan – recollects why we started the silent period –

- Ivan – Sensing does not work even when our own transmitter is in the vicinity.

- Ivan feels collaborative sensing should be removed if sensing is removed entirely. Othewise collaborative sensing is adding a great value.

- Geral Chouinard – suggested that this is a systems issue and should be discussed in the systems group meetings.

- Ivan – make sensing as a separate section and include collaborative sensing. Let sensing be a separate mass produced device that uses collaborative sensing.

- Ranga – TG1 beacon authentication – Ivan – is saying that we built the beacon but microphone folks themselves are not backing it and FCC has decided not to include it.

- Ivan – just like sensing this should be a different clause – separate devices that may be supported by some other countries.

- Ivan - in the view of the current FCC R&O beacon should be made an optinal feature of the standard.

- Winston – Don’ t stick literally to the R&O. If it makes a good engineering decision then we should use it

- Apurva – Is 802.22.1 a whitespace device – Winston, Gerald – No it is not.

- Apurva – If 802.22.1 is not a Whitespace device then it does not fall into FCC R&O and people can use it in the US also.

- Ranga – Last topic is the CBP authentication – Ivan is suggesting to remove it. If you have your own network – you can distribute the certificates and co-exist

- Ivan – Is it worth the added to cost to introduce CBP authentication. In the stage of technology at this point whether we should not be madating some plug in language. They can literllay be downloaded in the CPEs.

- Apurva – He recently saw a presentation from a local University professor who told the audience that CBP in 802.22 is extremely vulnerable and it has no protection. Apurva corrected him. The Professor wanted to add even the encryption on the CBP. 802.22 only has authentication so far.

- Ranga – we are defining the the capabilities that may or may not be used.

- Ivan – standards inter-operability issue. They need to define these features in the standard.

- Ranga – these issues need to be handled outside the standard –

- Ivan – does not agree – we need to define inter-operability.

- Overall, this meeting was very productive and we ended up agreeing and dis-agreeing on many issues.

- Gerald asked Apurva to take out some systems related issues which need further discussions in the Hawaii interim meeting.

7. October 5th - 2009 – Security Ad-Hoc Conference Call Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Ranga Reddy – US Army

Ivan Reede - Amerisys

Gerald Chouinard – CRC

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Ranga discussed his preliminary approach in designing of MIBs for the 802.22 standard. The approach consists of MIB Description, Management Primitives, ASN.1 Formatting of the MIB.

- Some of the main issues discussed were as follows:

- We may leverage 802.16 and/or 802.11 MIBs, we should provide for the proper attribution to the folks in those working groups.

- Do we need a flexible MIB for PHY MIB objects? .16/.11 PHY MIBs have to support MIB objects for multiple PHYs. In those standards there are some PHY MIB objects that are “common” to all PHYs, and a group of PHY MIB objects specific to each PHY. At this time, in 802.22 we do not have this problem, but we should try to avoid it.

- Do we need a flexible MIB for PHY MAC objects? In .16 there is support for relay stations, will have support for newer platforms like femtocell BS. In future 802.22 projects we decide to develop similar capabilities. So do we need a MIB object for MAC items that are common to all stations, then specific MAC MIB objects for stations?

- Should MIB objects for SM, SSA, and the Database Service be relegated to their own main MIB objects or be rolled into BS / CPE? – There was a lot of discussion on this issue. The fundamental question was as to where the intelligence in the network reside. Apurva Mody suggested that the essential intelligence for the 802.22 systems should reside within the BS, and the network operator is provide some controls to choose the channels from those that are available. Ivan Reede compared this to an operation of  a vehicle where the operator needs at least enough information to make informed decisions.

- Apurva Mody suggested that there were two issues in bringing the intelligence outside the 802.22 BS. 1. The 802.22 systems need to make decisions in a timely fashion and this may become difficult 2. We will need interfaces that can withstand information barrage if the intelligence was to be brought outside. For example, the SNMP interfaces break down when there is too much information coming in and out since they are not designed for so much network traffic.

- Most people on the call agreed that the essential decision making would take place within the BS, however essential hooks for monitoring and control will be provided.

- Roaming and Mobility - If we leverage .16/.11 MIBs, should we leave out MIBs related to roaming/mobility? The group discussed this and everyone agreed that we should leave out the MIBs related to roaming and mobility.

- Ranga Reddy discussed his thoughts on the MIB objects and their nomenclature. His essential idea was to create following classes – wranDevMib, wranIfBsMib, wranIfCpeMib, wranIfSmMib, wranIfSsaMib and wranIfDatabaseServiceMib.

- Ivan Reede suggested that a Wireless Microphone trap needs to be provided for the wranDevMib.

- Gerald Chuinard suggested that the Spectrum Automaton – does not need lot of MIB objects since the plan was to control the SSA from the BS and not from the user interface.

- There were some other discussions on issues going forward which included how the information contained in Table 238 and Annex B needs to be used to formulate the MIB objects.

8. October 12th - 2009 – Security Ad-Hoc Conference Call Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Ranga Reddy – US Army

Gerald Chouinard – CRC

Winston Caldwell - Fox

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Comments 698, 699, 700, 701, 704, 705, 706, 707, 708, 709 were discussed.

- Ranga Reddy - The difference between 802.16 and 802.22 is that 802.22 always has to provide protection to management messages such as spectrum sensing messages.  Need to provide reference to the fact that no state machine should start if the user traffic is not protected.  The user terminal has to have the state machine set for authorization. If there is no protection provided for the data, the management messages still need to be protected.

- Ranga Reddy - Even if the CPE cannot support any cryptographic suite for data (the user traffic), it will still need to authenticate and protect management messages.

- Ranga Reddy - Management messages should be protected for everybody through AES-128 but the user data may or may not be protected.

- Apurva Mody - Per Ivan's comment, he wants all the management information to be protected.  As a result, the user terminal needs to support at least one cryptographic suite namely AES-GCM.  Note that 802.11 also uses AES-CCMP so this should not be a problem.  In 802.11, there is no distinction between management and data plane.  This would require all CPEs to initiate and run the authorization state machine but they may chose not to use it for data.  This machine will generate the encryption key and authorization key which will allow the management messages to be encrypted.

- Ranga Reddy - EC bit set to 0 means that the payload is not encrypted but the CPE needs to be authorized at associating. The management messages map to the null SA.

- Ranga Reddy - Primary SA: Any traffic that will use protection

- Ranga Reddy - Secondary SA: would use encryption only.  It cannot use the Primary SA

- Ranga Reddy and Apurva Mody - Action: When referring to cryptogaphic suites, any reference to "no-authorization" should be changed for "no protection".

- Ranga Reddy - Action: Across all the SAs, the EC bit set to 0 means that the PDU is not encrypted whereas if the EC bit is set to 1, then the PDU is encrypted.  For the management messages (null SA), the EC bit will always be set to 1.

9. October 26th - 2009 – Security Ad-Hoc Conference Call Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Ranga Reddy – US Army

Gerald Chouinard – CRC

Winston Caldwell - Fox

2.1 Agenda

- Attendance

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Comments 710, 712, 713, 714, 715 were resolved.

- There was discussion on how to protect the privacy of the CPE.

- Ranga Reddy described his document 22-09-0114 Rev 0 where he has laid out some privacy concerns for the CPEs and two methods of protecting the privacy.

- In general, most people prefer Method 1, which utilizes the use of a dummy MAC address which is transmitted in the clear to register, and the true MAC address is used only after the CPE is registered.

- General Discussion - Part 15-711, FCC R&O requests that identification of the transmitting device and geolocation be known to everyone.

- Gerald Chouinard – What needs to be kept secret is the information between the Basic CID and the device ID.

- Gerald Chouinard suggested that perhaps we should send out what FCC is asking for in the CBP bursts which are in the clear periodically. This way, it becomes possible to use Method 1.

- CBP will be used to periodically transmit the device ID and the geolocation in the clear

- There were some discussions on the MMP Key

- Winston – May need another bit – Continue, Abort, Success, Re-range,

- Winston – Another IE may be required in the RNG, RSP.

- Gerald – If network entry is done, but there is mis-synchronization between the BS and the CPE authorization keys, then the CPE should not be forced into network re-entry.

- Add a third bit to the ranging status field of the RNG RSP message at bottom of Page 64. BS telling the CPEs that you are out of synch with your key and RE-AUTHORIZATION is required. If this happens then the CPE needs to re-start its authorization process but there is no need to go through the network re-entry and no need for re-registration.  If RNG-RSP requires CPE re-authorization, then the CPE authorization state machine will go back to the START State. Ranga needs to add this sentence to the appropriate section on Page 288.

- Winston Caldwell suggested that he would definitely want to sign and encrypt the management messages.

- Gerald - Ranga to search Section 7, remove reference to optional encryption of management messages. Also remove any text on any negotiation on the level of protection.  This is because the group feels that the management messages (except for basic CID, init ranging and broadcast.) always require encryption and authentication.

- Some discussion on Comments 731-734 - AES GCM allows flexibility for managing encryption overdead. 802.1 and 802.11 are adopting it. It is also faster.\

- The resolution for Comments 731-734 was tabled until the next teleconference call.

10. November 2nd - 2009 – Security Ad-Hoc Conference Call Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Ranga Reddy – US Army

Gerald Chouinard – CRC

Winston Caldwell - Fox

2.1 Agenda

- Attendance was taken

- Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Minutes  of  the Security Ad-Hoc Conference Calls  were approved  – The meeting minutes can be found at https://mentor.ieee.org/802.22/dcn/09/22-09-0140-08-0000-meeting-minutes-of-the-security-and-management-plane-procedures-ad-hocs.doc

- Minutes  of  all the Security Ad-Hoc Conference Calls  previously held can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Comments 716, 717, 718, 719, 722, 723, 724, 725, 726, 729, 730, 731, 732, 733, 734, 739, 740, 751, 752, 674 were discussed and resolved.

- Some of the discussions that took place during the conference call was as follows:

- Comment 716: Statement here says that there are different MMP_keys for DS and US. However, there is no reference to this difference in Section 7.2.4.6.1 and Figure 135, and in MMP_Key derivation described in 7.2.4.6.2 (pg 289, line 11-12). Resolution: Counter - Ranga needs to go in and find out all the instances where different DS and US MMP keys are supposed to be used and change it such that it reflects that only one MMP key will be used for both the DS and the US. The Security ad-hoc decided that in order to keep the system complexity low, only one derived MMP key will be used for both DS and US.

- Comment 717: second sentence that starts on line 6, refers to a MMP_key for broadcast. Broadcast traffic (traffic on broadcast CID) is not to be protected. This part of the paragraph (or section 7.2.4.6 for that matter) also doesn't describe how multicast management messages are protected. Resoulution: Defer – Security ad-hoc to check if this comment has already been addressed through some other comment. If not, then Security ad-hoc will discuss it during a meeting or a telecon.

- Comments 718 and 719 were superceded

- Comment 722: KEK and MMP_Key/MMP_PN should not have the scope of a particular SA. Their scope is that of CPE itself. This is because these constructs are used to protect management messages and deliver new TEKs for SAs. All this traffic happens on management connections, which are not mapped to a particular SA (e.g. they're mapped to the Null SAID). For GSA this is different, GKEK is only within the scope of a particular GSA. Resolution: Accept - Remove reference to KEK and MMP_Key from the definition of the contents of an SA, make sure these items are mapped to the Null SAID.

- Comment 723 – Superceded

- Comment 725 - Section 7.3 doesn't provide description of how KEKs and GKEKs are to be used by the CPE and BS – Resolution: Defer – Security ad-hoc will check to see where this has been described and if it is taken care of.

- Comment 726 – Superceded

- Comment 729 - There isn't an "encryption only" cryptographic suite. "Encryption only" avoids the overhead of an ciphertext ICV (e.g. it's good for protection of traffic for service flows carrying voice traffic) Resolution – Accept in Principle – Security ad-hoc will make appropriate changes in Section 7.4

- Comment 730 – Superceded

- Comment 731 - Consider adoption of AES-GCM for PDU authentication/encryption. GCM mode has a few advantages: i) faster processing of PDUs, ii) encryption process is not data-length dependent, iii) increasing support in other standards (802.11s 802.1ae) Resolution: Accept - 802.11 is moving to it.  It allows flexibility in managin encryption overhead.  It is faster.  There is more implementation of CCM in practice.  802.1, 802.2af. No hardware exists to support this. Security ad-hoc will provide the appropriate changes in Section 7.4 to the editor.

- Comment 732 – Superceded

- Comment 733 - Even if we move the GCM, the AES-based encryption, there is the overhead of the ciphertext ICV. This incurs large overhead for small PDUs (signalling headers, VoIP, etc.). Need to develop methods to reduce overhead Resolution: Accept - See the proposed resolution for Comment 729 where encryption only mode is introduced to reduce the overhead.

- Comment 734 – Superceded

- Comment 739 - Change reference of CCM on this line to GCM. Resolution: Accept

- Comment 740 – Superceded

- Comment 751 - CMAC is defined nowhere else in standard. In fact suggestion is to go to AES GCM, which has an authentication-only mode GMAC. Resolution: Accept – Security ad-hoc to bring appropriate resolution to the group. Propose appropriate changes to the rest of the document so that the use of GMAC can be reflected. Resolution: Accept – Security ad-hoc to bring appropriate resolution to the group. Propose appropriate changes to the rest of the document so that the use of GMAC can be reflected.

- Comment 674 - 802.22 shouldn't mandate factory-installed private/public key pairs. Operators should be able to self-sign their own certificates and install them for CPE authorization. There was a lot of discussion on this issue. Resolution: Defer - Open this discussion to the systems group at the Nov. 2009 Plenary along with the use of RSA.

11. November 9th - 2009 – Security Ad-Hoc Conference Call Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Ranga Reddy – US Army

Winston Caldwell - Fox

2.1 Agenda

- Attendance was taken

- IEEE Patent Policy - Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Approval of Meeting Minutes: Minutes  of  the Security Ad-Hoc Conference Calls  were approved  – The meeting minutes can be found at https://mentor.ieee.org/802.22/dcn/09/22-09-0140-09-0000-meeting-minutes-of-the-security-and-management-plane-procedures-ad-hocs.doc

- Reference: Minutes of  the Security Ad-Hoc Conference Calls previously held were approved. The meeting mintues can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- MIB and Management Primitives Outline: DiscusMIB and Management Primitives Outline - https://mentor.ieee.org/802.22/dcn/09/22-09-0201-00-0000-mibs-and-management-primitives-outline.doc

- MIB and Management Primitives Discussions: MIB and Management Primitives Description - https://mentor.ieee.org/802.22/dcn/09/22-09-0202-00-0000-mibs-and-management-primitives-description.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Call was cancelled due to lack of sufficient participation.

11. November 30th - 2009 – Security Ad-Hoc Conference Call Meeting Minutes

1. Attendance

Apurva Mody – BAE Systems

Ranga Reddy – US Army

Gerald Chouinard – CRC

Ivan Reede – Amerisys

2.1 Agenda

- Attendance was taken

- IEEE Patent Policy - Chair asked if everyone attending was familiar with the IEEE patent policy - http://standards.ieee.org/board/pat/pat-slideset.pdf - Everyone was familiar.

- Approval of Meeting Minutes: Minutes  of  the Security Ad-Hoc Conference Calls  were approved  – The meeting minutes can be found at https://mentor.ieee.org/802.22/dcn/09/22-09-0140-09-0000-meeting-minutes-of-the-security-and-management-plane-procedures-ad-hocs.doc

- Reference: Minutes of the Security Ad-Hoc Conference Calls previously held were approved. The meeting mintues can be found at  –  https://mentor.ieee.org/802.22/dcn/08/22-08-0163-22-0000-meeting-minutes-of-the-security-ad-hoc-group-in-802-22.doc

- MIB and Management Primitives Outline: DiscusMIB and Management Primitives Outline - https://mentor.ieee.org/802.22/dcn/09/22-09-0201-00-0000-mibs-and-management-primitives-outline.doc

- MIB and Management Primitives Discussions: MIB and Management Primitives Description - https://mentor.ieee.org/802.22/dcn/09/22-09-0202-00-0000-mibs-and-management-primitives-description.doc

- Discussion on Protection Mechanisms for Co-existence Beaconing Packets – led by Ranga Reddy. https://mentor.ieee.org/802.22/dcn/08/22-08-0296-03-0000-cbp-security-protection-mechanisms.doc

- Recommended Text for Section 7 on Security in 802.22 – included in Draft v 2.0 can be found at https://mentor.ieee.org/802.22/dcn/08/22-08-0174-18-0000-recommended-text-for-section-7-on-security-in-802-22.doc

- Reference Document - Table of Contents for Section 7. The reference document for the proposed text can be found at   
https://mentor.ieee.org/802.22/file/08/22-08-0165-00-0000-table-of-content-for-the-security-section-in-802-22.doc

- Reference Document - Scope and the Workplan for the Security Ad-Hoc Group –https://mentor.ieee.org/802.22/file/08/22-08-0159-00-0000-scope-agenda-workplan-and-timeline-for-the-security-ad-hoc-in-802-22.doc

- Reference Document - Presentation on the PRM and Security Enhancements in 802.22 – 802.22 Threat Analysis
https://mentor.ieee.org/802.22/dcn/08/22-08-0083-08-0000-security-and-prm-enhancements-in-80222-v3.ppt

- Proposed Reference Architecture for 802.22 - https://mentor.ieee.org/802.22/dcn/08/22-08-0121-10-0000-text-on-protocol-reference-model-enhancements-in-802-22.doc

- 802.22 Database Interface Architecture and Security Mechanisms - https://mentor.ieee.org/802.22/dcn/09/22-09-0111-02-0000-database-interface-architecture-and-security-mechanisms.ppt

- Whitespace Database Service - https://mentor.ieee.org/802.19/dcn/09/19-09-0047-00-tvws-white-spaces-database.ppt

Minutes and Discussions

- Ivan Reede’ s comments were discussed during this conference call.

- Comment IR 01 – The 802.22 security mechanisms are quite complex.

- Ivan Reede asked why he should pay for an extra cost of security in the CPE when we will not use it. Cost is definitely an issue. According to Ivan, those people who need security can use VPN to protect their information which can be provided easily at no cost.

- Apurva Mody – asked Ivan that one hand he wanted to protect the management message information and on the other hand he wanted a simple security architecture.

- Ranga said that one of the most important thing is authorization as to who is allowed to access the network. Ranga asked Ivan on how this is done.

- Ivan said that this is done right now in his network based on the MAC address which is used to derive the keys. If a particular MAC address is not allowed to access the network then he will end up generating a false key.

- IR wants the security ad-hoc to balance between the “Cost of Theft of Service” and “Cost of Complexity.” This coment was deferred and Gerald will insert this into the Comment Database to be discussed later.

- IR2 – No comments from Ivan

- IR3 – Counter to Ivan’ s comment is acceptable to Ivan

- IR4 - Counter to Ivan’ s comment is acceptable to Ivan

- IR5 – Whether 802.22 will be compatible with 802.16 architecture. Ranga is confident that it will. Ivan said that he will review the resolution to this comment in the comment confirmation.

- IR 6, IR7, IR8 – Modified as per Ivan’s comments

- IR 9 – The security ad-hoc agrees to some extent and dis-agrees to some extent also. Modifications proposed to the draft. Ivan will review during Comments confirmation.

- IR 10 – Same as comments 667- 668 which have been resolved.

- IR 11 – Modified as per Ivan’s  comments

- IR 12 – Counter to Ivan’ s comments. He will review during comments confirmation

- IR 13 – Ivan’ s comment is that the operators need to rest assured that the keying material will not be relinquished to an imposter or other BS. This comment has been addressed. Resolution - This essentially becomes a device implementation issue. Regardless of whether or not a CPE is re-authenticating to its serving BS or attempting to join a new network, it must stop any existing authorization and TEK state machines, as well as delete any SAs and keying material associated with them.

- IR 14 – Same as CID 677 which has been resolved.

- IR 15 – IR suggests deleting the introductory paragraph of 7.2. Group agreed – It does not add any normative value so just remove this paragraph – introductory paragraph in Section 7.2

- IR 16 – Modified as per Ivan’ s comments

- IR 17 – “This is broken as in CBP, from a CPE to a foreign BS must be in the clear for all foreign BS to understand it. Fix text accordingly,” same as Gerald’s comment that was resolved in November 2009 Plenary. CBP bust is authenticated ONLY and not encrypted. It is sent in the clear. Ivan suggested that if two people have the same mis-understanding them the text needs to be edited to make things clear. Ranga will do that.

- IR 18 - He will review the comments confirmation during comments confirmation

- IR 19 - "Complexity, timers, etc should be kept out of the CPE and pushed towards the BS as much as technically possible to reduce network costs." There were some discussions. Ivan suggests that all the complexity should be kept at the BS. Ranga will see if any timers can be pushed to the BS.

- Other Ivan’s comments were discussed. Ivan will see the proposed resolutions to his comments during the comments confirmation round.