IEEE P802.22
Wireless RANs

| PHY Status and Remaining Issues as in Sept 2009 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-9-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Zander Zhongding Lei | Institute for Infocomm Research, Singapore | 1 Fusionopolis Way #21-01 Connexis, Singapore 138632 | 65-6408-2436 | leizd@i2r.a-star.edu.sg |

Abstract

This document summarizes the current PHY status in the comments and resolution. The remaining PHY related system and PHY specific issues are listed, together with detailed updates.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- PHY Comments Summary

PHY has received total 104 TR/T comments, comprising 93 in section 8 and 11 in section 6 appointed by MAC team. Up to the beginning of the Sept. meeting, we have reviewed one round all 104 comments. The status is summarized as follows:

| Unresolved | Resolved |
| --- | --- |
| - Pending/superseded (42) - Defer (7) | - Accepted (26) - Rejected (5) - Withdrawn (2) - Counter (22) |
| Subtotal: 49 or ~45% | Subtotal 55 or ~55% |

- Detailed Updates on TR/T Comments

- Working Group System Issues

- General

- Apurva (759/760)

Replacing PHY with a stripped down version of 802.16-2009 OFDMA (No MIMO, No AAS, No HARQ) PHY

Jul09: it has been demonstrated that 802.16 PHY does not meet the FRD function requirements. It took 3 years for the PHY team to develop the current PHY specs to satisfy the FRD requirement which are different from that in 802.16. If the commenter can demonstrate that 802.16 PHY can meet the FRD, further consideration may take place. Comment will be rejected as non-actionable if no demonstration is completed by Nov. 2009.

- Sampling Rates

- Robert (ID 771)

Subcarrier spacing is 3.35, 3.91 and 4.46 KHz for 6, 7 and 8 MHz respectively. Suggest to use 7 MHz profile to unify the sampling rates for 6 MHz and 8 MHz while relax the frontend design.

Sept09CC:

Robert: propose a unification of sampling rates for 6 MHz, 7 MHz and 8 MHz. The proposed system will have:

- Every MAC frame of 10ms is a multiple of PHY clock cycle

- Same sampling frequency, Inter-carrier spacing, FFT period, time unit, symbol duration for all 3 bands

It can simplify the radio frontend design and relax the phase noise/frequency offset limitation for 6 MHz radio; and simplify the hardware and software implementation for MAC and PHY.

A number of concerns were raised during conference call:

Ivan: Does the new proposal lead to more dynamic range of ADC.

Zander: What and how important is the benefit of having full cycles of sampling rate in every 10 ms should be elaborated. In current PHY, we have the same elementary sampling clock frequency 8/7 MHz for all three 3 TV channels. The sampling frequency, inter-carrier spacing, FFT period and Time Unit scales nicely with a factor of BW=6, 7, 8 MHz respectively. This gives us the same number of guard sub-carriers, used sub-carriers, data sub-carriers, pilot sub-carriers and we can use the exactly same subcarrier allocation scheme, interleaver and FEC codecs etc.

The new proposal has different block size and may have impact to interleaver, FEC codecs and subcarrier allocation scheme.

The new proposal has different number of subcarriers for different bandwidths. 144 guard subcarriers are used in 8 MHz channels whereas 648 subcarriers are used in 6 MHz channels. It’s a big waste of spectrum.

Sung Hyun: The new proposal will lead to different preamble sequences for different channels.

Gerald: The payload of SCH/FCH may have to be revisited and impact need to be addressed.

Ivan: It seems dramatic change to the PHY is expected. It must bring to the group with heavy advantages to justify the risk of delaying the standard.

- Network Synchronization

- Robert (941)

BSs are synchronized in WRAN. The sync tolerance should be in usec rather than +/-25% of the shortest prefix (relevant to sampling rate issue)

- Steve K (942)

The +/- 25% of the shortest CP (1/32) gives +/- 2.333 usec for 6 MHz channels, which decreases the amount of channel delay spread that can be tolerated by 4.666 usec total.  Point out that the effective CP for the uplink is shorter by e.g. 4.666 usec due to imperfect synchronization.  If long filters are used to satisfy spectral masks, even more of the margin could be consumed.   The utility of short (1/32) CPs is consequently reduced.

Aug09CC:

Robert: We need a specific number in usec rather than +/-25% of the shortest prefix.

Gerald: This +/-25% corresponds to +/-16 TU or sampling clocks.  It works for 6, 7, or 8 MHz channels and the tolerance is different in usec for different channels (different channels are usually used in different geographic area/countries). This is why it is expressed this way rather than in absolute usec since the ranging control loop will work in terms of sampling clock whatever the actual sampling frequency is in the given bandwidth.

Steve: +/-25% eats up into the cyclic prefix range and reduces the multipath correction capability. This represents an effective reduction of the CP of 4.66 usec for the 6 MHx channel bandwidth.  Slightly smaller reduction applies to the 7 and 8 MHz channel bandwidths.  We could reduce it to +/-8TU or even +/-4 TU if further analysis can show that we can time synchronize to the BS frame with e.g. 95% confidence to a tolerance of 4 or 8 time units (instead of the present +/- 16 TU) at some minimum SNR and channel model averaged over all ranging codes.  In the end it comes down to whether the delay spread tolerance of the CP is borderline for a given environment and this 25% sync requires to go to the next larger CP, resulting in an efficiency hit.  802.16-2009 (clause 8.4.10.1.2) uses +/- 25% and it was likely just borrowed from there.  Were there contributions provided in 802.16 that justified this value?  Maybe we can save some work here.

Gerald: As there is another relevant comment on sampling clocks yet to be addressed, the comment is suggested to be addressed together /after."

- Symbol clock frequency tolerance

- Steve K. (952)

The symbol clock frequency is the 3348 Hz subcarrier spacing divided by 1+CP (as far as I understand it; symbol clock frequency is not defined in the draft).  For CP = 1/8, the symbol frequency is 2976 Hz (1/336 usec).  2% of this is 6.7 usec.  Over a 29 symbol frame, the timing could slip by 29 x 6.7 usec = 197 usec, over half a symbol, clearly intolerable.

To keep timing from slipping less than 25% of the smallest CP (1/32) (or 2.333 usec), the symbol clock would have to be accurate to 2.333 usec/10 msec = 233 ppm.  To allow for initial synchronization margin, it should be somewhat less, perhaps 100 ppm.

Demonstrate that 2% tolerance on the symbol clock frequency is acceptable.

Sept09CC:

It seems to be strange that the clock frequency would not be compensated more often than once per frame.

ETRI made simulations previously but these simulations considered only the timing and frequency offset and not the sampling rate offset.  There is a need to do simulations with a drift in sampling clock frequency.

WiLAN did simulations and the preliminary results showed that the synchronization issue (due to system/channel impairment such as timing/ carrier frequency/sampling clock) was challenging

Sung Hyun to investigate whether this 2% subcarrier spacing for the sampling rate offset requirement is sufficient and present the results in Hawaii meeting.

- Short/Long Training Sequences

- Gerald (806) Long Training Sequences (LTS)

There may not be a need for the multiple LTS since the SCH will define which frame belongs to which BS in coexistence mode.  Why do we need 114 orthogonal frame preambles if the actual frame assignment is known from the SCH in a coexistence situation?

Jul09: Accept. Multiple base station coexistence is handled by the TDM based co-existence mechanism. There is no requirement for using orthogonal LTS sequences for different cells. It is agreed in PHY to keep only one LTS sequence. Action: Zander will get comments from Monisha as the sequences are proposed and generated by Monisha.

Sept09 Updates through email

Monisha agree to have remove all 113 redundant sequences and keep the first one only.

- Gerald (807) Short Training Sequences (STS)

Given the decision to use the distributed SCH coexistence method in section 6, clarify whether there should be a number of orthogonal Short Training Sequences (16 or more) to differentiate the BSs right away at the PHY layer rather than having to decode the frame header to identify the BS from which it originates.  Consider the need for more than one STS for the superframe preamble and one for the CBP burst. Since the coexistence mode relies on the distribution of 16 frames per superframe to overlapping WRAN cells and at east one frame has to be transmitted for each overlapping WRAN cell per superframe, there would be a need for 15 more orthogonal STS sequences to implement the distributed SCH method so that CPEs only decode the first frame that is destined to them by correlating their proper superframe preamble.

Jul09: Accept. Multiple base station coexistence is handled by the TDM based co-existence mechanism of the superframe structure, which has 16 frames. There maybe a requirement for using up to 16 orthogonal short training sequences (STS) for different cells. This would allow the CPEs to identify which frame carries the SCH without having to demodulate the SCH to obtain the MAC address. It is agreed in PHY to generate 15 more STS sequences.

Zander will get comments and help in generating the 15 more STS sequences from Monisha.

Sept09 Comments from Monisha through email

There are pros and cons to this approach. The disadvantage of having multiple STS is that initial synchronization will take longer since correlation on the 16 sequences will always be needed. Maybe I’m missing something here, but I don’t see a problem with demodulating the SCH since that has to be done anyway whereas the correlators will be an added complexity. However, if it is decided that 16 STS sequences are needed, I could help generate them.

- FEC Coding

- Gerald (871/875/877/931/332/353) Robert (331/352)

There is no need to keep three advanced FEC coding schemes with almost equivalent performance as part of the standard.  This makes the standard unnecessarily complex (e.g., excessibe number of DIUC and UIUC in Tables 38 and 49). Select one of the three advanced coding schemes based on relative complexity, previous performance comparison (22-07-0319-01-0000-FEC_simulation_results.xls) and any new information, the best data burst mapping (22-07-0190-04-0000_OFDMA_Block_sizes.xls) and on what is used by other standards in the industry, and make it mandatory.

Jul09: The WG decided not to re-open the FEC issues given there was an approved motion to keep the advanced FEC codecs an optional feature

- Winston (872)

Optional FEC should not be included in the standard. Remove Duo-binary CTC mode, LDPC mode, and SBTC mode.

Aug09CC:

Zander: Without advanced FEC coding, the current Draft would not meet the performance requirements.

Ivan: We should choose only one advanced FEC coding and make it mandatory. It is complex for an operator to implement all 3 FEC codecs. If we want to accelerate the approval of the Standard, we should limit it to one advanced codec.  Wi-MAX forum voted for only one advanced FEC codec.

Zander: An operator has the choice of implementing only one of 3 advanced FEC codecs.

Robert: Furthermore, we should eliminate the basic convolutional code and use only one advanced FEC codec for all types of messages such as in LTE since there is no need to be backward compatible.  The interleaver does not seem to work for the basic convolutional code. With only one FEC codec, the interleaver could then be better optimized for this advanced codec.  For example, LTE and Wi-MAX selected the turbo code.

Gerald: It is not clear that the interleaver does work with all codecs. It was shown in 07-190r4 that not all block size combinations are possible for each FEC codec.  This may be the reason. A possibility could be to select only one advanced FEC for the first version of the Standard and add the other ones in subsequent versions if needed. The FEC ad-hoc group could be re-formed to do more comparison work to find the codec with the best performance and select it.  If no codec comes out as superior, one of the three advanced codecs could still be selected out of the three equally performing codecs.  The FEC adhoc group had produced key documents in 2007 on their results: 07-319r1 on FEC performance and 07-190r4 on interleaver capabilities.

Robert: Timing errors and frequency errors should be included in the evaluation of the codecs."

Zander Suggestion for FEC: The main concern of having 3 optional FEC codecs is the potential complexity at BS implanting 3 codecs. Although we should keep the system complexity as low as possible, the advanced FEC issue itself is not a hole in the draft. As the coming 802.22.3 on potable devices are more concerned about device complexity, it is suggested to have thorough investigation there and choose only one codec if possible. If that happens, it may give the group better information whether there is a need to have further amendment to the 802.22 standard. This is similar to the process in 802.16e.

- Interface Spec Between Non-Integrated Antenna And CPE

- Steve K. (916)

Since there is a communications interface between the non-integrated antenna and the 802.22 CPE device, don't we have to completely specify that interface for interoperability? Generate interface spec for IEEE 802.22 CPE device to non-integrated antenna.  There is no information on the format of the stored data or how it is accessed.

Jul09: Assign Ivan to provide a reference standard for transmitting the information from antenna to CPE through coaxial cables.

This is NOT a one person job, the group will have to address many of these antenna aspects and issues

- Antenna Pattern

- Charles/Tom (954/956/962)

The antenna pattern must apply in both azimuth and elevation.  Change text to read "…the CPE shall meet the reference antenna pattern in both azimuth and elevation depicted …".

Aug09CC: Suggested text accepted. The reference antenna pattern in elevation is missing in the draft if it is different from that in azimuth. To assign Ivan to help proposing the text/graph for the pattern in elevation if needed.

Updates through email Sept09:

Ivan: the antenna pattern will not be the same in elevation as it is in azimuth and I would therefore object to a requirement to have the same pattern in elevation as in azimuth.

It would be nice to specify very high gains but the antennaes would become unacceptably large and expensive especially at the lower frequencies. Gain and directivity are different yet highly correlated. So getting a high gain requires a high directivity. Directivity comes at a price. Directivity is highly correlated to antenna array size. Array size requires structural components... therby costs and unsightlyness. Directivity also limits bandwidth. The higher the directivity, the smaller the bandwith.

Antenna performance is frequency dependent. Although gemotrically designed antennaes, such a log periodics and spirals exist, in general, the specifications given by manufacturers are "the best perfomarnce attained" at a sweetspot frequency. We need to consider this and not rely on the specs at this sweetspot frequency but also the worst case performance at the worstcase frequency ... i.e. the antennae perfomrance at the "sourspot" frequencies... This is an important consideration as for example, on some sourspot frequencies, VSWR may be not so good and at those frequncies, and those in its vicinity where the antenna will be either capacitivly or inductivly reactive, the coax cable will no longer have the current on its shield out of phase with the current in its inner conductor and the cable will then start to radiate...

This is NOT a one person job, the group will have to address many of these antenna aspects and issues

- PHY Group

- Pilot PRBS Generator

- Sunghyun (829/839)

There is no text describing the sequences for pilot modulation to avoid the generation of line spectral frequencies. The sequence generator (e.g. PRBS) for 426 pilots (CBP) and 240 pilot (US/DS) should be described in Section 8.5

Jul09: Assign Sung Hyun to propose the PRBS generators, with reference to 802.16e (Figure 204 in 16e 2007, taking consideration the initialization of sequence and reference starting time for BS and each CPE)

Updates through email Sept09:

The proposed PRBS generator for pilot modulation has been circulated in the reflector.

- Subcarrier Mapping

- Apurva (841)

The subcarrier mapping in DS and US is still quite complicated. Is there a need to carry out sub-carrier mapping in a Turbo-like fashion. If the gains of implementation using turbo-like interleaving are not substantial, reduce the complexity and suggest simplified techniques for easier implementation.

Jul09: Assigned John Benko to response to the comments

- Gerald (846)

Formula notations are not clear.  This would need more explanation or a generic algorithm described in C-language.  For example, the function I is some time a single variable function and other time a two-variable function.

Jul09: Assigned John Benko to provide more explanation or an example of implementation to avoid mis-interpretation of the equation

- Bit Interleaving

- Gerald (857)

Notation in the formulas need to be clarified.  For example, the meaning of q' and p' is not clear. Provide more explanation on the notation used or provide a generic implementation of the algorithm in C-language or Matlab.

Jul09: Assigned John Benko to provide more explanation or an example of implementation to avoid mis-interpretation of the equation

- Gerald (858)

The last sentence of this third paragraph needs more explanation.  The function x(k) is not clearly defined.  The distance between input bits separated in the output sequence by s-1 positions when s= 1 should be zero because it would be the same bit but Table 232 says otherwise. The sentence most likely needs to  be modified to correct the inconsistency and should be modified to make it clearer.

Jul09: Assigned John Benko to provide more explanation or clearer text

- Jungsun (859)

The number of coded bit per each block of the CBP payload will be 836. However, there is no the K value of 836 in the table 232. To support the CBP payload, the 836 bits for coded block shall be additionally included in the table 232.

Aug09CC: John Benko is to investigate how this block length can be accommodated by the proposed interleaver."

- Gerald (861)

Notation in the Figure is unclear. Provide more explanation on the notation used or provide a generic implementation of the algorithm in C-language or Matlab.

Jul09: Assigned John Benko to provide more explanation or clearer text (for example, what is P.A_in, q.p.(V1)

- CC Tail Bits vs Tail Biting

- Jungsun (867)

If the PSDU data would use the tail bits for encoder memory to be zero state at the end of encoding, the useful data of the table 236 is not matched with the coded block in the table 232. Table 232 is generated with considering the tail biting. For example, when we assume QPSK, R=1/2, useful data payload=3, then the bits to be entered into convolutional encoder will be 30 bits(24 data bits and 6 tail bits). From the encoder, there will be 60 encoded bits. However, 60 bits for coded block is not defined in the table 232.

PSDU data shall also use the tail biting. Or the re-calculated coded block shall be added in the table 232

Aug09CC: There seems to be a misfit between the convolutional coding using the 6 tail bits and the interleaving scheme. Tail biting was used for the data before in simulations but the use of the 6 tail bits are now used for the PSDU but this no longer fits with the interleaver.  If 6 tail bits are used for convolutional coding, the block size is not covered by the interleaving block sizes available.  Tail biting convolutional encoder would resolve this problem but then the initial state of the viterbi decoder is no longer known and results in higher complexity in the receiver.  There is no performance difference between the tail bits and the tail biting.  Another possibility would be to add new block sizes.

To contact John Benko to see the possibility of adding these new block sizes to match the 6 tail bits and see the relative increase in complexity.

- Jungsun (864)

The block of tail bits in the figure 166 has two problems

1: tail bits are only required for convolutional encoder, not for advanced encoding method. However that figure is common for advanced coding schemes as well as CC. 2: If it is right that PSDU data is encoded with tail bits(refer to 8.7.2.1.1), each block shall be required tail bits. Suggest either 1: The block of tail bit shall be removed and described with in the section 8.7.2.1.1 of convolutional coding or 2: Tail bits shall be included in the each block

Aug09CC: If the convolution coder uses tail bits, each block would need to include these tail bits since the coder encodes on a block basis. Subject to resolution in comment 867

- CTC

- Sunghyun (873)

In Table 237, for QPSK and code rate 1/2, the code parameters for data block size of 3 bytes should be defined.

Jul09: Assigned John Benko to verify the feasibility and provide the parameter

- LDPC

- Gerald (876)

The size of the burst will be different as a function of number of sub-channels on the downstream and on the upstream because of the vertical laying and horizontal laying of data in the downstream and the upstream respectively. Specify whether this applies to the downstream or upstream and develop the formulas for the two cases if needed.

Jul09: propose to change all instances of "subchannel" to "ofdm slot". Pending Steve Kuffner for confirmation.

- SBTC

- Gerald (878)

Undefined parameters used in the Table 246. Clarify the parameters n' and K' in Table 246 and their use in the text.

Jul09: Assign Changlong Xu to provide clarification

- Gerald (882)

The size of the burst will be different as a function of number of sub-channels on the downstream and on the upstream because of the vertical laying and horizontal laying of data in the downstream and the upstream respectively. Specify whether this applies to the downstream or upstream and develop the formulas for the two cases if needed.

Jul09: propose to change all instances of "subchannel" to "ofdm slot". Pending Changlong Xu for confirmation.

- Modulation

- Sunghyun (883)

To avoid the confusion in the demapping metrics, for the first bit of I and Q phase, it is desirable to follow a consistent mapping rule for decision boundary. In the draft, for the I-axis, bit '1' is assigned to the (+) side of the decision boundary, but for the Q-axis, bit '1' is assigned to the (-) side of the decision boundary. In this case, the receiver should different demapping metrics for the first bit of I and Q phase. Modify the constellation mapping. For example, in case of QPSK, from right, 0,1 for I-phase, and from top, 0,1 for Q-phase. The performance is exactly same, not changed.

Jul09: Assign Sung Hyun to fix the diagram and add formulas making sure that the value of 0 is mapped to a positive value and 1 is mapped to a negative value

- Reporting Maximum Available PAPR for Different Modulation

- Gerald (933)

Update and clarify the paragraph to be consistent with the previous ones.  There is no reason why the maximum EIRP will be different for QPSK, 16 QAM and 64 QAM for the multi-carrier modulation used in this standard.  There is only a need to report this maximum achievable EIRP once. Modify the paragraph as follows: "The current transmitted EIRP is the EIRP of the burst that carries the message. The maximum available EIRP is reported for QPSK, 16-QAM and 64-QAM constellations. The current transmitted EIRP and the maximum achievable EIRP parameters is are reported in dBm. The parameters is are quantized in 0.5 dBm steps ranging from –64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme.

Aug09CC: Sung Hyun to verify that whether the PAPR is different or not depending on the type of modulation used on the upstream burst, (use the limit case of 1 sub-channel for the upstream burst = 28 subcarriers).

- Assigned by MAC Team

- Winston (322)

Where is the RSS field measured? How is it measured? Using what resolution?  What bandwidth?

Sept09CC: RSSIR_BS_nom is not a measured value, instead it is a nominal value predetermined by BS. However, clarification is needed for the text in section 6.17.2.6.3.1.

Assigned to Jungsun and Gerald to confirm and clarify the equation and text from line 8 to line 19 in page 161. (terms EIRPIR_MAX , RSSIR_BS_nom, consistency of the signs before the antenna gain GRX_BS  and GRX_CPE)

Updates through email Sept 15

The equation is for deriving the maximum transmit EIRP for initial ranging (EIRP_IR_MAX). EIRP_IR_MAX is calculated with channel loss and nominal equivalent isotropic received power of BS (EIRxP_BS).

Firstly, channel loss can be calculated by subtracting equivalent isotropic received power of CPE (EIRxP_CPE) from BS_EIRP.

Channel Loss(CL) = BS_EIRP - EIRxP_CPE = BS_EIRP -  (RSS - G_RX_CPE)

And, the EIRxP_BS can be defined with (RSS_IR_BS_nom - G_RX_BS).

Therefore, the following equation can be derived and it is the same equation in the draft v2.0.

EIRP_IR_MAX = EIRxP_BS + CL =  (RSS_IR_BS_nom - G_RX_BS) +  (BS_EIRP -  RSS + G_RX_CPE)

- Gerald (330)

DIUC 1 for PAPR reduction needs to be explained

Sept09CC: How DIUC 1 can be used for PAPR reduction should be clarified. Also it should be understand whether this PAPR reduction is for CPE or BS. In 802.16, both UIUC and DIUC tables include values for PAPR reduction. However, there is no UIUC in 802.22 draft. It is understood that CPE is more concerned about PAPR issue.

Assigned to Sunghyun to check the how DIUC/UIUC is used for PAPR reduction in WiBro/802.16.

- Gerald (338)

The calculation of the possible range of the codes seems to be wrong.  Same codes can be re-used for the different functions since they occupy different ranges of symbols in the upstream sub-frame.  The BS will be able to differentiate for the purpose.

Sept09CC: The reason why S value under “Start of ranging codes group” (row 5) in Table 44 can not be zero should be explained / clarified.

Assigned to Jungsun to investigate

- Sunghyun (343)

There are 12 values in normalized C/N table (Table 253). 5 bytes are 10 nibbles or represent 10 (difference) C/N values. 6 bytes are needed to represent the 12 values.

Sept09CC: The values should not be transmitted too often as it will increase the overhead considerably if many CPEs are served, as each CPE would use different values. However, these values are used for short message transmission instead of data transmission. The frequency is not expected to be high.

Assigned to Sung Hyun to investigate the frequency of the C/N values transmissions.