IEEE P802.22
Wireless RANs

| Proposed resolution for comment 322 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-09-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave. Ottawa, Ontario Canada K2H 8S2 | 613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This document proposes a resolution for comment 322 that deals with Table 32 and sub-clause 6.17.2.6.3.1 of the 802.22 Draft 2.0.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

#### Proposed resolution for comment 322

See page 161, line 11 for the equation where this information is used at the CPE.  This is not a measured value but a nominal field strength expected to be received at the BS.   This nominal value needs to be provided by the BS to the CPE to maximize the dynamic range of its A/D while avoiding clipping, thus controlling the input power "per carrier" at the output of the BS receive antenna.

Is the RSS a power density per carrier or the total received power for initial ranging?  The value will depend on how many sub-channels are used by the initial ranging.  It was decided that 2 sub-channels would be used for ranging, resulting in 56 subcarriers.  There should therefore be a correction between the EIRP of the BS for which 60 sub-channels are used versus 2 sub-channels used for initial ranging from the CPE, thus a 2/60 correction. Since there could be colliding CDMA ranging bursts during the same ranging time slot, the BS may want to include some more headroom before saturation and reduce the RSS_IR_BS_Nom accordingly.  This would be implementation specific.

Action: Jung Sun and Gerald to review the equation and text in lines 8 to 19 of page 161.

Email was received from Jung Sun Um on 09.09.14 on result of ETRI review of the equation and text on page 161.

Proposed action from Gerald:

1) Change the last column of the table from "Value" to "Description".

2) Change the description of the RSS parameter to read: "Nominal signal strength to be received at the BS by a 0 dBi gain antenna for initial ranging."

3) Change the parameter "BS EIRP" for "EIRP_BS".

4) Replace the text on page 161 by the following:

“The CPE shall calculate the maximum transmit EIRP for initial ranging, EIRPIR_MAX , from the following equation:

EIRPIR CPE MAX = EIRP_BS + RSSIR_BS_nom – (RSSIR CPE - GRX_CPE) + 10*log(2/60)

where the RSSIR_BS_nom and EIRP_BS are specified by DCD IEs (see Error: Reference source not found).  GRX_CPE is the receive antenna gain at the CPE. RSSIR CPE is the RSSI measured by the CPE, which is then corrected by the CPE antenna gain to represent the RSS for an isotropic receive antenna.

NOTE - The value of RSSIR_BS_nom corresponds to the signal strength that should be measured at the output of a 0 dBi gain receive antenna at the BS for a two sub-channel initial ranging signal coming from a CPE. The BS_EIRP is the equivalent isotropic radiated power of the base station in the direction of the CPE, which is computed for a simple single-antenna transmitter as PTX-BS + GTX-BS, where PTX-BS is the transmit power and GTX-BS is the transmit antenna gain of the BS in the direction of the CPE.  [Note: if the BS antenna is not omnidirectional, this single EIRP_BS value provided in the DCD will not be sufficient.]

The CPE shall send a CDMA code at a power level at or below EIRPIR CPE MAX. If the CPE does not receive a response, the CPE shall send a new CDMA code at the next appropriate Initial Ranging transmission opportunity at one step higher power level. [Where is this “one step” defined?  Where does it stop?] If the CPE receives a RNG-RSP message containing the parameters of the code it has transmitted and the status “continue”, it shall consider the transmission attempt unsuccessful but implement the corrections specified in the RNG-RSP and issue another CDMA code after the appropriate backoff delay. If the CPE receives an US-MAP containing a CDMA allocation IE with the parameters of the code it has transmitted, it shall consider the RNG-RSP reception successful, and proceed to send a unicast RNG-REQ on the allocated BW.”

______________________________