IEEE P802.22
Wireless RANs

| Suggested Changes for Constellation Mapping |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-09-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

As a resolution of comment ID #883 for draft v2.0, this document is the suggested changes for the constellation mapping.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related comments

As the resolution of comment ID #883 for draft v2.0, I submit the suggested changes of corresponding section 8.8.1. The relating comments are as follows:

As an example, for QPSK constellation mapping in current draft (refer to Figure 172), the value of 1 for Q-axis (b0) is assigned to the negative side of the decision boundary, while the value of 1 for I-axis (b1) is assigned to the positive side of the decision boundary. If then, the demapping matrics for b0 and b1 are different, as shown in Figure A and B. Figure A and B show the demapping matrics for b0 and b1, respectively. Here, 6-bit soft decision is assumed. To avoid the confusion, I propose to use same demapping matrics for b0 and b1, and change the gray mapping rule. The suggested mapping rule is the same as the 802.16e standard.

Figure A - Demapping matrics for b0                              Figure B - Demapping matrics for b1

2. Suggested text

    - Data modulation

The output of the bit interleaver is entered serially to the constellation mapper. The input data to the mapper is first divided into groups of NCBPC (seeTable 1) bits and then converted into complex numbers representing QPSK, 16-QAM or 64-QAM constellation points. The mapping for QPSK, 16-QAM and 64-QAM is performed according to Gray-coding constellation mapping, as shown in Figure 1, Figure 2 and Figure 3, respectively where b0 represents the most significant modulation bit for all constellations.

- Gray Mapping for QPSK

- Gray Mapping for 16-QAM

- Gray Mapping for 64-QAM

The complex valued number is scaled by a modulation dependent normalization factor KMOD. Table 1 shows the KMOD values for the different modulation types defined in this subclause. The number of coded bits per block (NCBPSS) and the number of data bits per block for the different constellation type and coding rate combinations are summarized in Table 2. Note that a block corresponds to the data transmitted in a single sub-channel.

- — Modulation dependent normalization factor

| Modulation Type | NCBPC | KMOD |
| --- | --- | --- |
| QPSK | 2 | 12 |
| 16-QAM | 4 | 110 |
| 64-QAM | 6 | 142 |

- — The number of coded bits per subchannel per OFDM symbol (NCBPSS) and the number of data bits per subchannel per OFDM symbol (NDBPSS) for the different constellation type and coding rate combinations

| Constellation type | Coding rate | NCBPSS | NDBPSS |
| --- | --- | --- | --- |
| QPSK | ½ | 48 | 24 |
| QPSK | 2/3 | 48 | 32 |
| QPSK | ¾ | 48 | 36 |
| QPSK | 5/6 | 48 | 40 |
| 16-QAM | ½ | 96 | 48 |
| 16-QAM | 2/3 | 96 | 64 |
| 16-QAM | ¾ | 96 | 72 |
| 16-QAM | 5/6 | 96 | 80 |
| 64-QAM | ½ | 144 | 72 |
| 64-QAM | 2/3 | 144 | 96 |
| 64-QAM | ¾ | 144 | 108 |
| 64-QAM | 5/6 | 144 | 120 |