IEEE P802.22
Wireless RANs

| Suggested Changes for Frequency Control Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

As a resolution of comment ID #951 and #952 for draft v2.0, this document is the suggested changes for the frequency control requirements in Section 8.12.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related comments

As the resolution of comment ID #951 and #952 for draft v2.0, I submit the suggested changes for the frequency control requirements in Section 8.12. The relating comments are as follows:

To investigate whether the tolerance of 2% subcarrier spacing for the symbol clock frequency is reasonable, I looked over the IEEE 802.16 standard which employs the OFDMA. The relating section is as follows:

| From IEEE 802.16 standard |  |
| --- | --- |
| 8.4.14 Frequency control requirements 8.4.14.1 Center frequency and symbol clock frequency tolerance At the BS, the transmitted center frequency, receive center frequency, and the symbol clock frequency shall be derived from the same reference oscillator. At the BS, the reference frequency accuracy shall be better than ±2×10–6. At the SS, both the transmitted center frequency and the sampling frequency shall be derived from the same reference oscillator. Thereby, the SS UL transmission shall be locked to the BS so that its center frequency shall deviate no more than 2% of the subcarrier spacing compared to the BS center frequency. During the synchronization period, the SS shall acquire frequency synchronization within the specified tolerance before attempting any UL transmission. During normal operation, the SS shall track the frequency changes by estimating the DL frequency offset and shall defer any transmission if synchronization is lost. To determine the Tx frequency, the SS shall accumulate the frequency offset corrections transmitted by the BS (e.g., in RNG-RSP message) and may add to the accumulated offset an estimated UL frequency offset based on the DL signal. |  |

The IEEE 802.16 standard specifies only the deviation of center frequency, not symbol clock frequency. It seems to make sense to me.

The relating section of IEEE 802.22 draft is as follows:

| From IEEE 802.22 draft v2.0 |  |
| --- | --- |
| 8.12 Frequency Control Requirements 8.12.1 Center frequency and symbol clock frequency tolerance At the BS, the transmitted center frequency, receive center frequency, and the symbol clock frequency shall be derived from the same reference oscillator. At the BS, the reference frequency tolerance shall be better than ±2 ppm. At the CPE, both the transmitted center frequency and the symbol clock frequency shall be synchronized and locked to the BS with a maximum tolerance of 2% of the subcarrier spacing. During the synchronization period, the CPE shall acquire frequency synchronization within the specified tolerance before attempting any uplink transmission. During normal operation, the CPE shall track the frequency changes and shall defer any transmission if synchronization is lost. |  |

If the transmit center frequency and symbol clock frequency are derived from the same reference oscillator, the deviation of all frequency (i.e., Tx/Rx center frequency, Tx/Rx symbol clock frequency) must be identical to that of the reference frequency (e.g, master clock). Thereby, if the deviation of center frequency after synchronization is defined by “the maximum tolerance of 2% of subcarrier spacing”, it does not need to specify the deviation of symbol clock frequency separately.

From the ETRI’s simulation results (DCN 22-07-0114-00-0000), the tolerance of 2% of subcarrier spacing can be easily achieved using FFO and IFO estimation around 0dB SNR in WRAN fading channel. The tolerance of 2% of subcarrier spacing is equivalent to the 66.96 Hz (=0.02*3.348 KHz). It amounts to 0.096 ppm (=66.96 Hz/695 MHz, 695 MHz center frequency is assumed). Since the sampling frequency can be derived from the same reference oscillator, the tolerance of sampling frequency is same as that of the center frequency. The sampling frequency deviation of 0.096 ppm means that this sampling offset can generate a periodic rob or stuff phenomenon in every 10,416,667 samples. While the maximum number of samples in a 10 ms WRAN frame is about 66,560 samples (=2560 samples/OFDM symbol * 26 OFDM symbols/frame, 1/4 CP ratio and 6 MHz channel bandwidth are assumed). Thus the maximum accumulated samping phase offset due to 0.096 ppm sampling frequency offset is about 0.00639 sample per frame.

The effect of sampling frequency offset is twofold: phase rotation and interference (i.e., ICI and ISI). The phase rotation by 0.096 ppm per sample can be easily compensated by frequency-domain equalizer (phase rotator). Since the relative sampling offset within each OFDM symbol is very small (amount of 0.096 ppm), the degradation due to ICI is negligible (less than 0.01 dB). While the accumulated sampling offset may cause the ISI. However, since the FFT starting point is generally moved forward into the cyclic prefix region, the ISI does not occur.

Because the rob/stuff period due to sampling tolerance is much smaller than the maximum number of samples in a frame, the effect of sampling offset (e.g., phase rotation and interference) will not degrade the BER performance in WRAN system.

2. Suggested text

Based on the opinion above, I propose the text changes as follows in RED:

8.12 Frequency Control Requirements

8.12.1 Center frequency and symbol clock frequency tolerance

At the BS, the transmitted center frequency, receive center frequency, and the symbol clock frequency shall be derived from the same reference oscillator. At the BS, the reference frequency tolerance shall be better than ±2 ppm.

At the CPE, the transmit center frequency and symbol clock frequency shall be derived from the same reference oscillator. Thereby, the transmit center frequency shall be synchronized and locked to the BS with a maximum deviation of 2% of the subcarrier spacing.

During the synchronization period, the CPE shall acquire frequency synchronization within the specified tolerance before attempting any uplink transmission. During normal operation, the CPE shall track the frequency changes and shall defer any transmission if synchronization is lost.