IEEE P802.22
Wireless RANs

| Clarification text for Interleaving |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Isabelle Siaud | Orange Labs | Rue du Clos Courtel 35510 Cesson-Sévigné France | +33 299 124 629 | isabelle.siaud@orange-ftgroup.com |
| John Benko | Orange Labs | 801 Gateway Blvd. Suite 500, South San Francisco, CA 94080 | 650-875-1593 | john.benko@orange-ftgroup.com |

Abstract

Clarifications to interleaver text in Draft 2.0, sections 8.6.2 and 8.6.3.  Note that the figure and table numbering is not in line with the draft numbering and should be modified.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

    - Sub-carrier allocation method

A sub-channel is composed of 28 sub-carriers (24 data and 4 pilot). A sub-channel is the basic unit used for sub-carrier allocation for both downstream and upstream. There is a total of 60 sub-channels per OFDMA symbol.

If pilot insertion is performed after interleaving (downlink), interleaving is performed upon a block of K =NSDC x Nsc data symbols. After interleaving, the K data symbols are split into Nsc groups of NSDC contiguous symbols. Pilots are then inserted in a periodic manner in each sub-group following the pilot pattern defined in the section 8.6.1. Pilot positions vary with the index of OFDMA symbol and sub-carrier index. Nsc sub-channels assorted with pilots associated to a block of K= NSDC x Nsc data symbols are then distributed in the frequency domain to form OFDMA symbols.  DL (downlink) sub-carrier mapping allocation method is illustrated on the Figure 115

For uplink transmission pilot insertion shall be performed before interleaving.  In this case a restriction is enforced such the block size is at least 7 OFDMA symbols long, such that each subcarrier is visited by a pilot tone. In this case, interleaving block size is equal to K'= N'SDCxNSC and successively applied upon 7 blocks of K' symbols. Interleaving is then directly followed with sub-channel generation resorting from a splitting of K'= N'SDCxNSC contiguous symbols into Nsc sub-channels. Sub-channels are then distributed in the frequency and time domain to form OFDMA symbols and generate thanks to OFDM modulator, OFDMA signals.

Sub-carrier mapping shall use the block interleaving algorithm denoted Turbo-Like Interleaving which is described in this subclause and illustrated by Figure 1.  A more detailed description of this interleaving algorithm is contained  in Annex A

- The interleaving algorithm is implemented by the Turbo-Like Interleaver (TLI) which generates specific interleaving patterns resulting from the selection of 4 parameters:

- The interleaving block size K

- The 3 integer parameters {p,q,j} that define the permutation rule L(k), where k is the logical index before interleaving and L(k) is the logical index after interleaving. The equation L(k) provides the input sample position of the input sequence X in the output sequence Y located at the k-th position (Y(k)=X(L(k))

- The iterative structure of the algorithm allows the generation of suitable interleaving pattern in a scalable way. The TLI generates dedicated interleaving patterns according to the permutation rule  and the selected integer parameters {p,q,j} for each block size K as shown in Table 2, where j is the iteration, p is the interleaving partitioning size and is an integer multiplicative factor of K, and q is an integer.

- The performance of L(k) is characterized by the interleaving spreading depth L(k) between interleaved indices for a k index distance before interleaving, or

- The interleaving parameters {p,q,j} have been set to maximize the interleaving spreading depth L(k)

For each parameter set {p,q,j} and K as in Table 2, the permutation rule  shall be calculated as:

where:

Finally For the iteration j=1, L(k) is expressed by :

we have for the iteration j:

- The interleaving structure of the TLI, L(k)

      - Downlink sub-carrier allocation

On the downstream, 60x24=1440 data units destined to modulating sub-carriers (e.g., 2 bits for QPSK, 4 bits for 16-QAM, 6 bits for 64-QAM), called data subcarriers for convenience are interleaved using the TLI algorithm. After interleaving, the K=1440 data symbols are split into Nsc=60 sub-groups composed of NSDC =24 subcarriers. Pilot sub-carriers are then inserted in a periodic manner, 4 per sub-group, following the pilot pattern defined in the section 8.6.1, where the pilot sub-carrier positions change over a cycle of 7 OFDMA symbols. The block of K= NSDC x Nsc =1440 data units along with the 240 inserted pilot subcarriers are then mapped in the frequency domain to form OFDMA symbols. This process is repeated for each data unit in the downstream sub-frame. The sub-carrier mapping allocation method for downstream is illustrated on Figure 115 and the supported block size K and interleaving parameters are shown in Table 2.

Figure 115:  Downstream subcarrier mapping allocation

The sub-carrier allocation in the downstream shall be performed using the following procedure:

1) All null sub-carriers are first allocated;

2) Each block of K=1440 data subcarriers is interleaved using the TLI algorithm;

3) Interleaved data subcarriers are split into Nsc=60 sub-group of  NSDC=24 data unit modulation (data sub-carriers)

4) Pilot subcarriers are then inserted with respect to the pilot pattern according to the rotating pilot pattern shown in figure 159, with one pilot between every 6 data subcarriers, periodically.

Figure 117 shows an example of mapping of the interleaved data unit modulation indices assigned to sub-carriers in a sub-channel before pilot insertion (sub-group), ie the mapping of input data unit modulation in a OFDMA slot. Three successive sub-channels are shown to illustrate the interleaving sub-pattern diversity associated with the selected interleaving pattern {K,p,q,j}={1440,40,2,2}. The interleaved data unit modulation index range is from 0 to K-1 and is represented on the y-axis. Each L(k=l+NSDC*m) data subcarrier index is assigned to the l-th data sub-carrier of the m-th sub-channel SC#m with m={0, NSC-1} and l=.{0,…,NSDC-1} with {NSC, NSDC}= {60,24}. The sub-carrier index in a sub-channel is represented on the x-axis (before pilot insertion). Sub-pattern diversity is not modified by pilot insertion because pilot insertion is repeated 4 times in a sub-channel and repeated in each sub-channel associated to an OFDMA symbol. The data subcarrier index per sub-channel ranges from 0 to NSDC-1 =23.

- Figure 117:  Downstream sub-channel interleaving pattern

Figure 162 shows an example allocation for the first OFDMA symbol in a burst, where P is a Pilot subcarrier,

and L(x), represents the location of the data subcarrier after before permutation:

Figure 162 Example allocations for the first OFDMA symbol in a burst

      - Uplink Sub-carrier allocation

On the upstream, two sub-channels are reserved for opportunistic or scheduled control signaling (i.e., Ranging/BW Request/UCS bursts) using the pre-determined tone indices shown in Table 231 after the appropriate tone offset has been applied as indicated below.  When these sub-channels are not used for control signaling, they can be used for data. In such case, the pilot subcarriers are logically allocated as shown in Figure 159 and then mapped to the appropriate tone indices.

For the remaining 58 sub-channels, pilot subcarrier insertion shall be performed before interleaving. The upstream burst size is at least 7 OFDMA symbols long, so that each subcarrier is visited at least once by a pilot tone. In this case, interleaving block size shall be equal to K'= N'SDCxNSC (60-2)x(24+4)= 1624.  Interleaving shall then be directly followed with sub-channel generation resorting from a splitting of the 1624 contiguous logical subcarriers into 58 sub-channels. These sub-channels are then distributed in the frequency and time domain by skipping over the 56 subcarriers of the two first sub-channels reserved for control signaling to form the OFDMA signals.

Selected interleaving parameters associated to the data block size K' and interleaving spreading are given in the Table 2.

The sub-carrier allocation in the uplink shall be performed using the following procedure:

1)	 All null sub-carriers are first allocated;.

2)	 The 56 sub-carriers of the 2 first sub-channels used for "Ranging/BW requests/UCS" are allocated using the tone index # given in Table 1 below and applying the offsets (-19, -11, +11, +19) cycled every four successive frames and starting at the beginning of each superframe, or applying the offset specified by the Permutation base UCD channel IE as indicated in Table 44 for the current frame.

3) 	Pilots subcarriers are inserted in eack block of subcarriers belonging to the sub-channels allocated to the specific CPE following the pilot pattern defined in the section 8.6.1 over 7 OFDMA symbols.. Each block of subcarriers belonging to the sub-channels allocated to the specific CPE is interleaved over K' = 1624 subcarriers using the TLI algorithm. These sub-channels are then distributed in the frequency domain.

Figure 118 shows an example of mapping of the subcarrier indices in a sub-channel including pilot subcarriers after interleaving. Three successive sub-channels are shown to illustrate the interleaving sub-pattern diversity associated with the selected interleaving pattern {K,p,q,j}={1624,4,2,3}. The sub-carrier index range is from 0 to K'-1 and is represented on the y-axis. Each L(k=l+28*m)  subcarrier index is assigned to the k-th sub-carrier of the m-th sub-channel SC#j with m=floor(k/28) and l=k mod 28. l is represented on the x-axis. m varies from 0 to 57. The sub-carrier index per sub-channel ranges from 0 to 27.

- Upstream sub-channel interleaving pattern

- — Tone sets for Ranging/BW Request sub-channels (see xxx)

| Reference Tone index # |  |  |  |
| --- | --- | --- | --- |
| -814 | -508 | 49 | 519 |
| -813 | -503 | 111 | 536 |
| -811 | -502 | 176 | 560 |
| -807 | -465 | 224 | 594 |
| -799 | -456 | 272 | 644 |
| -778 | -414 | 307 | 693 |
| -747 | -377 | 335 | 713 |
| -712 | -334 | 378 | 748 |
| -692 | -305 | 415 | 779 |
| -643 | -271 | 457 | 800 |
| -593 | -223 | 466 | 808 |
| -559 | -175 | 503 | 812 |
| -535 | -110 | 504 | 814 |
| -518 | -48 | 509 | 815 |

- Interleaving parameters for sub-carrier mapping allocation for DS , US and CBP burst

|  | Interleaving depth | Interleaving parameters |  |  | Interleaving spreading L(Δk) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk=24 | Δk= 48 |
|  | 1440 | 32 | 2 | 3 | 577 | 286 | 572 | 552 | 336 |
| UL | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk= 28 | Δk= 56 |
|  | 1624 | 4 | 2 | 3 | 743 | 138 | 388 | 532 | 560 |

| CBP burst | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk=6 | Δk= 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 720 | 12 | 2 | 1 | 311 | 98 | 213 | 294 | 328 |

    - Bit interleaving

The same Turbo-Like interleaver (TLI), used for sub-carrier interleaving, shall be used to interleave the data bits at the output of the encoder. The TLI algorithm is described in 8.6 and further details are available in Annex A. For each interleaving block size Kb, the permutation rule L(k) provides the position of the input bit of the input sequence in the output sequence at the position k, i.e., (Xout(k)=Xin(L(k)))

The TLI interleaving equation L(k) associated to Xout(k)=Xin(L(k)) is reminded below:

For each parameter set {p,q,j} and K, the permutation rule  shall be calculated as:

For the iteration j=1, L(k) is expressed by :

The interleaving parameters {p,q,j} are selected to maximize the interleaving spreading.

Table 3 provides the supported block size and sets of interleaving parameters {p,q,j}. For simplicity, the parameter q is set to "2" for all binary interleaving. Table 3 also provides informative indication on the interleaving spreading depth for different values of subcarrier index spacing between bits before interleaving.

— Interleaving pattern description

| Coded Block | Interleaver Parameters |  |  | Interleaver Spreading (informative) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K (bits) | p | q | j | ΔL(Δk=1) | ΔL(Δk=2) | ΔL(Δk=3) | ΔL(Δk=4) | ΔL(Δk=6) | ΔL(Δk=8) |
| 48 | 16 | 2 | 2 | 17 | 14 | 3 | 20 | 6 | 8 |
| 96 | 3 | 2 | 3 | 25 | 46 | 21 | 4 | 42 | 8 |
| 144 | 6 | 2 | 3 | 61 | 22 | 39 | 44 | 66 | 56 |
| 192 | 3 | 2 | 3 | 71 | 50 | 21 | 92 | 42 | 8 |
| 240 | 6 | 2 | 3 | 109 | 22 | 87 | 44 | 66 | 88 |
| 288 | 3 | 2 | 3 | 121 | 46 | 75 | 92 | 138 | 104 |
| 336 | 16 | 2 | 3 | 95 | 146 | 51 | 44 | 102 | 88 |
| 384 | 6 | 2 | 3 | 179 | 26 | 153 | 52 | 78 | 104 |
| 432 | 18 | 2 | 1 | 181 | 70 | 111 | 140 | 210 | 152 |
| 480 | 16 | 2 | 3 | 191 | 98 | 93 | 196 | 186 | 88 |
| 528 | 6 | 2 | 3 | 227 | 74 | 153 | 148 | 222 | 232 |
| 576 | 36 | 2 | 1 | 217 | 142 | 75 | 284 | 150 | 8 |
| 672 | 3 | 2 | 2 | 263 | 146 | 117 | 292 | 234 | 88 |
| 720 | 12 | 2 | 1 | 311 | 98 | 213 | 196 | 294 | 328 |
| 768 | 3 | 2 | 3 | 313 | 142 | 171 | 284 | 342 | 200 |
| 836 | 22 | 2 | 2 | 351 | 134 | 217 | 268 | 402 | 300 |
| 864 | 48 | 2 | 1 | 383 | 98 | 285 | 196 | 294 | 392 |
| 960 | 6 | 2 | 3 | 371 | 218 | 153 | 436 | 306 | 88 |
| 1008 | 36 | 2 | 1 | 361 | 286 | 75 | 436 | 150 | 136 |
| 1056 | 16 | 2 | 3 | 671 | 866 | 476 | 831 | 198 | 88 |
| 1152 | 36 | 2 | 1 | 359 | 434 | 75 | 284 | 150 | 568 |
| 1248 | 3 | 2 | 2 | 409 | 430 | 21 | 388 | 42 | 472 |
| 1344 | 6 | 2 | 3 | 589 | 166 | 423 | 332 | 498 | 664 |
| 1440 | 40 | 2 | 2 | 559 | 322 | 237 | 644 | 474 | 152 |
| 1536 | 6 | 2 | 3 | 589 | 358 | 231 | 716 | 462 | 104 |
| 1632 | 3 | 2 | 3 | 793 | 46 | 747 | 92 | 138 | 184 |
| 1680 | 40 | 2 | 2 | 559 | 562 | 3 | 556 | 6 | 568 |
| 1728 | 36 | 2 | 1 | 793 | 142 | 651 | 284 | 426 | 568 |
| 1824 | 48 | 2 | 1 | 769 | 286 | 483 | 572 | 858 | 680 |
| 1920 | 48 | 2 | 1 | 863 | 194 | 669 | 388 | 582 | 776 |
| 2016 | 16 | 2 | 3 | 767 | 482 | 285 | 964 | 570 | 88 |
| 2112 | 16 | 2 | 3 | 671 | 770 | 99 | 572 | 198 | 968 |
| 2208 | 3 | 2 | 3 | 743 | 722 | 21 | 764 | 42 | 680 |
| 2304 | 16 | 2 | 3 | 1055 | 194 | 861 | 388 | 582 | 776 |

# Annex A (Normative) Turbo-Like Interleaving (TLI) algorithm

## Dedicated Properties

The Turbo-Like Interleaver (TLI) algorithm described by the equation L(k) with Y(k)=X(L(k)) is derived from an empirical approach intended to fulfill three main technical requirements:

1) Control the interleaving spreading in an algebraic way in order to generate appropriate interleaving patterns in accordance with a multi-level interleaving spreading maximization criterion. The interleaving spreading denoted L(s), is the minimum distance between samples separated by s-1 samples at the interleaving output with. Multi-level interleaving spreading maximization is very efficient to interleave encoded bits mapped through high level modulated symbols, as well as to allocate data sub-carriers in OFDMA systems. The figure below shows the interleaving spreading functionality.

Figure A-1 Interleaving spreading definition and notations

2) Preserve an interleaving partitioning with a size p equivalent to a p-parallel multi-stream interleaving in order to make compatible special data multiplexing with the interleaving process.

The size of the interleaving partitioning size is given by one parameter of the interleaver, the integer parameter p. This property is represented by the equation:

(A-1)

This property is illustrated in Figure A-2.

- Figure A-2: Preservation of an interleaving partitioning with a size p

This property will be used to respect a given ordering of data in a sub-frame where the elementary data ordering has a size set to p.

This property is also particularly interesting to respect special multi-user data mapping ordering in a sub-frame while performing interleaving that benefits all users. Consequently, the multi-user frame shall use a single transmission scheme. The advantage of the structure is to allow a multi-stream interleaving in considering multi-user data mapping compressed into a single stream.

3) A scalable and dynamic interleaving algorithm which allows dynamic and variable permutation rules adapted to non-stationary transmission channels and variable frame formats. For that purpose, an iterative structure was selected with an elementary interleaver unit I included into a local loop.

The turbo transmitter structure is illustrated in Figure A-3 considering interleaving block size K and associated {p,q,j} interleaving parameters.

- Figure A-3: Iterative-based interleaving processMathematical description

The bloc interleaving algorithm  with a size K is described by four parameters: the block size K, an integer parameter p setting the interleaving partitioning size, an integer parameter q and the iteration j that modify, in conjunction with p, the interleaving spreadingL(s).

The interleaving algorithm L(k) is characterized by an iterative structure using a basic interleaver I(k, k1) characterized by two inputs {k,k1} and one output. The input k is fed with the usual position index incremental of samples in the initial sequence k={0, 1, …K-1} and k1 is fed with the interleaving pattern resulting from the previous iteration (j-1) of the algorithm  (See Figure A-4 for illustration). For j-1=0, k1 is identical to k.

I(k, k1) is built on a serial combination of two algebraic functions Lo,p(k,k1) and L1,p,q(k,k1) characterized by two inputs and one output. Inputs of the function Lo,p(k,k1) are both fed with the initial k position index and k1 is the feedback of the previous iteration of the algorithm: . Inputs of the function L1,p,q(k,k1) are both fed by k and the output of the function Lo,p(k,k1) associated with the current iteration.

The functions L0,p(k,k1) and L1,p,q(k,k1) are expressed as follow:

(A-2)

These two functions are related to each other where the output of Lop is the input k1 of L1,p,q for the current iteration. Furthermore, for the first iteration, we take k=k1 for Lop.

For the first iteration, j=1, L0,p, L1,p,q, and L(k) are expressed, as follow:

(A-2)

For the j-th iteration, we have:

(A-3)

- Figure A-4: Turbo-like structure of the interleaving algorithm

The interleaving spreading between samples separated with s-1 samples corresponds to lowest absolute value of the dispersion  and the function when k varies from 0 to K-1. It also coincides with the median value of the absolute value of the dispersion [B2] and depends only on s.

The interleaving spreading for the first iteration is given by:

(A-4)

For the iteration j, we have:

(A-5)

These algebraic functions are used to select appropriate interleaving parameters following the optimization criteria.

References (to be added to draft):

- Siaud.I, Ulmer-Moll A.M., "A Novel Adaptive Sup-carrier Interleaving Application to millimetre-wave WPAN OFDM System (IST MAGNET Project)," Conference IEEE Portable 2007, Orlando (USA), March 2007.

- Siaud.I, Ulmer-Moll A.M., "Turbo-like Processing for Scalable Interleaving Pattern Generation: application to 60 GHz UWB-OFDM systems," ICUWB'07, Singapore, September 2007.