IEEE P802.22
Wireless RANs

| Clarification text for Interleaving |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-04-02 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Isabelle Siaud | Orange Labs | Rue du Clos Courtel 35510 Cesson-Sévigné France | +33 299 124 629 | isabelle.siaud@orange-ftgroup.com |
| Lingjie Li | WiLAN Inc. | 11 Holland Ave, Ottawa, Ontario, Canada | 1-613-668-7074 | jli@wilan.com |
| Sung-Hyun Hwang | ETRI | 138 Gajeongno, Yuseong-gu, Daejeon, Korea | +82-42-860-1133 | shwang@etri.re.kr |

Abstract

Revised text for 8.6.  Note that the section, figure and table numbering is not in line with the draft numbering and should be modified.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

  - OFDM subcarrier allocation

Among the 2048 subcarriers in each OFDM symbol, 384 subcarriers are null subcarriers (left guard band, right guard band and DC subcarrier). The remaining 1680 subcarriers are for pilot and data, which are partitioned into 60 sub-channels. A sub-channel is composed of 28 subcarriers (24 data and 4 pilot subcarriers). A sub-channel is the basic unit used for subcarrier allocation in both downstream and upstream.

In the downstream, the 240 pilot values are allocated first. A sequence of 1440 complex data values from constellation mapper are interleaved using the TLI algorithnm described in 8.6.2. These interleaved values are then allocated to the 1440 data subcarriers.

In the upstream, 2 sub-channels are reserved for opportunistic or scheduled control signaling. Among the remaining 58 sub-channels, the sub-channels allocated to the CPE are assigned with pilot values and data from constellation mapper. Other sub-channels are assigned with 0. The 1624 assigned values are interleaved using the TLI algorithm described in 8.6.2. These interleaved values are then allocated to the 1624 remaining subcarriers.

    - Pilot pattern

The pilot insertion pattern is shown in Figure 1. The pilot pattern shall be repeated every 7 OFDM symbols and 7 subcarriers in the time and frequency domains, respectively. The pilot pattern is always the same independent of the channel bandwidth. The pilot pattern shall also be the same for the downstream and upstream. In the downstream, the pilots does not participate in the interleaving procedure, and the physical representation of the pilots in the time and frequency domains is the same as in Figure 155 except for the subcarrier index (physical subcarrier indices -840, -839, …, -834 should be used instead of logical indices 0, 1, …, 6). In the upstream, the pilots participate in the interleaving procedure, they will not have the same spacing after the interleaving, and their physical representation is different from Figure 155. These pilot signals shall be used by both BS and CPE for robust channel estimation and tracking against frequency offset and phase noise. Note that the pattern described in this subclause does not apply to the CBP packet.

- Pilot pattern (in the logical domain before interleaving)

- The pilot signals in data packet shall be BPSK modulated, as described in 8.8.2, by a pseudo random binary sequence to avoid the generation of line spectral frequencies. The polynomial for the PRBS generator shall be the same as that used for the data scrambler, as defined in 8.7.1. The PRBS generator shall be initialized by the same seed value, 0 1 1 0 1 1 1 0 0 0 1 0 1 0 1, starting with MSB on the left. The sequence shall be used for pilot subcarriers from the first symbol in a successive manner. The first 240 bits are used for pilot modulation in the first OFDMA symbol, while the next 240 bits are used for pilot modulation in the second OFDMA symbol, and so on. The first OFDMA symbol starts after the preamble in every downstream subframe, while the first OFDMA symbol starts from the initial OFDMA symbol in every upstream burst.

    - Turbo-Like Interleaving (LTI) algorithm

The LTI algorithm uses four parameters K, p, q and j to generate an permutation rule L(k). K is the interleaving block size, p and q are integers, and j specifies the number of iterative calculations in the algorithm.

For both subcarrier interleaving and bit interleaving, an input sequence is interleaved to generate an output sequence. After the interleaving, the value at index L(k) in the input sequence is relocated to index k in the output sequence. In other words, index L(k) in the input sequence corresponds to index k (k=0,⋯,K−1) in the output sequence.

Let Ip,q,K(0)(k)=kdenotes the initial index pattern, i.e., the index pattern of the input sequence. The algorithm iteratively updates the index pattern. At the output of iteration m, the index pattern can be calculated as

Ip,q,K(m)=K−p+k+q⋅p⋅−k−p⋅Ip,q,K(m−1)(k)modKmodK.

After j times of iterations, the jth iteration output index pattern is used as the permutation rule:L(k)=Ip,q,K(j)(k).

- The performance of L(k) is characterized by the interleaving spreading depth L(k). For any indices k and k+k in the output sequence, L(k) represents the minimum distance between the corresponding indices L(k) and L(k+k) in the input sequence. L(k)  can be calculated as

- The interleaving parameters {p,q,j} have been set to maximize the interleaving spreading depth L(k) for small values of k.

    - Downstream subcarrier allocation

For each symbol in the downstream, every 4 pilot values and every 24 complex data from the constellation mapper are assigned to a sub-channel. For example, sub-channel 1 is consisted of pilot values at indices 0,…, 3 and complex data values at indices 0, …, 23; sub-channel 2 is consisted of pilot values at indices 4, …, 7 and complex data values at indices 24, …, 47.

The pilot values shall be directly allocated (without interleaving) to pilot subcarriers following the pilot pattern defined in 8.6.1. A sequence of 1440 complex data from the constellation mapper shall first be interleaved, then the interleaved data sequence shall be allocated to the data subcarriers sequentially.

The permutation rule L(k) used for the interleaving shall be determined using the TLI algorithm described in 8.6.2 with the parameters {K,p,q,j}={1440,40,2,2} (specified in Table 4). Index L(k) in the constellation mapper output data sequence shall correspond to index k in the interleaved data sequence. Table 1 shows the relationship between the constellation mapper output sequence and the interleaved sequence for sub-channels 1, 2 and 3. In the table, constellation mapper output sequence is the input, interleaved sequence is the output.

Table 1 – Interleaving patterns for sub-channels 1, 2 and 3 in the upstream

| Sub-channel 1 |  | Sub-channel 2 |  | Sub-channel 3 |  |
| --- | --- | --- | --- | --- | --- |
| Input index | Output index | Input index | Output index | Input index | Output index |
| 0 | 521 | 24 | 65 | 48 | 1049 |
| 1 | 1402 | 25 | 946 | 49 | 490 |
| 2 | 843 | 26 | 387 | 50 | 1371 |
| 3 | 284 | 27 | 1268 | 51 | 812 |
| 4 | 1165 | 28 | 709 | 52 | 253 |
| 5 | 606 | 29 | 150 | 53 | 1134 |
| 6 | 47 | 30 | 1031 | 54 | 575 |
| 7 | 928 | 31 | 472 | 55 | 16 |
| 8 | 369 | 32 | 1353 | 56 | 897 |
| 9 | 1250 | 33 | 794 | 57 | 338 |
| 0 | 691 | 34 | 235 | 58 | 1219 |
| 11 | 132 | 35 | 1116 | 59 | 660 |
| 12 | 1013 | 36 | 557 | 60 | 101 |
| 13 | 454 | 37 | 1438 | 61 | 982 |
| 14 | 1335 | 38 | 879 | 62 | 423 |
| 15 | 776 | 39 | 320 | 63 | 1304 |
| 16 | 217 | 40 | 1201 | 64 | 745 |
| 17 | 1098 | 41 | 642 | 65 | 186 |
| 18 | 539 | 42 | 83 | 66 | 1067 |
| 19 | 1420 | 43 | 964 | 67 | 508 |
| 20 | 861 | 44 | 405 | 68 | 1389 |
| 21 | 302 | 45 | 1286 | 69 | 830 |
| 22 | 1183 | 46 | 727 | 70 | 271 |
| 23 | 624 | 47 | 168 | 71 | 1152 |

The subcarrier allocation in the downstream shall be performed using the following procedure:

1) All null subcarriers are first allocated.

2) The pilot values are allocated to the pilot subcarriers following the pilot pattern described in 8.6.1.

3) K=1440 complex values from the constellation mapper are interleaved using the TLI algorithm with parameters {K,p,q,j}={1440,40,2,2}.

4) The interleaved data sequence is allocated sequentially to the remaining subcarriers.

Figure 162 shows an example allocation for the first OFDMA symbol in a burst, where P is a pilot subcarrier,

and L(k) represents an index in the constellation mapper output data sequence.

Figure 162 Example allocation for the first OFDMA symbol in a burst

    - Upstream subcarrier allocation

In the upstream, two sub-channels are reserved for opportunistic or scheduled control signaling (i.e., Ranging/BW request/UCS notification bursts) using the pre-determined subcarrier indices shown in Table 3 after the appropriate subcarrier offset has been applied as indicated below.  When these sub-channels are not used for control signaling, they can be used for data. In such case, the pilot subcarriers are logically allocated as shown in Figure 159 and then mapped to the appropriate subcarrier indices.

Every 4 pilot values are associated with a sub-channel. For example, pilot values at indices 0,…,3 are associated with sub-channel 1, pilot values at indices 4,…,7 are associated with sub-channel 2.

For the remaining 58 sub-channels, 0 shall be assigned to all the subcarriers in the sub-channels not assigned to the CPE. For each sub-channel assigned to the CPE, the 4 associated pilot values and 24 complex data from the constellation mapper shall be assigned. For example, if sub-channel 1 and 2 are assigned to the CPE, then sub-channel 1 is consisted of pilot values at indices 0, …,3 and complex data values at indices 0,…,23; sub-channel 2 is consisted of pilot values at indices 4, …,7 and complex data values at indices 24,…,47. If sub-channel 5 and 6 are assigned to the CPE, then sub-channel 5 is consisted of pilot values at indices 16, …,19 and complex data values at indices 0,…,23; sub-channel 6 is consisted of pilot values at indices 20, …,23 and complex data values at indices 24,…,47.

Within each sub-channel assigned to the CPE, the 4 pilot values shall be inserted following the pilot pattern described in 8.6.1. Then complex data values shall be sequentially allocated to the remaining 24 logical subcarriers.

Interleaving shall be performed on the subcarriers in the 58 sub-channels. The interleaved sequence shall be allocated sequentially to the 1624 remaining physical subcarriers in the symbol.

The permutation rule L(k) used for the interleaving shall be determined using the TLI algorithm described in 8.6.2 with the parameters {K,p,q,j}={1624,4,2,3} (specified in Table 4). Index L(k) in the interleaver input sequence shall correspond to index k in the interleaver output sequence. Table 2 shows the relationship between the interleaver input sequence and the interleaver output sequence for the case that sub-channels 1 and 2 are assigned to the CPE. In the table, Pi represents the i’th pilot value, Dj represents the j’th data value from the constellation mapper.

Table 2 – Example interleaving patterns in the upstream:

Sub-channel 1 and 2 are allocated to the CPE, 1st symbol in burst

| Sub-channel 1 |  |  | Sub-channel 2 |  |  |
| --- | --- | --- | --- | --- | --- |
| Input index | value | Output index | Input index | value | Output index |
| 0 | P0 | 341 | 28 | P4 | 1377 |
| 1 | D0 | 494 | 29 | D24 | 1530 |
| 2 | D1 | 647 | 30 | D25 | 59 |
| 3 | D2 | 800 | 31 | D26 | 212 |
| 4 | D3 | 953 | 32 | D27 | 365 |
| 5 | D4 | 1106 | 33 | D28 | 518 |
| 6 | D5 | 1259 | 34 | D29 | 671 |
| 7 | P1 | 1412 | 35 | P5 | 824 |
| 8 | D6 | 1565 | 36 | D30 | 977 |
| 9 | D7 | 94 | 37 | D31 | 1130 |
| 10 | D8 | 247 | 38 | D32 | 1283 |
| 11 | D9 | 400 | 39 | D33 | 1436 |
| 12 | D10 | 553 | 40 | D34 | 1589 |
| 13 | D11 | 706 | 41 | D35 | 118 |
| 14 | P2 | 859 | 42 | P6 | 271 |
| 15 | D12 | 1012 | 43 | D36 | 424 |
| 16 | D13 | 1165 | 44 | D37 | 577 |
| 17 | D14 | 1318 | 45 | D38 | 730 |
| 18 | D15 | 1471 | 46 | D39 | 883 |
| 19 | D16 | 1624 | 47 | D40 | 1036 |
| 20 | D17 | 153 | 48 | D41 | 1189 |
| 21 | P3 | 306 | 49 | P7 | 1342 |
| 22 | D18 | 459 | 50 | D42 | 1495 |
| 23 | D19 | 612 | 51 | D43 | 24 |
| 24 | D20 | 765 | 52 | D44 | 177 |
| 25 | D21 | 918 | 53 | D45 | 330 |
| 26 | D22 | 1071 | 54 | D46 | 483 |
| 27 | D23 | 1224 | 55 | D47 | 636 |

The subcarrier allocation in the upstream shall be performed using the following procedure:

1)	 All null subcarriers are first allocated.

2)	 The 56 subcarriers of the 2 first sub-channels used for "Ranging/BW requests/UCS notification" are allocated using the subcarrier indices given in Table 3 below and applying the offsets (-19, -11, +11, +19) cycled every four successive frames and starting at the beginning of each superframe, or applying the offset specified by the Permutation base UCD channel IE if specified for the current frame as indicated in Table 44.

3) For each sub-channel allocated to the CPE, the 4 associated pilot values are inserted following the pilot pattern described in 8.6.1. Complex data values from the constellation mapper are then sequentially allocated to the remaining 24 logical subcarriers.

4) 0 is assigned to all the sub-channels not allocated to the CPE,

5) The 1624 assigned logical subcarriers are interleaved using the TLI algorithm with parameters {K,p,q,j}={1624,4,2,3}.

6) The interleaved sequence is allocated sequentially to the remaining 1624 subcarriers.

Table 3

— Subcarrier indices for Ranging/BW/UCS notification request sub-channels (see xxx)

| Reference subcarrier index |  |  |  |
| --- | --- | --- | --- |
| -814 | -508 | 49 | 519 |
| -813 | -503 | 111 | 536 |
| -811 | -502 | 176 | 560 |
| -807 | -465 | 224 | 594 |
| -799 | -456 | 272 | 644 |
| -778 | -414 | 307 | 693 |
| -747 | -377 | 335 | 713 |
| -712 | -334 | 378 | 748 |
| -692 | -305 | 415 | 779 |
| -643 | -271 | 457 | 800 |
| -593 | -223 | 466 | 808 |
| -559 | -175 | 503 | 812 |
| -535 | -110 | 504 | 814 |
| -518 | -48 | 509 | 815 |

- Table 4 -- Interleaving parameters for subcarrier mapping allocation for DS, US and CBP burst

|  | Interleaving depth | Interleaving parameters |  |  | Interleaving spreading L(Δk) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk=24 | Δk= 48 |
|  | 1440 | 40 | 2 | 2 | 559 | 322 | 644 | 456 | 528 |
| UL | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk= 28 | Δk= 56 |
|  | 1624 | 4 | 2 | 3 | 743 | 138 | 388 | 532 | 560 |

| CBP burst | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk=6 | Δk= 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 720 | 12 | 2 | 1 | 311 | 98 | 213 | 294 | 328 |

    - Bit interleaving

The Turbo-Like Interleaving algorithm described in 8.6.2 shall be used to interleave the data bits at the output of the puncturing procedure to generate an interleaved bit sequence. The index L(k) in the bit sequence at the output of the puncturing procedure shall correspond to the index k in the interleaved bit sequence.

Table 5 provides the supported block sizes and sets of interleaving parameters {p,q,j}. For simplicity, the parameter q is set to "2" for all binary interleaving. Table 5 also provides informative indication on the interleaving spreading depth L(k) for some k values.

Table 5 — Interleaving pattern description

| Coded Block | Interleaver Parameters |  |  | Interleaver Spreading (informative) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| K (bits) | p | q | j | ΔL(Δk=1) | ΔL(Δk=2) | ΔL(Δk=3) | ΔL(Δk=4) | ΔL(Δk=6) | ΔL(Δk=8) |
| 48 | 16 | 2 | 2 | 17 | 14 | 3 | 20 | 6 | 8 |
| 96 | 3 | 2 | 3 | 25 | 46 | 21 | 4 | 42 | 8 |
| 144 | 6 | 2 | 3 | 61 | 22 | 39 | 44 | 66 | 56 |
| 192 | 3 | 2 | 3 | 71 | 50 | 21 | 92 | 42 | 8 |
| 240 | 6 | 2 | 3 | 109 | 22 | 87 | 44 | 66 | 88 |
| 288 | 3 | 2 | 3 | 121 | 46 | 75 | 92 | 138 | 104 |
| 336 | 16 | 2 | 3 | 95 | 146 | 51 | 44 | 102 | 88 |
| 384 | 6 | 2 | 3 | 179 | 26 | 153 | 52 | 78 | 104 |
| 432 | 18 | 2 | 1 | 181 | 70 | 111 | 140 | 210 | 152 |
| 480 | 16 | 2 | 3 | 191 | 98 | 93 | 196 | 186 | 88 |
| 528 | 6 | 2 | 3 | 227 | 74 | 153 | 148 | 222 | 232 |
| 576 | 36 | 2 | 1 | 217 | 142 | 75 | 284 | 150 | 8 |
| 672 | 3 | 2 | 2 | 263 | 146 | 117 | 292 | 234 | 88 |
| 720 | 12 | 2 | 1 | 311 | 98 | 213 | 196 | 294 | 328 |
| 768 | 3 | 2 | 3 | 313 | 142 | 171 | 284 | 342 | 200 |
| 836 | 22 | 2 | 2 | 351 | 134 | 217 | 268 | 402 | 300 |
| 864 | 48 | 2 | 1 | 383 | 98 | 285 | 196 | 294 | 392 |
| 960 | 6 | 2 | 3 | 371 | 218 | 153 | 436 | 306 | 88 |
| 1008 | 36 | 2 | 1 | 361 | 286 | 75 | 436 | 150 | 136 |
| 1056 | 16 | 2 | 3 | 671 | 866 | 476 | 831 | 198 | 88 |
| 1152 | 36 | 2 | 1 | 359 | 434 | 75 | 284 | 150 | 568 |
| 1248 | 3 | 2 | 2 | 409 | 430 | 21 | 388 | 42 | 472 |
| 1344 | 6 | 2 | 3 | 589 | 166 | 423 | 332 | 498 | 664 |
| 1440 | 40 | 2 | 2 | 559 | 322 | 237 | 644 | 474 | 152 |
| 1536 | 6 | 2 | 3 | 589 | 358 | 231 | 716 | 462 | 104 |
| 1632 | 3 | 2 | 3 | 793 | 46 | 747 | 92 | 138 | 184 |
| 1680 | 40 | 2 | 2 | 559 | 562 | 3 | 556 | 6 | 568 |
| 1728 | 36 | 2 | 1 | 793 | 142 | 651 | 284 | 426 | 568 |
| 1824 | 48 | 2 | 1 | 769 | 286 | 483 | 572 | 858 | 680 |
| 1920 | 48 | 2 | 1 | 863 | 194 | 669 | 388 | 582 | 776 |
| 2016 | 16 | 2 | 3 | 767 | 482 | 285 | 964 | 570 | 88 |
| 2112 | 16 | 2 | 3 | 671 | 770 | 99 | 572 | 198 | 968 |
| 2208 | 3 | 2 | 3 | 743 | 722 | 21 | 764 | 42 | 680 |
| 2304 | 16 | 2 | 3 | 1055 | 194 | 861 | 388 | 582 | 776 |

-

-

- -

-

-

- -

# -

# -

References (to be added to draft):

- Siaud.I, Ulmer-Moll A.M., "A Novel Adaptive Sup-carrier Interleaving Application to millimetre-wave WPAN OFDM System (IST MAGNET Project)," Conference IEEE Portable 2007, Orlando (USA), March 2007.

- Siaud.I, Ulmer-Moll A.M., "Turbo-like Processing for Scalable Interleaving Pattern Generation: application to 60 GHz UWB-OFDM systems," ICUWB'07, Singapore, September 2007.