IEEE P802.22
Wireless RANs

| Update of the Spectrum Manager policy table (Comments 1023-1030) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |
| Apurva Mody | BAE Systems | Nashua, NH 03054 | 404-819-0314 | apurva.mody@baesystems.com, apurva_mody@yahoo.com |

Abstract

This contribution is proposing an updated version of the Spectrum Manager policy table contained in 22-09-0122-09-0000-802-22-spectrum-manager-policy.xls.  This new version translated the Excel version in a Word version for inclusion in the 802.22 Draft Standard and includes some flowcharts to describe the logic elements.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Update of the Spectrum Manager policy table

| Policy ID | 802.22 component involved | Event trigger | Event description | Action |
| --- | --- | --- | --- | --- |
| 1a | BS | DB | If the SM is directed by the database service that the current operating channel is no longer available for the BS. | Initiate a channel switch of the entire cell to a new operating channel within (Tch_move - 0.5) seconds from the time when database informed the SM. The new operating channel should be the highest priority backup channel. Tch_move is to be found in Annex B for the various Regulatory domains where the WRAN operation takes place. The default value of the Tch_move shall be 2 seconds. |
| 1b | CPE | DB | If the SM is directed by the database service that the current operating channel is no longer available for some of the CPEs. | Option 1: Initiate a channel switch of the entire cell to a new operating channel within (Tch_move - 0.5) seconds from the time when database informed the SM. The new operating channel should be the highest priority backup channel. The default value of the Tch_move shall be 2 seconds. Option 2: Dis-associate the CPEs that are not allowed to operate on the current channel within (Tch_move - 0.5) seconds from the time when the database informed the SM and continue normal operation with the other CPEs. A DREG-CMD (6.10.16), aimed at dropping their association on the current operating channel, shall be sent to these CPEs so that they no longer wait for an allocation in the US-MAP and/or transmit an opportunistic BW request UCS or Ranging request. Further, the BS may signal the affected CPEs to move to a particular channel using the DREG-CMD, in order to re-associate with another BS and continue their operation. The default value of the Tch_move shall be 2 seconds. |
| 1c | BS | DB | If the SM obtains information from an incumbent database that indicates the current operating channel will become unavailable for the BS at a specific time in the future. | Option 1: Initiate a channel switch of the entire cell to a new operating channel no later than (Tch_move - 0.5) seconds after the time specified by the database. The new operating channel should be the highest priority backup channel. The default value of the Tch_move shall be 2 Seconds. Option 2: Dis-associate the CPEs that are not allowed to operate on the current channel within (Tch_move - 0.5) seconds after the time specified by the database and continue normal operation with the other CPEs. A DREG-CMD (6.10.16) aimed at dropping their association on the current operating channel shall be sent to these CPEs so that they no longer wait for an allocation in the US-MAP and/or transmit an opportunistic BW request UCS or Ranging request. Further, the BS may signal the affected CPEs to move to a particular channel using the DREG-CMD, in order to re-associate with another BS and continue their operation. The default value of the Tch_move shall be 2 Seconds. |
| 1d | CPE | DB | If the SM obtains information from an incumbent database that indicates the current operating channel will become unavailable for some of the CPEs at a specific time in the future. | Option 1: Schedule a channel switch for the entire cell to a new operating channel at least 0.5 seconds before the expected time (as obtained from the incumbent database) at which its current channel will become unavailable. The new operating channel should be the highest priority backup channel. Option 2: Schedule a dis-association of the CPEs that are not allowed to operate on the current channel at least 0.5 seconds before the expected time at which the current channel will become unavailable for these CPEs and continue normal operation with the other CPEs. A DREG-CMD (6.10.16) aimed at dropping their association on the current operating channel shall be sent to these CPEs so that they no longer wait for an allocation in the US-MAP and/or transmit an opportunistic BW request UCS or Ranging request. Further, the BS may signal the affected CPEs to move to a particular channel using the DREG-CMD, in order to re-associate with another BS and continue their operation. The default value of the Tch_move shall be 2 Seconds. |
| 1e | BS | DB | Where a database service exists for the regulatory domain of operation, if such a service becomes unavailable for greater than TDBavail specified in Annex B for that domain (default value for TDBavail shall be 1 hour). | The BS shall de-register its associated CPEs and terminate its own operation. |
| 2 | BS/CPE | Signal detected | If the signal detected on the operating channel or either of its first adjacent channels is a TV signal through the BS spectrum sensing function or through a combination of sensing results from multiple CPEs. 1 Detection threshold is to be found in Annex B for the various Regulatory domains where the WRAN operation takes place. | Note the information about the detection of TV signal and make it available according to the local regulatory requirements. No Yes Initiate a channel switch of the entire cell to a new operating channel within (Tch_move - 0.5) seconds from the time when the TV signal was detected. The new operating channel should be the highest priority backup channel . The value for Tch_move is to be found in Annex B for the various Regulatory domains where the WRAN operation takes place. The default value of the Tch_move shall be 2 seconds. Does the local regulatory domain require to vacate the channel on confirmation of the presence of a TV signal? No further action |
| 3a | BS/CPE | Signal detected | If the signal detected on the operating channel is a wireless microphone signal through the BS spectrum sensing function or through the sensing results from a CPE or a combination of multiple CPEs.2 1 Detection threshold is to be found in Annex B for the various Regulatory domains where the WRAN operation takes place. 2 The following action should apply to any of the nodes (BS or CPE) that detected the wireless microphone signal | Note the information about the detection of wireless microphone signal and make it available according to the local regulatory requirements. Initiate a channel switch of the entire cell to a new operating channel within (Tch_move_wm - 0.5) seconds from the time when the wireless microphone signal was detected. The new operating channel should be the highest priority backup channel. The default value of the Tch_move_wm is to be found in Annex B for the various Regulatory domains where the WRAN operation takes place. The default value for the timer Tch_move_wm shall be 2 seconds. Dis-associate the CPEs that are within the protected radius of the wireless microphone operation (MPR) within (Tch_move_wm - 0.5) seconds from the time when the wireless microphone signal was detected and continue normal operation with the other CPEs. A formal MAC message shall be sent to these CPEs before dropping their association so that the CPEs no longer wait for an allocation in the US-MAP and/or transmit an opportunistic BW request, UCS or Ranging request. Optionally, the BS may signal the next channel to go to for the dis-associated CPEs before shutting down the communication. Note: the variable MPR will be defined in Annex B for the various Regulatory domains where the WRAN operation takes place. The default value for MPR shall be 4 km. |
| 3b | BS/CPE | Signal detected | If the signal detected on the operating channel is a 802.22.1 wireless microphone beacon signal .1 1 The following action should apply to any of the nodes (BS or CPE) that detected the 802.22.1 wireless microphone beacon signal. Note: the variable MPR (Microphone_Protected_Radius) will be defined in Annex B for the various Regulatory domains where the WRAN operation takes place. The default value for MPR shall be 4 km. | Note the information about the detection of wireless microphone beacon signal and make it available according to the local regulatory requirements. Complete cell move Option I (No Authentication): BS skips the beacon authentication step and initiates the switching of the entire cell to a new operating channel within (Tch_move - 0.5) seconds of the time when the beacon was detected. The new operating channel shall be the highest priority backup channel. Option II (Authentication): A (Tch_move - 0.5) second timer is set. BS shall authenticate the beacon by scheduling quiet periods within (Tch_move - 0.5) seconds, ask the CPEs to capture the required portion of the 802.22.1 payload and decode the MSF1, MSF2, and/or MSF3 fields as found sufficient by the operator. If the 802.22.1 beacon is found to be authentic or (Tch_move - 0.5) seconds timer expires, then the BS shall initiate the switching of the entire cell to a new operating channel which should be the highest priority backup channel. If the beacon is found to be non-authentic, no action is taken. The default value of Tch_move shall be 2 seconds. Specific CPEs cease to operate or move Option I (No Authentication) : BS skips the beacon authentication step and dis-associates the CPEs that are within less than MPR from the wireless microphone operation within (Tch_move - 0.5) seconds from the time when the 802.22.1 signal was detected and continue normal operation with the other CPEs. A DREG-CMD MAC message shall be sent to these CPEs before dropping their association so that the CPEs no longer wait for an allocation in the US-MAP and/or transmit an opportunistic BW request, UCS or Ranging request. Optionally, the BS may signal the next channel to go to for the dis-associated CPEs in the DREG-CMD message before shutting down the communication. The default value of Tch_move shall be 2 seconds. Specific CPEs cease to operate or move Option I (No Authentication) : BS skips the beacon authentication step and dis-associates the CPEs that are within less than MPR from the wireless microphone operation within (Tch_move - 0.5) seconds from the time when the 802.22.1 signal was detected and continue normal operation with the other CPEs. A DREG-CMD MAC message shall be sent to these CPEs before dropping their association so that the CPEs no longer wait for an allocation in the US-MAP and/or transmit an opportunistic BW request, UCS or Ranging request. Optionally, the BS may signal the next channel to go to for the dis-associated CPEs in the DREG-CMD message before shutting down the communication. The default value of Tch_move shall be 2 seconds. Option II (Authentication): A (Tch_move - 0.5) second timer is set. BS shall authenticate the beacon by scheduling quiet periods within (Tch_move - 0.5) seconds, ask the CPEs to capture the required portion of the 802.22.1 payload and decode the MSF1, MSF2, and/or MSF3 fields as found sufficient by the operator. If the 802.22.1 beacon is found to be authentic or (Tch_move - 0.5) seconds timer expires then the BS shall dis-associate the CPEs that are within MPR from the wireless microphone operation within (Tch_move - 0.5) seconds from the time when the 802.22.1 signal was detected and continue normal operation with the other CPEs. A DREG-CMD MAC message shall be sent to these CPEs before dropping their association so that the CPEs no longer wait for an allocation in the US-MAP and/or transmit an opportunistic BW request, UCS or Ranging request. Optionally, the BS may signal the next channel to go to for the dis-associated CPEs in the DREG-CMD message before shutting down the communication. If the beacon is found to be non-authentic, no action is taken. The default value of Tch_move shall be 2 seconds. |
|  |  |  |  | Option II (Authentication): A (Tch_move - 0.5) second timer is set. BS shall authenticate the beacon by scheduling quiet periods within (Tch_move - 0.5) seconds, ask the CPEs to capture the required portion of the 802.22.1 payload and decode the MSF1, MSF2, and/or MSF3 fields as found sufficient by the operator. If the 802.22.1 beacon is found to be authentic or (Tch_move - 0.5) seconds timer expires then the BS shall dis-associate the CPEs that are within MPR from the wireless microphone operation within (Tch_move - 0.5) seconds from the time when the 802.22.1 signal was detected and continue normal operation with the other CPEs. A DREG-CMD MAC message shall be sent to these CPEs before dropping their association so that the CPEs no longer wait for an allocation in the US-MAP and/or transmit an opportunistic BW request, UCS or Ranging request. Optionally, the BS may signal the next channel to go to for the dis-associated CPEs in the DREG-CMD message before shutting down the communication. If the beacon is found to be non-authentic, no action is taken. The default value of Tch_move shall be 2 seconds. |
| 4 | BS/CPE | DB or Signal detected | If the database indicates that the current operating channel is not available or the signal detected on the operating channel is a wireless microphone or an 802.22.1 signal, or, in case of a TV signal, the signal is detected on the operating or any of its first adjacent channels and there is no backup channel available. | Terminate the operation of the entire cell in the current operating channel within Tch_move seconds. The default value of Tch_move shall be 2 seconds. |
| 5 | CPE | Signal detected | If the signal detected on the operating channel is a wireless microphone or an 802.22.1 signal, or, in case of a TV signal, the signal is detected on the operating or any of its first adjacent channels before the CPE has registered with a BS on the same channel. | The CPE shall not transmit on that channel. |
| 6 | CPE | Signal detected | If the signal detected on the operating channel is a wireless microphone or an 802.22.1 signal, or, in case of a TV signal, the signal is detected on the operating or any of its first adjacent channels and the CPE has already registered with the BS. | The CPE should send a UCS to alert the BS of the presence of the detected signal in its coverage even if it did not plan to transmit because of the potential interference from the downstream transmission. |
| 7 | BS/CPE | DB or sensing | If the spectrum manager confirms the presence of any other device which is granted regulatory protection. | Comply to local regulations.] |