IEEE P802.22
Wireless RANs

| Antenna Primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90035 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

In this document antenna primitives are proposed to accommodate the communication of antenna gain information between the MAC and the antenna.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Wendong Hu> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

    - Antenna Primitives

Essential antenna information is provided to the MAC by the antenna through the M-SAP. The M-SAP is an interface that provides a means of exchange information between the SM at the BS MAC and the SSA at the CPE MAC and their respective antenna. Table 298 summarizes the primitives supported by the MAC to access antenna information through the M-SAP interface. The primitives are discussed in the subclauses referenced in the table.

- — Antenna Primitives supported by the M-SAP

| Name | Request | Indication | Confirm | Response |
| --- | --- | --- | --- | --- |
| M-ANTENNA-INTEGRATED | 9.7.6.1 |  | 9.7.6.2 |  |
| M-ANTENNA-INFORMATION | Error: Reference source not found |  |  | Error: Reference source not found |

      - M-ANTENNA-INTEGRATED.request

The M-ANTENNA-INTEGRATED.request primitive allows the MAC to identify whether the device’s antenna is integrated or non-integrated through the M-SAP in order to know whether it is required to get antenna gain information for calculation of EIRP. The M-ANTENNA-INTEGRATED.request primitive has no attributes.

        - When generated

The M-ANTENNA-INTEGRATED.request primitive shall be generated by the MAC and issued to its antenna to identify the whether its antenna is integrated or non-integrated.

        - Effect on receipt

When the antenna receives the M-ANTENNA-INTEGRATED.request primitive, the antenna shall generate an M-ANTENNA-INTEGRATED.confirm primitive to indicate whether the antenna is integrated or non-integrated.

      - M-ANTENNA-INTEGRATED.confirm

The M-ANTENNA-INTEGRATED.confirm primitive allows the antenna to inform the MAC whether it is integrated or non-integrated through the M-SAP. Table 29999 specifies the parameters for the M-ANTENNA-INTEGRATED.confirm primitive.

- — M-ANTENNA-INTEGRATED.confirm parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Antenna Type | Integer | 0-1 | The value indicates whether the antenna is integrated or non-integrated. 0 = integrated antenna 1= non-integrated antenna |

        - When generated

The M-ANTENNA-INTEGRATED.confirm primitive shall be generated by the antenna and issued to its MAC when an M-ANTENNA-INTEGRATED.request primitive is received to indicate whether the antenna is integrated or non-integrated through the M-SAP.

        - Effect on receipt

When the MAC receives the M-ANTENNA-INTEGRATED.confirm primitive, the SM at the BS and the SSA at the CPE shall identify whether the antenna is integrated or non-integrated.

      - M-ANTENNA-INFORMATION.request

The M-ANTENNA-INFORMATION.request primitive allows the MAC to request antenna information from the antenna. The M-ANTENNA-INFORMATION.request primitive has no attributes.

        - When generated

The M-ANTENN-INFORMATION.request primitive shall be generated by the SM of a BS or the SSA of the CPE and issued to their respective antenna for antenna information.

        - Effect on receipt

When the antenna receives the M-ANTENNA-INFORMATION.request primitive, the antenna shall generate an M-ANTENNA-INFORMATION.response containing information that describes the attributes of the antenna.

      - M-ANTENNA-INFORMATION.response

The M-ANTENNA-INFORMATION.response primitive is used to respond to the MAC request with antenna information. Table 300300 specifies the parameters for the M-ANTENNA-INFORMATION.response primitive.

- — M-ANTENNA-INFORMATION.repsonse parameters

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| For (i=1 to Number of Channels, i++) { Channel_Number Maximum Gain } | List channels and max gain per channel |  | List Channel Numbers and corresponding maximum gain (dBi). |

        - When generated

The M-ANTENNA-INFORMATION.response primitive shall be generated by the antenna and issued to the MAC to respond with information about the antenna.

        - Effect on receipt

When the MAC receives the M-ANTENNA-INFORMATION.response, MAC shall store the maximum gain (dBi) for each channel so that the device can convert from transmit power to EIRP.

References: