IEEE P802.22
Wireless RANs

| RF and FFT Sample Clock Tolerances |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2009-11-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Stephen Kuffner | Motorola, Inc | Schaumburg, IL | 847-538-4158 | stephen.kuffner@ motorola.com |

Abstract

Assuming independent references for the RF and FFT sample clock frequencies for the sake of not constraining the implementation, the tolerances are derived to keep the maximum subcarrier error within limits defined to maintain a specified C/I degradation due to ICI.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The RF frequency of the base station is some multiple M of the reference frequency fref.  Riding on this frequency is an OFDM modulation that has maximum and minimum subcarrier frequencies defined by ±K/N fref.  The errors at the end subcarriers have to be less than a multiple  (e.g., 0.02) of the subcarrier spacing, fref / N.  The transmit frequencies can be written as follows:

M fref+KN fref=ftx,maxM fref−KN fref=ftx,min	(1)

At the receiver, there is an error α in the RF frequency and an error  in the FFT clock, resulting in

M fref(1+α)+KN fref(1+β)=frx,maxM fref(1+α)−KN fref(1+β)=frx,min	(2)

The absolute difference between ftx,max and frx,max has to be less than  fref/N.  Likewise, the absolute difference between ftx,min and frx,min has to be less than  fref/N.  These absolute differences can be expressed as

|αMfref±βKNfref| < δ frefN|αMN±βK| < δ	(3)

This equation is satisfied by ordered pairs (α, ) that fall within the boundaries of a rhombus defined by the four lines

β<(δ−α MN)Kβ>−(δ+α MN)Kβ>(α MN−δ)Kβ<(δ+α MN)K	(4)

Example:

fref = 6 MHz x 8/7 = 6.857142MHz

fRF = 700 MHz (M = 102.08333)

For an N = 2048 FFT, with Kmax = 840 and  = 0.02

|αMN±βK| < δ|209,066.666 α±840β| < 0.02

The rhombus is bounded by | | < .02/840 =23.809523ppm and |α | < .02/840/248.888 = .09566 ppm.

The boundaries are shown graphically below in Figure 1.  Any ordered pair that falls within these boundaries should satisfy the worst case subcarrier offset constraints.



α

0.09566 ppm

0.09566 ppm

23.809523 ppm

23.809523 ppm

Figure 1	Boundaries of frequency tolerances for CPE RF (α) and FFT sampling clock ().

References: