IEEE P802.22
Wireless RANs

| Spectrum Sensing Automaton (Comments 1098-1116) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This contribution proposes resolutions for comments #1098 up to #1116 of the comment database 22-09-0120r22 on the Spectrum Sensing Automaton.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Resolution of comments related to the Spectrum Sensing Automaton

Introduction

A number of comments were made related to section 9.3 of the 802.22 Draft v2.0 which deals with the Spectrum Sensing Automaton.  What follows is an attempt at resolving these comments.

Proposed comments resolutions

TR comment #1098:

| Do we have a message that defines the configuration of the Spectrum Automaton and the sensing at the CPEs? How is the SSF at the CPE supposed to know how and when to sense for how much time, how often and on which channels. Also, which signals should the SSF be looking for? This depends on the regulatory domains | Define these messages in case they do not exist. | This process is described in section 9.3.3. Beyond section 9.3.3, there is a need to provide a description of what needs to be done when the CPE receives a MAC message to carry out sensing, and how and what to report as results back to the BS. This may or may not require additional text to section 9.3.3 or another section. The BLM-REQ and BLM-RSP MAC messages are defined in Section 6 for this purpose but need to be supplemented with a flow diagram and text to show how this will be executed at the CPE. Action: Gerald to develop the text and the flow diagram to describe the process taking place at the CPE. |
| --- | --- | --- |

It is not clear that the spectrum automaton needs to be configured by the BS.  Each CPE will be initiated with the right pre-set regulation domain as specified in the Regulatory Domain Classes Annex and this should provide the necessary information about the frequency bands of interest, the frequency for each channel and any specific requirement for the automaton.

For all sensing tasks requested by the BS, all the information on when, how much time, how often and on which channels to sense will be provided to the automaton through the BLM-REQ MAC message (see section 6.10.22.1) and the information to be sent back to the BS is defined in the BLM-RSP MAC message in section 6.10.22.3.  All the input and output parameters for the sensing requested by the BS are thus defined and the way it will actually take place at the CPE is a matter of implementation which does not need to be standardized.  A flow diagram and text to show how this will be executed at the CPE is therefore not needed.

With respect to the sensing conducted during initialization and idle time, which is not under the control of the BS and thus not defined by MAC messages, the detailed diagrams are provided in sections 9.3.1 and 9.3.3 respectively.  The input of the automated sensing process to be carried out during CPE idle time is the list of backup/candidate channels provided by the DCD message sent by the BS (see Table 33).

The output of the automated idle time sensing process at the CPE should be in the form of two unsolicited MAC messages sent from the CPE and two solicited MAC messages sent in response to a BS query.  The two unsolicited MAC messages are intended to signal either that a backup channel is no longer available (see proposed BLM-RSP IE message “Unavailable Backup Channel” listed in Table 163 and described in new Table 169) or that the sensing scheme did not have time to clear all the backup channels (see proposed BLM-RSP IE message “Backup channel list clearance depth” listed in Table 163 and described in new Table 170).  These unsolicited messages are to be carried by a normal upstream bandwidth allocation assigned to the CPE in the US-MAP or, if not present, using the opportunistic BW Request mechanism to allow reporting to the BS as soon as possible.

The two solicited CPE MAC messages will be responses to either a query from the BS carried by the BLM-REQ message on the depth of the backup/candidate channel list reached by the CPE out-of-band sensing (in view of trying to increase the list of available backup channels) (see the additional BLM-REQ IE message “Backup/candidate channel list clearance depth “ listed in Table 150 and the additional BLM-RSP IE message “Backup/candidate channel list clearance depth “ listed in Table 164 and described in new Table 172), or a query by the BLM-REQ message “Status of CPE out-of-band sensing results” listed in Table 150 for a detailed report from the CPE on the status of its out-of-band sensing on the most recently sensed channels as indicated in Table 157.  The response to this query is listed in Table 175 and corresponds to the parameters presented in Table 256 of section 9.3.4.

TR comment 1101:

Proposed wording change to the first sentence of section 9.3.3: “In addition to being able to carry out the in-band sensing process and any higher priority sensing requests coming from under the control of the base station through the specific MAC messages described in 6.10.22.1, …”

TR comment 1102:

See resolution of comment #1098.

TR comment 1103:

See footnote 16 on page 393 that deals with the sensing time for two tuners.  No change required to the text.

TR comment 1104:

See resolution of comment #1098.

TR comment 1111:

See resolution of comment #1098. The RSSI value is reported by the Signal Specific Measurement Report described in section 6.10.22.3.1.1.

TR comment 1112:

SINR is only found twice in the definition and acronym sections.  This would be a new parameter to measure and report on.  SNR is used for adjusting the modulation level at the BS.  Is it really needed at the CPE?  Modulation and coding format are known at the BS since it controls it through the UIUC.  Consideration of the effect of impulse noise on the sensing results is not clear.  Multiple measurements would be needed and statistics would need to be done.  BLM message provides for such measurements.

TR comment 1113:

See resolution of comment #1098.

TR comment 1114:

The RSSI value is reported by the Signal Specific Measurement Report described in section 6.10.22.3.1.1.  SINR is only found twice in the definition and acronym sections.  CINR is found 6 times and is used to adjust the DIUC (for DS modulation and coding level) by the DBPC-REQ or RNG-REQ message by the BS.  The actual value is not sent to the BS.  It is not really useful to quantify RF signal levels to avoid interference.  This would be a new parameter to measure and report on.  SNR is used for adjusting the modulation level for the uplink at the BS.  Modulation and coding format are known at the BS since it controls it through the UIUC, thus no need to ask the CPE for it.  Should operating channel CINR and RSSI (instantaneous and average) at the CPE be added to the BLM-REQ message?

TR comment 1115:

See resolution of comment #1114.

TR comment 1116:

See description of the BLM-REQ and BLM-RSP MAC messages which completely describe the input and output of the sensing requests and responses from and to the BS.  The actual mechanism to implement sensing measurements as requested by the BS through MAC messages should be implementation specific and does not need to be standardized.

Proposed changes to sections 6.10.22.1 (BLM-REQ), 6.10.22.3 (BLM-RSP) and 9.3.3 of the Draft:

_________________________________