IEEE P802.22
Wireless RANs

| Proposed Amendments to Annex A and Annex B |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | P.O. Box 868, MER 15-2350, Nashua, NH 03061-0868 | 603-885-2621 404-819-0314 | apurva.mody@baesystems.com, apurva_mody@yahoo.com |
| Ranga Reddy | US Army (CERDEC) | Ft Monmouth, NJ | - | Ranga.reddy@us.army.mil |
| Gerald Chouinard | CRC | 3701 Carling Avenue, Ottawa, Ontario, Canada K2H 8S2 | 613-998-2500 | Gerald.chouinard@crc.ca |
| Victor Tawil | MSTV | 4100 Wisconsin Ave. NW Washington, DC | 202-360-8941 | Vtawil@mstv.org |

Abstract

This document proposes modifications to the new Annex A of Draft 3.0 that combines the previous version of Annex A and B in an attempt to address the various comments submitted in the 802.22 Draft v3.0.as well as inputs suggested in document dcn/10/22-10-0050-00-0000-towards-resolution-of-comments-4-and-1203-to-802-22-draft-v2-0.docand other proposed changes suggested during the comment resolution process.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Annex A

## 802.22 Regulatory Domain and Regulatory Classes Requirements

## This Annex describes the various technical parameters and specifications required  by the various regulatory domans for operation of the 802.22 standard in the TV bands.

## A. 1 Regulatory Domains and Regulatory Classes

Table 290 specifies the regulatory domains and licensing regime where the 802.22 systems are planned to be authorized to operate in the TV bands.

Table 290—Regulatory Domains

| Geographic area | Regulatory Domain - IETF Country Code (2 Bytes) | Licensing Regime | Approval authority |
| --- | --- | --- | --- |
| United States | US | unlicensed | FCC |
| Canada | CA | licensed | IC |
| United Kingdom | GB |  | OFCOM |

Table 291 specifies the authorized regulatory classes under their respective regulatory domains

Table291— Regulatory Classes

| Regulatory Domain | Regulatory class and Profile |  |
| --- | --- | --- |
|  | Fixed | Personal Portable |
| US | Stationary fixed | . Mode II * |
| CA | Stationary fixed |  |
| GB |  |  |

*The behavioral limits sets for Modes I and II are defined in the FCC Report and Order.  However, 802.22 will only operate in portable nomadic Mode 1I.

## A.2  Radio Performance Requirements

### A.2.1 Transmit Power Limits and EIRP Requirement

Table 292 specifies the transmitted EIRP levels authorized by the various regulatory domains for the different regulatory classes described in Table 291

Table 292 Transmit Power Level by Regulatory Domain and Classes

| Regulatory Domain | Regulatory Class | Maximum BS EIRP | Maximum CPE EIRP |
| --- | --- | --- | --- |
| US | . stationary fixed | 4 W | 4 W |
| US | . Portable (Modes II) | 100 mW | 100 mW |
| CA | Stationary fixed | 500 W | 4 W |

### A.2.2 Transmit Spectrum Mask Requirements

Table 293 specifies the transmit spectrum mask requirements authorized by the various regulatory domains for the different regulatory classes described in Table 291. Table 293 refers to the appropriate Figures xxx and xxx which illustrate the various RF Masks applicable to the 802.22 systems.  The power spectrum density (PSD) measurement shall be done over a measurement bandwidth of 100 kHz and a video bandwidth of 100 kHz with an average detector.

Table 293—Transmit Spectrum Mask Requirements

| Regulatory Domain | Regulatory Class | Transmit Spectrum Mask |  |
| --- | --- | --- | --- |
| US | Stationary fixed | See Figure 186. First adjacent channel Beyond the outer edge of the first adjacent channel | 55 dB below the highest power in a 6 MHz operating channel in 100 kHz bandwidth Comply with FCC section 15.209(a) |
| US | Portable ( Mode II) | See Figure 186. First adjacent channel Beyond the outer edge of the first adjacent channel | 55 dB below the highest power in a 6 MHz operating channel in 100 kHz bandwidth Comply with FCC section 15.209(a) |
| CA | Stationary fixed (Δf is referenced to the edge of the operating channel, Measurement bandwidth is 100 kHz, levels are expressed in dBc, relative to the total power in the operating channel) | See Figure 187. 0.05 ≤ Δf ≤ 6 6 ≤ Δf ≤ 12 12 ≤ Δf ≤ 18 Δf > 18 AND within 54-72 MHz, 76-88 MHz, 174-216 MHz, 470-608 MHz and 614-698 MHz Outside the above cases | 44.9 +1.1*( Δf)1.6 37.8 + 4.4* Δf 70.2 + 1.7* Δf 100.8 BS: 43+10*log(P in Watts) CPE: Comply with RSS 210 Table 2 |
| UK |  |  |  |

Figure 186   WRAN 802.22 transmission RF Mask to be used in the USA

Figure 187   WRAN 802.22 transmission RF Mask to be used in Canada

Table 294 Unavailable channels inside the protected contour of a DTV station operating on channel “N”

| Regulatory Domain | Regulatory Class | Unavailable channels |
| --- | --- | --- |
| US | (Stationary fixed) | N N±1 |
| US | (Portable nomadic Mode II) | N N±1 |
| CA | Stationary fixed | N N±1 N±2 |
| UK |  |  |

Table294— Medium Access Requirements

## Table 295— Device Identification Requirements

Table 296— Signal Type Array

- Table 296— Spectrum Sensing Requirements for the Regulatory Domain and Classes

- Table 297— Spectrum Sensing Requirements for the Regulatory Domain and Classes

- A.3 Channel Move Timing Requirements

Table 295  identifies channel move timing specifications authorized or recommended by the 802.22 standard for various requlatory domains

- Table 295— Channel Move Timing Specifications

| Regulatory Domain | Regulatory Class | Channel Move Time once incumbent signal is detected (seconds) |
| --- | --- | --- |
| US | stationary fixed | 2 |
| US | Portable (Mode II) | 2 |

## A.44Database Access Requirements

Table 296 speficies the frequency of accessing an incumbent data base for the different regulatory classes of devices for the various regulatory domains

- Table 296— Database Access Requirements

| Regulatory Domain | Regulatory Class | Maximum Time without Database Refresh (hours) |
| --- | --- | --- |
| US | (stationay fixed and nomadic) | 24 hours |
| US | Portable( Mode 1I) | 24 hours |
| GB |  |  |

## A.6 Channelization Based on the Regulatory Domain

The system frequencies are the frequencies at which the transmitter and the receiver equipment operates.  These frequencies and their utilization shall conform to the Radio Regulations of the International Telecommunication Union (ITU) as well as the various Regional ITU Agreements as appropriate. The following Tables list the frequencies corresponding to the TV channels used for WRAN operation in various regulatory domains.

- Table  300— Frequency of TV Channels in the ‘US’ Regulatory Domain
(Note that channels 37 and 52 and above are not available in the USA.)

| Country | Regulatory Class (2 Bytes) | Channel Number (1 Byte) | Center Frequency | Max. Bandwidth (MHz) |
| --- | --- | --- | --- | --- |
| USA | US | 2 | 57 | 6 |
| USA | US | 3 | 63 | 6 |
| USA | US | 4 | 69 | 6 |
| USA | US | 5 | 79 | 6 |
| USA | US | 6 | 85 | 6 |
| USA | US | 7 | 177 | 6 |
| USA | US | 8 | 183 | 6 |
| USA | US | 9 | 189 | 6 |
| USA | US | 10 | 195 | 6 |
| USA | US | 11 | 201 | 6 |
| USA | US | 12 | 207 | 6 |
| USA | US | 13 | 213 | 6 |
| USA | US | 14 | 473 | 6 |
| USA | US | 15 | 479 | 6 |
| USA | US | 16 | 485 | 6 |
| USA | US | 17 | 491 | 6 |
| USA | US | 18 | 497 | 6 |
| USA | US | 19 | 503 | 6 |
| USA | US | 20 | 509 | 6 |
| USA | US | 21 | 515 | 6 |
| USA | US | 22 | 521 | 6 |
| USA | US | 23 | 527 | 6 |
| USA | US | 24 | 533 | 6 |
| USA | US | 25 | 539 | 6 |
| USA | US | 26 | 545 | 6 |
| USA | US | 27 | 551 | 6 |
| USA | US | 28 | 557 | 6 |
| USA | US | 29 | 563 | 6 |
| USA | US | 30 | 569 | 6 |
| USA | US | 31 | 575 | 6 |
| USA | US | 32 | 581 | 6 |
| USA | US | 33 | 587 | 6 |
| USA | US | 34 | 593 | 6 |
| USA | US | 35 | 599 | 6 |
| USA | US | 36 | 605 | 6 |
| USA | US | 37 | 611 | 6 |
| USA | US | 38 | 617 | 6 |
| USA | US | 39 | 623 | 6 |
| USA | US | 40 | 629 | 6 |
| USA | US | 41 | 635 | 6 |
| USA | US | 42 | 641 | 6 |
| USA | US | 43 | 647 | 6 |
| USA | US | 44 | 653 | 6 |
| USA | US | 45 | 659 | 6 |
| USA | US | 46 | 665 | 6 |
| USA | US | 47 | 671 | 6 |
| USA | US | 48 | 677 | 6 |
| USA | US | 49 | 683 | 6 |
| USA | US | 50 | 689 | 6 |
| USA | US | 51 | 695 | 6 |
| USA | US | 52 | 701 | 6 |
| USA | US | 53 | 707 | 6 |
| USA | US | 54 | 713 | 6 |
| USA | US | 55 | 719 | 6 |
| USA | US | 56 | 725 | 6 |
| USA | US | 57 | 731 | 6 |
| USA | US | 58 | 737 | 6 |
| USA | US | 59 | 743 | 6 |
| USA | US | 60 | 749 | 6 |
| USA | US | 61 | 755 | 6 |
| USA | US | 62 | 761 | 6 |
| USA | US | 63 | 767 | 6 |
| USA | US | 64 | 773 | 6 |
| USA | US | 65 | 779 | 6 |
| USA | US | 66 | 785 | 6 |
| USA | US | 67 | 791 | 6 |
| USA | US | 68 | 797 | 6 |
| USA | US | 69 | 803 | 6 |

- Table 301— Frequency of TV channels in Western Europe and many other countries in Africa, Asia and the Pacific (BW= 7 MHz) (System B)

| Channel | Center Frequency | Channel | Center Frequency | Channel | Center Frequency |
| --- | --- | --- | --- | --- | --- |
| 2 | 50.5 | 6 | 184.5 | 10 | 212.5 |
| 3 | 57.5 | 7 | 191.5 | 11 | 219.5 |
| 4 | 64.5 | 8 | 198.5 | 12 | 226.5 |
| 5 | 177.5 | 9 | 205.5 |  |  |

- Table 302— Frequency of TV channels in Western Europe and many other countries in Africa, Asia and the Pacific (BW= 8 MHz) (System H)

| Channel | Center Frequency | Channel | Center Frequency | Channel | Center Frequency |
| --- | --- | --- | --- | --- | --- |
| 21 | 474 | 38 | 610 | 55 | 746 |
| 22 | 482 | 39 | 618 | 56 | 754 |
| 23 | 490 | 40 | 626 | 57 | 762 |
| 24 | 498 | 41 | 634 | 58 | 770 |
| 25 | 506 | 42 | 642 | 59 | 778 |
| 26 | 514 | 43 | 650 | 60 | 786 |
| 27 | 522 | 44 | 658 | 61 | 794 |
| 28 | 530 | 45 | 666 | 62 | 802 |
| 29 | 538 | 46 | 674 | 63 | 810 |
| 30 | 546 | 47 | 682 | 64 | 818 |
| 31 | 554 | 48 | 690 | 65 | 826 |
| 32 | 562 | 49 | 698 | 66 | 834 |
| 33 | 570 | 50 | 706 | 67 | 842 |
| 34 | 578 | 51 | 714 | 68 | 850 |
| 35 | 586 | 52 | 722 | 69 | 858 |
| 36 | 594 | 53 | 730 |  |  |
| 37 | 602 | 54 | 738 |  |  |