IEEE P802.22
Wireless RANs

| Suggested Changes for Maximum Available EIRP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-03-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Byung-Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |
| Gerald Chouinard | CRC | Canada | +1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

To verify whether or not the maximum available EIRP should be different for the modulation and the number of used subchannel, we evaluate the distribution of peak to average power ratio (PAPR) for different modulation order and burst size. As a resolution of comments ID #378 and #933 for draft v2.0, this document includes the suggested changes for maximum available EIRP in Section 6.10.15.3.3.1 and 8.10.3.2.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related comments

As the resolution of comments ID #378 and #933 for draft v2.0, I submit the suggested changes for the maximum available EIRP in Section 6.10.15.3.3.1 and 8.10.3.2. The relating comment is as follows:

The maximum allowable EIRP shall be determined by the regulatory requirements. In the upstream direction, however, the maximum EIRP can be restricted to prevent the BER performance degradation due to high peak to average power ratio (PAPR). A high PAPR causes the distortion in nonlinear high power amplifier (HPA) and increases the quantization noise in DAC and ADC. The PAPR is given by

where Pavg is the average power of the OFDM symbol, and Ppeak is the maximum power of the OFDM symbol.

To investigate whether or not the maximum available EIRP should be different depending on the transmission parameters of the upstream, such as modulation scheme and number of used subchannel, we evaluate the effect of PAPR under the 802.22 OFDM parameters. The effect of PAPR is evaluated using the cumulative distribution function (CDF) which is the probability that the PAPR is below some threshold level. The theoretical derivation of the CDF in OFDM system is given by

where N is the FFT size. The theoretical CDF with a=2.8 is quite accurate for N>64 [1]. The simulated CDF is plotted against theoretical values in Figure 1 for 2048-FFT and QPSK. From Figure 1, it can be seen that the simulated CDF with 4-times oversampling is almost same as the theoretical CDF as well as the simulated CDF of continuous-like time signal with 64-times oversampling. Thus we use the oversampling by a factor of 4 for all computation of PAPR. Oversampling can be performed by padding IFFT sequence data with zeros.

Figure 1 – CDF of the PAPR for 2048-FFT

Figure 2 shows the distribution of the PAPR in the WRAN system. For the random QPSK constellations, most of PAPR is distributed in the range of 8 dB to 11 dB.

Figure 2 – PDF of the PAPR

Figure 3 shows the CDF of the PAPR in the 802.22 WRAN upstream for different modulation scheme and number of used subchannel. From Figure 3, it can be seen that the PAPR is greatly affected by the number of used subchannel for the data burst, instead of the modulation scheme. It means that the maximum available EIRP should be different depending on the number of used subchannel for the data burst, instead of the modulation scheme.

Figure 3 – CDF of the PAPR in the WRAN upstream for a modulation order and a number of used subchannel

Figure 4 shows the CDF of the PAPR in the 802.22 WRAN upstream for a number of used subchannel. The data subcarriers are modulated by 64-QAM, and the number of used subchannel has a range of 1 to 60, which is allocated for the data burst. As the number of used subchannel increases, the CDF of the PAPR becomes worse rapidly.

Figure 4 – CDF of the PAPR in the WRAN upstream for a number of used subchannel

In the draft, there are several information elements which are used to adjust the maximum transmit EIRP, such as, in Section 6.10.4.1.2.2 CPE Maximum Transmit EIRP IE and 6.10.15.3.3.1 Maximum Transmit EIRP. Among them, Maximum Transmit EIRP IE already contains the 3 Bytes to adjust the maximum transmit EIRP depending on the modulation scheme. Thus, we can still adjust the maximum available EIPR by replacing the modulation scheme with a number of used subchannel within the Maximum Transmit EIRP IE.

2. Suggested text

Based on the opinion above, I propose the text changes as follows in RED:

          - Maximum Transmit EIRP

This field indicates the maximum EIRP achievable at the CPE while still meeting the required RF mask (see 8.14) and other performance limits set by the manufacturer for the transmission of all the 60 subchannels. The maximum EIRP parameters are reported in dBm and quantized in 0.5 dB steps ranging from -64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme.

- — Information elements

| Element ID | Length (bytes) | Value | Scope |
| --- | --- | --- | --- |
| 3 | 1 | Maximum EIRP for the transmission of all the 60 subchannels. | CBC-REQ |

      - Transmit power control mechanism

The other part of this section is not changed.

…

The CPE shall report, at registration, the maximum EIRP that it can achieve in the case of the transmission of all the 60 subchannels while still meeting the required RF mask (see 8.14) and other performance limits set by the manufacturer.. The maximum EIRP achievable at the CPE (see 6.10.15.3.3.1) may be reported in the CBC-REQ MAC message (see 6.10.15.1). This parameter may be used by the BS to determine the maximum EIRP density per subcarrier achievable as a function of the number of used subchannels allocated to the CPE for its upstream burst, noting that the PAPR tends to increase with the number of transmitted subcarriers in the burst. This parameter may also be used by the BS for optimal assignment of coding schemes and modulations and also for optimal allocation of subchannels. These algorithms are vendor-specific.

References:

[1] R. Van Nee and R. Prasad, “OFDM for Wireless Multimedia Communications,” Artech House, 2000

[2] F. M. Bui and D. Hatzinakos, “An Interpolation and Resampling Framework for Efficient Reduction of Peak-to-average Power Ratio in OFDM Systems,” PIMRC’07, pp.1-5, 2007.