IEEE P802.22
Wireless RANs

| Proposed Additional MIBs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90035 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

This document contains proposed MIBs related to the BS, CPE, database service, geolocation, antenna, operating channel, and selected WRAN service for addition into Draft 3.0.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Wendong Hu> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

      - wranIfBSID

This MIB object contains the ID granted to the BS by the federal regulatory body when it was certified for authorized use in the TV bands.  This MIB object is readable by all users.  It is not writeable.

      - wranIfBSSerialNumber

This MIB object contains the serial number of the BS provided to it by its manufacturer.  This MIB object is readable by all users.  It is not writeable.

      - wranIfBSResponsibleParty

This MIB object is an array that contains the contact information for the responsible party for the BS.  Each of these MIBS are readable by all users and are writeable only by the responsible party.

        - wranIfBSResponsiblePartyName

This MIB object represents the name of the responsible party for the BS.

        - wranIfBSResponsiblePartyPhysicalAddress

This MIB object represents the physical address of the responsible party for the BS.

        - wranIfBSResponsiblePartyEmailAddress

This MIB object represents the email address of the responsible party for the BS.

        - wranIfBSResponsiblePartyTelephoneNumber

This MIB object represents the telephone number of the responsible party for the BS.

      - wranIfCPEListOfAvailableWRANServices

This MIB object represents a list that contains the available WRAN services.  This MIB is readable by all users.  The CPE autonomously generates this list using spectrum sensing to discover the available WRAN services in an area.

      - wranIfCPESelectedWRANServiceTVChannel

This MIB object contains the TV channel number for the selected WRAN service.  This MIB object is readable and writeable by all users.  However, only a TV channel from the list of available WRAN services may be entered as a value.  Any other TV channel not on the list of available WRAN services shall be prohibited.

      - wranIfDatabaseServiceBSMib

This MIB group is existent only at the BS and deals with objects related to the configuration of access to, as well as the interaction with the Database Service.

      - wranIfDatabaseServiceBSNetworkAddress

This MIB object represents the network address of the approved database service which provides the BS its list of available TV channels.  The default network address is loaded at the time of initial power on.  This MIB object is readable by all users and writeable by only authorized administrators.

      - wranIfDatabaseServiceBSPortNumber

This MIB object represents the port number to be used to access the approved database service which provides the BS its list of available TV channels.  The default network address is loaded at the time of initial power on.  This MIB object is readable by all users and writeable by only authorized administrators.

      - wranIfDatabaseServiceBSListOfAvailableTVChannels

This MIB object represents a list that stores the list of available TV channels on which the WRAN network may operate.  The WRAN network is prohibited from operating on any TV channel not on this list.  The list is readable by all users and writeable by only direct access of an authorized database service.

      - wranIfGeolocationBS

This MIB object contains the location NMEA data string.  This MIB object is readable by all users and writeable only by the BS satellite-based geolocation module.

      - wranIfGeolocationCPE

This MIB object contains the location NMEA data string.  This MIB object is readable by all users and writeable only by the CPE satellite-based geolocation module.

      - wranIfGeolocationBSCPE

This MIB object, located at the BS, contains the location NMEA data string for the CPE for which the current or most recent database query is being or has been performed.  This MIB object is readable by all users and is writeable by the BS SM after parsing the MAC message provided by the CPE.

      - wranIfTXAntennaInformationBS

This MIB object, located at the BS, is an array that contains information about the transmit/receive antenna that is connected to the BS.  These related MIB objects are readable by all users and writeable only by the antenna.

        - wranIfTXAntennaInformationBSGain

This MIB object is an array that contains the maximum gain of the transmit/receive antenna connected to the BS for each TV channel.

        - wranIfTXAntennaInformationBSHeight

This MIB object contains the height in m above ground level of the transmit/receive antenna connected to the BS.

      - wranIfTXAntennaInformationCPE

This MIB object, located at the CPE, is an array that contains information about the transmit/receive antenna that is connected to the CPE.  These related MIB objects are readable by all users and writeable only by the antenna.

        - wranIfTXAntennaInformationCPEGain

This MIB object is an array that contains the maximum gain of the transmit/receive antenna connected to the CPE for each TV channel.

        - wranIfTXAntennaInformationCPEHeight

This MIB object contains the height in m above ground level of the transmit/receive antenna connected to the CPE.

      - wranIfOperatingTVChannel

This MIB object contains the current operating TV channel of the WRAN network.

        - wranIfOperatingTVChannelBS

This MIB object is located at the BS and contains the current operating TV channel of the WRAN network.  This MIB object is readable by all users and writeable only by either the authorized responsible party or the BS SM.

        - wranIfOperatingTVChannelCPE

This MIB object is located at the CPE and contains the current operating TV channel of the WRAN network.  This MIB object is readable by all users and is not writeable.  It is set by the BS through MAC messages.

References: