IEEE P802.22
Wireless RANs

| Modifications to sub-clause 9.5.2 on Terrestrially-based Geolocation in response to comments #655 and 656 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-08-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave., Ottawa, Ontario, Canada K2H8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This contribution contains proposed modifications to sub-clause 9.5.2 of the Draft v3.0 on Terrestrially-based Geolocation as a result of comments #655 and #656 as well as the presentation made during the Orlando session in March 2010 (22-10-0054-02-0000_OFDM-based Terrestrial Geolocation.ppt).

Following discussions in San Diego in July, it was decided to make further modifications to the text so that it represents a generic description of the process to be used rather than a mandatory implementation.

Further modifications were made following discussion on the System teleconference call held on Wednesday, 4 August 2010.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Proposed modifications to sub-clause 9.5.2 of the 802.22 Draft v3.0

    - Terrestrially-based Geolocation

The 802.22 standard has been designed to support terrestrially-based geolocation capability by providing all the necessary PHY and MAC tools to make it possible.  Terrestrially-based geolocation can be achieved in a two-step process. First, the range between the BS and a number of its CPEs involved in the process is determined with high accuracy. Second, in order to establish the geolocation of a CPE through triangulation, the precise distances between the CPE to be geolocated and a number of reference CPEs belonging to the same cell are determined.  This can be achieved using the capabilities of the coherent multicarrier modulation inherent to the OFDM/OFDMA technique used by the 802.22 standard.

The 802.22 operational conditions under which terrestrial geolocation can be achieved with sufficient accuracy are described in section 9.5.2.1 and the schemes, including the actual MAC messaging, to carry out the BS-to-CPE and CPE-to-CPE ‘fine’ ranging are detailed in 9.5.2.2 and 9.5.2.3.  Once the necessary timing information has been acquired through these specific schemes, a geolocation process for which an example is given in section 9.5.2.4 and 9.5.2.5 can be applied to obtain the new geolocation of a CPE.  Annex B gives additional details on the scientific approach to support the terrestrial geolocation process based on the properties of the coherent multicarrier modulation technique used in the 802.22 Standard, allowing the reader to better understand this terrestrial geolocation  process.

      - Conditions to carry out precise ranging

It is first assumed that the WRAN cell is operating normally and that all the CPEs are synchronized in frequency and time to the BS on the downstream within the tolerance specified in section 8.11 through the normal acquisition of the super-frame and frame preambles as described in 8.9.1.  It is also assumed that the transmit frequency at the CPEs will be synchronized and locked to the frequency transmitted from the BS within the tolerance specified in 8.11 and that proper upstream time synchronization is achieved through initial ranging and periodic ranging processes as described in 8.9.3.  Note that these ranging processes, based on TU increments (i.e., nominal sampling period, see Table 212) are called ‘coarse’ ranging processes for the purpose of this sub-clause as opposed to the more accurate ‘fine’ ranging processes needed to carry out terrestrial geolocation as described below.

As part of the ‘coarse’ ranging process, the BS keeps an indication of the absolute timing advance required at the CPE in integer number of TU’s to compensate for the signal propagation delay on both downstream and upstream RF paths as specified in Table 54.  Such absolute timing advance is referenced to a zero value when the CPE is co-located with the BS and will increase as the distance between the BS and the CPE increases.  This timing advance can therefore readily be used as a coarse indication of the range between the BS and the CPE.  This parameter will be used in the ‘fine’ ranging processes described below.

The ‘fine’ ranging processes will need to work with accuracy in the range of nanoseconds and the various amounts of residual delay present in different CPEs will need to be taken into account.  Such residual delay may vary from one CPE model to another as well as from one antenna setup to another in the case where the antenna is not integrated to the CPE.  Such residual delay will need to be measured by the manufacturer with an accuracy of at least +/-30 ns as specified in 6.10.7.3.6.8 by co-locating the CPE with the BS (i.e., BS and CPE antennas are co-located or the BS and CPE are connected through the proper length of feed cable) and setting the Timing Advance (see 6.10.6) to zero.  This CPE residual delay will be declared during the CPE registration process through the REG-REQ MAC message (see 6.10.7.3.6.8).

The normal time unit used by the 802.22 Standard in its time synchronization and ‘coarse’ ranging is the sampling period or Time Unit (TU, see Table 218).  Better time accuracy will be needed to carry out the ‘fine’ ranging described below and the use of a sort of “vernier” comes in handy.   This “vernier” will allow an improvement of some two orders of magnitude in timing accuracy through the use of the phase correlation between the orthogonal carriers of the OFDM modulation.

      - BS to CPE fine ranging

The ‘fine’ BS-to-CPE ranging process shall take place as follows to provide the all necessary information to the terrestrial geolocation process described in sections 9.5.2.4 and 9.5.2.5:

- The BS transmits a RNG-RSP MAC message (see 6.10.6) to the specific CPE to re-range (ranging status= 011) and starts its counter Trange1 at the exact time when the downstream burst leaves the BS (i.e., at the start of the frame preamble). The Trange1 increments in TU’s at the BS.

- The BS keeps in memory (Range_map) the symbol number scheduled in the US-MAP (see 6.10.4) for the RNG-REQ message (UIUC= 6).

- The BS notes the size of the TTG in TU’s for the given channel bandwidth and the cyclic prefix used in the frame (see Table 222).

- The CPE acquires the values of the amplitude and phase rotation (or I&Q values) of the received subcarriers sent during the frame preamble in the downstream, removes the STS encoding and keeps the resulting information locally as the vector ‘Vernier1’.  Optionally, this amplitude and phase information can also include the results of the pilot carrier tracking taking place during the downstream sub-frame to increase accuracy.

- The CPE stores this amplitude and phase rotation information, i.e., ‘Vernier1’, in a local register (1680x2 bytes) and reports it to the BS when requested by the BLM-REQ message (see 6.10.18.3.1.10).

- The BS can then arrange for this data to be processed to precisely determine the time of arrival of the key multipath (usually the first) relative to the synchronization time at the CPE (which is recovered by the preamble correlator)..  The precise time of arrival of the downstream burst (in ns) relative to the timing of the CPE clock (incremented in TU’s) can then be deduced from the values contained in ‘Vernier1’.  Unlike the process to gather the basic timing information described in this sub-clause, the calculations to accomplish this signal processing are beyond the scope of the standard and belong to the geolocation process for which a simple example is presented in sub-clauses 9.5.2.4 and 9.5.2.5. Note that the amplitude and phase rotation of the subcarriers recovered from the decoded frame preamble actually represent the frequency domain of the channel impulse response. Note also that typical OFDM demodulation processes readily use this information to correct the constellations in amplitude and phase at the output of the FFT for proper data decoding.

- The CPE responds to the RNG-RSP MAC message by sending a RNG-REQ CDMA message in the time slot allocated in the US-MAP (UIUC = 6).

- At the time of arrival of the CDMA ranging burst from the CPE, the BS stops the Trange1 timer precisely at the time of the first sampling period belonging to the CDMA burst.

- The BS acquires the values of the amplitude and phase rotation (or I&Q values) of the first 56 logical static subcarriers (see 8.6.2.2) from the output of the FFT and removes the CDMA signature.  These corrected values, which constitute ‘Vernier2’ (56x2 bytes), are stored at the BS and sent to the geolocator for processing.

      - CPE to CPE fine ranging

Inter-CPE communication is possible by the means of the CBP bursts (6.23.1) transmitted during the Self-coexistence window (Table 44, UIUC = 0 and 1). These bursts can be used for self-coexistence between adjacent and overlapping WRAN cells, for device identification as may be required by local regulation and for carrying out the CPE-to-CPE ‘fine’ ranging process described described in this sub-clause.

The ‘fine’ CPE-to-CPE ranging process shall take place as follows to provide all the necessary information to the terrestrial geolocation process described in sections 9.5.2.4 and 9.5.2.5:

- The BS starts the process by signaling in the US-MAP the presence of an active mode SCW at the end of the frame for the CPE that needs to be ranged (see Table 44, UIUC = 0).

- The BS indicates the Timing Advance that it wants this CPE to use for its CBP burst so that this burst is received within the cyclic prefix at the nearby reference CPEs unlike the RNG-RSP Timing Advance, which is adjusted so that the CPE burst is received within the cyclic prefix at the BS.  The BS can also adjust the EIRP Density Level for sufficient local coverage noting that the CBP burst may be transmitted through the antenna backlobe and received by another CPE through its receive antenna backlobe depending on the geometry of the RF path.

- The BS signals the presence of a passive mode SCW at the end of the frame for the reference CPEs that have been selected to carry out this ‘fine’ CPE-to-CPE ranging process.  This is done at the BS by including IE’s with UIUC = 1 addressing each reference CPE to be used for ‘fine’ CPE to CPE ranging in the US-MAP (see Table 44).

- The BS indicates that the channel to be listen to during the SCW is the operating channel and the BS specifies that the reference CPEs are to use the synchronization mode = 0 so that the CBP burst capture is done synchronously with the internal frequency and time synchronization of each reference CPE which are locked to the BS (see Table 44, UIUC = 1).

- The BS uses the same frame to carry out the ‘fine’ CPE-to-CPE ranging process between the CPE to be geolocated and the reference CPEs as that used for the BS-to-CPE ‘fine’ ranging process with its reference CPEs as well as with the CPE to be geolocated as described in 9.5.2.2.    Note that the adjacent frames can also be used if frequency and time synchronization slippage between these consecutive frames is tightly controlled.

- The CPE in active mode transmits its CBP burst with the specified Timing Advance relative to the start of the fourth symbol before the end of the frame and the specified EIRP Density Level for proper RF path propagation margin (see Table 47) and it should contains, at least, the CBP Identification IE (see 6.8.1.3.1.7) to confirm that the burst comes from the right CPE.

- Each reference CPE signaled by the BS to be in SCW passive mode during this frame captures the CBP burst using its own internal synchronization locked to the BS (i.e., Synchronization Mode = 0, see Table 47) and acquires the amplitude and phase rotation (or I&Q values) of the subcarriers contained in the CBP preamble.  Optionally, this amplitude and phase information can also include the results of the pilot carrier tracking taking place during the CBP burst decoding to increase accuracy.  This information constitutes the vector ‘Vernier3’, and is stored in a local register (840x2 or 1680x2 bytes).

- The BS then queries the values contained in ‘Vernier3’ from each reference CPE and arrange for this data to be processed by the terrestrial geolocation process.

      - Terrestrial geolocation process

The terrestrial geolocation process is responsible for managing the WRAN device ‘fine’ ranging process and collecting, through the MAC messages between the BS and its CPEs, all the information necessary to carry out the calculations required to geolocate a CPE.

When the terrestrial geolocation process needs to locate a CPE, it determines which reference CPEs will be used for this purpose using existing geolocation information and its own criteria. The BS as well as a minimum of two reference CPEs, for which the location has already been determined through either surveying, satellite-based initial geolocation or by having pre-applied the current triangulation method based on other reference devices, are needed to carry out the new geolocation process.  The terrestrial geolocation process signals to the BS the CPEs that it has selected to carry out the process and the BS initiates the necessary signaling process with these reference CPEs and the new CPE to be located and, once done, transfers the collected information to the terrestrial geolocation process. Triangulation calculations can then be carried out on the resulting device-to-device distances to find the location of the new CPE. All the signaling required takes place within the WRAN cell.

The BS establishes the precise distance to each of the reference CPEs as well as to the new CPE using the BS-to-CPE ranging process described in 9.5.2.2.  This multiple ‘fine’ ranging process is conducted in parallel with the CPE-to-CPE ranging process to establish the distance between the new CPE and each reference CPE involved using the ranging process described in 9.5.2.3.  It is assumed that the terrestrial geolocation process already knows the absolute location of the BS and of the reference CPEs and the BS only needs to gather the information that it has acquired from these various BS-to-CPE and CPE-to-CPE ranging processes.

Using all this information, the terrestrial geolocation process computes the location of the new CPE and returns a NMEA longitude-latitude string.

      - Geolocation calculations

Once the terrestrial geolocation process has acquired all the information generated by the ‘fine’ BS-to-CPE ranging process described in 9.5.2.2, the calculations to determine the precise BS-to-CPE propagation time and propagation distance can be carried out as follows:

BS-to-CPE Propagation time = Trange1 – (Range_map+TTG) – CPE residual delay + Vernier1 + Vernier2

BS-to-CPE Propagation distance = c * Propagation time / 2

Furthermore, the various propagation distances between the BS and the reference CPEs and the CPE to be geolocated will be augmented with the information on the propagation distances obtained from the ‘fine’ CPE-to-CPE ranging process between the CPE to be geolocated and each of these reference CPEs and calculated as follows based on the Vernier3 acquired in 9.5.2.3:

CPE1-to-CPE2 Propagation time = Local transmit timing reference at CPE1 + TACBP
 - Local receive timing reference at CPE2 + Vernier3

CPE1-to-CPE2 Propagation time = -PT1 + TACBP + (PT2 - CPE2 residual delay) + Vernier3

where:

PT1: propagation time between the BS and CPE1 as calculated by the previous equation for BS-to-CPE

PT2: propagation time between the BS and CPE2 as calculated by the previous equation for BS-to-CPE

TACBP: Timing Advance of the CBP burst (see step 2 of 9.5.2.3)

(Note that the geolocation process will have to pre-adjust TACBP depending on the distance between the two CPE to be geolocated and the reference CPEs so that the delays measured by Vernier3 fall within the symbol cyclic prefix (e.g., 74.68 usec corresponding to 22.4 km).

CPE1-to-CPE2 Propagation distance = c * Propagation time / 2

Once these BS-to-CPE and CPE-to-CPE distances have been found,triangulation calculations can be carried out to come up with the precise position of the CPE to be geolocated.

The accuracy of the process will be limited first by the accuracy of the CPE residual delays reported at registration by each CPE involved through the RNG-REQ MAC message, which is specified to be +/-30 ns in sub-clause 6.10.7.3.6.8.  It will also be limited by potential multipath contained in the channel impulse response received at each CPE.  If complex multipath is experienced, it is possible that the right impulse that reflects the correct propagation distance may be hidden and/or diffused by other multipaths due to reflections on the RF channel.  Low SNR resulting from blockage or distant reference CPEs may also impact the accuracy.  However, accuracy can be improved in building redundancy in the process by using more reference CPEs.  This would help in selecting the reference CPEs that provide the cleanest channels among those used for the ranging process.  This choice will be made by the terrestrial geolocation process since it can have access to the channel impulse response at each reception point as the time domain representation of the different ‘Vernier’ vectors.  More accuracy in the triangulation calculations can also be obtained by using more reference CPEs in these calculations.

The definition of “Geolocator” currently in section 3 has to be removed.

Vernier: A vernier scale is an additional scale which allows a measurement to be read more precisely than directly reading a uniformly divided measurement scale (from: http://en.wikipedia.org/wiki/Vernier_scale)