IEEE P802.22 Wireless RANs

| Minutes from PHY Conference Call Held on Jun 3, 2010 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Zander Zhongding Lei | Institute for Infocomm Research, Singapore | 1 Fusionopolis Way #21-01 Connexis, Singapore 138632 | 65-6408-2436 | leizd@i2r.a-star.edu.sg |

Abstract

This document contains the minutes from the PHY conference call on Jun. 3, 2010

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## Agenda

### Attendance

### IEEE patent policy located at: http://standards.ieee.org/board/pat/pat-slideset.pdf

### Approve the agenda

### Review Session 8 (PHY) of Draft version 3.6 (Track Change)

- 8.9 Control Mechanisms

### AOB

## Attendance

| Attendee | Affiliation | Jun 3 |
| --- | --- | --- |
| Sung Hyun Hwang | ETRI | x |
| Jung Sun Um | ETRI | x |
| Gerald Chouinard | CRC | x |
| Zander Zhongding Lei | I2R | x |
| Ivan Reede | Amerisys |  |
| Jason Li | Wi-LAN |  |
| Apurva Mody | BAE | x |
| Ranga Reddy | US Army |  |

## Notes

- The meeting started at 10:00 pm and ended at 11: 40 pm ET

- All attendees were aware of the IEEE patent policy

- The agenda was approved.

8.9 Control Mechanisms

The group discussed and had some agreement to the amendment in sessions 8.9.3, 8.9.3.1, and 8.9.3.2.

- 8.9.3

- It was agreed to append “when needed” after “subframe, ” in the first sentence as the opportunistic burst does not have to be present in all frames

- It was agreed to remove “less the extent of the self-coexistence window when present” as the self-coexistence window was decided not to be part of the US sub-frame in the last round of comments and resolution, proposed by Wendong according to Gerald

- It was agreed to change “12 additional subchannels” to “14 additional sub-channels” since 4 bits are used in UIUC indicating up to 16 subchannels (2 static subchannels are reserved for geolocation)

- 8.9.3.1

- It was agreed to expand the 1st sentence and add in “CDMA initial ranging, CDMA periodic ranging, CDMA BW request and CDMA UCS notification” to highlight the 4 types of CDMA bursts.

- It was agreed to reject the insertion of sentence “The initial ranging always occupies the first 7 OFDM symbols of the US sub-frame over the specified number of sub-channels” at the end of the 1st paragraph. The initial ranging details would be addressed in next section.

- 8.9.3.1.2

The group had a good discussion on how many ranging symbols are required to cover 100 km. As the symbol duration is roughly the time for a signal travels 100 km. To take account the round trip delay, a 2-symbol buffer time is needed. Therefore, 5 symbols in total (3 symbols for ranging signal) are to be reserved in the burst MAP. It was agreed to reject the proposed sentence “The initial-ranging transmission shall therefore always occupy the first 7 OFDM symbols of the ranging burst.”

- It was agreed to rephrase the last 2 sentences in the first paragraph to “The initial-ranging transmission is performed using three consecutive symbols starting, as indicated in the US-MAP for the CPE, on the first symbol after the TTG. To allow for the absorption of the signal propagation delay for the forward and return paths from a CPE located at a distance of up to a maximum of 100 km, 2 more buffer symbols are needed at the BS for this initial ranging burst to avoid spilling onto other signals received from synchronized CPEs.  The window for the initial-ranging transmission shall therefore always occupy the first 5 OFDM symbols of the upstream sub-frame.”

8.6.4 Update

For the action item taken by Jason in Beijing meeting to amend 8.6.4, Gerald updated that Jason has had an initial draft and been discussed with Gerald. They will upload to the Mentor once ready.

Other Editorial comments

Gerald felt that some of the editorial comments could be technical, e.g. ID# 511, 512, 513, 534, and 545. Zander will look at them and schedule time for discussion if needed.

## Interim Meeting

Zander reminded the group that the teleconference based Interim meeting will be held next week and PHY group meeting had been scheduled from 10:30 am to 12:30 pm Eastern Time, June 8.