IEEE P802.22
Wireless RANs

| Role of CHO-UPD message: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Byung Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Antony Franklin | ETRI | Korea | +82-42-860-0752 | antony@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

This document summarizes role of CHO-UPD message. This document related CID #  220, 317, 351, 464, 466.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Introduction of CHO-UPD

In Section 6.10.20.7 (Draft v.3.1), CHO-UPD message is defiend in order to consolidate channel occupancy information over the entire cell. This CHO-UPD message transmits both “Protected” and “Unclassified” channels from BS to CPE (See table 140). However, “Candidate/Backup” channels can be transmitted to all CPEs using Backup/Candidate IE (See table 31) in DCD message and transmitted to neiboring cells using Backup/Candidate IE in CBP (See Section 6.8.1.2.1).

- Role of CHO-UPD

Difference between CHO-UPD message and Backup/Candidate IE is that CHO-UPD message broadcasts “Protected” and “Unclassifed” channels whereas Backup/Candidate IE broadcasts “Backup” and “Candidate” channels.

Generally, “Protected” channels information is collected from the incumbent data base. In this case, updating of “Protected” channels information depends on update duration of incumbent data base. Both BS and CPE can estimate this updating duration. Therefore BS and CPE do not require this CHO-UPD message. However, if incumbent data base does not contain all the incumbent activity information or if some region do not support incumbent data base, the BS and CPE need synchronization of channel information. This is to keep the common channel occupancy information between BS and CPE or between BSs to avoid wrong decisions.

For example, there are many WMP operations that are not registered in the incumbent data base (Unregistered WMP: UWMP). Due to this UWMP, there will be harmful interference between WRAN systems and UWMP. To solve this problem, BS should know the existence of UWMP. This is done by spectrum sensing function managed by the BS using some intelligent functions. The BS request the CPEs to perform sensing using BLM-REQ and the CPEs report the sensing result through BLM-REP. Therefore, CPE do not need information of “Protected” channels.

- Need for CHO-UPD message

The knowledge of “Protected” and “Unclassified” channels information is required at the CPEs for the following considerations.

1. In the sensing schedule given by the BS, there are idle times during which there is no sensing by CPEs. During this time the CPEs can sense other channels with the information of “Protected” channels.

2. The CPEs can be used to communicate the channel information to other BSs in an efficient way. For this the CPEs need to be updated with the “Protected” channel information through CHO-UPD message.

3. In future, if we would like to use CPEs to provide the “Protected” channel information to other TVWS devices (assume that the 802.19.1 co-existence standard comes up with some solution), the CPEs has to be updated with “Protected” channel information.

- Conclusion

If we ignore all the above considerations, there is not much necessitiy to keep the CHO-UPD message in the standard. This may be used in future.