IEEE P802.22
Wireless RANs

| Inclusion of RF Masks in Annex A |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave., Ottawa, Ontario, Canada K2H8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This document contains the proposed changes to section 8.13 and Annex A for the transfer of the RF Masks related to each regulatory domain to Annex A.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Proposed modifications to section 8.13

  - RF Mask

The RF mask shall meet the requirements applicable to the regulatory domain where the CPE is to be operated using the appropriate regulatory class described in Table 291 and the corresponding RF Masks described in Table 293 (see Error: Reference source not found). Where such requirements have not been specified, at least one of the RF masks described in Tablw 293shall be met as default by both base stations and CPEs for worldwide deployment of the technology.

##### Proposed modifications to Annex A as contained in document 22-10-23r4

### A.2.2 Transmit Spectrum Mask Requirements

Table 293 specifies the transmit spectrum mask requirements authorized by the various regulatory domains for the different regulatory classes described in Table 291. Table 293 refers to the appropriate Figures 186 and 187 which illustrate the various RF Masks applicable to the 802.22 systems.  The power spectrum density (PSD) measurement shall be done over a measurement bandwidth of 100 kHz and a video bandwidth of 100 kHz with an average detector.

Table 293—Transmit Spectrum Mask Requirements

| ###### Regulatory Domain | Regulatory Class | ###### Transmit Spectrum Mask |  |
| --- | --- | --- | --- |
| US | (Stationary fixed) | See Figure 186. First adjacent channel Beyond the outer edge of the first adjacent channel | 55 dB below the highest power in a 6 MHz operating channel in 100 kHz bandwidth Comply with FCC section 15.209(a) |
| US | (Portable nomadic Mode II) | See Figure 186. First adjacent channel Beyond the outer edge of the first adjacent channel | 55 dB below the highest power in a 6 MHz operating channel in 100 kHz bandwidth Comply with FCC section 15.209(a) |
| CA | Stationary fixed (Δf is referenced to the edge of the operating channel, Measurement bandwidth is 100 kHz, levels are expressed in dBc, relative to the total power in the operating channel) | See Figure 187. 0.05 ≤ Δf ≤ 6 6 ≤ Δf ≤ 12 12 ≤ Δf ≤ 18 Δf > 18 AND within 54-72 MHz, 76-88 MHz, 174-216 MHz, 470-608 MHz and 614-698 MHz Outside the above cases | 44.9 +1.1*( Δf)1.6 37.8 + 4.4* Δf 70.2 + 1.7* Δf 100.8 BS: 43+10*log(P in Watts) CPE: Comply with RSS 210 Table 2 |
| UK |  |  |  |

Figure 186   WRAN 802.22 transmission RF Mask to be used in the USA

Figure 187   WRAN 802.22 transmission RF Mask to be used in Canada

Table 294 Unavailable channels inside the protected contour of a DTV station operating on channel “N”

| Regulatory Domain | Regulatory Class | Unavailable channels |
| --- | --- | --- |
| US | (Stationary fixed) | N N±1 |
| US | (Portable nomadic Mode II) | N N±1 |
| CA | Stationary fixed | N N±1 N±2 |
| UK |  |  |

_________________________