IEEE P802.22
Wireless RANs

| Proposed resolution for comments 393, 397, 400-403 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-25 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jianfeng Wang | Philips | 345 Scarborough Road, Briarcliff Manor, NY 10510 | 1-914 945 6247 | jianfeng.wang@philips.com |
| Gerald Chouinard | Communications Research Cente | 3701 Carling Avenue, Ottawa, Canada K2H-8S2 | 613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This document contains the proposed resolution for comment 393, 397, 400-403.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of situation

- There exist two types of SCWs in the draft: contention based SCW and reservation based SCW.

  - Contention based SCW:  SCW is shared by all neighboring WRANs.

    - Pros:  Simple to setup. Efficient for shared use in low load case.

    - Cons:  not as efficient as reservation based method in high load case

  - Reservation based SCW:  WRAN reserving a SCW has exclusive access to that SCW.

    - Pros:  once successfully setup, efficient for high load (of CBP transmissions)

    - Cons:  it has scalability issue (w/ the number of WRANs). The setup procedure could be also complicated and/or time consuming.

- WG agreed in principle for joint use of reservation and contention scheme

- US-MAP based Mechanisms have been already in place (see 6. 10.4.1) to allow a BS to coordinate its associated CPE of CBP transmissions (active mode) and CBP receptions (passive mode). In other words, US-MAP based Mechanisms can be used by a BS to schedule which CPEs to transmit in a specified SCW  to mitigate intra-WRAN CBP collisions while maximizing the (spatial) use of the SCW whether the SCW is reserved by the BS or shared with other WRANs (contention based). If the SCW is contention based, frame based (e.g. backoff) collision avoidance algorithm should be used (done internally in the BS) to mitigate potential collision with other WRANs. Given SCW schedule, the intelligence of selecting which CPE(s) for CBP transmission or reception should be left for implementation.

- Further actions needed

  - Expand SCH to indicate the scheduling of both types of SCWs (reservation vs contention). Note SCH is included in CBP to allow neighboring WRANs to get informed of the SCW schedule.

  - In using contention based SCW,  frame based (e.g. backoff) collision avoidance  algorithm should be defined to mitigate collision with neighboring WRANs.

Suggested remedy

[In Table 1 (SCH format), to add SCW scheduling information. Two types of SCW can be scheduled]

- Table 1— Superframe control header format

| Syntax | Size | Notes |
| --- | --- | --- |
| Superframe_Control_Header_Format() { |  | 1 OFDM symbol long and transmitted with modulation/coding described in 8.4.2.1 |
| … | .. | … |
| SCW Cycle Length | 5 bits | Specified in number of superframes. If this field is set to 0, then no SCW cycle is scheduled. This field has to be 1 or larger to be effective. To limit the number of possibilities, the field shall be one of five following choices {1, 2, 4, 8, 16}. For example, if this field is set to 1, SCW Cycle repeats every superframe, if it is set to 2, SCW Cycle repeats every 2 superframes, etc. |
| SCW Cycle Offset | 8 bits | Specified in number of superframes, it indicates the offset from this SCH transmission to the superframe (i.e., the superframe contains SCWs and is specified by SCW Cycle Frame Bitmap) that SCW cycle starts, or repeats. For example, if this field is set to 0, the SCW cycle starts from the current superframe. The value of the field shall be less than SCW Cycle length unless initial countdown. For initial countdown, this field can equal or be larger than SCW Cycle Length. Larger initial countdown gives neighboring WRANs longer time to discover and avoid any potential SCW reservation collision. |
| SCW Cycle Frame Bitmap | 32 bits | Valid for a unit of superframe, each 2-bit in the bitmap corresponds to one frame within the superframe. If the 2-bit is set to 00, this means that there is no SCW scheduled for this frame. If the 2-bit is set to 11, a reservation-based SCW (reserved by the current WRAN) is scheduled in the corresponding frame. If the 2-bit is set to 10, a reservation-based SCW has been scheduled by a direct-neighbor WRAN cell in the corresponding frame and needs to be avoided by other WRAN cells receiving this SCH. If the 2-bit is set to 01, a contention-based SCW (that could be shared with other WRANs) is scheduled by the current WRAN cell in the corresponding frame. The number of reservation based SCWs cannot exceed 2 per WRAN cell per SCW Cycle. At least one contention-based SCW shall be scheduled in one SCW Cycle (code 01). The BSs shall start scheduling their contention-based SCWs from the last frame of the superframe, going backward for multiple contention-based SCWs. This bitmap applies only to the superframes scheduled by the SCW Cycle. Note: quiet period scheduling should be done prior to the SCW scheduling so that SCWs avoid frames already reserved for QP. If SCW conflicts with QP, QP shall override the SCW. |
| Reserved | 3 bits |  |
| IEs | Variable (integer of bytes) | Optional Information Elements that could be transmitted with the SCH. They are: MAC version (6.9.1) Current transmit EIRP (Error: Reference source not found) Incumbent detection reporting inhibit timer (Error: Reference source not found) |
| HCS | 8 bits | Header Check Sequence See Error: Reference source not found |
| } |  |  |

- 6. 23. 1.2 SCW Schedule and CBP Packet Transmission / Reception Scheduling

802.22 base stations and CPEs are capable of transmitting coexistence beacons (see Error: Reference source not found) which provide its recipients enough information to achieve satisfactory and good coexistence amongst overlapping 802.22 cells. The CBP also allows for CPE identification as may be required by local regulation for interference resolution and geolocation ranging enhancement process between CPEs within a cell (see Error: Reference source not found).

Coexistence beacons are scheduled by the BS through the use of UIUCs 0 and 1for Passive and Active modes respectively as indicated by the US-MAP (see Table 47Error: Reference source not found). When selecting a UIUC equal to 0 or 1 for coexistence scheduling, the connection ID contained in the MAP IE indicates which CPEs shall send the beacon, in case of Active mode, or which CPEs shall listen to the medium for detecting a beacon, in case of Passive mode, within the scheduled time of the SCW. This connection ID can be either unicast (e.g., a CPE’s primary connection ID), multicast (i.e., a multicast management connection ID), or even broadcast. In case of multicast, the BS can implement clustering of CPEs to improve spectrum utilization and maximize the effectiveness of the coexistence beacons, as multiple CPEs in a given WRAN cell would transmit a coexistence beacon during the same scheduled time.  However, the BS needs to select the CPEs that will belong to each  multicast group to minimize potential collisions.

CBP packet transmissions shall be performed in the operating channel, but base stations and CPEs shall be capable of receiving CBP packets in any channel it is capable of operating in. The BS specifies the channel number in which the CPE shall listen to the medium for a coexistence beacon (SCW in Passive mode). The channel number is included in the US-MAP for the UIUC=1 Error: Reference source not found. This allows the coexistence amongst 802.22 systems operating in the same channel and the monitoring of the quiet period scheduling carried by the CBP bursts in different channels for proper RF detection of incumbent signals in those channels that would otherwise be hidden by other WRAN transmissions.

The coexistence UIUCs identify the presence of a SCW at the end of the frame during which CBP packets can be exchanged.  The SCW, when scheduled, occupies the last 5 symbols of the frame (see Error: Reference source not found) and consists in one symbol buffer to absorb the propagation time, 3 symbols for the CBP burst and one last symbol to absorb propagation time. Their access to the medium within the SCW is controlled by the BS.

The BS shall decide which CPEs transmit CBP packets in each scheduled active UIUC=0. Example of mechanisms that can be used by the BS for the selection of which CPEs are to transmit in each SCW in Active mode can be based on location information, clustering, or be as simple as selecting all CPEs.

All BSs involved in coexistence on the operating channel will need to align their operation with all of the declared SCWs of neighboring WRANs so that none continue its normal operation during these SCWs. All overlapping WRAN cells need to comply with the SCW scheduling either through active or passive use. When a WRAN cell reserves a SCW, this means that only that BS and its CPEs will be able to be active (i.e., transmit CBP bursts) during this SCW.  For contention-based SCWs, all neighboring WRAN cells could be active during the SCW.  This SCW scheduling takes place and applies only to the operating channel.

The CBP protocol can be used for communication and coexistence of WRANs on the same operating channel as well as synchronization to scheduled quiet periods across channels, which is enabled by the use of the Channel Number parameter for the UIUC=0 as described above. Note that different CPEs associated to the same BS can simultaneously communicate using CBP. For example, if two CPEs are associated with the same BS, during the same self-coexistence window, both can be transmitting in active mode, one of them can be transmitting a CBP packet on the operating channel while the other could be receiving on the same channel or on another channel. This is controlled by the BS when it schedules the self-coexistence windows in the US-MAP. It is assumed that within a cell, the BS will have the intelligence to select the right number and the right location of the 'active' CPEs to minimize collisions without going to internal contention.

In order to facilitate network discovery and to enable coexistence with neighboring WRANs, the BS shall maintain a minimal regular pattern of  Active CBP transmissions (UIUC= 0).  The BS will also augment this schedule with  Active and Passive mode SCW operation, to enable CPE identification, over-the-air geolocation and additional inter-WRAN communication.

The BS shall  schedule SCWs through the SCH when the BS initiates its normal operation and continuously thereafter according to its varying requirement as well as the requirements of the other WRAN cells in the area through coordination of these schedules by exchange of the SCH information through the CBP mechanism.

The SCH includes SCW Cycle Length, SCW Cycle Offset, and SCW Cycle Frame Bitmap for SCW scheduling, as defined in Table 1.  SCW Cycle Length represents the cycle interval length between each successive superframes carrying SCWs, in number of superframes. The SCW Cycle Frame Bitmap specifies which frames (in a scheduled superframe) have SCW scheduled and the SCW mode (reservation based SCW vs. contention based SCW). The field encoding of SCW Cycle Frame Bitmap is specified in Table 1.

The  SCW Cycle Length and SCW Cycle Frame Bitmap parameters should be set according to the requirements for coexistence and inter-WRAN communications, and it determines a trade-off between the performance of inter-WRAN communications and the overhead represented by the SCWs.

Multiple WRANs operating on the same channel may share the same SCWs  via contention or each WRAN may reserve its own SCWs depending on how they set up their regular SCWs patterns. Sharing of SCWs reduces the total overhead in the channel, but on the other hand, reservation of SCWs enables contention-free CBP transmissions. The decision on whether to share or reserve SCWs will depend on the total overhead generated by SCWs and the coexistence scenarios (e.g., number of neighboring WRANs on the same channel and number of CPEs in the overlap area which can result in non-negligible transmission redundancy and, hence, probability of successful CBP transmission using contention-based SCWs). Moreover, the WRANs may change their schedule of the SCWs dynamically and therefore they may adjust the schedule to adapt to the coexistence scenarios.

The reservation of SCWs facilitates the contention-free CBP transmissions among the neighboring WRAN cells sharing the same channel, and it can be achieved by scheduling the reservation based SCWs such that it does not overlap with  those of other neighboring WRANs. The procedure for that is as follows:

- During the initialization stage, the WRAN monitors the channel for at least 16 superframes (the maximum SCW Cycle Length) to discover neighboring WRANs and identifies their schedules of reservation-based SCWs by capturing and decoding the CBP packets from a neighboring WRAN.

- If a WRAN identifies another WRAN’s schedule of reservation-based SCW (specified in SCW Cycle Frame Bitmap with 2-bit set to 11 or 10 in the corresponding frame. See Table 1), it shall regard these SCWs as reserved and shall set its own  schedule of reservation-based SCWs so that it does not overlap with the SCWs reserved by the neighboring WRAN(s).

- In the process of establishing an SCW schedule, a WRAN can include its SCW schedule in its SCH with the initial SCW Cycle Offset set to be larger than the SCW Cycle Length to allow time to advertise/countdown its SCW schedule. The WRAN shall also convey its SCW schedule through CBP via contention-based SCWs.

- With the above mechanism, the reservation conflict is low but it could still happen. To detect a potential conflict after the reservation is done, a WRAN can purposely skip the transmission of CBPs in its reserved SCWs and schedule its associated CPE to listen to the channel (using US-MAP with UIUC=1, passive mode) once in a while (irregularly) during the reserved SCWs. If a reservation conflict is identified, the WRAN will restart the process of scheduling SCWs.

- A WRAN can change its schedule of SCWs by  updating SCW related parameters in SCH.

Contention-based SCWs can be shared by neighboring WRANs. In other words, its neighbors can use these contention-based SCWs (combined with its own scheduled SCWs if available) for the transmission of CBPs following the random backoff mechanisms defined later in this paragraph. However, the WRAN scheduling those contention-based SCWs has the ownership of those SCWs.  Other WRANs cannot use those SCWs for other purposes of transmissions nor can they cancel the schedules of these contention based SCWs.  The WRAN owning the contention based SCWs can reduce the frequency of contention based SCWs by canceling some of contention based SCWs (Note at least one contention-based SCW shall be scheduled in one SCW Cycle. Note also that co-ownership of these contention-based SCWs would avoid one BS removing the SCW used by other BSs.). Hence, two neighboring WRANs can be the co-owner of a certain set of contention based SCWs by scheduling them in their SCHs respectively. In that case, if one WRAN cancels the schedule, it does not affect that of the second WRAN. The second one as well as the neighbor WRANs of the second one can still share those contention based SCWs. To mitigate collision of contention with neighboring WRANs, access to contention-based SCW shall use random backoff in the unit of contention-based SCWs. When a BS schedules its associated CPEs to transmit CBPs via contention-based SCWs, it will wait for a random number (from 0 to 15 with uniform probability) of next available contention based SCWs. For example, if the random number chosen is 0, the BS shall schedule a CBP transmission via an US-MAP IE by accessing the first available contention based SCW from the transmission of the US-MAP IE. If the random number is 5, the BS will schedule a CBP transmission via an US-MAP IE by accessing the sixth available contention based SCWs from the transmission of the US-MAP IE.

A BS can schedule CPEs in Passive mode (via US-MAP IE) during SCWs reserved or shared by other neighboring WRANs in order to receive CBP packets from its own WRAN cell (BS and/or CPEs) or from the neighboring WRANs operating on the same channel. A BS can also schedule CPEs in Passive mode to listen to CBP bursts on either of the adjacent channels for the purpose of localizing and synchronizing the quiet periods for proper in-band sensing. CPEs executing out-of-band sensing during their idle time will listen to CBP bursts on other channels for the purpose of localizing the quiet periods for proper out-of-band sensing in coexistence situations in these channels (see 9.3.3).  Note that WRANs on different operating channels could reserve the same SCW for transmitting CBP packets. For the SCW in Passive mode, the BS shall define the schedule such that it can capture the required coexistence information from neighboring WRANs that may impact the WRAN operation. Note that SCWs in Passive mode does not increase the overhead in the channel since it only requires listening for other CBP transmissions from neighboring cells.  By definition, no other type of transmission shall be allowed during the SCWs. Therefore, any CPE may be scheduled in Passive mode during SCWs.

The BS could also use other information available at the MAC layer to decide when and in which mode to generate a Coexistence UIUC, as long as the above generation rules are satisfied. One example can be found in Error: Reference source not found, where the BS uses the CPE statistics report as the basis for triggering the execution of CBP.

Other suggested remedy

        - CBP Information Elements

CBP packets shall carry at least one information element (IE) in their payload among the set described in Table 17 since it provides the basic information required to enable self-coexistence. CBP packets shall at least carry a Backup Channel information element (IE) in their payload. This is to allow WRANs to execute the spectrum etiquette mechanism before deciding to execute the other spectrum sharing mechanisms described in 6.23.3.

- — CBP IEs

| Element ID | Name |
| --- | --- |
| 0x00 | Backup_and_Candidate_Channel List IE |
| 0x01 | FC_REQ IE |
| 0x02 | FC_RSP IE |
| 0x03 | FC_ACK IE |
| 0x04 | FC_REL IE |
| 0x05 | CBP_Location IE |
| 0x06 | CBP_Geolocation IE |
| 0x07 | Signature IE |
| 0x08 | CERT-REQ IE |
| 0x09 | CERT-RSP IE |

- — CBP Pattern Identification IE format

- 6. 16 Contention Resolution

…

The BS has much flexibility in controlling the contention resolution. At one extreme, the BS may choose to set up the Request (or Ranging or UCS Notification) Backoff Start and Request (or Ranging or UCS Notification) Backoff End to emulate an Ethernet-style backoff with its associated simplicity and distributed nature as well as its fairness and efficiency issues. This would be done by setting Request (or Ranging or UCS Notification) Backoff Start = 0 and Request (or Ranging or UCS Notification) Backoff End = 10 in the UCD message. At the other end, the BS may make the Request (or Ranging or UCS Notification) Backoff Start and Request (or Ranging or UCS Notification) Backoff End identical and frequently update these values in the UCD message so that all CPE are using the same, and hopefully optimal, backoff window.

…

- 6. 23. 1.1 CBP Packet Structure

The structure of a CBP packet (i.e., CBP PHY PDU) is shown in Figure 105Error: Reference source not found. The burst starts with a CBP preamble which shall be common across all 802.22 networks (see subclause Error: Reference source not found), and which shall be different from the superframe preamble. After the CBP preamble, the CBP MAC PDU shall be transmitted, starting with the SCH data (as defined in Table 1), followed by the CBP header (Table 5) and other CBP IEs. The CBP MAC PDU may be one or two OFDM symbols long. The second data symbol may be needed depending on the number of CBP IEs included in the CBP MAC PDU.

By including the SCH data (which contains information about the 802.22 cell) as part of the CBP MAC PDU, the transmitting CPE or BS conveys necessary information to allow neighboring network discovery and coordination of quiet periods and self-coexistence windows. Including the SCH is a way to advertise the schedule of QPs and SCWs to CPEs in other neighboring cells.

The SCH information is needed in situations where WRANs are operating in different channels as well as when they are operating co-channel or adjacent channels. In the first case, the SCH information obtained through detecting and demodulation the SCH or through reception of the CBPs allows other WRANS to discover the schedule of QPs, which can be used for out-of-channel sensing. In case WRANs are operating co-channel or adjacent channels, the SCH, received through the CBPs will signal the schedule of QPs and SCWs in addition to other IEs that can be used to signal frame allocations, if necessary.

For communication using CBP over the backhaul, the CBP MAC PDU (see Error: Reference source not found) should be encapsulated into the IP packet.

_________________________