IEEE P802.22
Wireless RANs

| Modifications to the upstream subcarrier allocation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-06-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jason Li | Wi-LAN Inc. | 11 Holland Avenue, Suite 608, Ottawa, ON, Canada K1Y 4S1 | 1-613-688-8967 | jli@wilian.com |
| Gerald Chouinard | CRC | 3701 Carling Ave., Ottawa, Ontario, Canada K2H8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This contribution contains proposed modifications to sub-clause 8.6.2.2 of the Draft v3.0 on upstream subcarrier allocation as a result of comments #526 and #527 and further discussions in the PHY ad-hoc group.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Proposed modifications to sub-clause 8.6.2.2 of the 802.22 Draft v3.0

      - Upstream Subcarrier allocation

In the upstream, the first two sub-channels, which are usually reserved for opportunistic or scheduled control signalling (i.e., Ranging/BW request/UCS notification bursts, see Error: Reference source not found), use the pre-determined static subcarrier indices shown in Table 219 after the appropriate subcarrier offset has been applied as indicated below.  When these sub-channels are not used for control signaling, they can be used for data.  In such case, the pilot subcarriers are logically allocated as shown in Error: Reference source not found and then mapped to the appropriate static subcarrier indices. The remaining static subcarriers are then sequentially assigned with data values from the constellation mapper.

All the subcarriers in the sub-channels not assigned to the CPE shall be assigned 0 amplitude and 0 phase at the CPE. For each sub-channel assigned to the CPE, 4 associated pilot values shall be assigned and 24 complex data from the constellation mapper shall be assigned following the pattern specified in Section 6.6.

Among the 240 pilot values, every 4 pilot values are associated with a sub-channel. For example, pilot values at indices 0, …, 3 are associated with sub-channel 1, pilot values at indices 4, …, 7 are associated with sub-channel 2, and so on. However, if a sub-channel is used for CDMA opportunistic signaling, the pilot values normally associated with this sub-channel are not used.

Within each sub-channel assigned to the CPE, the 4 pilot values shall be inserted following the pilot pattern described in Error: Reference source not found. Then complex data values shall be sequentially allocated to the remaining 24 logical subcarriers.

While the logical subcarriers in the two first sub-channels are allocated to static physical subcarriers as indicated in Table 219, interleaving shall be performed on the subcarriers in the remaining 58 sub-channels. The interleaved sequence shall be allocated sequentially to the 1624 remaining physical subcarriers in the symbol. In other words, each sub-channel is mapped to a group of 28 static physical subcarriers following the interleaving scheme described below.

The permutation rule L(k) used for interleaving shall be determined using the TLI algorithm described in Error: Reference source not found with the parameters {K,p,q,j}={1624,4,2,3} (as specified in Table 3). Index L(k) in the interleaver input sequence shall correspond to index k in the interleaver output sequence. Table 1 shows the relationship between the interleaver input sequence and the interleaver output sequence for sub-channels 3 and 4 with the pilot allocation shown for the first symbol of the upstream sub-frame. In the table, Pi represents the i’th pilot value, Dj,k represents the k’th data value in the group of 24 data values (obtained from the constellation mapper) allocated to sub-channel j.

- – Example of interleaving patterns in the upstream for the first symbol of the interleaved subchannels 3 and 4

| Sub-channel 3 |  |  | Sub-channel 4 |  |  |
| --- | --- | --- | --- | --- | --- |
| Input index | Value | Output index | Input index | Value | Output index |
| 0 | P0 | 341 | 28 | P4 | 1377 |
| 1 | D3,0 | 494 | 29 | D4,0 | 1530 |
| 2 | D3,1 | 647 | 30 | D4,1 | 59 |
| 3 | D3,2 | 800 | 31 | D4,2 | 212 |
| 4 | D3,3 | 953 | 32 | D4,3 | 365 |
| 5 | D3,4 | 1106 | 33 | D4,4 | 518 |
| 6 | D3,5 | 1259 | 34 | D4,5 | 671 |
| 7 | P1 | 1412 | 35 | P5 | 824 |
| 8 | D3,6 | 1565 | 36 | D4,6 | 977 |
| 9 | D3,7 | 94 | 37 | D4,7 | 1130 |
| 10 | D3,8 | 247 | 38 | D4,8 | 1283 |
| 11 | D3,9 | 400 | 39 | D4,9 | 1436 |
| 12 | D3,10 | 553 | 40 | D4,10 | 1589 |
| 13 | D3,11 | 706 | 41 | D4,11 | 118 |
| 14 | P2 | 859 | 42 | P6 | 271 |
| 15 | D3,12 | 1012 | 43 | D4,12 | 424 |
| 16 | D3,13 | 1165 | 44 | D4,13 | 577 |
| 17 | D3,14 | 1318 | 45 | D4,14 | 730 |
| 18 | D3,15 | 1471 | 46 | D4,15 | 883 |
| 19 | D3,16 | 1624 | 47 | D4,16 | 1036 |
| 20 | D3,17 | 153 | 48 | D4,17 | 1189 |
| 21 | P3 | 306 | 49 | P7 | 1342 |
| 22 | D3,18 | 459 | 50 | D4,18 | 1495 |
| 23 | D3,19 | 612 | 51 | D4,19 | 24 |
| 24 | D3,20 | 765 | 52 | D4,20 | 177 |
| 25 | D3,21 | 918 | 53 | D4,21 | 330 |
| 26 | D3,22 | 1071 | 54 | D4,22 | 483 |
| 27 | D3,23 | 1224 | 55 | D4,23 | 636 |

The subcarrier allocation in the upstream shall be performed using the following procedure:

1) All null subcarriers and the DC subcarrier are first allocated with 0 amplitude and 0 phase.

2) The 56 subcarriers of the two first sub-channels, which are usually reserved for "Ranging/BW requests/UCS notification", are allocated using the subcarrier indices given in Table 2 below and applying the offsets (-19, -11, +11, +19) cycled every four successive frames starting with –19 at the beginning of each superframe, or applying the offset specified by the Permutation base UCD channel IE if specified for the current frame as indicated in Error: Reference source not found. In the casewhere the US-MAP indicates that these two first sub-channels are assigned to the CPE for data transmission, the pilot subcarriers are logically allocated as shown in Error: Reference source not found as a function of the symbol order in the upstream sub-frame and then mapped to the appropriate static subcarrier indices; the remaining static subcarriers are then sequentially assigned with data values from the constellation mapper and then mapped to the appropriate static subcarrier indices.

3) For each sub-channel allocated to the CPE among the remaining 58 sub-channels, the 4 associated pilot values are logically inserted following the pilot pattern described in Error: Reference source not found as a function of the symbol order in the upstream sub-frame. Complex data values from the constellation mapper are then sequentially allocated to the remaining 24 logical subcarriers. Note that in the upstream, a CPE is assigned 7 or more symbols that can span one or more sub-channels, and data values are inserted into the same sub-channel on a symbol by symbol basis (over all assigned symbols) before the next sub-channel is used. If the US-MAP indicates that more than 2 sub-channels are assigned for “Ranging/BW request/UCS notification”, the logical subcarriers for these sub-channels shall be allocated accordingly.

4) All the subcarriers of the remaining sub-channels are allocated with 0 amplitude and 0 phase not allocated to the CPE.

5) The 1624 assigned logical subcarriers are interleaved using the TLI algorithm with parameters {K,p,q,j}={1624,4,2,3}.

6) The interleaved sequence is allocated sequentially to the remaining 1624 subcarriers.

- — Tone sets for Ranging/BW request/UCS Notification sub-channels (see Error: Reference source not found)

| Reference Tone index # |  |  |  |
| --- | --- | --- | --- |
| -814 | -508 | 49 | 519 |
| -813 | -503 | 111 | 536 |
| -811 | -502 | 176 | 560 |
| -807 | -465 | 224 | 594 |
| -799 | -456 | 272 | 644 |
| -778 | -414 | 307 | 693 |
| -747 | -377 | 335 | 713 |
| -712 | -334 | 378 | 748 |
| -692 | -305 | 415 | 779 |
| -643 | -271 | 457 | 800 |
| -593 | -223 | 466 | 808 |
| -559 | -175 | 503 | 812 |
| -535 | -110 | 504 | 814 |
| -518 | -48 | 509 | 815 |

- Interleaving parameters for the downstream subcarrier mapping allocation

|  | Interleaving depth | Interleaving parameters |  |  | Interleaving spreading L(k) (informative) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DS | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk=24 | Δk= 48 |
|  | 1440 | 32 | 2 | 3 | 557 | 286 | 572 | 522 | 336 |
| US | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk= 28 | Δk= 56 |
|  | 1624 | 4 | 2 | 3 | 743 | 138 | 388 | 532 | 560 |
| CBP burstSCH | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk=6 | Δk= 8 |
|  | 720 | 12 | 2 | 1 | 311 | 98 | 213 | 294 | 328 |
| CBP burst | K | p | q | j | Δk=1 | Δk=2 | Δk=4 | Δk=6 | Δk= 8 |
|  | 836 | 22 | 2 | 2 | 351 | 134 | 268 | 402 | 300 |