IEEE P802.22
Wireless RANs

| ARQ Parameter Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | US Army |  |  | Ranga.Reddy@us.army.mil |

Abstract

This contribution seeks to fill in values for Table 289 in Section 11.2, and text for Section 6.12.3.4 to resolve CID 324, 691, 692

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[-------------------------------------------------------Beginning of Text Proposal---------------------------------------------]

[Replace Table 289 in Section 11.2 with following]

- Table 289 — ARQ State Machine Parameters

| Entity | Name | Description | Length (octets) | Value |
| --- | --- | --- | --- | --- |
| BS, CPE | ARQ_BSN_MODULUS | See 6.12.3.1 | 2 | 2^11 |
| BS, CPE | ARQ_WINDOW_SIZE | See 6.12.3.2 | 2 | See Table 104, Section 6.10.8.9.18.2 |
| BS, CPE | ARQ_BLOCK_LIFETIME | See 6.12.3.3 | 2 | See Table 106, Section 6.10.8.9.18.4 |
| BS, CPE | ARQ_RETRY_TIMEOUT | See 6.12.3.4 | 2 | See Table 105, Section 6.10.8.9.18.3 |
| BS, CPE | ARQ_SYNC_LOSS_TIMEOUT | See 6.12.3.5 | 2 | See Table 107, Section 6.10.8.9.18.5 |
| BS, CPE | ARQ_PURGE_TIMEOUT | See 6.12.3.6 | 2 | See Table 108, Section 6.10.8.9.18.6 |
| BS, CPE | ARQ_BLOCK_SIZE | See 6.12.3.7 | 2 | See Table 109, Section 6.10.8.9.18.7 |

[Editor’s Note: In looking through ARQ parameters, contributor noticed that ARQ_RETRY_TIMEOUT also needs to be modified. This is brought up in CID 324. What follows is a modification to the resolution in current comment database for CID 324. The proposed text borrows from Section 6.4.3.4 and 11.13.17.3 of IEEE 802.16-2009]

[Modify the text for Section 6.12.3.4 as follows. Red strikeout text pertains to the original resolution modification proposed in CID 324. Blue text pertains to the added text provided as part of a modified resolution]

ARQ_RETRY_TIMEOUT is the minimum time interval a transmitter shall wait before retransmission of an unacknowledged block for retransmission. The interval begins when the ARQ block was last transmitted. This interval shall be used to compensate for delays on both transmitter and receiver side. On the transmitter side this includes time to transmit MAC PDUs and ARQ blocks. On the receiver side, this includes the amount of time to receive and process MAC PDUs as well as transmit ARQ feedback. The transmitter-side and receiver side delays are distinguished in Table 105, distinctly. These delays may include scheduling and propagation when the BS is the transmitter or the receiver. On connections that use both HARQ and ARQ, the ARQ_RETRY_TIMEOUT value shall be set accordingly to allow HARQ retransmission operation of the ARQ block to be completed before ARQ retransmission occurs. An ARQ block is unacknowledged if it has been transmitted but no acknowledgment has been received.

[-------------------------------------------------------End of Text Proposal---------------------------------------------]

References:

[1] Chounaird, Gerald, “WRAN Draft 3.0 Ballot Comments Database”, DCN# 22-10/78r7, IEEE P802.22 Working Group, https://mentor.ieee.org/802.22/dcn/10/22-10-0078-07-0000-wran-draft-3-0-ballot-comments-database.xls

[2] “IEEE Standard for Local and metropolitan area networks – Part 16: Air Interface for Broadband Wireless Access Systems”, IEEE Std 802.16-2009.