IEEE P802.22
Wireless RANs

| Proposed Modifications to Section 7.6.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems |  | 404-819-0314 | apurva.mody@baesystesms.com, apurva_mody@yahoo.com |
| Ranga Reddy | US Army - CERDEC |  |  | ranga.reddy@us.army.mil |

Abstract

The following document provides the proposed modifications to Section 7.6.7 in response to Comment 503 and Comment 504 from Tom Gurley and Winston Caldwell

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

    - 7.6.7 Signal Type Determination, Distributed Sensing and Policy Enforcement

      - General

The essential functions of the Security Sublayer 2 at the cognitive plane shall be to provide protection for the incumbents as well as protection to the 802.22 systems against DoS attacks of various types. Error: Reference source not found shows the security sublayer architecture for the Cogitive Plane. One of the most important mechanisms employed by this security sublayer is that of distributed spectrum sensing, signal type determination, correlation with geolocation information from various sensors (CPEs) located in the network and policy enforcement. This information from various sensors shall be combined in a certain way which is referred to as information fusion followed by policy enforcement. These steps enable the 802.22 systems to provide protection against some types of DoS attacks. Since this functionality is essentially a part of the Spectrum Manager, the signal type determination, information fusion, and policies are defined in Clause Error: Reference source not found, on Cogntive Radio Capability,[16] however a brief description is provided in the following section.

      - Signal Type Determination

The IEEE P802.22 systems shall have the capability to determine the signal type that have been detected as an outcome of spectrum sensing. The signal type determination is dependent on the regulatory domain requirements and has been specified in Clause 9 and normative Annex A. In some regulatory domains, the signal type determination through the use of spectrum sensing may be necessary since it determines the radio transmission behavioral control using various policies and procedures.

      - Distributed Sensing Mechanisms

Distributed spectrum sensing is a scheme wherein more than one Radio Frequency (RF) sensors are used to sense the spectrum. The local sensing information shall be combined with information from the incumbent database to see if any of the subscribers (CPEs) lie in the protected contour of an incumbent. This local sensing information is then passed on to a central node such as the BS which makes a decision on whether to make opportunistic usage of the spectrum.

Error: Reference source not found shows some comparisons of distributed spectrum sensing and information fusion to improve security in a network consisting of cognitive radios such as the one being considered for P802.22 systems. In Appendix E, various rules are considered for information fusion where it has been shown that rather than using simple OR or AND type rules, Voting based rules provide flexibility and accuracy needed to provide security protection in cognitive radio systems. However, in certain regulatory domains such as the US, only ‘OR’ rule based distributed sensing is allowed wherein even if one sensor detects and determines the signal type, the policies defined in Clause 9 for such a detected signal need to be enforced.

Distributed sensing may take into account the knowledge on the geographic location of the specific sensing devices providing sensing reports.  The correlation of the reports of the sensing devices that may have sent an “Urgent Coexistence Situation (UCS)” message to the BS and their respective geographic location may provide means to identify the characteristics of the sensed signal. This may improve the decision making to determine the presence of an incumbent and its type versus presence of other devices based on their expected extent of coverage.

In the case of DTV detection, the area where correlation will exist between sensing devices is likely to be large and will be in the direction of the DTV transmitter.  In the case of a wireless microphone, the correlation will exist over a more limited area.  This limited area correlation will also be used in the case of the detection of an 802.22.1 beacon and will help increasing the level of confidence of the decision that it is indeed an 802.22.1 beacon based on its expected area of coverage, and possibly reducing the need to rely on the full PPDU decoding.

Since all the information may be available at the BS, based on what is allowed in the regulatory domain requirements, it will be up to the manufacturers to implement various levels of complexity in fusing the sensing reports and the geographic location of the sensing devices to come up with a reliable assessment of the presence of incumbents, that is true positives, while minimizing the probability of false positives. Since incumbents shall be protected in all cases and in case of doubt, action shall be taken to avoid interference to incumbents, more advanced data fusion and decision processes will have the advantage of avoiding false positives and false alarms and therefore reducing cases where WRAN systems would have to change frequency while it is not necessary.

Policy enforcement through the use of signal type determination and distributed sensing will provide protection to the primary users of the spectrum since the probability of missed detection reduces with the number of sensors. Signal type determination and distributed sensing provide protection to the secondary users of the spectrum against DoS attacks since it is easy to fool one sensor into believing the state of the spectrum, however, it is difficult to fool many sensors located in proximal, but disparate geographical locations. However, the use of the distributed sensing schemes shall be used in accordance with the specified regulatory domain requirements.

      - Policy Enforcement and Radio Transmission Behavior Control

The IEEE P802.22 Systems shall control the radio transmission behavior via rule-based policy sets as defined in Clause 9. The detailed policy sets can be found in Section 9.2.5.