IEEE P802.22
Wireless RANs

| Modifications of Power Control Equation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-07-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-Joo Kim | ETRI | Korea | +82-42-860-1230 | cjkim@etri.re.kr |
| Byung-Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Jung-Sun Um | ETRI | Korea | +82-42-860-4844 | korses@etri.re.kr |

Abstract

To clarify the definition of , , and   in Eq. (186) of sub-clause 8.9.4.2, I propose the modifications of the corresponding sub-clauses as follows.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- I. Related comments

Regarding the Eq. (186) of sub-clause 8.9.4.2, we need to clarify the definition of  from the discussions in PHY ad-hoc group.  can be determined by the RNG-RSP MAC message for the last data burst. In case that the last burst is used for control signal,  is same as the EIRP density of the last data burst. It means that  is always specified by the RNG-RSP MAC message for the last data burst regardless of the type of the last burst. Moreover,  of  is confused with the number of subcarriers newly transmitted, . Thus  will be replaced by  over the whole 802.22 draft. The definition of  is newly added in Eq. (186) of sub-clause 8.9.4.2.

II. Proposed modifications of the 802.22 Draft v3.3

- Abbreviations and Acronyms

| AAA | Authentication, Authorization and Accounting |
| --- | --- |
| AES | Advanced Encryption Standard |
| AK | Authorization Key |
| ARQ | Automatic Repeat Request |
| BCC | Binary Convolutional Coding |
| BE | Best Effort |
| BS | Base Station |
| BSN | Block Sequence Number |
| BTC | Block Turbo Code |
| C-SAP | Control Service Access Point |
| CA | Certificate Authority |
| CBP | Coexistence Beacon Protocol |
| CC | Confirmation Code |
| CCM | CTR mode with CBC-MAC |
| CDMA | Code Division Multiple Access |
| CID | Connection Identifier |
| CINR | Carrier to Interference and Noise Ratio |
| CN | Channel Number |
| CNR | Carrier-to-Noise Ratio |
| CoS | Class of Service |
| CP | Cyclic Prefix |

.

.

.

- — Information elements

| Name | Element ID (1 byte) | Length (bytes) | Description |
| --- | --- | --- | --- |
| Ranging data ratio | 151 | 1 | Reduction factor, in units of 0.5 dB, between the EIRP per subcarrier used for this burst and the EIRP per subcarrier that should be used for CDMA Ranging. |
| Normalized CNR override | 152 | 7 | This is a list of numbers, where each number is encoded by one nibble, and interpreted as a signed integer. The number encoded by each nibble represents the difference in 0.5 dB from the normalized CNR indicated in Table 242. The last nibble shall be set to 0000. This proposes a vector for all the modulation and FEC levels whereas a UIUC only refers to one modulation and FEC level. Only one byte should be used to give only one difference value. |

      - Transmit Power Control mechanism

A power control algorithm shall be supported for the US channel with both an initialization procedure through initial ranging and a periodic adjustment procedure to be carried out without loss of data. The BS shall be capable of providing accurate power measurements of the received burst signal. This value shall then be compared against a reference level, and the resulting EIRP level per subcarrier to be used by the CPE shall be fed back to the CPE through the RNG-RSP MAC message (see Error: Reference source not found). The power control algorithm shall support EIRP adjustment as required (due to propagation loss and power fluctuations) at rates of up to 6 dB/s. The CPE shall adjust its transmit EIRP in response to a TPC command in the next scheduled upstream burst following receipt of the command. The power control algorithm at the BS shall take into account the various SNR requirements resulting from the different burst profiles while preventing violation of the emission masks (see Error: Reference source not found) and maximum EIRP levels (see Error: Reference source not found).

A transmitting CPE shall maintain the same transmitted EIRP density (EIRP per OFDMA subcarrier) regardless of the number of sub-channels assigned, unless the maximum allowable total EIRP level (EIRP per subcarrier multiplied by the number of subcarriers used at any particular time) is reached. In other words, when the number of active sub-channels allocated to a user is reduced, the total transmitted EIRP shall be reduced proportionally by the CPE, without additional power control messages (i.e., the gain of the RF transmission path is kept constant). When the number of sub-channels is increased, the total transmitted EIRP shall also be increased proportionally. However, the transmitted EIRP level shall not exceed the maximum allowable EIRP level in an entire TV channel as dictated by regulatory requirements (see Error: Reference source not found). The CPE shall interpret power control messages as the required transmitted EIRP density (EIRP per OFDMA subcarrier) as described above.

To maintain at the BS a received power density (received signal strength level (RSSL) per subcarrier) consistent with the modulation and FEC rate used by each CPE, the BS may change the EIRP density transmitted by the CPE, through the RNG-RSP message (see Error: Reference source not found), as well as the CPE-assigned modulation and FEC rate. There are, however, situations where the CPE should automatically update its transmitted EIRP density without being explicitly instructed by the BS. This happens when the CPE transmits in the region marked by UIUC = 2 to 7 (see Error: Reference source not found).  In all these situations, the CPE shall use a temporary transmitted EIRP density value set according to Equation (186) (in dBm).

(186)

where

is the maximum allowable transmitted EIRP on the current operating TV channel

is the temporary transmitted EIRP density (per subcarrier)

is the EIRP density (per subcarrier) indicated by the BS through the RNG-RSP MAC message

is the number of subcarriers transmitted

is the temporary total transmitted EIRP

is the normalized CNR of new modulation/FEC rate instructed by the UIUC

is the normalized CNR of the last used modulation/FEC rate

is the EIRP density (per subcarrier) indicated by the BS through the RNG-RSP MAC message

is the  specified by the RNG-RSP MAC message for the last used modulation/FEC rate

is the temporary transmitted EIRP density (per subcarrier)

Note that the “normalized CNR” corresponds to the subcarrier power over the noise power present in the subcarrier nominal bandwidth which corresponds numerically to the subcarrier spacing as indicated in Error: Reference source not found, expressed in dB. In other words, the “normalized CNR” corresponds to the subcarrier power density over the noise power density expressed in dB.

In all other situations, the CPE shall use the EIRP density value set according to Equation (187) (in dBm).

(187)

This resulting value  is updated based on the value  transmitted regularly to the CPE by the BS through the RNG-RSP MAC message (see Error: Reference source not found) to keep the TPC of the RF link up-to-date. The CPE shall be calibrated by the manufacturer so that the actual EIRP density per subcarrier transmitted by the CPE corresponds to the level indicated by the  variable resulting from the RNG-RSP message (within 0.5 dB).  The default normalized CNR values per modulation for the binary convolutional code (BCC), except for the CDMA code, are given in Table 242. These values may be overridden by the BS by using a dedicated UCD message (see Table 42). The second column is the default value and third column is informative and indicative of the modulation performance in a multipath channel.

- — Normalized CNR per modulation for BER= 2*10-4

| Modulation - FEC rate | Normalized CNR AWGN (default) | Normalized CNR Multipath Channel (informative) |
| --- | --- | --- |
| CDMA code | 1.2 | 5 |
| QPSK, rate: ½ | 4.3 | 8.1 |
| QPSK, rate: 2/3 | 6.1 | 11.6 |
| QPSK, rate: ¾ | 7.1 | 14.0 |
| QPSK, rate: 5/6 | 8.1 | 17.8 |
| 16-QAM, rate: 1/2 | 10.2 | 14.8 |
| 16-QAM, rate: 2/3 | 12.4 | 20.3 |
| 16-QAM, rate: 3/4 | 13.5 | 24.6 |
| 16-QAM, rate: 5/6 | 14.8 | 28.6 |
| 64-QAM, rate: 1/2 | 15.6 | 20.5 |
| 64-QAM, rate: 2/3 | 18.3 | 26.2 |
| 64-QAM, rate: 3/4 | 19.7 | 31.8 |
| 64-QAM, rate: 5/6 | 20.9 | 40.4 |

The CPE shall report, at registration, the maximum EIRP that it can achieve in the case of the transmission of a full multiplex (60 sub-channels) while still meeting the required RF mask (see Error: Reference source not found) and other performance limits set by the manufacturer.  The maximum EIRP achievable at the CPE (see Error: Reference source not found) may be reported in the CBC-REQ MAC message (see Error: Reference source not found).  This parameter may be used by the BS to determine the maximum EIRP density per subcarrier achievable as a function of the number of used subchannels allocated to the CPE for its upstream burst, noting that the PAPR tends to increase with the number of transmitted subcarriers in the burst. This parameter may also be used by the BS for optimal assignment of coding schemes and modulations and also for optimal allocation of subchannels. These algorithms are vendor-specific.

The maximum EIRP parameter is quantized in 0.5 dB steps ranging from –64 dBm (encoded 0x00) to 63.5 dBm (encoded 0xFF). Values outside this range shall be assigned the closest extreme.

For the periodic ranging, once a CPE sends a periodic ranging code and fails to receive a RNG-RSP (Error: Reference source not found), the CPE may adjust its EIRP for the transmission of subsequent periodic ranging codes up to EIRPIR_MAX (Error: Reference source not found).

              - wranIfBsOfdmaUsNormalizedCnrOverride

This is a list of numbers, encoded by a nibble and interpreted by a signed integer. The the nibbles are defined in section 8.10.3.2. The number encoded by each nibble represents the difference in normalized CNR relative to the previous one.

              - wranIfBsOfdmaUsNormalizedCnrValue

A signed integer that corresponds to the normalized CNR value in the previous line.

              - wranIfBsOfdmaUsNormalizedChSounding

Signed integer for the required CNR for Channel Sounding.

The multipath channel used for the calculations is defined on 6 paths as follows: excess delay: -3, 0, 2, 4, 7 and 11 μsec; relative amplitude: -6, 0, -7, -22, -16 and -20 dB; the phase for each path is random.  The delay, amplitude and phase are assumed to be constant over the period of one symbol.