IEEE P802.22
Wireless RANs

| Proposed Changes to Section 9.7.1 of Draft v3.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-08-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90035 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

This document contains the proposed changes to Section 9.7.1 of Draft v3.0.  The changes have been proposed in order to resolve comment numbers 673-675.

R1 adds a modification to the antenna information fields of primitive in 9.7.1.3.

R2 adds text to specify a length vector for the variable length character string variables in the primitives.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Wendong Hu> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

    - Database Service Primitives

The following list of messages, present in the Draft 802.22 Standard, defines the necessary messaging to support access to the database service by the BS.  The format described below shall be used for the messages sent directly to the database service as well as those received directly from the database service.  Some parameters in the following primitives are variable length character strings. The length of these parameters is given in terms of the number of characters in those strings. The total size of those parameters is the number of characters (the length) multiplied by the size of each character. For ASCII character sets, each character is 1 octet. For Unicode character sets, each character is 2 octets. Note that all variable length character strings shall be null terminated.

      - M-DB-AVAILABLE-REQUEST

M-DB-AVAILABLE-REQUEST: message that allows the BS to verify that it is connected to the database service in order to receive channel availability and maximum allowed EIRP updates.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Base station-ID Length | Integer | 2 octets | length of Base station-ID field (# of characters) |
| Base station-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 octets | length ot Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |
| Access Type | Integer | 1 byte | The value identifies the type of access being used for communication to the database. 0x00 = URL 0x01 = IPv4 IP address 0x02 = IPv6 IP address 0x03-0xFF = Reserved |
| Database Service Network Address { |  |  |  |
| If (Access Type = 0x00) |  |  |  |
| Database Service URL Length | Integer | 2 octets | length of database service url field (# of characters) |
| Database Service URL | Character String | Variable | A fully qualified URL starting with, http:// or https:// |
| Else If (Access Type = 0x01) |  |  |  |
| Database Service IPv4 Address | Integer | 4 bytes | IPv4 network address for database service |
| Else if (Access Type = 0x02) |  |  |  |
| Database Service IPv6 Address | Integer | 16 bytes | IPv6 network address for database service |
| Else (Access Type = 0x03 – 0xFF) |  |  |  |
| Reserved mechanism | Character String and/or Integer | Variable | Reserved to allow identification of other access mechanisms |
| } |  |  |  |
| Database port number | Integer | 2 bytes | Port number of the internet connection |
| Base Station Network Address { |  |  |  |
| If (Access Type = 0x00) |  |  |  |
| Database Service URL Length | Integer | 2 octets | length of database service url field (# of characters) |
| Database Service URL | Character String | Variable | A fully qualified URL starting with, http:// or https:// |
| Else If (Access Type = 0x01) |  |  |  |
| Database Service IPv4 Address | Integer | 4 bytes | IPv4 network address for database service |
| Else if (Access Type = 0x02) |  |  |  |
| Database Service IPv6 Address | Integer | 16 bytes | IPv6 network address for database service |
| Else (Access Type = 0x03 – 0xFF) |  |  |  |
| Reserved mechanism | Character String and/or Integer | Variable | Reserved to allow identification of other access mechanisms |
| } |  |  |  |
| Base station port number | Integer | 2 bytes | Port number of the internet connection |
| Timestamp Length | Integer | 2 octets | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string. |

      - M-DB-AVAILABLE-CONFIRM

M-DB-AVAILABLE-CONFIRM: message that allows the database service to confirm that the BS is still connected to the database service.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Base station-ID Length | Integer | 2 octets | length of Base station-ID field (# of characters) |
| Base station-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 octets | length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |
| Timestamp Length | Integer | 2 octets | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE-REQUEST. |

      - M-DEVICE-ENLISTMENT-REQUEST

M-DEVICE-ENLISTMENT-REQUEST: message that allows the BS to enlist with the database service a device that has joined its WRAN network.

| Name |  | Type |  | Length | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| Device Type |  | Integer |  | 1 byte | The value identifies the type of device obtained as part of its process to associate 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 2 0x03-0xFF = Reserved |  |
| Device-ID Length |  | Integer |  | 2 octets | length of Device-ID field (# of characters) |  |
| Device-ID |  | Character String |  | Variable | In US, this is FCC-ID |  |
| Serial Number Length |  | Integer |  | 2 octets | length of Serial Number field (# of characters) |  |
| Serial Number |  | Character String |  | Variable |  |  |
| Proxy Device-ID Length |  | Integer |  | 2 octets | length of Proxy Device-ID field (# of characters) |  |
| Proxy Device-ID |  | Character String |  | Variable | This element is the device ID for the device (most likely the controlling BS) that is acting as the proxy to the database. In US, this is FCC-ID |  |
| Proxy Serial Number Length |  | Integer |  | 2 octets | length of Proxy Serial Number field (# of characters) |  |
| Proxy Serial Number |  | Character String |  | Variable | This element is the serial number for the device (most likely the controlling BS) that is acting as the proxy to the database. |  |
| Location Data String Length |  | Integer |  | 2 octets | length of Location Data String field (# of characters) |  |
| Location Data String |  | Character String |  | NMEA 0183 | The value identifies the location of the device (latitude, longitude). |  |
| Responsible Party Name Length |  | Integer |  | 2 octets | length of Responsible Party Name field (# of characters) |  |
| Responsible Party Name |  | Character String |  | Variable |  |  |
| Antenna height |  | Integer |  | 1 byte | Antenna height above ground level in meters. |  |
| If (Device Type = 0x00 or 0x01) { |  |  |  |  |  |  |
| Contact Name Length |  | Integer |  | 2 octets | length of Contact Name field (# of characters) |  |
| Contact Name |  | Character String |  | Variable |  |  |
| Contact Physical Address field |  | Integer |  | 2 octets | length of Contact Physical Address field (# of characters) |  |
| Contact Physical Address |  | Character String |  | Variable |  |  |
| Contact Email Address length |  | Integer |  | 2 octets | length of Contact Email address length field (# of characters) |  |
| Contact Email Address |  | Character String |  | Variable |  |  |
| Contact Telephone Number Length |  | Integer |  | 2 octets | length of contact Telephone Number field (# of characters) |  |
| Contact Telephone Number |  | Character String |  | Variable |  |  |
| } |  |  |  |  |  |  |
| If (Device Type = 0x00) { |  |  |  |  |  |  |
| Access Type |  | Integer |  | 1 byte | The value identifies the type of access being used for communication to the database. 0x00 = URL 0x01 = IPv4 IP address 0x02 = IPv6 IP address 0x03-0xFF = Reserved |  |
| Base Station Network Address { |  |  |  |  |  |  |
| If (Access Type = 0x00) |  |  |  |  |  |  |
| Database Service URL Length |  | Integer |  | 2 octets | length of Database Service URL field (# of characters) |  |
| Database Service URL |  | Character String |  | Variable | A fully qualified URL starting with, http:// or https:// |  |
| Else If (Access Type = 0x01) |  |  |  |  |  |  |
| Database Service IPv4 Address |  | Integer |  | 4 bytes | IPv4 network address for database service |  |
| Else if (Access Type = 0x02) |  |  |  |  |  |  |
| Database Service IPv6 Address |  | Integer |  | 16 bytes | IPv6 network address for database service |  |
| Else (Access Type = 0x03 – 0xFF) |  |  |  |  |  |  |
| Reserved mechanism |  | Character String and/or Integer |  | Variable | Reserved to allow identification of other access mechanisms |  |
| } |  |  |  |  |  |  |
| Base station port number |  | Integer |  | 2 bytes | Port number of the internet connection |  |
| If (wranIfDatabaseServiceBSAntennaInformationSupportedMib) { |  |  |  |  |  |  |
| Antenna information |  | Character String |  | 72 bytes | Antenna directionality information of the device in dB relative to the main lobe maximum gain for every 5 degree azimuth clockwise starting from the direction of the maximum antenna gain expressed in unit of 0.25 dB over the range –63.75 dB (encoded 0x00) to 0 dB (0xFF). (to allow the database calculation of the channel availability and the maximum allowed EIRP values at the registering location). |  |
| Antenna rotation |  | Integer |  | 2 bytes | Antenna rotation in degrees clockwise from true North. |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| Timestamp Length |  | Integer |  | 2 octets | length of Timestamp field (# of characters) |  |
| Timestamp |  | Character String |  | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string. |  |

      - M-DEVICE-ENLISTMENT-CONFIRM

M-DEVICE-ENLISTMENT-CONFIRM: message that allows the database service to confirm to the BS that the new device has been successfully registered.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID Length | Integer | 2 octets | length of Device-ID field (# of characters) |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number length | Integer | 2 octets | length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |
| Timestamp Length | Integer | 2 octets | length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE-REQUEST. |

      - M-DB-AVAILABLE-CHANNEL-REQUEST

M-DB-AVAILABLE-CHANNEL-REQUEST: message that allows the BS to request a list of available channels and maximum allowed EIRP per channel from the database service for the specified type of device at the particular location.

| Name | Type |  | Length |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Device Type | Integer |  | 1 byte |  |  | The value identifies the type of device at the geo-location registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 2 0x03-0xFF = Reserved |
| Device-ID Length | Integer |  | 2 octets |  |  | length of Device-ID field (# of characters) |
| Device-ID | Character String |  | Variable |  |  | In US, this is FCC-ID |
| Serial Number length | Integer |  | 2 octets |  |  | length of Serial Number field (# of characters) |
| Serial Number | Character String |  | Variable |  |  |  |
| Location Data String Length | Integer |  | 2 octets |  |  | length of Location Data String field (# of characters) |
| Location Data String | Character String |  | NMEA 0183 Character String |  |  | The value identifies the location of the device (latitude, longitude). |
| Timestamp Length |  | Integer |  | 2 octets | length of Timestamp field (# of characters) |  |
| Timestamp |  | Character String |  | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string. |  |

      - M-DB-AVAILABLE-CHANNEL-INDICATION

M-DB-AVAILABLE-CHANNEL-INDICATION: message that is used to return to the BS the list of available channels as provided by the database service in the form of channel, maximum allowed EIRP, and availability schedule.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID Length | Integer | 2 octets | length of Device-ID field (# of characters) |
| Device- ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number length | Integer | 2 octets | length of Serial Number field (# of characters) |
| Serial number | Character String | Variable |  |
| Number of Channels Available | Integer | 1 byte |  |
| { If( Number of Channels Available > 0) |  |  | If the number of channels is equal to 0, this means that the device cannot operate. |
| For (i=1 to Number of Channels Available, i++) { Channel_Number Max_Allowed_EIRP (dBm) Availability schedule } | Vector of 2xN bytes and a number of pairs of NMEA 0183 $ZDA strings | Variable | List of available channel numbers and corresponding maximum allowed EIRP expressed in dBm over the range the range –64 dBm (encoded 0x00) to +63.5 dBm (encoded 0xFF) as well as the availability schedule (start and stop date/time) for each channel in Universal date and time system. |
| } |  |  |  |
| Status Message | Character String | Variable | Various status messages coming from the Database service (e.g. unapproved device flag) |
| Timestamp Length | Integer | 2 octets | length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE-CHANNEL-REQUEST. |

      - M-DB-DELIST-REQUEST

M-DB-DELIST-REQUEST: message that allows the BS to request the database service to remove the enlistment of a device that was associated with that base station.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID Length | Integer | 2 octets | length of Device-ID field (# of characters) |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number length | Integer | 2 octets | length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |
| Responsible Party Name Length | Integer | 2 octets | length of Responsible Party Name field (# of characters) |
| Responsible Party Name | Character String | Variable |  |
| Location Data String | Char | NMEA 0183 Character String | The value identifies the location of the device (latitude, longitude). |

      - M-DB-DELIST-CONFIRM

M-DB-DELIST-CONFIRM: message that is used to inform the BS whether its request to remove the enlistment of a device that was associated with that base station was successfully received and executed by the database service.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number | Character String | Variable |  |
| Responsible Party Name | Character String | Variable |  |
| Location Data String Length | Integer | 2 octets | length of Location Data String field (# of characters) |
| Location Data String | Character String | NMEA 0183 Character String | The value identifies the location of the device (latitude, longitude). |

References:

Note that this interface allows enlistment of TVBD devices (beyond 802.22 BS and CPE) that may not need to be formally  “registered” as required by the FCC R&O 08-260 for potential broader capability and applicability of the database service.

Antenna directionality will represent the antenna gain pattern in the horizontal plane in dB referred to the gain of its main lobe and it is assumed that the database service will use its knowledge of the geolocation of the base station and the device being enlisted to calculate the azimuth of the device antenna main lobe for interference calculations in the case of base station and CPE operation.  Omnidirectional antennas shall be assumed as the default.

Antenna directionality will represent the antenna gain pattern in the horizontal plane in dB referred to the gain of its main lobe and it is assumed that the database service will use its knowledge of the geolocation of the base station and the device being enlisted to calculate the azimuth of the device antenna main lobe for interference calculations in the case of base station and CPE operation.  Omnidirectional antennas shall be assumed as the default.

See footnote 3.

See footnote 3.