IEEE P802.22
Wireless RANs

| Updated On-demand Frame Contention Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave., Ottawa, Ontario, Canada K2H8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This document contains a proposal for reduced and re-arranged material on On-demand Frame Contention Protocol to be re-inserted in section 6.23.2.2 in the main body of the Standard based on the fact that the way by which the CBP IEs for frame contention need to be used by all the BSs needs to be standardized to have a workable and fair frame contention process.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Updated On-demand Frame Contention Protocol

A) Re-insert the following version of the On-demand Frame Contention Protocol in section 6.23.3.2.2 of the main body of the 802.22 Draft Standard.

6.23.3.2.2 On-demand Frame Contention Protocol

The On-demand Frame Contention protocol is used to resolve contentions of frame resource among the neighboring WRAN cells. It is assumed that when an operating BS switches from the normal mode of operation to the coexistence mode of operation, it initially occupies all 16 frames of the superframe.  Once in coexistence mode, the BS shall schedule at least one contention SCW to monitor potential frame requests from new overlapping WRAN cells. Those WRAN cells will then be able to schedule more SCWs (reservation-based or contention-based) as needed, as described in 6.23.1.2 and contend for frames used by the original WRAN cell by sending the appropriate CBP bursts during these SCWs.

Once these CBP bursts are received by the original BS or any of its associated CPEs which shall relay it to the BS, the content of these CBP bursts shall be decoded and the On-demand Frame Contention Protocol described in this sub-clause shall be applied to determine whether the distribution of the 16 frames of future superframes should be changed.

6.23.3.2.2.1 Top-level Procedure of the On-demand Frame Contention Protocol

On an on-going basis, each BS in coexistence mode shall execute the ODFC procedure shown in Figure 1 every time it receives an internal (intra-WRAN) or external (inter-WRAN) request for additional frames.

- Top-level Procedure of the On-demand Frame Contention Protocol

6.23.3.2.2.2 Frame Contention Procedure at the Frame Contention Source

Figure 2 shows the frame contention procedure that is followed by the contention source.

- Frame Contention Procedure at the Contention Source

6.3.2.2.3 Frame Contention Procedure at the Frame Contention Destination

Figure 3 shows the frame contention procedure that is followed by the contention destination.

- Frame Contention Procedure at the Contention Destination

6.23.3.2.2.4 Algorithm to identify the winner FC-SRC

The On-demand Frame Contention algorithm is used to resolve contentions of frame resource among the neighboring WRAN cells when the frame contention number (FCN) of the destination cell is not the smallest one (see Figure 3). For each frame that is contended for, the algorithm shall identify the frame contention source (FC-SRC) that has the smallest frame contention number. The algorithm can be implemented as follows:

On-Demand Frame Contention (N, WRAN, FCN, Frame)

1)  FCNwinner(Frame) ← MIN(FCN [1] : FCN [N])

2)  for i ← 1 to N

3)	if FCN [i]==FCNwinner(Frame)

4)	return WRANwinner(Frame) ← WRAN [i] ,

FCNwinner(Frame)

where in the algorithm:

N : total number of contending WRAN cells;

WRAN : the array of IDs of the contending WRAN cells, WRAN [i] for i ← 1 to N ;

FCN : the array of frame contention numbers of the contending WRAN cells, in which FCN [i] is the frame contention number of WRAN [i] for i ← 1 to N ;

Frame : the data frame (spectrum resource) being contended for;

FCNwinner(Frame): the winner frame contention number for accessing the Frame;

WRANwinner(Frame): the ID of the winner WRAN cell to access the Frame.

If the algorithm finds more than one smallest frame contention number, it shall select randomly one winner frame contention source amongst the FC-SRCs represented by these equal frame contention numbers.

6.23.3.2.2.5 Algorithm for Generation of Frame Contention Number

The frame contention number, FCN [i], for WRAN cell i, i.e., WRAN [i], shall be generated as follows:

FCN [i] = RANDOM (0, 2n-1)

where n = FCN_Range, an adjustable parameter appearing in Table 273.

The random number generator shall generate a uniform distribution from 0 to 2n-1.

B) Delete the Annex B as it now appears in Draft v4.0.

C) Change the variable Frame_Contention_Number in Table 273 for FCN_range

_________________________