IEEE P802.22
Wireless RANs

| Updated set of CDMA Ranging Tones |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave., Ottawa, Ontario, Canada K2H8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

In order to fit in 2 sub-channels, the upstream fine ranging burst needs to be limited to 56 subcarriers and, despite the resulting sub-sampling in the frequency domain, provide a time-domain function that is well localized.  Further search for optimal sub-carrier location has been conducted recently using a ‘genetic search algorithm’ and a better pattern has been found.  These results are used to update Table 206 of the 802.22 Draft.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Search for the best fixed CDMA subcarrier pattern
for the upstream fine ranging process

The 802.22 standard uses 1680 subcarriers spread over 60 sub-channels.  This represents a bandwidth of 5.624 MHz for the system operating in a 6 MHz channel.  The carrier spacing is 3.35 kHz resulting in a 300 μs symbol period (not counting the cyclic prefix). In the downstream, the PN-sequence used for the frame preamble modulates 820 subcarriers, that is one active carrier over two.  This allows a characterization of the channel response over 150 μs (beyond which aliasing in the time domain would occur).

On the upstream, the fixed CDMA subcarrier ranging burst is limited to the use of a minimum of 2 sub-channels corresponding to 56 subcarriers.  Most multipath in the TV bands has been found to fall within 25 μs for typical TV transmit and receive installations.  Similar channel characteristics are expected for larger coverage WRAN systems in the TV bands.  As a result, if the channel impulse response can be characterized over 30 μs, that is 1/10 of a full symbol length, this should be sufficient for all practical cases.  A 30 μs time span can be quantified in a WRAN system by taking one subcarrier over 10 in the frequency domain, thus 1680/10 = 168 subcarriers.

Since the minimum fixed CDMA carrier ranging burst is limited to 56 subcarriers, the problem is therefore to find the allocation of these 56 subcarriers over 168 that would give the best time domain localization to characterize the channel impulse response while avoiding aliasing.  In other words, one needs to find the allocation of 56 subcarriers over 168 for which the IDFT would give the narrowest practical main lobe while keeping all sidelobes as low as possible.

In order to make the problem more tractable, the number of possible subcarrier permutations was reduced by half by assuming that there would be a symmetry around DC (making the resulting time-domain waveform a ‘real’ waveform rather than ‘complex’).  This allowed a reduction of the number of permutations to to correspond to 28 subcarriers over 84.  A second simplification was made by limiting the frequency span of these CDMA carriers to half of the channel bandwidth, that is +/-1.5 MHz.  This resulted of course in a broadening of the main lobe by a factor of two but it was found that the fine ranging scheme described in Annex B of the 802.22 Draft can produce a time localization of channel multipath in excess of what is needed for proper device geolocation.  This resulted in a number of permutations corresponding to 28 over 42 subcarriers which, due to the resulting lower sub-sampling in the frequency domain, could keep the sidelobes which would appear as false echoes in the time domain at a much more reasonable level.

A systematic search was originally used at CRC on a 14/21 set of permutations to keep it within reasonable computation time (the resulting pattern being repeated by two to produce a 28/42 allocation) and the results formed the 56 fixed CDMA subcarrier allocation that was included in Table 206 of the Draft 802.22 r5.0. The performance of this winning permutation can be found in Figure 1.

Since then, a ‘genetic search’ algorithm[1] has been used by CRC to search for a superior subcarrier allocation over the 28/42 permutations and a better allocation has been found for which the performance is shown in Figure 2.  As can be seem, most sidelobes can be kept below 20 dB relative to the main lobe to provide a proper time-domain localization over 30 μs even if only 2 sub-channels over a total of 60 (i.e., 1/30 sub-sampling in the frequency domain) are used.

It is propose to update the Table of static CDMA subcarriers in Clause 8 of the 802.22 Draft standard to reflect this superior subcarrier allocation.

Figure 1: Relative amplitude of the time-domain channel impulse response of the current 56-carrier upstream burst (512 samples correspond to 30 μs)

Figure 2:	Relative amplitude of the time-domain channel impulse response of the improved 56-carrier upstream burst  (512 samples correspond to 30 μs)

Modify Table 206 to include the latest more optimum subcarrier allocation for the 56 static CDMA subcarriers used for fine ranging as shown in the Table below.

- Tone sets for Ranging/BW request/UCS Notification sub-channels
(see Error: Reference source not found)

| Reference Tone index # |  |  |  |
| --- | --- | --- | --- |
| -420 | -10 | 10 | 420 |
| -400 | -20 | 20 | 400 |
| -390 | -30 | 30 | 390 |
| -370 | -40 | 40 | 370 |
| -340 | -50 | 50 | 340 |
| -330 | -60 | 60 | 330 |
| -300 | -70 | 70 | 300 |
| -290 | -80 | 80 | 290 |
| -270 | -90 | 90 | 270 |
| -250 | -100 | 100 | 250 |
| -230 | -110 | 110 | 230 |
| -220 | -160 | 160 | 220 |
| -210 | -170 | 170 | 210 |
| -190 | -180 | 180 | 190 |

Reference:

[1]	Oktay Üreten and Selçuk Tascioglu, "Autocorrelation Properties of OFDM Timing Synchronization Waveforms Employing Pilot Subcarriers," EURASIP Journal on Wireless Communications and Networking, vol. 2009, Article ID 538978, 14 pages, 2009. doi:10.1155/2009/538978

_________________________________