IEEE P802.22
Wireless RANs

| Comments to the ECC Report on White Space Operation |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-11-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90035 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

This document contains the IEEE P802.22 comments on the ECC Report 159 on white space operation.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 1.

A minimum occupied bandwidth should be specified.  IEEE 802 believes that a 2.5 MHz minimum bandwidth requirement would ensure that WSDs provide broadband services and would help WSDs operate in a “fair” and sharing manner.

Comment 2.

Additional key parameters for sensing are the probability of detection and the probability of false alarm.

Comment 3.

The database system plan and operation should be augmented to allow online, near real time operation.  We believe that the database push functionality, as well as pull functionality, should be a requirement.  Many Internet transaction systems operate continuously, with redundant elements and very high reliability.  Our recommendation is to have each master mode device provide an Internet contact address, and the TV bands database push changes in channel availability to affected master devices. We believe that the 24 hour grace period for continued operation can lead to an extended duration of uncontrollable interference. In our recommendation, the requirement for daily contact with the TV bands database is replaced by the requirement for master mode devices to verify their Internet connectivity hourly or cease operation. There are a broad range of standards-based paging and messaging technologies available that the TV bands database could use to push messages that reflect changes in channel availability to master mode devices for near real time updates.

Comment 4.

The WSD should be required to supply a certification of identity in order to be granted operational access by the database.  Reciproclally the database should be required to supply a certification of identity in order for the WSD to become certain the WSD is being controlled by an officially approved database.

Comment 5.

The IEEE 802.22 standards project is designing a protocol that utilizes a combination of cognitive abilities.

WSDs connect in a point-to-multipoint architecture where the customer premises equipments (CPEs) are controlled by the cetralized base station (BS).  The BS not only controls the operational parameters of all its registered CPEs but also acts as a proxy to the database service.  A CPE reports its location information to the BS and the BS uses this information to query for available TV channels from the database on behalf of the CPE.  The BS manages TV channel selection for its network using an integrated cognitive brain – called the “spectrum manager.”

Each of devices on the network is capable of spectrum sensing.  The exact spectrum sensing capability employed is left open to implementation.  However, the BS keeps track of the statistical performance of the spectrum sensing capabilities.  These statistics along with the individual sensing measurments made by the individual devices on the network can be combined to accomplish distributed sensing.

The IEEE 802.22 standards project has recognized the great difficulty in protecting wireless microphone usage during its design and compatibility research.  Spectrum sensing is more suitable than database registration to protect wireless microphone usage due to the dynamic and unpredictable nature of on-location news and emergency reporting.  However, the detection range of the low-power signals is unacceptably shorter than the range of interference potential from a 1 W WSD.  The IEEE 802.22 standards project has accounted for this difficulty by publishing the IEEE 802.22.1 standard for enhanced protection of low-power devices.  IEEE 802.22.1 specifies a beaconing system that announces the TV channels in use by protected wireless microphones in an area.  The IEEE 802.22 protocol detects and reacts to the IEEE 802.22.1 beacon.

The IEEE 802.22 standards project has attained an approval rate of 100% of its Working Group membership.

References: