IEEE P802.22
Wireless RANs

| Modifcations to Parameter Table in 11.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-11-23 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | Self |  |  | ranga.reddy@ieee.org |

Abstract

This document contains modifactions to parameter table in 11.1

R0 is the initial version of this document

R2 is an amendment adding parameters that set the bound of T46, as well as deal with loss of synchronization to US/DS during CPE initialization

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Comments related to Parameter Table development in 22-10/174 for D5 (all T)

The following text is proposed to replace the current structure for Section 11.1. This section is being restructured in such a way to highlight min/default/max values for various parameters, timers, constants, message fields/IEs found throughout the MAC, Security, Coexistence, Cognitive Radio Capabilities, and PHY. These values will be used to build MIBs throughout the Sponsor Ballot process.

In Section 11.1.5, that describes Cognitive Radio Capability parameters, a few parameters are highlighted in green. The green text highlights parameters that were pulled from the existing table (272) in D5, that don’t correspond to timers/parameters provided by the Cognitive Radio Capability ad-hoc chair.

New Timer (Txx) numbering is provided that can be used to update figures in other parts of the draft.

[change the title for Clause 11 to “Parameter and Connection Management”]

[replace the text for 11.1 with the following]

[------------------------------------Start of text Modification---------------------------------------]

- 11.1	Parameters, Timers, Message IEs

This sections defines bounds for various parametes, timers, and message/IE fields that are specified throughout the draft.

11.1.1	MAC (Dynamic Service Flow, Multicast, ARQ, Capability, and Bandwidth Management)

- Table 11.1.1 – MAC Parameters, Timers, Message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| CPE, BS | DSx Request Retries | Number of Timeout Retries on DSA/DSC/DSD Requests | -- | 3 | -- |
| CPE, BS | DSx Response Retries | Number of Timeout Retries on DSA/DSC/DSD Responses | -- | 3 | -- |
| CPE | T6 | Wait for registration response | -- | -- | 3 s |
| CPE, BS | T7 | Wait for DSA/DSC/DSD Response timeout | -- | -- | 1 s |
| CPE, BS | T8 | Wait for DSA/DSC Acknowledge timeout | -- | -- | 300 ms |
| BS | T9 | Registration Timeout, the time allowed between the BS sending a RNG-CMD (success) to a CPE, and receiving a CBC-REQ from that same CPE | 300 ms | 300 ms | -- |
| CPE, BS | T10 | Wait for Transaction End timeout | -- | -- | 3 s |
| BS | T13 | The time allowed for an CPE, following receipt of a REG-RSP message to send a TFTP-CPLT message to the BS | 15 min | 15 min | -- |
| CPE | T14 | Wait for DSx-RSP/DSX-RVD Timeout | -- | -- | 200 ms |
| BS | T15 | Wait for MCA-RSP | 20 ms | 20 ms |  |
| CPE | T16 | Wait for bandwidth request grant | 10 ms | -- | Service QoS dependent |
| CPE | T18 | Wait for CBC-RSP timeout | -- | 5 ms | << T9 |
| CPE, BS | T22 | Wait for ARQ-Reset |  |  | 0.5 s |
| CPE | T26 | Wait for TFTP-RSP | 10 ms | 10 ms | 200 ms |
| BS | T27 as idle timer | Maximum time between unicast grants to CPE when BS believes CPE upstream transmission quality is good enough | CPE Ranging Response Processing Time |  |  |
| BS | T27 as active timer | Maximum time between unicast grants to CPE when BS believes CPE upstream transmission quality is not good enough | CPE Ranging Response Processing Time |  |  |
| BS | T28 | Time allowed for the BS to complete the transmission of the backup/candidate channel list to its CPEs after initial registration by a new CPE, including the database service query |  | 60 s |  |
| CPE | CBC Request Retries | Number of retries on CBC Request | 3 | 3 | 16 |
|  | DSx Flow Control | Maximum # of ongoing dynamic service flow (DSx) transactions that are ongoing | 1 | 4 | Infinite |
|  | MCA Flow Control | Maximum # of ongoing multicast group assignment (MCA-REQ/RSP) transactions | 1 | -- | Infinite |
|  | Max # of multicast groups | Maximum # of multicast groups the BS supports in a cell | 1 | -- | 511 |
| BS, CPE | T30 | CPE registration Timer (see 6.9.7.3.5 and 6.16.2.10) | 160 ms | 40.8 s | 10,485.6 s |
| BS, CPE | ARQ_BSN_MODULUS | Number of unique BSN values | -- | 2^10 | -- |
| BS, CPE | ARQ_WINDOW_SIZE | Max # of un-acknowledged ARQ blocks at a given time | -- | -- | ≤ (ARQ_BSN_MODULUS)/2 |
| BS, CPE | ARQ_BLOCK_LIFETIME | Max time interval an ARQ block shall be managed by the Tx ARQ state machine | 10 µs | -- | 655.36ms |
| BS, CPE | ARQ_RETRY_TIMEOUT (TRANSMITTER_DELAY/RECEIVER_DELAY) | Minimum time interval a transmitter shall wait before retransmission of a unacknowledged block | 10 µs | -- | 655.36ms |
| BS, CPE | ARQ_SYNC_LOSS_TIMEOUT | Max amount of time ARQ_TX_WINDOW_START or ARQ_RX_WINDOW_START shall be allowed to remain at the same value before declaring a loss of synchronization of the sender and receiver state machines for an ongoing transfer. | 10 µs | -- | 655.36ms |
| BS, CPE | ARQ_RX_PURGE_TIMEOUT | Time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START | 10 µs | -- | 655.36ms |
| BS, CPE | ARQ_BLOCK_SIZE | Size of ARQ block that SDU is fragmented into | 1 octet | -- | 2040 octet |
| BS | Max CPE Transmit EIRP | Maximum CPE Transmit EIRP as negotiated during registration | -64 dBm | -- | +63.5 dBm |
| CPE | Registration Request Retries | Number of retries on registration requests | 1 |  | 3 |
| CPE | Request Retries | Number of retries on bandwidth allocation requests | 16 |  |  |

11.1.2	PHY (Initialization, Operation, and DS/US synchronization)

- Table 11.1.2 – PHY Parameters, Timers, Message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| BS | DCD Interval | Time between transmission of DCD messages |  |  | 10 s |
| BS | UCD Interval | Time between transmission of UCD messages |  |  | 10 s |
| BS | UCD Transition | The time the BS shall wait after repeating a UCD message with an incremented Configuration Change Count before issuing a US-MAP message referring to Upstream_Burst_Profiles defined in that UCD message | 2 MAC frames |  |  |
| BS | DCD Transition | The time the BS shall wait after repeating a DCD message with an incremented Configuration Change Count before issuing a DS-MAP message referring to Downstream_Burst_Profiles defined in that DCD message | 2 MAC frames |  |  |
| BS | Initial Ranging Interval | Time between Initial Ranging opportunities assigned by the BS |  |  | 2 s |
| BS | CLK-CMP Interval | Time between the clock compare measurements used for the generation of CLK-CMP messages. | 50 ms | 50 ms | 50 ms |
| CPE | Lost DS-MAP Interval (T56) | Time since last received DS-MAP message before downstream synchronization is considered lost |  |  | 600 ms |
| CPE | Lost US-MAP Interval (T57) | Time since last received US-MAP message before upstream synchronization is considered lost |  |  | 600 ms |
| CPE | Lost SCH (T58) | Number of SCH that can be lost until synchronization is considered lost |  |  | 15 |
| CPE | CDMA Ranging Retries | Number of retries on CDMA Ranging Requests | 1 |  | 4 |
| CPE, BS | Invited Ranging Retries | Number of retries on inviting Ranging Requests | 16 |  |  |
| BS | US-MAP Process Time | Time provided between arrival of the last bit of a US-MAP at a CPE and effectiveness of that map | 5 symbols |  |  |
| BS | CPE Ranging Response Processing Time | Time allowed for a CPE following receipt of a ranging response before it is expected to reply to an invited ranging request | 10 ms |  |  |
| CPE | T1 | Wait for DCD timeout |  |  | 5 * DCD interval maximum value |
| CPE | T2 | Wait for broadcast ranging timeout |  |  | 5 * ranging interval |
| CPE | T3 | Ranging Response reception timeout following the transmission of a Ranging Request |  | 200 ms | 200 ms |
| CPE | T4 | Wait for unicast ranging opportunity. If the pending-until-complete field was used earlier by this CPE, then the value of that field shall be added to this interval. | 1 s | 30 min. (fixed) 10 min. (portable | 30 min. |
| BS | T5 | Wait for Upstream Channel Change response |  |  | 2 s |
| CPE | T12 | Wait for UCD descriptor |  |  | 5 * UCD Interval maximum value |
| CPE | T20 | Time the CPE searches for preambles on a given channel | 2 MAC frames |  |  |
| CPE | T21 | Time the CPE searches for DS-MAP on a given channel |  |  | 10 s |
| BS | EIRPbs | EIRP of BS (DS) | -64 dBm |  | 63.5 dBm |
| BS | TTG | Transmit/Receive Transition Gap | 105 µs | 210 µs | 333 µs |
| BS | DIUC Mandatory Exit Threshold | CINR at or below which this DIUC can no longer be used and where change to a more robust DIUC is required. | -64 dB |  | +63.5 dB |
| BS | DIUC Mandatory Entry Threshold | The minimum CINR required to start using this DIUC when changing from a more robust DIUC is required | -64 dB |  | +63.5 dB |
| BS | Boosting | Boosting applied to a DS allocation | -12 dB | 0 dB | +9 dB |
| BS, CPE | BW Request Backoff Start | Initial size of BW Request opportunity used by CPEs to contend to send BW requests to BS | 0 | -- | 15 |
| BS, CPE | BW Request Backoff End | Final size of BW Request opportunity used by CPEs to contend to send BW requests to BS | 1 | -- | 15 |
| BS, CPE | UCS Notification Backoff Start | Initial backoff window size in units of UCS Notification opportunity used by CPEs to contend to send UCS notifications to BS. | 0 | -- | 15 |
| BS, CPE | UCS Notification Backoff End | Final size of UCS Notification opportunity used by CPEs to contend to send UCS Notification to BS | 1 | -- | 15 |
| BS, CPE | Contention-based reservation Timeout | Number of US-MAPs to receive before contention-based reservation is attempted again for the same connection | 1 | -- | 255 |
| BS, CPE | BW Request opportunity size | Size (in OFDM slots) of PHY bursts, that a CPE may use to format and transmit a bandwidth request message in a conention request opportunity. | 1 | -- | 255 |
| BS, CPE | UCS Notification request opportunity size | Size (in OFDM slots) of PHY bursts, that a CPE may use to transmit a UCS Notification. | 1 | -- | 255 |
| BS, CPE | # of initial ranging codes | Number of initial ranging CDMA codes (N) | 1 | -- | 255 |
| BS, CPE | # of periodic ranging codes | Number of periodic ranging CDMA codes (M) | 1 | -- | 255 |
| BS, CPE | # of bandwidth request codes | Number of bandwidth request CDMA codes (L) | 1 | -- | 255 |
| BS, CPE | # of UCS notification codes | Number of UCS notification CDMA codes (I) | 1 | -- | 255 |
| BS, CPE | Start of CDMA codes group | Indicates the starting number, S, of the group of codes used for the US. | 0 | See 6.10.3 | 255 |
| BS, CPE | EIRP Density Level | EIRP Transmitted per subcarrier | -104 dBm | -- | +23.5 dBm |
| BS, CPE | EIRP Control | EIRP per subcarrier that the CPE should apply to correct its’ current transmission EIRP | -104 dBm | -- | +23.5 dBm |
| BS | EIRP Per subcarrier | EIRP transmitted per subcarrier | -104 dBm | -- | +23.5 dBm |

11.1.3	Coexistence

- Table 11.1.3 – Coexistence Parameters, Timers, Message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| BS | Tse (T33) | Time between transmission of the broadcast message of the operating backup and candidate channel sets for the purpose of spectrum etiquette. |  |  | 60 s |
| BS | T32 | Wait for the Frame Contention Response message | 1 superframe |  | 32 superframes |
| BS | FCW | Frame Contention Window: number of superframes during which a BS (FC_DST) accumulates the frame contention requests before reacting to it and responding to the FC_SRC’s. | 0 superframe |  | 16 superframes |
| BS | SCWBackoff_Max | Integer denoting the maximum superframes for the SCW backoff window |  |  |  |
| BS | Frame_Contention_Min | Number of frames not available for contention at a BS in a coexistence situation | 0 | 2 | 8 |
| BS | FCN_range | Exponent in base 2 defining the extent of the range of the random number FCN | 4 |  | 16 |
| BS | SF_release | Number of super-frame after which the BS releases the frames wan by another BS through frame contention |  | 5 |  |
| CPE | TCBP (T34) | The minimum time between transmission of a CBP packet carrying its MAC address for identification by nearby CPEs and BSs for coexistence purpose, as well as by spectrum monitoring systems to identify potential interference situations. Timing value may depend on the regulatory domain where the WRAN system operates (see Anenx A) | 8 s |  | 15 min |
| BS | SCWBackoff_Timer (T35) | Backoff timer that controls exiting or continuation of Frame contention procedure | -- | -- | SCWBackoff_Max |

11.1.4	Security

- Table 11.1.4 – Security Parameters, Timers, Message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| CPE | EAP Authentication Timer (T36) | Timeout period between sending SCM EAP-Start or EAP-Transfer (Subclause 7.2.2.5) | 2 s | 10 s | 30 s |
| CPE | Authentication Grace Timer (T37) | Amount of time after authentication is complete that must pass before re-authentication is complete (Subclause 7.2.2.5) | 5 min (300 s) | 10 min (600 s) | 35 days (3,024,000 s) |
| CPE | Max #of Authentication Attempts | Maximum # of Times a CPE is allowed to attempt EAP Authentication (Subclause 7.2.2.5) | 1 | 5 | 10 |
| CPE | Operational Wait Timeout (T38) | Timeout period between sending of Key Request messages from the Op Wait state (Subclause 7.2.3.2.4) | 1 s | 1 s | 10 s |
| CPE | Rekey Wait Timeout (T39) | Timeout period between sending of Key Request messages from the Rekey Wait state (Subclause 7.2.3.2.4) | 1 s | 1 s | 10 s |
| CPE | GTEK/TEK Grace Time (T40) | Time interval, in seconds before the estimated expiration of a GTEK/TEK (Subclause 7.2.3.2.4) | 5 min (300 s) | 1 h (3600 s) | 3.5 days (302,400 s) |
| BS | AK Lifetime | Lifetime BS assigns to new AK | 1 day (86,400 s) | 7 days (604,800 s) | 70 days (6,048,000 s) |
| BS | TEK Lifetime | Lifetime BS assigns to new TEK | 30 min (1,800 s) | 12 h (43,200 s) | 7 days (604,800 s) |
| BS, CPE | SCM Flow Control | The maximum number of concurrent SCM transactions | 0 (Default: unlimited # of transactions | -- | 255 |
| BS, CPE | Number of Supported Security Associations | The maximum # of supported security associations | 2 | -- | 2+m where m is the number of multicast groups |
| BS, CPE | PN_WINDOW_SIZE | Window that defines the acceptable PNs for received PDUs that are to be processed by encryption/decryption process | 16 | -- | 512 |
| BS | T17 | Time allowed for CPE to complete CPE Authorization and Key Exchange | 5 min | 5 min |  |

11.1.5	Cognitive Radio Capabilities (SM, SSA, Incumbent Protection, QP Management)

- Table 11.1.5 – Cognitive Radio Capability Parameters, Timers, Message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| CPE | T19 | Time DS-channel remains unusable |  |  |  |
| CPE | T29 | Wait for BLM-ACK timeout | 10 ms | -- | 300 ms |
| BS | T31 | Wait for BLM-REP timeout | 1 MAC Frame |  |  |
| CPE | BLM-REP Retries | Number of retries allowed for sending BLM-REP |  | 3 |  |
| BS, CPE | Channel Availability Check Time | The time during which a TV channel SHALL be checked for the presence of licensed incumbent signals having a level above the Incumbent Detection Threshold prior to the commencement of WRAN operation in that channel, and in the case of TV, a related channel at an EIRP level that can affect the measured TV channel. |  | 30 s |  |
| BS, CPE | Non-Occupancy Period | The required period during which WRAN device transmissions SHALL NOT occur in a given TV channel because of the detected presence of an incumbent signal in that channel above the Incumbent Detection Threshold, or in the case of TV, above a given EIRP level. | 10 min |  |  |
| BS, CPE | Channel Detection Time | The maximum time taken by a WRAN device to detect a licensed incumbent signal above the Incumbent Detection Threshold within a given TV channel during normal WRAN operation. |  | <=2 s to >=90% Probability of Detection with a False Alarm rate of <=10% |  |
| BS, CPE | Channel Setup Time | The window of time that may be taken by a WRAN CPE to transmit control information to a WRAN base station in order to establish operation with that base station at the prescribed power or, in the case of TV, at or below the allowable EIRP within a given TV channel. |  | 2 sec |  |
| BS, CPE | Channel Opening Transmission Time (Aggregate transmission time) | The aggregate duration of control transmissions by WRAN devices during the Channel Setup Time, which starts at the end of the Channel Availability Check Time |  | 100 ms |  |
| BS, CPE | Channel Move Time (In-service monitoring) | The time taken by a WRAN system to cease all interfering transmissions on the current TV channel upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold, or in the case TV, to alternatively reduce its EIRP to that which is allowable within a given TV channel upon detection of a TV signal in the same or a related channel |  | 2 s |  |
| BS, CPE | Channel Closing Transmission Time (Aggregate transmission time) | The aggregate duration of control transmissions by the WRAN devices during the Channel Move/EIRP Reduction Time, which starts upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold |  | 100 ms |  |
| BS, CPE | Channel Number | The channel number which is to be sensed by the SSF (SSF Subclause 9.4) | 0 |  | 255 |
| BS, CPE | Channel Bandwidth | The bandwidth of the channel to be sensed by the SSF (SSF Subclause 9.4) |  | 6/7/8 MHz (depending on regulatory domain) |  |
| BS, CPE | Maximum Probability of false alarm | In sensing modes 0 and 1 this value specifies the maximum probability of false alarm for each sensing mode decision in the signal present array | 0.0 |  | 0.255 |
| BS, CPE | NumSensingPeriods | Number of sensing periods field in a Sensing Window Specification Array entry | 0 | 1 | 127 |
| BS, CPE | SensingPeriodDuration | Duration of sensing period field (in units of OFDM symbols) in a Sensing Window Specification Array entry | 0 | 16 | 1023 |
| BS, CPE | SensingPeriodInterval | Periodicity of Sensing period field in a Sensing Window Specification Array entry | 0 | 200 | 2047 |
| BS, CPE | Sensing Window Specification Array | Array containing Sensing Window specification for each signal type enumerated in Signal Type Array | 1*sizeof(NumSensingPeriods+SensingPeriodDuration+SensingPeriodInterval) | -- | 32*sizeof(NumSensingPeriods+SensingPeriodDuration+SensingPeriodInterval) |
| BS, CPE | Signal Type Array | Bitmap that indicates which signal types are to be sensed for in a given regulatory domain |  | Set on a regulatory domain-by-domain basis (see Annex A, Table 236) |  |
| BS, CPE | Sensing Mode | The sensing mode a CPE supports. Negotiated during CPE initialization. Various modes are described in Table 238. |  | No Sensing |  |
| BS, CPE | Signal Present Decision | Indicaiton of whether or not a signal of a specific type has been detected. All unused values are reserved | 0x00 (Absent) | 0x7F (No Decision could be made) | 0xFF (Signal Present) |
| BS, CPE | Confidence Metric | Confidence with which sensing can determine the signal type | 0x00 (No Confidence) | -- | 0xFF (Full confidence) |
| BS, CPE | Mean RSSI | Mean of M RSSI measurements | -104 dBm | -- | +23.5 dBm |
| BS, CPE | Standard Deviation of RSSI | Standard deviation of the M RSSI measurements | +0.0 dB | -- | +25.5 dB |
| BS, CPE | RSSI_detection_threshold | Energy detection threshold indicating the presence of an incumbent, other WRAN system, or interference | -120 dBm | -- | -10 dBm |
| BS, CPE | Microphone Proection Radius | Radius of the contour within which the WRAN system cannot operate due to potential interference with the microphone | 0.1 km | -- | 100 km |
| BS, CPE | TCandidate_Channel_Refresh (T41) | Maximum time interval allowed before sensing is performed on the candidate channel to ensure that no incumbents are detected. | 1 s | 6 s | 10 s |
| BS, CPE | TBackup_Channel_Refresh (T42) | Maximum time interval allowed before sensing is performed on the backup channel to ensure that no incumbents are detected. | 1 s | 6 s | 10 s |
| BS, CPE | TCandidate_to_Backup_Transition (T43) | Minimum time duration without detection of any incumbent for a candidate channel to transition to the backup channel. | 1 s | 30 s | 100 s |
| BS, CPE | TCh_Move (T44) | Maximum time to ensure that the channel move information is successfully conveyed to all the associated CPEs and BS (self coexistence mode). | 1 s | -- | 10 s |
| BS, CPE | TNo_DB (T45) | Maximum WRAN operation time without access to the incumbent database service | 0.1 hr | -- | 72 hrs |
| BS, CPE | TWait_Before_Channel_Move (T46) | Waiting time before which the BS moves to a given operating channel of the backup channel. This is used to make sure that all the CPEs have moved to a backup channel before BS switches operation to the backup channel. | T46_min | -- | T46_max |
| BS, CPE | T46_min | Maximum allowable value that T46 can be set to |  | 100 ms |  |
| BS, CPE | T46_max | Minimum allowable value that T46 can be set to |  | 1 ms |  |
| BS, CPE | TRefresh_Database_Info (T47) | The prescribed time by the WRAN operator to refresh the incumbent database service. | 0.1 hr | -- | 72 hrs |
| BS, CPE | TClear_N (T48) | Lapser Timer keeps track of whether the Operating Channel N has been cleared using spectrum sensing | 0.1 s | -- | 60 s |
| BS, CPE | TClear_N-1 (T49) | Lapser Timer keeps track of whether the Adjacent Channel N-1 has been cleared using spectrum sensing | 0.1 s | -- | 60 s |
| BS, CPE | TClear_N+1 (T50) | Lapser Timer keeps track of whether the Adjact Channel N+1 has been cleared using spectrum sensing | 0.1 s | -- | 60 s |
| BS, CPE | TLoss_of_BS_Contact (T51) | Initiated when the CPE loses contact with the BS | 1 s | -- | 600 s |
| BS, CPE | TRange1 (T52) | Used for terrestrial geolocation. Initiated when the downstream burst leaves the BS (i.e., at the start of the frame preamble). | 1 TU | -- | 1000 TU |
| BS, CPE | TINsens (T53) | The parameter TINsens is used to verify that in-band sensing has been done within the required In-service monitoring period. The TINsens parameters is driven by the regulatory domain requirements. See Annex A. | 0.1 s | -- | 60 s |
| BS, CPE | TOUTsens (T54) | The parameter TOUTsens is used to verify that out-of-band sensing has been done within the required "Acquiring a channel monitoring period" specified in Annex A, Table 285 (30 s in the US). This value would be used to either initialize a ''lapse timer'' for each channel in the backup/candidate list at each CPE so that it is compared to Tsens | 0.1 s | -- | 60 s |
| BS, CPE | Tsens (T55) | Tsens parameter corresponds to the maximum length of time required to carry out the sensing process. Manufacturers need to specify the sensing time required to detect the specified signals with required accuracy. | 1 ms | -- | 160 ms |
| BS, CPE | IETF Country code | 2 character, ASCII string denoted the regulatory domain of operation (e.g. “US” is for United States” | -- | 2 characters | -- |
| BS, CPE | TACBP | Timing Advance of the CBP burst (see step 2 of 9.5.2.3) (Note that the geolocation process will have to pre-adjust TACBP depending on the distance between the two CPE to be geolocated and the reference CPEs so that the delays measured by Vernier3 fall within the symbol cyclic prefix (e.g., 74.68 usec corresponding to 22.4 km). | -1024 TU | -- | +1024 TU |

[------------------------------------End of text Modification---------------------------------------]

References: