IEEE P802.22
Wireless RANs

| RP Edits to Draft v1.0 to Resolve Comment #25 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-01-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90035 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

This document contains edits to the RP Draft v.1.0 to resolve comment #25 of comment database IEEE 802.22.2-10/161r0.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

    - Characteristics

Use a coverage and interference prediction model for the planning of a WRAN service that incorporates at least the following:

- Point-to-point propagation model.

- Desired to undesired protection ratios (D/U) for the various channel relationships considered should be selected from a scientific reference that provides results from analysis examining the specific modulations used by both the desired and undesired signals.

- Transmitting and receive antenna pattern characteristics for the incumbent, BS, and CPEs.

- Height of the transmitting antenna Above Ground Level (AGL).

- 30 meter terrain data.

- Population from current census data.

- K-factor and other atmospheric effects on signal fading.

- Ground cover and surface of the Earth electrical characteristics.

- Ground clutter (trees, urban buildings, etc).

- Long term fading effects, such as those described in Environmental Science Services Administration (ESSA) Technical Report ESSA Research Laboratories 79-Institute for Telecommunication Sciences 67.

      - Point-to-Point Propagation Model

A point-to-point propagation model determines the energy loss between a point where a transmitter might be located and a point where a receiver might be located.  A point exists in a cell which is the unit of a grid which is created by subdividing a larger area to be analyzed.  Each energy loss determination between two points is a link.  Typically a transmitter is positioned at one point while it is assumed that a receiver is located in each cell in the grid.  Over the entire area to be analyzed, each cell is independent and its propagation link is valued by its energy loss from the point where the transmitter is located.

The point-to-point propagation model is significantly more precise than empirical propagation models based on field test measurements.  The estimations and approximations inherent to empirical propagation models decrease processing time but increase the chances of inaccurate propagation predictions.  Popular empirical propagation models are outdated and processing power is more readily available to perform more complex point-to-point propagation modeling.  A systems planner should invest in any additional processing resources in order to gain as much precision as possible in order to ensure that the invested service operates efficiently and without harmful interference.

      - Desired to Undesired Protection Ratios (D/U)

D/U ratios express the desired field strength above any undesired field strength in dB that is required in order to avoid harmful interference and to allow acceptable reception of an RF service.  The desired field strength is that which is produced by the transmitter providing the desired RF service while the undesired field strength in that which is produced by a potentially interfering signal.  The D/U ratio is measured at the antenna terminals of a receiver.

D/U ratios should be selected depending on the channel relationship between the desired and undesired signals.  For example, D/U ratios are usually defined for co-channel and adjacent channel relationships.  D/U ratios are also often defined for channel relationships that can extend out to +/- 15 channels of separation between the desired and undesired signals.  D/U ratios are specified to account for any interference mechanism, including co-channel interference, out-of-band leakage, and higher-order inter-modulation interference.

D/U ratios will vary depending on the modulation used for the desired signal and the modulation used for the undesired signal.  The D/U ratios should be determined through compatibility testing in the laboratory using the specific modulations.  One modulation can be a more aggressive interferer than another modulation and the D/U ratio should reflect this difference.

      - Antenna Pattern Characteristics

More data that can be determined about the characteristics of the antennas being used in the coverage and interference prediction model will result in more precision in the predictions.  A system planner should use antenna data for the BS, the CPEs, neighboring WRANs, and incumbents.  A system planner should collect antenna data for each transmitting antenna and receiving antenna, which might be different for the same radio.  Use antenna data for the azimuth pattern and the elevation pattern for both horizontal and vertical polarizations.  Azimuth pattern data is often available for numerical gain values in degree increments from 0 to 359 degrees.  Elevation pattern data is often available in quarter degree increments from 90 to -90 degrees.  The system planner should also take in to account any antenna rotation, electrical tilt, or mechanical tilt that is being used.  Not considering the detailed data of the antennas will result in less accurate predictions and possible harmful interference.

      - Antenna Height (AGL)

The height above ground level of the transmitting and receiving antennas used by the BS, CPEs, neighboring WRANs, and incumbents is important to include in the coverage and interference prediction calculations to ensure accuracy.  The heights should be within 1 m of accuracy.

      - Terrain Data

Terrain data for at least the entire geographic area that is being analyzed is essential to include in the coverage and prediction analysis.  Terrain is one of the most influential factors in a propagation analysis, especially in mountainous or hilly regions.  The system planner should use terrain data with a resolution of at least 30 m.

      - Population Data

In order to translate propagation results into population served or affected by interference, the system planner should include population data.  Population data can be obtained from my populous areas of the world through census data.  Since population density can change significantly over several years, the most current census should be used in a population count.

      - K Factor

K factor is surface refractivity that a system planner should apply to each link analyzed in the point-to-point propagation model.  K factor is only relevant to those links where the transmitter and receiver are beyond LOS.  K factor is used to determine the amount of tropospheric scatter loss experienced in the link.

A system planner should determine the K factor for the geographic region in which the WRAN service deployment is planned and should use a propagation model that uses K factor to calculate accurate losses due to tropospheric scatter.

      - Ground Cover and Surface of the Earth Electrical Characteristics

Radio transmissions can propagate differently depending upon the surface of the Earth electrical characteristics over which they are traveling.  The propagation model should account for and make use of the conductivity and permittivity of the surface by determining the type of ground cover that is present.  The conductivity and permittivity – and therefore radio propagation – will be different depending upon whether the ground cover is: sea water, fresh water, wet ground, medium dry ground, very dry ground, or ice.  Conductivity and permittivity values for the types of ground cover listed above are provided by sources such as CCIR Report 236-6 [B2] and ITU-R P.527-3 [B3].

      - Ground Clutter

Depending on the type of ground clutter present upon an area over which they are propagating, the field strength of radio transmissions might be attenuated by a varying amount.  The common practice is to set a height above ground level trigger for each type of ground clutter along with an attenuation value in dB for a range of operating frequencies.

In the areas where the path profile between transmitter and receiver drops below the height trigger for the ground clutter type that is present, the field strength is attenuated the number of dBs listed for the ground clutter type at the operating frequency.  Ground clutter information can be attained from the USGS [B4].

      - Long Term Fading Effects

Long term fading effects cause median fade variability on a radio transmission link at frequencies in the TV band.  The total loss in the link should be properly adjusted by the median long term fade variability.

A system planner should determine the median fade variability using the calculation procedure give in ESSA ERL 79 – ITS 67 [B5] and providing to the calculation procedure the path length, the height of the center of radiation for both transmitter and receiver, the operating frequency, the climate type, and the desired percent availability.

References: