IEEE P802.22
Wireless RANs

| RP Edits to Resolve Comment #33 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Winston Caldwell | Fox | 10201 W. Pico Blvd. Los Angeles, CA 90035 | 310-369-4367 | Winston.caldwell@fox.com |

Abstract

This document contains edits to RP Draft v1.0 in order to resolve comment #33 in comment database IEEE 802.22.2-10/161r0.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

    - Statistics

Use the following statistical information when performing a coverage and interference prediction analysis:

- 802.22 coverage simulations should use 99% and 10% for time availability for desired and undesired signals, respectively and 50% for location availability.

- Digital television (DTV) interference analysis should use 90% and 10% for time availability for desired and undesired signals, respectively and 50% for location availability.

- Analog interference analysis desired and undesired signals use 50% and 10%, respectively for time availability and 50% for location availability.

      - 802.22 Location and Time Availability

When modeling the coverage of an 802.22 network, a system planner should design the service so that it is extremely reliable.  The system planner should consider a cell (usually 500 m by 500 m) to be reliably served if the desired transmissions exceed the minimum field strength for at least 50% of the locations in the cell and at least 99% of the time.

During the stage of the design where potential harmful interference from 802.22 devices is considered, the system planner should again use 50% for the location statistic but use 10% for time.  These statistics would determine where an 802.22 device would exceed the D/U ratio (section 4.1.1.2) for 50% of the locations in the cell for at least 10% of the time.

      - DTV Location and Time Availability

Since the incumbent DTV service has been operating for many years, coverage and interference statistics have long since been established.  The system planner should conform to these established specifications when determining the locations where the DTV service is receivable; and is therefore legally protected from harmful interference.  The system planner should consider that a DTV service is receivable if the DTV transmissions exceed the minimum field strength for at least 50% of the locations in the cell and at least 90% of the time.

The system planner should change the statistics of the DTV service so that the model shows potentially harmful interference with a greater likelihood.  Instead the system planner should plan for potentially harmful interference from DTV into the WRAN service or other TV services by using 50% of the locations and 10% of the time.

      - Analog TV Location and Time Availability

Similarly for DTV, the incumbent analog TV service statistics have been well established through many years of modelling its coverage and interference.  The analog TV service is considered receivable with a lower coverage statistic due to its graceful degradation capabilities.  Unlike the DTV service’s “cliff effect,” where the service is either perfectly clear or totally dark, the analog TV service has many grades in between.  As either the field strength that is present from an analog TV transmission decreases or the noise or interference increases, the analog TV service visibly degrades; but it is still considered receivable – or useable.  Therefore, a system planner should determine the locations where the analog TV service is receivable using a location reliability statistic of 50% and a time reliability statistic of 50%.

While modeling potentially harmful interference from the analog TV service into WRAN services or other TV services, the system planner should again use 50% for the location reliablility statistic and 10% for the time reliability ststistic.

References: