IEEE P802.22
Wireless RANs

| IEEE 802.22 WG Inputs to the FCC Notice of Inquiry 10-198 - Promoting More Efficient Use of Spectrum Through Dynamic Spectrum Use Technologies |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2010-11-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Apurva Mody | BAE Systems | P. O. Box 868, Nashua, NH 03061 | 404-819-0314 / 603-809-0459 | apurva.mody@baesystesms.com, apurva_mody@yahoo.com |
| Winston Caldwell | Fox |  |  | Winston.caldwell@fox.com |
| Gerald Chouinard | CRC |  | 613-998-2500 | gerald.chouinard@crc.ca |
| Victor Tawil | MSTV |  |  | vtawil@mstv.org |
| Tom Gurley | CBS |  |  | tgurley@ieee.org |

Abstract

The following document contains the 802.22 response on the FCC Notice of Inquiry 10-198 seeking comments on wider usage of technologies such as DSA for efficient spectrum use.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The IEEE 802 would like to acknowledge FCC’ s leadership for proposing the use of new and novel technologies such as Dynamic Spectrum Access (DSA), spectrum sensing, geolocation database and beacons for spectrum sharing and enhanced usage of the spectrum for licensed and un-licensed use.

In response to the FCC Notice of Inquiry (FCC NoI 10-198), here is some information and suggestions that we would like to make:

Clause 18: Information on the Current State of the Art in Dynamic Spectrum Access Radios

Question: What is the current state of equipment, system/ standards development?

Response:

Over the last five years IEEE 802 has gained tremendous amount of knowledge on the technologies that are necessary to successfully develop standards for efficient usage of the spectrum using new technologies such as Dynamic Spectrum Access (DSA), spectrum sensing, access to the database, beaconing etc.

For example, the IEEE 802.22 standard project (www.ieee802.org/22) has developed a PHY / MAC air interface to provide cognitive radio (CR) and Dynamic Spectrum Access (DSA) based broadband wireless access in the TV White spaces. This project has been concurrent with the FCC’ s proceeding on un-licensed operation in the TV Whitespaces and the resulting proposed standard has optimized robustness and spectrum utilization.

The IEEE 802.22 membership comprises the entire ecosystem of interested parties such as TV broadcasters, service providers, chip manufacturers, device manufacturers, Department of Defence contractors, Govt. and non-Govt. academic institutions and universities. The 802.22 member organizations not only bring state of the art research to convert these technologies into products but also the knowledge on how to successfully develop and deploy these systems.

The IEEE 802.22 project on cognitive radio based Wireless Regional Area Networks (WRANs) has three projects under its umbrella.

- The IEEE 802.22.1-2010 standard on enhanced protection of low power licensed devices was completed and published in Nov. 2010.

- The IEEE 802.22 standard specifying the broadband wireless air-interface is on-track to be completed in the 2011 time-frame.

- The IEEE 802.22.2 standard for recommended practice for deployment of 802.22 systems is being developed.

For more information on the Overview of the IEEE 802.22 Standard, please refer to our website at www.ieee802.org/22 (under the Core Technologies)

Question: What approaches have developers taken with respect to mitigating hidden nodes where shadowing may make it appear that the spectrum is unoccupied when in fact it is being used nearby? Which methods have met with the most success? What solutions are available for dynamic radio operations in the vicinity of (primarily) receive-only sites?

The IEEE 802.22 standards project has recognized the challenge posed by the hidden node issues (e. g. protecting wireless microphone in the TV bands).  Spectrum sensing is more suitable than database registration to protect wireless microphone usage due to the dynamic and unpredictable nature of on-location news and emergency reporting.  However, the detection range of the low-power signals is unacceptably shorter than the range of interference potential from a 4 W EIRP White Space Device (WSD).  The IEEE 802.22 standards project has accounted for this difficulty by publishing the IEEE 802.22.1-2010 standard for enhanced protection of low-power devices.  IEEE 802.22.1 specifies a beaconing system that indicates the presence of protected wireless microphones in an area.  The IEEE 802.22 protocol detects and reacts to the IEEE 802.22.1 beacon. Hence we believe that all the means for protection of the primary users (e. g. spectrum sensing, database service and beacons for alerting the presence of the primary users) need to be taken into account to avoid the hidden node issues.

Clause 21: Spectrum Sensing Technologies:

Questions: What innovations to sensing are contemplated? Has there been any industry wide consensus regarding methods of implementing sensing? Have there been studies regarding which sensing methods work best among using matched filters, simple energy detection, or cyclostationary detection or other techniques? Are there ways to generalize sensing requirements, or do they need to be determined band-by-band based on the incumbent services? Advocates for band by band requirements should also address methods for updating sensing algorithms based on usage changes over time. How has filter technology advanced such that false positive detections due to adjacent channel signals can be minimized? Can a common standard for spectrum sensing be developed? What would need to be included in such a standard? How should the detection threshold for spectrum sensing be determined? How can dynamic spectrum access radios avoid adjacent channel interference to incumbent systems? Upon detection, should a minimum frequency offset be established to avoid adjacent channel interference? What factors impact detection time and how do they vary for different incumbent radio services (e.g., land mobile systems versus radar systems)?

Response:

IEEE 802.22 investigated a variety of spectrum sensing techniques such as cyclostationary approaches, higher order statistics, covariance based approaches, matched filter, energy detection etc. to detect and recognize the signals of interest. The proposed standard has created a dedicated Annex that specifies these techniques and provides detailed quantitative performance comparisons for various techniques using over-the-air captured signals that were provided to the various participants. The IEEE 802.22 standard specification accomodates the use of spectrum sensing for deployment of the WRAN systems. In fact the IEEE 802.22 standard specification recommends signal type determination/ characterization in addition just signal detection for security reasons in order to ensure protection against cognitive attacks such as primary user emulation and spoofing.

In general we believe that no one technique is sufficient to accurately detect and characterize the signals with reasonable computational complexity in all the bands. For example, cyclostationary and higher order statistics based approaches are suitable to detect and characterize a wide variety of signals at low Signal to Noise Ratios (SNR) however they suffer from relatively high computational complexity. In cases where signals are above the noise floor, simple techniques such as energy detection and interference detection (Clause 23 – correlation in the rise of the noise floor) may be used. IEEE 802.22 specification recommends the use of a combination of techniques such as energy (interference) detection through Receiver Signal Strength Indication (RSSI) monitoring as well as more complex feature based techniques as described above to detect and determine the signal type.

We continue to believe that spectrum sensing will play a very important role in DSA systems.

Member companies of the IEEE 802.22 are working with other Government agencies such as DARPA to continue the advancements in spectrum sensing technologies.

Clause 29: Policy Radios

Questions: How should policies be developed for these dynamic radios? Who should be responsible for developing these policies? Should the policies be incorporated as part of individual service rules? What is the appropriate hierarchy among the Federal Government, industry, and users of policy radios? If policies are developed by non-governmental entities, should the Commission or other appropriate agencies have the option to review or modify policies before they become effective? What factors should be considered in developing dynamic radio policies? How often should policies be reviewed for possible modification? How do we ensure that dynamic policies are distributed to all devices in a timely manner? What procedures can be used to ensure the distributed polices implemented properly?

Response: IEEE 802.22 has devoted a significant thought into the issues of enforcing the policies in the Cognitive Radios. While developing the policies for the radios, the Working Group considered aspects such as how to ensure policy enforcement and security, minor differences in policies proposed by various regulatory domains etc. Although other generic formats of defining the policies may be available, IEEE 802.22 currently uses rule-based policies specific to the bands in which these system are likely to operate. The policy frame-work is generic enough to allow its operation in various regulatory domains. The policies are driven by certain regulatory dependent parameters (e. g. Channel Move Time, Database Refresh Time etc.) which are provided as MIB objects that shall be pre-configured by the manufacturer before selling to ensure appropriate operation in the given regulatory domain.

Clause 38: Secondary Market Mechanisms Time, Frequency and Spatial Approach for Interference Management and Spectrum Sharing

IEEE 802 supports FCC’ s position that effective spectrum sharing and interference management may require the use of all the dimensions that may be available and that are best suited for effective communication (e. g.  frequency, time, space, sub-space etc.).

IEEE 802 at this time does not have any position on the private commons option as proposed by the FCC.

Clause 48: Real-time Database Service:

Question: We seek comment on whether such an approach (use of real time database service) would also work in fixed microwave bands. How might such an approach be instituted in these bands? Should the Commission consider promoting such an approach, and if so, how? Are there other candidate bands? How would a real-time database be maintained so that it contains up-to-date information? Similarly, we seek comment on whether and how such an approach would work in highly mobile bands.

Response:

Although the real-time database service approach is suitable for the TV bands, IEEE 802 urges the FCC to use this approach with caution for other bands. IEEE 802 further recommends:

- Access model of the TV bands database needs to be defined and developed. No company has as yet developed a complete access model that shows success in the field. Neither has FCC finalized the database service provider.

- Push technologies such as synchronous audio and video conferencing as well as instant messaging have been successfully developed and are widely used today. Push technology needs to be considered for DSA systems where the spectrum usage patterns are likely to change more rapidly. Push technology will also reduce the amount of internet traffic, since the database service provider can selectively send messages to the interested parties in case changes to transmit characteristics (e. g. frequency, power etc.) of the devices are needed.

- Security aspects and database access – Entities that are present on the internet are susceptible to various kinds of Denial of Service (DoS), cyber attacks and information assurance issues. The level of effort required to stage such attacks are low and the risk of system failure is very high. The other issue of manual entry database is that of trust. That is how can one corroborate a request that proposes to use the spectrum at a certain time by ensuring that they will indeed use the spectrum for the suggested purpose. Hence adequate considerations need to be made to establish a secure connection between the DSA device and the database service. Security aspects such as authorization to access, authentication, identification, data integrity, non-repudiation, confidentiality and privacy need to be considered.

Clause 50: Real-time Spectrum Monitoring

Questions: Generally, we seek comment on whether such a system of monitoring equipment could be deployed at a reasonable cost. Should such a network be administered by the U.S. Government or the private sector? If the private sector, how should the collected information be disseminated? For example, should a fee be charged? Could a low-cost sensor be developed to effectively detect the presence of weak radio signals? How would a system of sensors perform in urban areas or in areas where there is irregular terrain?

Response: IEEE 802 would like to propose the development of a Country-wide network of distributed sensors that monitors the spectrum and reports any non-standard signals to the FCC official database for further investigation. Such a network of sensors would be very beneficial to complement other techniques of interference avoidance such as use of database and beacons. It may also help to act as a sentry on the ground to corroborate the information has been provided to the database service. This will further help to make the proposed DSA network more secure.

Clause 55. FCC Participation in Technology and Standards Development.

Question: Is there more that the Commission can do through its participation in the technical community to help advance this technology and help identify and remove any regulatory barriers?

Response: The IEEE 802 LAN MAN Standards Committee has been at the fore-front of innovation since many years and has created many successful standards resulting in products that have become house-hold names (e. g. IEEE 802.11 synonymous with WiFiTM, IEEE 802.15 synonymous with ZigBeeTM, IEEE 802.16 synonymous with WiMAXTM). IEEE 802 has actively worked with the FCC to shape the rules and is also creating standards that promote DSA. Other regulatory domain representatives such as the ones from the Office of Communications (UK), Govt. of Egypt, Govt. Singapore etc. have regularly attended the IEEE 802 meetings. Hence, it would be nice if the representatives of the FCC occasionally attend the IEEE 802 meetings and engage directly in the discussions to promote these new technologies.

Concluding Remarks:

IEEE 802 appreciates this opportunity to provide information on our standards projects related to dynamic spectrum use.  As these standards are deployed and implementation experience is gained, we will continue to apprise the Commission of our progress.