IEEE P802.22
Wireless RANs

| Comment 63 Resolution to P802.22/D2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-03-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | Self |  |  | ranga.reddy@ieee.org |

Abstract

This document contains modified text for various editorial comments that Ranga Reddy is proposing for P802.22/D2.

R0 Is the intial version of this document.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

I. Introduction

This contribution provides resolution to Comment 63 against P802.22/D2.

II. Modified Text Proposals

[modify Table 16 as follows]

[Start of Text Modification]

| Syntax | Size | Description |
| --- | --- | --- |
| Signature_IE_Format { |  |  |
| Element ID | 8 bits | 0x06 |
| Key ID | 9 bits | Identifier of the key associated with the BS implicit certificate used to generate the signature. This identifier is generated by the Certification Authority (CA) when the certificate is created. |
| Time Stamp | 44 bits | Derived from a NMEA 0183 ($..ZDA) string (each letter represents a digit, encoded by different numbers of bits): - X: year= 2010+X, X is 6 bits - M: month, e.g., 01-12, total is 4 bits - D: day, e.g., 01-31, total 5 bits - H: hour, e.g., 00-23, total 5 bits - m: minute, e.g., 00-59, total 6 bits - ss: seconds, e.g., 00-59, total 6 bits - .ss: 10 ms boundary, e.g., .00-.99, 7 bits - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is number of hours, e.g., 1-13, 4 bits, total 5 bits |
| Version | 1 bit | 0: current 1: reserved |
| Signature | 32 bytes | Output of signature process. This includes the Recoverable Message part (C) and the Signature Data (d) as described in 8.6.2.5.2. The signature is calculated over the entire CBP MAC PDU. The signature process is detailed in 8.6.2.3. |
| Padding | 2 bits | All bits shall be set to 0. |
| } |  |  |

[End of Text Modification]

[Section 7.7.21, pg 107, lines 7, modify Table 160 as follows]

[Start of Text Modification]

| Syntax | Size | Description |
| --- | --- | --- |
| CERT-REQ_IE_Format { |  |  |
| Element ID | 8 bits | 0x07 |
| Destination BS ID | 48 bits | ID of BS to which that request is directed. |
| CA ID | 8 bits | ID of Certificate Authority that issued the certificate to the BS that is initiating the certificate request. |
| Key ID | 9 bits | Identifier of public key associated with certificate as assigned by CA. This identifier is generated by the Certification Authority (CA) when the certificate is created. |
| Key Validity Date (Not Before) | 31 bits | Date that signifies the start of period for which the certificate of the BS that is making the request is valid. Derived from a NMEA 0183 ($..ZDA) string (each letter represents a digit, encoded by different numbers of bits): - X: year= 2010+X, X is 6 bits - M: month, e.g., 01-12, total is 4 bits - D: day, e.g., 01-31, total 5 bits - H: hour, e.g., 00-23, total 5 bits - m: minute, e.g., 00-59, total 6 bits - s: seconds, assumed to be 00, not actually encoded - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is the number of hours, e.g., 1-13, 4 bits, total 5 bits |
| Key Validity Time Period | 7 bits | Amount of time, in 6 month increments, that the certificate is valid. |
| Version | 1 bit | 0: current 1: reserved |
| Public Key Reconstruction Data | 248 bits | Key data used to reconstruct the public key, i.e., 22 bytes for 163 bit ECC keys. |
| } |  |  |

[End of Text Modification]

[modify Table 18 as follows]

[Start of Text Modification]

| Syntax | Size | Description |
| --- | --- | --- |
| CERT-RSP_IE_Format { |  |  |
| Element ID | 8 bits | 0x08 |
| Source BS ID | 48 bits | ID of BS to which that Certificate Response is directed. |
| CA ID | 8 bits | ID of Certificate Authority that issued the certificate to the BS that is transmitting the certificate response. |
| Key ID | 9 bits | Identifier of public key associated with certificate as assigned by CA. This identifier is generated by the Certification Authority (CA) when the certificate is created. |
| Key Validity Date (Not Before) | 31 bits | Date that signifies the start of period for which the certificate of the BS, that is transmitting the certificate response, is valid. Derived from a NMEA 0183 ($..ZDA) string (each letter represents a digit, encoded by different numbers of bits): - X: year= 2010+X, X is 6 bits - M: month, e.g., 01-12, total is 4 bits - D: day, e.g., 01-31, total 5 bits - H: hour, e.g., 00-23, total 5 bits - m: minute, e.g., 00-59, total 6 bits - s: seconds, assumed to be 00, not actually encoded - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is the number of hours e.g., 1-13, 4bits, total 5bits |
| Key Validity Time Period | 7 bits | Amount of time, in 6 month increments, that the certificate is valid. |
| Version | 1 bit | 0: current 1: reserved |
| Public Key Reconstruction Data | 248 bits | Key data used to reconstruct the public key, i.e., 22 bytes for 163 bit ECC keys. |
| Time Stamp | 44 bits | Copied from Signature IE of the CBP MAC PDU in which the CERT-REQ IE was received. |
| Reserved | 4 bits | All bits shall be set to zero. |
| } |  |  |

[End of Text Modification]

[modify Table 195 as follows]

[Start of Text Modification]

| Item | Size | Description |
| --- | --- | --- |
| CA ID | 8 bits | Id of CA that issued implicit certificate to BS |
| Key ID | 9 bits | Identifier of public key associated with certificate as assigned by CA. This identifier is generated by the Certification Authority (CA) when the certificate is created. |
| Key Validity Date (Not Before) | 31 bits | Derived from ZDA NMEA 0183 string (each letter represents a digit encoded by different # of bits): - X: year= 2010+X, X is 6 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - s: seconds, assumed to be 00, not actually encoded - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Key Validity Time Period | 7 bits | Amount of time in 6 month increments, that the certificate is valid. |
| Version | 1 bit | 0:current 1: reserved |
| BS Public Key Reconstruction Data | 31 byte | Key data used to reconstruct public key: - 22 bytes for 163 bit ECC keys |

[End of Text Modification]

[modify Table 196 as follows]

[Start of Text Modification]

| Item | Size | Description |
| --- | --- | --- |
| Key ID | 9 bits | Identifier of the key associated with the BS implicit certificate used to generate the signature. This identifier is generated by the Certification Authority (CA) when the certificate is created. |
| Time Stamp | 44 bits | Derived from ZDA NMEA 0183 string (each letter represents a digit encoded by different # of bits): - X: year= 2010+X, X is 6 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - ss: seconds, 00-59, 6 bits - .ss: 10 ms boundary, .000-.99, 7 bits - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Version | 1 bit | 0: current 1: reserved |
| Signature | 32 bytes | Output of signature process. This includes the RecoverableMessage part (C) and the Signature Data (d) as described in The signature process is detailed in 8.6.2.5.2. The signature calculated over the entire CBP MAC PDU. |

[End of Text Modification]

[modify Table 197 as follows]

[Start of Text Modification]

| Item | Size | Description |
| --- | --- | --- |
| CA ID | 8 bits | Unique identifier of the CA |
| Key ID | 9 bits | Identifier of CA Root certificate as assigned by CA |
| Key Validity Date (Not Before) | 31 bits | Derived from ZDA NMEA 0183 string (each letter represents a digit encoded by different # of bits): - X: year=2010 + X, X is 6 bits - M: month, e.g. 01-12, total is 4 bits - D: day, e.g. 01-31, total 5 bits - H: hour, e.g. 00-23, total 5 bits - m: minute, e.g. 00-59, total 6 bits - s: seconds, assumed to be 00, not actually encoded - zZ: hours off of GMT; z is 1bit -/+ indication, 2nd Z is # hours 1-13 4bits, total 5bits |
| Key Validity Time Period | 7 bits | Amount of time in 6-month increments, that the certificate is valid. |
| Version | 1 bit | 0: current version, EC Domain parameters in compressed form & 256bit keys 1: reserved |
| EC Domain Parameters | 162 bytes | EC Domain parameters of the form D = (p, a, b, G, n, h) where G is expressed in compressed form |
| CA Public Key Reconstruction Data | 31 bytes | Data used to reconstruct CA public key |

[End of Text Modification]

[Section 8.6.4.2.1, pg 301, lines 3-7 as follows]

[Start of Text Modification]

  - Recommended EC domain parameters to be used shall be for binary fields on either 163-bit random or Koblitz curves. Example domain parameters can be found in:

    - K-233 or B-233 elliptic curves defined in FIPS 186-3,

    - sect233k1 and sect233r1 curves defined in [SEC2]

    - In the EC domain parameters, elliptic curve points shall be represented in compressed form

[End of Text Modification]

References: