IEEE P802.22
Wireless RANs

| IEEE 802.22-2011 Standard Press Release |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-05-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | Merrimack, NH | 603-809-0459 | Apurva.mody@baesystems.com |
| Victor Tawil | MSTV |  |  | vtawil@mstv.org |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Unapproved DRAFT 13, May 2011

NOT FOR RELEASE

Contact:

Dr. Apurva N.Mody, Chair of IEEE 802.22 Working Group

IEEE 802.22-2011 STANDARD FOR WIRELESS REGIONAL AREA NETWORKS IN TV WHITESPACES COMPLETED

The 802.22 standard[1] was published as an official IEEE Standard in July 2011.  IEEE 802.22 standards based systems have a potential to provide broadband access to wide regional areas around the world to bring information and communication technologies to unserved and under-served communities.

It incorporates cognitive radio capabilities such as dynamic spectrum access (DSA), access to the incumbent database, accurate geolocation techniques, spectrum sensing, primary user protection beacon, regulatory domain dependent policies, spectrum etiquette and coexistence. It provides broadband access while protecting the incumbent users and using the spectrum efficiently.

The 802.22 Working Group started its work in January 2005 following the Notice of Inquiry (NOI) issued by the United States’ Federal Communications Commission (“FCC”)[2] and the first NPRM[3] on unlicensed’ operation  in the TV broadcast bands.

IEEE 802.22 WG has developed a point-to-multipoint wireless broadband air interface standard optimized for operation in the VHF and UHF TV bands, in the frequency range between 54 MHz and 862 MHz.  The standard is especially useful for serving less densely populated areas (e.g., rural areas) where most empty TV channels can be found and where population is mostly unserved or underserved by Internet broadband access services.  The base station (BS) can be accessed by fixed customer premise equipment (CPE) as well as portable devices as per local regulatory requirements.

A messaging interface for the purpose of accessing an incumbent database service is included in the standard to accommodate new US regulatory requirements. It should be noted that since the BS controls the DSA operation in this point-to-multipoint operation, the base station is used as the central point for querying the database service.  This is done by acquiring the location of each CPE and portable terminal early in the process of association, querying the database service on behalf of these terminals and locally managing the responses from the database service to decide on the next DSA action for the specific CPE or the entire network

The standard contains mechanisms for the BS to control sensing operation taking place at the CPEs and portable devices, and to report sensing results to the BS. Furthermore, regulatory dependent policies used at the BS to control the DSA behavior under various conditions are embedded in the standard. Satellite and terrestrial geolocation techniques have been incorporated into the standard.  The IEEE 802.22 Working Group took a special care for embedding security features for not just the data, control and management functions but also for cognitive functions. Based on the regulatory requirements, certain features, (e.g., sensing) can be disabled if they are not required.

Recognizing the difficulty of reliably sensing wireless microphone operations in the TV bands, a parallel standardization activity was undertaken under the 802.22.1 Task Group to develop a related standard for an RF beacon to signal the presence of wireless microphone operation in a more reliable manner. This has resulted in the publication of the IEEE Std 802.22.1™-2010 in November 2010[4].

Besides being able to detect broadcast incumbents to avoid interference, a spectrum efficient mechanism is included to detect other 802.22 WRAN operations to implement self-coexistence. This is achieved by having the available channels properly distributed among 802.22 WRAN’s operating in the same area and, if needed, distributing frame capacity amongst co-channel operating 802.22 systems for more efficient use of the spectrum.

References

[1]	P802.22-D3, “Standard for Information Technology — Telecommunications and Information Exchange Between Systems — Local and Metropolitan Area Networks – Specific Requirements — Part 22: Cognitive Wireless RAN Mediu Access Control (MAC) and Physical Layer (PHY) Specification: Policies and Procedures for Operation in TV Bands,” March 2011. www.ieee802.org/22

[2]	US FCC, ET Docket 02-380: Notice of Inquiry, In the Matter of “Additional Spectrum for Unlicensed Devices Below 900MHz and in the 3GHz Band”, Released December 20, 2002

[3]	US FCC, ET Docket 04-186: Notice of Proposed Rule Making, In the Matter of “Unlicensed Operation in the TV Broadcast Bands”, Released May 25, 2004

[4]	IEEE Std 802.22.1TM-2009, “Standard for Information Technology — Telecommunications and Information Exchange Between Systems — Local and Metropolitan Area Networks – Specific Requirements — Part 22.1: Enhanced Protection for Low-Power, Licensed Devices Operating in Television Broadcast Bands,” November 2009.

[5]	US FCC, ET Docket 04-186: First Report and Order and Further Notice of Proposed Rule Making, In the Matter of “Unlicensed Operation in the TV Broadcast Bands”, Released October 18, 2006.

[6]	U.S. FCC, ET Docket 08-260, “Second Report and Order and Memorandum Opinion and Order in the Matter of Unlicensed Operation in the TV Broadcast Bands,” November 14, 2008.

____________________________