IEEE P802.22
Wireless RANs

| Update to MIB Description & MIB Definition for Clause 13 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-10-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | Self |  |  | ranga.reddy@ieee.org |

Abstract

This contribution is to be used to update the MIB Description and Definition in clause 13.

R0 Contains additional modifications applied after 802.22-2011 as part of new project.

R1 Contains the ASN.1 Encoding of the wranDevMib MIB group

R2 Contains the ASN.1 Encoding of the wranIfCpeMib MIB group and updates to the MIB description in 13.1.4 for wranIfCpeMib

R3 Contains modifications to wranIfCpeMib resulting from 10/10 telecon and ASN.1 Encoding of wranIfDatabaseServiceMib MIB group and updates to the MIB description in 13.1.7 for wranIfDatabaseServiceMib

R4 Contains ASN.1 Encoding of wranIfSmMib MIB in 13.2.5, updates to wranIfSmMib MIB description in 13.1.5, updates to how octet string-based variables are handled in wranIfDatabaseServiceMib as well as updates requested during November 2011 Plenary

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- MIB Structure

The definition of managed objects in this standard is expressed in Structure of Management Information Version 2 (SMIv2). It supports a management protocol agnostic approach, including SNMP.

The basic MIB objects are the following:

- wranDevMib: basic MIB for BS/CPE device management. Can be used to track software versioning of BS/CPE HW/SW and what SNMP traps can be configured on those devices

- wranIfBsMib: basic MIB for BS-related MIB objects related to providing fixed AND portable service.

- wranIfBsSfMgmt: basic MIB for managing items related to Service Flow configuration, instantiation, and management

- wranIfCpeMib: basic MIB for CPE-related MIB objects related to operation of fixed AND portable CPEs

- wranIfSmMib: basic MIB for Spectrum Manager related MIB objects

- wranIfSsaMib: basic MIB for Spectrum Automaton related MIB objects

- wranIfDatabaseServiceMib: basic MIB for Database Service access related MIB objects

  - MIB Description

    - wranDevMib

wranDevMib details objects that will be managed by the SNMP agent in the BS and CPE. This MIB element is broken down into the following MIB groups:

- wranDevBsObjects: MIB objects to be implemented by the SNMP agent in BS

- wranDevCpeObjects: MIB objects to be implemented by the SNMP agent in CPE

- wranDevCommonObjects: MIB objects  to be implemented by the SNMP agent in BS/CPE

      - wranDevBsObjects

wranDevBsObjects is broken down into the following two items

- wranDevBsSoftwareUpgradeTable: contains objects related to BS SW upgrade

- wranDevBsNotification: managed objects related to SNMP traps on BS

        - wranDevBsSoftwareUpgradeTable

This table defines objects associated with BS software configuration. It may have one or more entry, although only one software configuration shall be active at any given time. Each entry is defined by a compound attribute, wranDevBsSoftwareUpgradeEntry.

          - wranDevBsSoftwareUpgradeEntry

Each entry in the wranDevBsSoftwareUpgradeTable is made up of the following items:

            - wranDevBsDeviceIndex

Index of entry in the table

            - wranDevBsVendorId

This value identifies the managed BS vendor to which SW upgrade was applied

            - wranDevBsHwId

Version of HW that SW upgrade is applied to

            - wranDevBsCurrentSwVersion

Version of SW currently running on the BS. This value is set by the vendor specified by the Vendor ID. The SW version and HW ID (wranDevBsHwId) should be a unique tuple. After the downloaded software is activated, the value in this object shall be replaced with the version in wranDevBsDownloadSwVersion.

            - wranDevBsDownloadSwVersion

Version of the SW to be downloaded. This value is set by the vendor specified by the Vendor ID. The SW version and HW ID (wranDevBsHwId) should be a unique tuple. This should be initialized before software is downloaded or activiated.

            - wranDevBsUpgradeFileName

Fully qualified path name that points to the location of SW version that is to be downloaded/activated

            - wranDevBsSoftwareUpgradeAdminState

This value can take on two states. When set to Download, the software listed by wranDevBsDownloadSwVersion will be downloaded. When set to Activate, the software recently downloaded will be activated. The Download and Activate procedures are vendor specific operations that are not defined in this standard.

            - wranDevBsDowloadSwProgress

This value shows the progress of the SW download highlighted by wranDevBsDownloadSwVersion, encoded as the percentage of the download successfully completed.

            - wranDevBsSoftwareUpgradeTimeStamp

This value is a timestamp to indicate when the last SW download or activation took place.

        - wranDevBsNotification

This group of objects relates to SNMP traps on the BS. There is a control element that enables/disables the traps (wranDevBsTrapControl) and what event information is logged  when a trap is sent (wranDevBsTrapDefinition).

          - wranDevBsTrapControl

Defines control elements for traps. This is a 2bit field that enables setting a trap when a BS event is logged (wranDevBsEvent) or to indicate when a event log buffer size overruns configured threshold (wranDevBsLogBuffExceedThreshold).

          - wranDevBsTrapDefinition

This object groups all of the notification objects for the BS. It is defined to be compatible with SNMPv1, following Section 8.5 and 8.6 RFC 2758. It’s a compound object made up of the contents of a logged BS event (wranDevBsEventTrap) and the indication of when the ratio used portion to total size of event log has been exceeded (wranDevBsLogBuffExceedThresholdTrap).

            - wranDevBsEventTrap

This object is a compound object that contains information the objects in wranDevCmnEventEntry in wranDevCmnEventLogTable that describes the event that is logged. This trap is caught when wranDevBsEvent in wranDevBsTrapControl is set.

            - wranDevBsLogBuffExceedThresholdTrap

This object is a compound object that indicates the index of an entry (defined by wranDevCmnEventLogEntry) in wranDevCmnEventLogConfigTable, and the object in that entry that defines the ratio (defined by wranDevCmnEventLogResidualBuffThreshold) of used capacity in Event Log vs. total capacity. This trap is caught when wranDevBsLogBuffExceedThreshold

in wranDevBsTrapControl is set.

      - wranDevCpeObjects

wranDevCpeObjects are broken down into the following elements:

- wranDevCpeConfigFileEncodingTable: related to configuration information about the CPE

- wranDevCpeNotification: related to SNMP traps that are particular to the CPE

        - wranDevCpeConfigFileEncodingTable

This table defines objects associated with CPE software configuration. This table may only have one entry, defined by a compound attribute, wranDevCpeConfigFileEncodingEntry.

          - wranDevCpeConfigFileEncodingEntry

The single entry in the wranDevCpeConfigFileEncodingTable is made up of the following items:

            - wranDevCpeDeviceIndex

Index of entry in the table, defaults to 0.

            - wranDevCpeMicConfigSetting

This value contains the MIC (Message Integrity Code) calculated for the CPE configuration file.

            - wranDevCpeVendorId

This value identifies the vendor of the managed CPE to which a configuration file upgrade is to be applied.

            - wranDevCpeHwId

This value identifies the hardware version of the CPE which the configuration file upgrade is to be applied.

            - wranDevCpeConfigFileVersion

Version of the configuration file to be downloaded. This value is set by the vendor specified by the Vendor ID. The SW version and HW ID (wranDevCpeHwId) should be a unique tuple. This should be initialized before software is downloaded or activiated.

            - wranDevCpeUpgradeFileName

Fully qualified path name that points to the location of configuration file that is to be downloaded

            - wranDevCpeSwTftpServer

IP address of the TFTP server on which the new configuration file resides.

            - wranDevCpeTftpServerTimeStamp

The time the configuration file was sent in seconds, as defined by IETF RFC 868.

        - wranDevCpeNotification

This group of objects relates to SNMP traps on the CPE. There is a control element that enables/disables the traps (wranDevCpeTrapControl) and what event information is logged when a trap is sent (wranDevCpeTrapDefinition).

          - wranDevCpeTrapControl

Defines control elements for traps. This is a 2bit field that enables setting a trap when a CPE event is logged (wranDevCpeEvent) or to indicate when a event log buffer size overruns configured threshold (wranDevCpeLogBuffExceedThreshold).

            - wranDevCpeTrapDefinition

This object groups all of the notification objects for the CPE. It is defined to be compatible with SNMPv1, following Section 8.5 and 8.6 RFC 2758. It’s a compound object made up of the contents of a logged BS event (wranDevCpeEventTrap) and the indication of when the ratio used portion to total size of event log has been exceeded (wranDevCpeLogBuffExceedThresholdTrap)

            - wranDevCpeEventTrap

This object is a compound object that contains information the objects in wranDevCmnEventEntry in wranDevCmnEventLogTable that describes the event that is logged. This trap is caught when wranDevCpeEvent in wranDevCpeTrapControl is set.

            - wranDevCpeLogBuffExceedThresholdTrap

This object is a compound object that indicates the index of an entry (defined by wranDevCmnEventLogEntry) in wranDevCmnEventLogConfigTable, and the object in that entry that defines the ratio (defined by wranDevCmnEventLogResidualBuffThreshold) of used capacity in Event Log vs. total capacity. This trap is caught when wranDevCpeLogBuffExceedThreshold in wranDevCpeTrapControl is set.

      - wranDevCommonObjects

This object contains managed elements that are common to CPE and BS:

- wranDevCmnEventLog:  contains managed objects related to the Event Log

- wranDevCmnSnmpAgent:  contains managed objects related to SNMP agent configuration

- wranDevCmnDeviceConfig:  contains common managed objects related to device configuration

        - wranDevCmnEventLog

Event Log is a compound attribute made up of:

- wranDevCmnEventLogConfigTable: configuration of parameters for Event Log operation

- wranDevCmnEventTable: defines events that are supported by the CPE and BS

- wranDevCmnEventLogTable: used to store local Events that shall reside in non-volatile memory

          - wranDevCmnEventLogConfigTable

Each entry stores the Event Log configuration for a device. Each entry is defined as wranDevCmnEventLogConfigEntry.

            - wranDevCmnEventLogConfigEntry

This object is a compound object that Event Log configuration for a particular device, indicated by wranDevCmnDeviceIndex.

              - wranDevCmnDeviceIndex

Index value that identifies a BS or CPE entry in the wranDevCmnEventLogConfigTable.

              - wranDevCmnEventLogEntryLimit

Maximum number of entries in wranDevCmnEventLogConfigTable. If this value is changed while entries exist wranDevCmnEventLogTable, old entries will be discarded until limit is reached.

              - wranDevCmnEventLogLifeTimeLimit

A value of 0 means that an entry is kept indefinitely. Any other value, it’s the maximum time an entry can exist in wranDevCmnEventLogTable. If this value is changed while entries exist in wranDevCmnEventLogTable, entries older than this limit will be discarded.

              - wranDevCmnEventLogEntryLimitPerEventId

The number of log entries that can be logged per event.

              - wranDevCmnEventLogSeverityThreshold

Minimum severity level of an event that can be logged into the Event Log.

              - wranDevCmnEventLogWrapAroundBuffEnable

Indication of whether or not the Event Log can wrap around when full or is emptied when full.

              - wranDevCmnEventLogLatestEvent

Index of latest event in Event Log.

              - wranDevCmnEventLogPersistenceSupported

Indication of whether or not Event Log is to persist through power cycle or reset of device

              - wranDevCmnEventLogResidualBuffThreshold

Threshold ratio of used capacity for Event Log to total capacity of Event Log, that when reached a TRAP is issued.

          - wranDevCmnEventTable

This compound object defines the types of events that are supported by a BS or CPE. Each event is defined wranDevCmnEventEntry.

            - wranDevCmnEventEntry

This object defines the parameters of an event entry into the wranDevCmnEventTable. Each entry is indexed by wranDevCmnDeviceIndex and wranDevCmnEventIdentifier.

              - wranDevCmnEventIdentifier

Event Identifier encoded as a numeric value

              - wranDevCmnEventDescription

Description of the event in the form of an SnmpAdminString.

              - wranDevCmnEventSeverity

The severity of the event as assigned by the device. The Severity assigned to an event is configurable by the system.

              - wranDevCmnEventNotification

Notification will be made when event occurs.

              - wranDevCmnEventNotificationOid

This is the object identifier of a notification object. If wranDevCmnEventNotification true, a trap identified by the OID will be reported

          - wranDevCmnEventLogTable

This is the log table that stores local events as they happen. This table shall reside in non-volatile memory that may persist after power cycle or reset of the device. The maximum number of entries in this table is determined by the wranDevCmnEventLogEntryLimit. If it is setup as a wrap-around log, then the oldest entry will be removed to make room for the newest entry. If it is not setup as a wrap-around log, then the log will be flushed. Multiple entries are store in the table. Each entry is defined by wranDevCmnEventLogEntry.

            - wranDevCmnEventLogEntry

Each entry in the Event Log table is made up of several parameters.

              - wranDevCmnEventId

A counter used to index entries in the Event Log. When it reaches the maximum value, it will  wrap-around if configured to wrap-around or the log will be flushed if it is not configured to wrap-around.

              - wranDevCmnEventLoggedTime

The time that the entry was placed into the Event Log. If this event happened just before the last initialization of the management system, then this value is set to 0.

              - wranDevCmnEventLogDescription

The Description of the event.

              - wranDevCmnEventLogSeverity

The severity of the recorded event.

        - wranDevCmnSnmpAgent

This compound object deals with the common objects related to SNMP agent configuration.

          - wranDevCmnSnmpV1V2TrapDestTable

This compound object deals with the configuration items of the SNMP agent. Each configuration item is represented by a wranDevCmnSnmpV1V2TrapDestEntry.

            - wranDevCmnSnmpV1V2TrapDestEntry

This compound object contains the parameters that identify the destination of an SNMP trap.

              - wranDevCmnSnmpV1V2TrapDestIndex

Identifies the trap in the table. This parameter shall have a maximum value of 8.

              - wranDevCmnSnmpV1V2TrapDestIpAddrType

Type of IP address stored in wranDevCmnSnmpV1V2TrapDestIpAddr.

              - wranDevCmnSnmpV1V2TrapDestIpAddr

SNMP manager’s IP address that is configured as a destination for traps.

              - wranDevCmnSnmpV1V2TrapDestPort

Port number of SNMP manager application configured as a trap destination.

              - wranDevCmnSnmpV1V2TrapDestRowStatus

This object is used to make sure that any write operation to multiple columns is treated as an atomic operation.

        - wranDevCmnDeviceConfig

This compound attribute contains:

- wranDevCmnResetDevice: object that is used to reset the device

- wranDevMibGroups: MIB objects that are required for conformance

- wranDevMibCompliance: MIB objects that are required for conformance

          - wranDevCmnResetDevice

There are two actions defined for the object. When set to actionResetDeviceNoAction, no action is taken. When set to actionResetDevice, device will reset itself.

          - wranDevMibGroups

This object helps define which MIB groups are necessary to meet conformance and what MIB objects are part of each group.

            - wranDevMibBsGroup

This MIB group is mandatory. It is made up of wranDevBsTrapControl.

            - wranDevMibBsSwUpgradeGroup

This MIB group is mandatory. It is a compound object made up of wranDevBsVendorId, wranDevBsHwId, wranDevBsCurrentSwVersion, wranDevBsDownloadSwVersion, wranDevBsUpgradeFileName, wranDevBsSoftwareUpgradeAdminState, wranDevBsDownloadSwProgress, wranDevBsSoftwareUpgradeTimeStamp. It contains the values of the most recent/current entry in wranDevBsSoftwareUpgradeTable.

            - wranDevMibCpeGroup

This MIB group is mandatory. It is a compound object made up of wranDevCpeMicConfigSetting, wranDevCpeVendorId, wranDevCpeHwId, wranDevCpeConfigFileVersion, wranDevCpeUpgradeFileName, wranDevCpeSwTftpServer, wranDevCpeTftpServerTimeStamp, wranDevCpeTrapControl. It contains the values of the most recent/current entry in wranDevCpeConfigFileEncodingTable for a particular CPE.

            - wranDevMibCmnGroup

This MIB group is mandatory. It is a compund object made up of wranDevCmnSnmpV1V2TrapDestIpAddrType, wranDevCmnSnmpV1V2TrapDestIpAddr, wranDevCmnSnmpV1V2TrapDestPort, wranDevCmnSnmpV1V2TrapDestRowStatus, wranDevCmnResetDevice, wranDevCmnDeviceIndex, wranDevCmnEventLogEntryLimit, wranDevCmnEventLogLifeTimeLimit, wranDevCmnEventLogEntryLimitPerEventId, wranDevCmnEventLogSeverityThreshold, wranDevCmnEventLogWrapAroundBuffEnable, wranDevCmnEventLogLatestEvent, wranDevCmnEventLogPersistenceSupported, wranDevCmnEventLogResidualBuffThreshold, wranDevCmnEventDescription, wranDevCmnEventSeverity, wranDevCmnEventNotification, wranDevCmnEventNotificationOid, wranDevCmnEventId, wranDevCmnEventLoggedTime, wranDevCmnEventLogDescription, wranDevCmnEventLogSeverity. It contains the values of the most recent/current entry in wranDevCmnSnmpV1V2TrapDestTable and wranDevCmnEventLogTable.

            - wranDevMibBsNotificationGroup

This MIB group is optional. It is a compound object that contains wranDevBsEventTrap, wranDevBsLogBuffExceedThresholdTrap.

            - wranDevMibCpeNotificationGroup

This MIB group is optional. It is a compound object that contains wranDevCpeEventTrap, wranDevCpeLogBuffExceedThresholdTrap.

          - wranDevMibCompliance

This object indicates which MIB groups in wranDevMibGroups are optional and which ones are mandatory.

    - wranIfBsMib

This MIB represent core MIB functionality that all base stations shall support. It is based on the FCAPS reference model, which is comprised of MIBs relating to management of the following items: Fault, Configuration, Accounting, Performance,  and Security. The following list defines MIBs that compose wranIfBsMib:

- wranIfBsFm: Fault management

- wranIfBsCm: Configuration management

- wranIfBsAm: Accounting management

- wranIfBsPm: Performance management/measurement

- wranIfBsScm: Security management

      - wranIfBsFm

Exceptions and fault events can be reported by using the traps defined in this MIB. wranIfBsFm is made up of the following MIBs: wranIfBsTrapControlRegister, wranIfBsCpeNotifactionObjectsTable, and wranIfBsThresholdConfigTable.

        - wranIfBsTrapControlRegister

This MIB is a bitmap that is used to disable/enable the following BS traps: wranIfBsCpeDynamicServiceFailNotification, wranIfBsCpeRssiStatusChangeNotification, wranIfBsCpeEIRPStatusChangeNotification, wranIfBsCpeRegisterNotification, wranIfBsCpeScmFailNotification, wranIfBsCpeStartupStatusChangeNotification, wranIfBsThroughputMetricsNotification, wranIfBsNetworkEntryMetricsNotification, wranIfBsPacketErrorRateChangeNotification, wranIfBsUserMetricsChangeNotification, wranIfBsCoexistenceStatusNotification, wranIfBsCpeCbpReceptionNotification, wranIfBsCpeWiMicBeaconMSF1Notification, wranIfBsCpeWiMicBeaconMSF12Notification, wranIfBsCpeWiMicBeaconMSF123Notification, wranIfBsInterFrameSensingStatusNotification, wranIfBsMeasurementStatusNotification. Enable of a trap indicates that the trap will be recorded in wranIfBsCpeNotificationObjectsTable

        - wranIfBsCpeNotificationObjectsTable

This table contains objects that represents notifications reported in CPE fault traps. The table is made up of one more entries.

          - wranIfBsCpeNotificationObjectsEntry

This MIB is a compound object that represents an entry in the MIB object wranIfBsCpeNotificationsObjectsTable.

            - wranIfBsCpeNotificationMacAddr

The MAC address of the CPE reporting the notification.

            - wranIfBsCpeStartupStatusChange

This object that is used to indicate the status of a CPE as it proceeds through the network entry and initialization procedures.

            - wranIfBsDynamicServiceFail

This object indicates the reason for failure if a dynamic service flow command failed and the SFID of that service flow.

            - wranIfBsCpeRssiStatus

This object is an indication of what type of RSSI alarm (see 13.1.2.1.3) has recently been triggered.

            - wranIfBsCpeEirpStatus

This object an indication of what type of EIRP alarm (see 13.1.2.1.3) has recently been triggered.

            - wranIfBsCpeRegister

This object contains the status of CPE registration.

            - wranIfBsCpeScmFail

This object contains the status of CPE’s SCM Configuration.

            - wranIfBsPacketErrorRateChange

The recent change in Packet Error rate metrics.

            - wranIfBsUserMetricsChange

The recent change in User metrics.

            - wranIfBsCoexistenceStatus

Indication when cell moves between normal operation and coexistence operation.

            - wranIfBsCpeCbpReception

This object would contain the contents of a recently received and decoded CBP.

            - wranIfBsCpeWiMicBeaconMSF1

This object would contain the contents of a recently received and decoded MSF1 of a wireless microphone beacon.

            - wranIfBsCpeWiMicBeaconMSF12

This object would contain the contents of a recently received and decoded MSF1+MSF2 of a wireless microphone beacon.

            - wranIfBsCpeWiMicBeaconMSF123

This object would contain the contents of a recently received and decoded MSF1+MSF2+MSF3 of a wireless microphone beacon.

            - wranIfBsMeasurementStatus

Indication when sensing measurement report has been received.

            - wranIfBsInterFrameSensingStatus

Indication when Inter-frame sensing configuration has been changed.

            - wranIfBsIntraFrameSensingStatus

Indication when Intra-frame sensing configuration has been changed.

        - wranIfBsThresholdConfigTable

This MIB provides a table that allows the setting to thresholds that can be used to detect the crossing of RSSI and EIRP thresholds. Each table is made up of entries for low and high thresholds for RSSI and EIRP.

          - wranIfBsThresholdConfigEntry

This MIB provides entries into the wranIfBsThresholdConfigTable.

            - wranIfBsRssiLowThreshold

Low threshold for generating an RSSI alarm.

            - wranIfBsRssiHighThreshold

High threshold for generating an RSSI alarm.

            - wranIfBsEirpLowThreshold

Low threshold for generating an EIRP alarm.

            - wranIfBsEirpHighThreshold

High threshold for generating an EIRP alarm.

      - wranIfBsCm

This MIB contains various objects related to Configuration Management. Within this MIB, the following tables are handled: wranIfBsCpeRngCapabilityReqTable, wranIfBsCpeRngCapabilityCmdTable, wranIfBsCpeBasicCapabilityReqTable, wranIfBsCpeBasicCapabilityRspTable, wranIfBsCpeRegCapabilityReqTable, wranIfBsCpeMeasurementSupportReqTable, wranIfBsCpeRegCapabilityRspTable, wranIfBsCpeMeasurementSupportRspTable, wranIfBsCpeAntennaGainTable, wranIfBsCapabilitiesConfigTable, wranIfBsMeasurementSupportTable, wranIfBsActionsTable, wranIfBsCpeMulticastConfigTable, wranIfBsCoexistenceConfigTable, wranIfBsScmCapabilityConfiguration, wranIfBsCpeScmCapabilityConfigTable, and wranIfBsScmAuthConfigTable.

        - wranIfBsCpeRngCapabilityReqTable

This object provides a table containing the ranging configuration requested by the CPE in the RNG-REQ during network entry. Each table is made up of multiple entries, one for each CPE that has sent a RNG-REQ, that is defined by wranIfBsCpeRngCapabilityReqEntry.

          - wranIfBsCpeRngCapabilityReqEntry

This object is a compound object that contains the ranging parameters transmitted in RNG-REQ by the CPE during network entry.

            - wranIfBsCpeMacAddress

The MAC address of the CPE that has attempted ranging with the BS.

            - wranIfMmpPn

Current value of MMP_PN that CPE is using if authenticated ranging (see 8.2.4.6.1.2) is used.

            - wranIfCiphertextIcv

Calculated value of Ciphertext ICV, calculated over RNG-REQ is authenticated ranging (see 8.2.4.6.1.2).

            - wranIfRngAnomaly

Inidication of any error condition detected by CPE during ranging process.

        - wranIfBsCpeRngCapabilityCmdTable

This object provides a table containing the ranging configuration the BS is specifiying for CPE in the RNG-CMD during network entry. Each table is made up of multiple entries, one for each CPE that a RNG-CMD is sent to, that is defined by wranIfBsCpeRngCapabilityRspEntry.

          - wranIfBsCpeRngCapabilityCmdEntry

This object is a compound object that contains the ranging parameters selected for the CPE by the BS and transmitted in RNG-CMD by the BS to CPE during network entry.

            - wranIfBsCpeMacAddress

MAC Address of CPE that the RNG-CMD is sent to. This is used to fill in MAC Address field of RNG-CMD, when RNG-CMD is sent in response to initial ranging.

            - wranIfBsCpeStationId

Station ID of CPE that RNG-CMD is sent to. This is used to fill in MAC Address field of RNG-CMD, when RNG-CMD is sent in response to initial ranging.

            - wranIfTimingAdvance

Timing advance parameter to Table 44.

            - wranIfEirpPerSubcarrier

EIRP per transmitted subcarrier parameter of RNG-CMD in Table 44.

            - wranIfOffsetFreqAdjust

Offset frequency adjustment parameter of RNG-CMD in Table 44.

            - wranIfRangingStatus

Ranging status field of RNG-CMD in Table 44.

            - wranIfActionSuperFrameNum

The Action Superframe Number field of RNG-CMD in Table 44.

            - wranIfCdmaCode

The CDMA Code field of RNG-CMD in Table 44.

            - wranIfTxOpportunityOffset

The Transmission Opportunity Offset field of RNG-CMD in Table 44.

        - wranIfBsCpeBasicCapabilityReqTable

This object provides a table containing the basic capability information CPEs that have requested configuration of from the BS in the CBC-REQ. Each table is made up of multiple entries, one for each CPE that has transmitted the CBC-REQ, that is defined by wranIfBsCpeBasicCapabilityReqEntry.

          - wranIfBsCpeBasicCapabilityReqEntry

This object is a compound object that contains the requested configuration of basic capabilities by a CPE during network entry. After wranIfBsCpeBasicCapabilityReqEntryIndex, the remaining list of objects used to make up wranIfBsCpeBasicCapabilityReqEntry can be found in 13.1.2.2.17 (wranIfBsCpeBasicCapabilityCmn).

            - wranIfBsCpeBasicCapabilityReqEntryIndex

A unique index for the entry in this table.

        - wranIfBsCpeBasicCapabilityRspTable

This object provides a table containing the basic capability information BS has configured for CPEs BS in the CBC-RSP. Each table is made up of multiple entries, one for each CPE that has transmitted the CBC-RSP, that is defined by wranIfBsCpeBasicCapabilityRspEntry.

          - wranIfBsCpeBasicCapabilityRspEntry

This object is a compound object that contains the configuration of basic capabilities BS has selected for a CPE during network entry. This table reflects the current configuration of a CPE’s basic capabilities. After wranIfBsCpeBasicCapabilityRspEntryIndex, the remaining list of objects used to make up wranIfBsCpeBasicCapabilityRspEntry can be found in 13.1.2.2.17 (wranIfBsCpeBasicCapabilityCmn).

            - wranIfBsCpeBasicCapabilityRspEntryIndex

A unique index for the entry in this table.

        - wranIfBsCpeRegCapabilityReqTable

This object provides a table containing the capability information that a CPE has requested configuration of from the BS during REG-REQ. Each table is made up of multiple entries, one for each CPE, that is defined by wranIfBsCpeRegCapabilityReqEntry.

          - wranIfBsCpeRegCapabilityReqEntry

This object is a compound object that contains the capabilties information that a CPE has requested configuration of by the BS, e.g. through sending a REG-REQ to the BS. Each each entry is uniquely identified by wranIfBsCpeMacAddress. The objects that make up the entry are defined in wranIfBsCpeRegCapabilityCmn (13.1.2.2.18). A REG-REQ message is comprised of objects within this compound object, wranIfBsCpeAntennaGainTable (13.1.2.2.9), and wranIfBsCpeMeasurementSupportReqTable (13.1.2.2.6).

""""

        - wranIfBsCpeMeasurementSupportReqTable

A compound object representing the Measurement Support IE of REG-REQ/RSP in 7.7.7.3.4.7. It is made up of multiple entries, one for each signal type that sensing is supported for. Each entry is defined by wranIfBsCpeMeasurementSupportReqEntry. Entries for a CPE are only present if the value for wranIfSensModeSupportArray is anything other than "No Sensing".

          - wranIfBsCpeMeasurementSupportReqEntry

A compound object representing entries of Measurement Support IE of REG-REQ in 7.7.7.3.4.7. It is made up of multiple objects. A CPE will have one entry for each Signal Type in the Signal Type Array of the Measurement Support IE. Each entry is uniquely identified by wranIfMeasurementSupportReqEntryIndex. The rest of the objects that represent each entry are defined in wranIfBsCpeMeasurementSupportCmn (13.2.1.1.19).

            - wranIfMeasurementSupportReqEntryIndex

A unique index for every entry in wranIfBsCpeMeasurementSupportReqTable.

        - wranIfBsCpeRegCapabilityRspTable

This object provides a table containing the capability information that a BS has configured for a CPE in REG-RSP. Each table is made up of multiple entries, one for each CPE, that is defined by wranIfBsCpeRegCapabilityRspEntry.

          - wranIfBsCpeRegCapabilityRspEntry

This object is a compound object that contains the capabilties information that a CPE has requested configuration of by the BS, e.g. through sending a REG-RSP to the BS. wranIfPermanentStationId is unique to REG-RSP. The remaining objects that make up the entry are defined in wranIfBsCpeRegCapabilityCmn (13.1.2.2.18). A REG-RSP message is comprised of objects within this compound object and wranIfBsCpeMeasurementSupportRspTable (13.1.2.2.8).

            - wranIfMeasurementSupportEntryIndex

A unique index for every entry in wranIfBsCpeMeasurementSupportReqTable.

        - wranIfBsCpeMeasurementSupportRspTable

A compound object representing the Measurement Support IE of REG-REQ/RSP in 7.7.7.3.4.7. It is made up of multiple entries, one for each signal type that sensing is supported for. Each entry is defined by wranIfBsCpeMeasurementSupportRspEntry.

          - wranIfBsCpeMeasurementSupportRspEntry

A compound object representing entries of  Measurement Support IE of REG-RSP in 7.7.7.3.4.7. It is made up of multiple objects. A CPE will have one entry for each Signal Type in the Signal Type Array of the Measurement Support IE. Entries for a CPE are only present if the value for wranIfSensModeSupportArray is anything other than "No Sensing". Each entry is uniquely identified by wranIfMeasurementSupportRspEntryIndex. The rest of the objects that represent each entry are defined in wranIfBsCpeMeasurementSupportCmn (13.2.1.1.19).

            - wranIfMeasurementSupportRspEntryIndex

A unique index for every entry in wranIfBsCpeMeasurementSupportReqTable.

        - wranIfBsCpeAntennaGainTable

At the CPE this object represents a compound object representing entries of the CPE Antenna Gain IE (7.7.7.3.4.9), that carries a CPE’s receive antenna gain when a CPE transmits a REG-REQ to the BS. At the BS, this object is a compound object that represents the entries for its’ own transmit antenna gain as well as the receive antenna gain information for each CPE that sent a CPE Antenna Gain IE in a REG-REQ (and has successfully completed registration). This object is a table that is made up of multiple entries, each defined by wranIfBsCpeAntennaGainEntry.

          - wranIfBsCpeAntennaGainEntry

A compound object representing the on-axis gain (in dB) for each channel the CPE’s or BS’s antenna is capable of operating on.

            - wranIfBsCpeMacAddress

MAC address of the of the device

            - wranIfTvChannel

TV Channel number

            - wranIfOnAxisGain

Maximum gain in the specified TV channel

        - wranIfBsScmCapabilityConfiguration

This MIB provides a bitmap that describes the cryptographic suites that the BS supports. The list of suites is provided in Table 193.

        - wranIfBsCpeScmCapabilityConfigTable

This MIB provides a table that provides a listing for the security capabilities for each CPE. Each CPE stores this table. On the BS, this table is made up of multiple entries, one for each CPE. Entries on the BS table do not contain the wranIfBsCpeEapTlsTtlsCredential object. Each entry is defined by wranIfBsCpeScmCapabilityConfigEntry. The list of capabilities is listed in Table 193.

          - wranIfBsCpeScmCapabilityConfigEntry

This object is a compound object that provides the definition of the entries wranIfBsCpeScmCapabilityConfigTable.

            - wranIfBsCpeMacAddress

The MAC address of the CPE.

            - wranIfBsCpeScmCapabilityConfiguration

This MIB provides a bitmap that describes the cryptographic suites that the CPE supports. The list of suites is provided in Table 193.

            - wranIfBsCpeEapTlsTtlsCredential

X.509 certificate that defines the credential the CPE exchanges with the AAA server to perform authentication

        - wranIfBsScmAuthConfigTable

This MIB provides a table that provides the configuration of the SCM attributes (e.g. timers and other items relating to the authorization process). This table is made up of one entry, defined by wranIfBsScmAuthConfigEntry.

          - wranIfBsScmAuthConfigEntry

This object is a compound object that defines an entry in wranIfBsScmAuthConfigTable.

            - wranIfBsT36

EAP Authentication Timer, T36.

            - wranIfBsT37

Authentication Grace Timer, T37.

            - wranIfBsMaxNumAuthAttempts

Maximum # of Authentication Attempts.

            - wranIfBsT38

Operational Wait Timeout, T38.

            - wranIfBsT39

Rekey Wait Timeout, T39.

            - wranIfBsT40

GTEK/TEK Grace time, T40.

            - wranIfBsAkLifetime

Lifetime BS assigns to new AK.

            - wranIfBsTekLifetime

Lifetime BS assigns to new TEK.

            - wranIfBsMaxNumSa

Maximum number of SAs a CPE can be authorized for.

            - wranIfBsT17

Time for CPE to complete authentication and key exchange.

        - wranIfBsActionsTable

This object provides a table that stores actions that can be configured to have BS direct a CPE to act upon receiving unsolicited MAC management messages such as RNG-CMD and DREG-CMD. Each table is made up of an entry for each action directed to a particular CPE. Each entry is defined by wranIfBsActionsEntry.

          - wranIfBsActionsEntry

This object is a compound object that contains actions that are requested for a particular CPE. A BS may skip directing the CPE to execute a particular action based on the current state of the CPE. Only one action is executable at a given time.

            - wranIfBsCpeActionsMacAddress

This uniquely identifes the CPE that is the target of the action by the CPE’s MAC address.

            - wranIfBsCpeActionsRangeCpe

When set, the BS will send an unsolicited RNG-CMD to a CPE with the Ranging Status field set to ‘abort’ without any new parameter or the Ranging Status field set to ‘abort’ with a new set of parameters. No action is to be taken if this object is read.

            - wranIfBsCpeActionsDeRegCpe

When set to an Action Code value as defined in Table 115, the BS will send a DREG-CMD to the CPE with that Action Code. No action is to be taken if this object is read or an invalid ActionCode is specified.

            - wranIfBsCpeActionsStatus

This object contains the status of the write operation for a component of the wranIfBsCpeActionsEntry, to make sure the write operation has been executed properly.

        - wranIfBsCpeMulticastConfigTable

This table contains the configuration of multicast groups and what CPEs are assigned to them. Each table is made up of multiple entries, defined by wranIfBsCpeMulticastEntry. A CPE may have multiple entries in this table, one for each multicast group that it belongs to. Entries will be deleted for a CPE whenever a CPE is asked to leave a multicast group.

          - wranIfBsCpeMulticastEntry

This object is a compound object that contains information about the configuration of a CPE’s multicast group membership configuration.

            - wranIfBsCpeMacAddress

This object refers to a CPE’s MAC address. It is used to uniquely index the multicast group configuration for a particular CPE in wranIfBsCpeMulticastConfigTable, along with wranIfBsCpeMulticastCid.

            - wranIfBsMulticastSid

This object refers to the multicast SID that is assigned to a multicast group.

            - wranIfBsMulticastPeriodicAllocationParameterM

This object defines the ‘M’ value (see Table 103) that is used to calculate the periodic allocation for multicast transmission.

            - wranIfBsMulticastPeriodicAllocationParameterK

This object defines the ‘K’ value (see Table 103) that is used to calculate the periodic allocation for multicast transmission.

            - wranIfBsMulticastPeriodicAllocationParameterN

This object defines the ‘N’ value (see Table 103) that is used to calculate the periodic allocation for multicast transmission.

        - wranIfBsCoexistenceConfigTable

This table contains the configuration items related to Coexistence operation and CBP transmission. It is made up of one entry represents the default values for Coexistence operation and CBP transmission.

          - wranIfBsCoexistenceConfigEntry

This object represents a single entry that is in wranIfBsCoexistenceConfigTable.

            - wranIfBsT34

This governs how often (in Seconds) the Device Identification IE is transmitted in a CBP burst. Default value is 300s.

            - wranIfBsT33

Time between transmission of CBP bursts with backup/candidate channel list information (see 7.6.1.3.1.1) to facilitate spectrum etiquette.

            - wranIfBsT32

Wait for Frame Contention Response (FC-RSP) message.

            - wranIfBsFcw

Frame contention window, the number of superframes that a contention destination accumulates frame contention requests before responding to them.

            - wranIfBsScwBackoffMax

Maximum number of superframes for the SCW backoff window.

            - wranIfBsFcMin

Number of frames not available for contention at the BS.

            - wranIfBsFcnRange

Exponent, base 2, defining the range of random numbers for the FCN.

            - wranIfBsSfRel

Number of superframes, after which a BS  releases the frames won by another BS through frame contention.

            - wranIfBsT35

SCW Backoff Timer, timer controlling continuation or exiting of Frame Contention Procedure.

        - wranIfBsCpeBasicCapabilityCmn

This MIB object is a group containing objects that are common to wranIfBsCpeBasicCapabilityReqEntry and wranIfBsCpeBasicCapabilityReqEntry.

          - wranIfBsCpeBasicCapabilityReqNumAttempts

The current number of attempts that a CPE as attempted basic capability configuration during network entry. This item gets set to 0 upon successful completion of registration process and a CPE is admitted into the network. This item gets incremented every time a CPE attempts basic capability configuration, but is unsuccessful. If this value reaches the limit set by wranIfBsMaxNumCbcReqAttempts, then BS shall re-attempt network entry.

          - wranIfBsCpeMacAddress

The MAC address of the CPE attempting basic capability configuration.

          - wranIfBsCpeStationId

The Station ID of the CPE attempting basic capability configuration.

          - wranIfMacPduTxandConstruction

An integer value that indicates the methods for transmission and construction of MAC PDUs that the CPE supports. This reflects the setting of the IE defined in 7.7.11.3.1.

          - wranIfMaxCpeTxEirp

A integer value, encoded in hexadcimal, that indicates the maxium EIRP that the CPE is configured for. This reflects the setting of the IE defined in 7.7.11.3.2.1.

          - wranIfCpeDemodulator

An bit map that encodes the DIUC’s that the CPE supports. This reflects the setting of the IE defined in 7.7.11.3.2.2.1.

          - wranIfCpeModulator

An bit map that encodes the UIUC’s that the CPE supports. This reflects the setting of the IE defined in 7.7.11.3.2.2.2.

          - wranIfCpeScmVersionSupport

Indicator of what version of the SCM protocol that the CPE supports. This reflects the setting of the IE defined in 6.9.11.3.3.1.

          - wranIfCpePnWindowSize

Size of PN_WINDOW (see 7.4) that is used to protect against replay attacks. This reflects the setting of the IE defined in 7.7.11.3.3.2.

          - wranIfCpeScmFlowControl

Maximum number of ongoing SCM transactions that the CPE can support. This reflects the setting of the IE defined in 7.7.11.3.3.3.

        - wranIfBsCpeRegCapabilityCmn

This MIB object is a group containing objects that are common to wranIfBsCpeRegCapabilityReqEntry and wranIfBsCpeRegCapabilityReqEntry.

            - wranIfBsCpeMacAddress

MAC addresss of CPE that is currently registered with BS. This object uniquely identifies the entry associated with a particular CPEs configured capabilities.

            - wranIfBsRegisteredCpeRegReqNumAttempts

The current number of attempts that a CPE as attempted network entry. This item gets set to 0 upon successful completion of registration process and a CPE is admitted into the network. This item gets incremented every time a CPE attempts registration, but is unsuccessful. If this value reaches the limit set by wranIfBsMaxNumRegReqAttempts, then BS shall send a DREG-CMD to CPE.

            - wranIfNMEALocString

NMEA location string of the CPE in REG-REQ (see 7.7.7.3.1).

            - wranIfICsConfig

Indication in REG-REQ/RSP of how the provider will operate the CPE on an ongoing basis; either with the Ethernet CS only, the IP CS, or both (see 7.7.7.3.2).

            - wranIfIpVersion

What version of the IP protocol (either v4 or v6) indicate in REG-REQ/RSP the CPE supports (see 7.7.7.3.3).

            - wranIfIpRohcSupport

Indicator in REG-REQ/RSP of whether or not CPE supports IP Robust Header Compression (ROHC), see 7.7.7.3.4.1.

            - wranIfArqSupport

ARQ Support IE of REG-REQ/RSP in 7.7.7.3.4.2

            - wranIf2ndMgmtArqWindowSize

Secondary Management flow - ARQ Window Size IE of REG-REQ/RSP defined in 7.7.8.9.17.2.

            - wranIf2ndMgmtArqRetryTxDelay

Secondary Management flow - Transmitter Delay component of ARQ Retry Timeout IE of REG-REQ/RSP defined in 7.7.8.9.17.3.

            - wranIf2ndMgmtArqRetryRxDelay

Secondary Management flow - Receiver Delay component of ARQ Retry Timeout IE of REG-REQ/RSP defined in 7.7.8.9.17.3.

            - wranIf2ndMgmtArqBlockLifetime

Secondary Management flow - ARQ Block Lifetime IE of REG-REQ/RSP defined in 7.7.8.9.17.4.

            - wranIf2ndMgmtArqSyncLossTimeout

Secondary Management flow - ARQ Sync Loss Timeout IE of REG-REQ/RSP defined in 7.7.8.9.17.5.

            - wranIf2ndMgmtArqDeliverInOrder

Seconday Management flow -ARQ Deliver In Order IE of REG-REQ/RSP defined in 7.7.8.9.17.6.

            - wranIf2ndMgmtArqRxPurgeTimeout

Secondary Management flow - ARQ Rx Purge Timeout IE of REG-REQ/RSP defined in 7.7.8.9.17.7.

            - wranIf2ndMgmtArqBlockSize

Secondary Management flow - ARQ Block Size IE of REG-REQ/RSP defined in 7.7.8.9.17.8.

            - wranIfDsxFlowControl

DSx Flow Control IE of REG-REQ/RSP in 7.7.7.3.4.4

            - wranIfMcaFlowControl

MCA Flow Control IE of REG-REQ/RSP in 7.7.7.3.4.5

            - wranIfMaxNumMcastGroups

Maximum Number of Multicast Groups IE of REG-REQ/RSP in 7.7.7.3.4.6

            - wranIfSensModeSupportArray

Value fo the "Sensing Mode Support Array" of the Measurement Support IE in REG-REQ/RSP in 7.7.7.3.4.7. If the value of this is set to "No Sensing" then wranIfBsCpeMeasurementSupportReqTable will not be stored for the CPE.

            - wranIfAntennaModel

Manufacturer Specific Antenna Model IE of REG-REQ in 7.7.7.3.4.8.

            - wranIfCpeResidualDelay

CPE Residual Dealy IE of REG-REQ in 7.7.7.3.4.10

            - wranIfSecMgmtIpAllocationMethod

Method for allocating IP Addresses on Secondary Mangement Connection IE of REG-REQ/RSP in 7.7.7.3.4.11

            - wranIfCpeOperationalCapability

CPE Operation Capability IE of REG-REQ in 7.7.7.3.4.13

            - wranIfCpeRegistrationTimer

CPE Registration Timer IE of REG-REQ/RSP in 7.7.7.3.5. This value is used to set T30 for the CPE.

            - wranIfPermanentSid

Permanent station ID assigned to CPE, when CPE is entering the network under the CPE Privacy method (see 8.7). The format of this IE shall be as defined in 7.7.7.3.4.12.

        - wranIfBsCpeMeasurementSupportCmn

This MIB object is a group containing objects that are common to wranIfBsCpeMeasurementSupportReqEntry and wranIfBsCpeMeasurementSupportRspEntry.

          - wranIfBsCpeRegisteredMacAddress

MAC address of CPE. This corresponds to an entry in wranIfBsCpeRegistrationReqTable for a registered CPE.

          - wranIfMeasurementSignalType

Signal type that measurement configuration this entry pertains to (see Table 237).

          - wranIfMeasurementThreshold

Signed number that signifies the sensitivity threshold for the signal type.

          - wranIfMeasurementPd

Probability of detection (PD) for the signal type.

          - wranIfMeasurementMpfa

Maximum Probablility of False Alarm for the signal type.

          - wranIfMeasurementRecNumSensPeriods

Recommended number of sensing periods required to sense the signal type.

          - wranIfMeasurementRecSensPeriodDuration

Recommended duration of sensing periods, units of symbols.

          - wranIfMeasurementRecSensPeriodInterval

Recommended length of sensing period interval, units of integer number of frames.

        - wranIfBsCpeSystemParametersCmn

This MIB object is a group containing objects that define system constants for REG-REQ/RSP, DSx-REQ/RSP, and MCA-RSP transactions.

          - wranIfBsDsxReqRetries

Maximum number of timeout retries for DSx-REQ messages

          - wranIfBsDsxRspRetires

Maximum number of timeout retries for DSx-RSP messages

          - wranIfBsT7

Wait for DSx-RSP timeout

          - wranIfBsT8

Wait for DSA/DSC-RSP timeout

          - wranIfBsT10

Wait for Transaction End timeout

          - wranIfBsT13

Time allowed for a CPE, follwing receipt of a REG-RSP to send a TFTP-CPLT message to the BS.

          - wranIfBsT15

Wait for MCA-RSP

          - wranIfBsT22

Wait for ARQ-Reset

          - wranIfBsT27Idle

Maximum time between unicast grants to CPE when BS believe CPE transmission quality is good enough.

          - wranIfBsT27Active

Maximum time between unicast grants to CPE when BS believes CPE transmission quality is not good enough

          - wranIfBsT28

Time allowed for BS to complete the transmission of the backup/candidate channel list to its CPEs after initial registration by a new CPE, including the database service query.

        - wranIfBsCpeRegCapabilityDefault

This object is a compound object containing default values of REG-REQ/RSP IEs. This table can be used by the BS to judge/verify a CPE's REG-REQ, and used to construct the REG-RSP message. The objects that make up this table are defined in wranIfBsCpeRegCapabilityCmn (13.1.2.2.17).

        - wranIfBsCpeBasicCapabilityDefault

This object is a compound object containing default values of CBC-REQ/RSP IEs. This table can be used by the BS to judge/verify a CPE's CBC-REQ, and used to construct the CBC-RSP message. The objects that make up this table are defined in wranIfBsCpeBasicCapabilityCmn (13.1.2.2.16).

        - wranIfBsCpeMeasurementSupportDefault

This Table is a compound object containing default values of used to configure the Measurement Support REG-REQ/RSP IE. This table can be used by the BS to judge/verify the Measurement Support IE in CPE's REG-REQ, and used to construct the Measurement Support IE sent by the BS in the REG-RSP message. There is one entry for each signal type, defined by wranIfBsCpeMeasurementSupportDefaultEntry.

          - wranIfBsCpeMeasurementSupportDefaultEntry

This object is a compound object that defines an entry in wranIfBsCpeMeasuremetnSupportDefault.  Each entry is identified by the following objects: wranIfMeasurementSignalType, wranIfMeasurementThreshold, wranIfMeasurementPd, wranIfMeasurementRecNumSensPeriods, wranIfMeasureRecSensPeriodInterval (which can be found in sections 13.1.2.2.18.2-13.1.2.2.18.8).

        - wranIfBsPhy

This MIB object contains managed objects related to the PHY configuration. All object described are related to the OFDMA PHY that is supported.

          - wranIfBsOfdmaPhyUsChannelTable

This object provides a table to describe attributes of upstream channels. It is a compound object that is made up of multiple entries, each described by wranIfBsOfdmaPhyUsChannelTableEntry.

            - wranIfBsOfdmaPhyUsChannelTableEntry

This object is a compound object that represents an entry for the upstream channel BS.

              - wranIfBsOfdmaCtBasedResvTimeout

The number of US-MAPs to receive before contention-based reservation is attempted again for the same connection.

              - wranIfBsOfdmaUsCenterFrequency

Upstream center frequency in kHz.

              - wranIfBsOfdmaUsRadioResource

Indicates the average percentage (ratio) of non-assigned US radio resource to total usable US radio resources.

              - wranIfBsOfdmaUsConfigChangeCount

Current UCD change count.

              - wranIfBsOfdmaUsUcsNotificationCodes

Number of CDMA codes for UCS Notification.

              - wranIfBsOfdmaUsInitRngCodes

Number of CDMA codes for Initial Ranging

              - wranIfBsOfdmaUsPeriodicRngCodes

Number of CDMA codes for periodic ranging.

              - wranIfBsOfdmaUsBWReqCodes

Number of CDMA codes for bandwidth requests.

              - wranIfBsOfdmaUsPeriodicRngBackoffStart

Represented in powers of 2, initial size of backoff window used for periodic ranging contention.

              - wranIfBsOfdmaUsPeriodicRngBackoffEnd

Represented in powers of 2, final size of backoff window used for periodic ranging contention.

              - wranIfBsOfdmaUsStartofCodes

Includes first code in block of codes to be used in a particular BS, known as S. The total set of codes ranges from S to (wranIfBsOfdmaUsInitRngCodes+wranIfBsOfdmaUsPeriodicRngCodes+wranIfBsOfdmaUsBWReqCodes+wranIfBsOfdmaUsUcsNotificationCodes) mod 256.

              - wranIfBsOfdmaUsNormalizedCnrOverride

This is a list of numbers, encoded by a nibble and interpreted by a signed integer. The the nibbles are defined in section 8.10.3.2. The number encoded by each nibble represents the difference in normalized CNR relative to the previous one.

              - wranIfBsOfdmaUsNormalizedCnrValue

A signed integer that corresponds to the normalized CNR value in the previous line.

              - wranIfBsOfdmaUsCpeUpPowerAdjStep

CPE-specific up power offset adjustment step.

              - wranIfBsOfdmaUsInitialRangingInterval

Number of frames between initial ranging interval allocation

              - wranIfBsOfdmaUsTxPowerReport

Tx Power report

              - wranIfBsOfdmaUsUcsNotificationBackoffStart

Expressed as a power of 2, initial backoff window size for UCS notification contention.

              - wranIfBsOfdmaUsUcsNotificationBackoffEnd

Expressed as a power of 2, final backoff window size for UCS notification contention.

              - wranIfBsOfdmaUsInitialRngBackoffStart

Expressed as a power of 2, initial backoff window size for initial ranging contention.

              - wranIfBsOfdmaUsInitialRngBackoffEnd

Expressed as a power of 2, final backoff window size for initial ranging contention.

              - wranIfBsOfdmaUsBwRequestBackoffStart

Expressed as a power of 2, initial backoff window for contention-based BW requests.

              - wranIfBsOfdmaUsBwRequestBackoffEnd

Expressed as a power of 2, the final backoff window size for contention-based BW requests.

              - wranIfBsOfdmaUsRelPwrOffsetMacMgmtBurst

Relative to normal traffic bursts, the power offset used for US MAC management message transmission.

              - wranIfBsOfdmaUsInitialTxTiming

Initial timing reference for US transmissions.

              - wranIfBsOfdmaUsRangingRegion

US ranging region definition.

              - wranIfBsOfdmaUsUcdInterval

Time between transmission of UCD messages.

              - wranIfBsOfdmaUsUcdTransition

Time BS shall wait after repeating a UCD message with an incremented Configuration Change Count before issuing a US-MAP message referring to Upstream_Burst_Profiles defined in that UCD message.

              - wranIfBsOfdmaUsClkCmpInterval

Time between the clock compare measurements used for the generation of CLK-CMP messages.

              - wranIfBsOfdmaUsT57

Lost US-MAP interval, time since last received US-MAP message before upstream synchronization is considered lost (used on the CPE).

              - wranIfBsOfdmaT58

Number of SCH that can be lost until synchronization is considered lost.

              - wranIfBsOfdmaUsCdmaRngRetries

Number of retries on CDMA RNG-REQs.

              - wranIfBsOfdmaUsInvRngReq

Number of retries on inviting RNG-REQs.

              - wranIfBsOfdmaUsMapProcTime

Time provided between arrival of the last bit of a US-MAP at a CPE and the effectiveness of that map.

              - wranIfBsOfdmaUsT3

RNG-CMD reception timeout following the transmission of a RNG-REQ.

              - wranIfBsOfdmaUsT4

Time to wait for unicast ranging opportunity.

              - wranIfBsOfdmaUsT5

Time to wait for Upstream Channel Change response.

              - wranIfBsOfdmaUsT12

Wait for UCD descriptor.

          - wranIfBsOfdmaPhyDsChannelTable

This object provides a table to describe attributes of downstream channels. It is a compound object that is made up of multiple entries, each described by wranIfBsOfdmaPhyDsChannelTableEntry.

            - wranIfBsOfdmaPhyDsChannelTableEntry

This object is a compound object that represents an entry for the downstream channel in each BS.

              - wranIfBsOfdmaDsEirp

The equivalent isotropic radiated power of the base station, which is computed for a simple single-antenna transmitter.

              - wranIfBsOfdmaDsChannelNumber

Current operating channel.

              - wranIfBsOfdmaDsPhyMaxEirp

Initial ranging max EIRP, at BS in units of 1 dBm.

              - wranIfBsOfdmaDsCenterFreq

DS center frequency in kHz.

              - wranIfBsOfdmaDsBsId

Base Station Id.

              - wranIfBsOfdmaDsMacVersion

The MAC version to which the BS is conformant to.

              - wranIfBsOfdmaDsCyclicPrefix

Ratio of CP time to useful symbol time; possible values are ¼, 1/8, 1/16, 1/32.

              - wranIfBsOfdmaDsRadioResource

Average ratio of non-assigned DS radio resources to total usable DS radio resources.

              - wranIfBsOfdmaDsHysteresisMargin

When the CINR of a neighbor BS is larger than the sum of the CINR of the current serving BS and the hysteresis margin for the time-to-trigger duration, than the neighbor BS is included in the list of possible target BS to attempt initial network entry with.

              - wranIfBsOfdmaDsCellType

This object identifies classes of BSs that can be used by CPE when selecting which cell to attempt network entry with.

              - wranIfBsOfdmaDsConfigChangeCount

Current BS DCD configuration change count.

              - wranIfBsOfdmaDsPowerControlMode

Defines the default power control information to CPE.

              - wranIfBsOfdmaDsFrameDuration

Duration of the frame.

              - wranIfBsOfdmaDsRssiCinrAvgParameter

Bits 0-3 of default RSSI and CINR averaging parameter.

              - wranIfBsOfdmaDsThresholdAddBsServiceSet

Threshold used by CPE to add a neighbor BS to the list of available WRAN services.

              - wranIfBsOfdmaDsThresholdDelBsServiceSet

Threshold used by CPE to delete a neighbor BS to the list of available WRAN services.

              - wranIfBsOfdmaDsDcdInterval

Time between transmission of DCD messages.

              - wranIfBsOfdmaDsDcdTransition

Time BS shall wait after repeating a DCD message with an incremented Configuration Change Count before issuing a DS-MAP message referring to Downstream_Burst_Profiles defined in that DCD message.

              - wranIfBsOfdmaDsT56

Time since last received DS-MAP message before downstream synchronization is considered lost.

              - wranIfBsOfdmaDsT1

Wait for DCD timeout.

              - wranIfBsOfdmaDsT2

Wait for broadcast ranging timeout

              - wranIfBsOfdmaDsT20

Time CPE searches for preambles on  a given channel.

              - wranIfBsOfdmaDsT21

Time the CPE searches for a DS-MAP on a given channel.

              - wranIfBsOfdmaDsTtg

Transmit/Receive Transition Gap

          - wranIfBsOfdmaUcdBurstProfileTable

This table contains the UCD burst profile configurations for each upstream channel. Each entry in the table is represented by wranIfBsOfdmaUcdBurstProfileEntry.

            - wranIfBsOfdmaUcdBurstProfileEntry

This is a compound object that defines each entry in wranIfBsOfdmaUcdBurstProfileTable.

              - wranIfBsOfdmaUcdUiucIndex

The UIUC that indicates the upstream burst profile in the UCD message

              - wranIfBsOfdmaUcdFecCodeType

Modulation and FEC for upstream.

              - wranIfBsOfdmaUcdUiucExitThreshold

CINR at or below which this UIUC can no longer be used where change to a more robust UIUC is required.

              - wranIfBsOfdmaUcdUiucEntryThreshold

Minimum CINR required to starting using this UIUC when changing from a more robust UIUC is required.

              - wranIfBsOfdmaUcdRangingDataRatio

Difference in power from burst UCD and power to be used for CDMA ranging in units of 1 dB.

          - wranIfBsOfdmaDcdBurstProfileTable

This table provides configuration for each DCD burst profile. It is made up of multiple entries defined by wranIfBsOfdmaDcdBurstProfileEntry.

            - wranIfBsOfdmaDcdBurstProfileEntry

This is a compound object that defines each entry in wranIfBsOfdmaDcdBurstProfileTable.

              - wranIfBsOfdmaDcdDiucIndex

The DIUC of the DS burst profile in the DCD message.

              - wranIfBsOfdmaDcdFecCodeType

Modulation and FEC for downstream.

              - wranIfBsOfdmaDcdDiucExitThreshold

CINR at or below which this DIUC can no longer be used where change to a more robust DIUC is required.

              - wranIfBsOfdmaDcdDiucEntryThreshold

Minimum CINR required to starting using this DIUC when changing from a more robust DIUC is required.

          - wranIfBsOfdmaDsRegionTable

This table provides the configuration of the DS subframe. It is made up of entries defined by wranIfBsOfdmaDsRegionEntry.

            - wranIfBsOfdmaDsRegionEntry

This is a compound object that describes each entry in wranIfBsOfdmaDsRegionTable.

              - wranIfBsOfdmaDsRegionIndex

Index DS region in table.

              - wranIfBsOfdmaDsDuration

Number of OFDMA slots linearly allocated to a DS burst region

          - wranIfBsOfdmaUsRegionTable

This table provides the configuration of the US subframe. It is made up of entries defined by wranIfBsOfdmaUsRegionEntry.

            - wranIfBsOfdmaUsRegionEntry

This is a compound object that describes each entry in wranIfUsOfdmaDsRegionTable.

              - wranIfBsOfdmaUsRegionIndex

Index US region in table.

              - wranIfBsOfdmaUsDuration

Number of OFDMA slots linearly allocated to a US burst region

      - wranIfBsAm

This MIB group contains various objects related to Accounting Management.

        - wranIfBsOtaUsageDataRecordTable

This object contains usage entries that track the number of octets/packets transmitted or received over the air interface. Records may be transferred to an external database, such as an AAA server, after which they can be deleted from this table. Each entry is defined by wranIfBsOtaUsageDataRecordEntry.

          - wranIfBsOtaUsageDataRecordEntry

This object is a compound object that identifies entries in the wranIfBsOtausageDataRecordTable.

            - wranIfBsSid

A 9-bit Station ID that identifies the station that is carrying traffic.

            - wranIfBsFid

A 3-bit Flow ID that identifies the specific flow assigned to a station that is carrying traffic. For data traffic this identifies the FID that is mapped to a service flow id (i.e. wranIfBsServiceFlowId).

            - wranIfBsSessionId

An identifier for a session. A session is a segement in time when a service flow is active. Multiple sessions can be created during the service flow activation time to allow the BS to track usage during periods when the service flow configuration (e.g. QoS) is changing.

            - wranIfBsServiceFlowId

32-bit identifer of the service flow. This value is set to 0, when wranIfBsFid is a basic, primary management, secondary management for unicast SIDs (single CPEs), as wel as the multicast management and polling FID of a multicast SID (group of CPEs).

            - wranIfBsMacSduCount

Counter of the number of MAC SDUs transmitted on a FID. If the FID represents a basic, primary/secondary management, or multicast management CID, then this deals with traffic transmited on both the DS and US.

            - wranIfBsOctetCount

Counter of the number of MAC SDUs/messages that have been transmitted and received over the air interface.

            - wranIfBsSessionStartTime

Date and time a session was established.

            - wranIfBsSessionEndTime

Date and time a session was ended.

            - wranIfBsOtaQoSServiceFlowList

A list of SFIDs for each service flow active during this session (i.e. a list of SFIDs that pertains to service flows as defined in wranIfBsActiveSfTable).

            - wranIfBsOtaQoSProfileIndexList

A list of indexs into wranIfBsScTable, that point to the definition of the QoS Parameter set of each of service flow listed in wranIfBsOtaQoSServiceFlowList (and hence were active during this session).

      - wranIfBsPm

This MIB group contains objects related to Performance Management. The following tables are provided in this MIB:

- wranIfBsPmConfigrationTable: Provides configuration determining which tables are enabled.

- wranIfBsRssiCinrMetricsTable: Help track metrics related to measurement of CPE upstream signal by BS and BS downstream signal by CPE.

- wranIfBsStartupMetricsTable: Help track non-timing metrics related to network entry and re-entry.

- wranIfBsThroughputMetricsTable: Help track metrics related to peak/average data rate.

- wranIfBsNetworkEntryMetricsTable: Help track timing metrics related to network entry and re-entry .

- wranIfBsPacketErrorRateTable: Help track metrics related to packet error rate measurements.

- wranIfBsUserMetricsTable: Help track metrics related to current status of CPEs in the cell.

- wranIfBsServiceFlowMetricsTable: Helps track metrics related to service flows.

- wranIfBsArqMetricsTable: Helps track the performance of ARQ.

- wranIfBsAuthenticationMericsTable: Table helps track the number of authentication/encryption errors that occur.

- wranIfBsCoexistenceStatusTable: Table helps track the status of ongoing coexistence (e.g. Frame Contention) transaction

- wranIfBsCoexistenceSourceTable: Table helps track what neighbor BSs are attempting a coexistence (Frame Contention) transaction with a particular source BS

- wranIfBsCoexistenceResourceListTable: Table helps track what resources neighboring WRANs are making use of. This includes the Backup/Candidate channel lists, DS/US splits, SCW schedule.

- wranIfBsCoexistenceCurrentConfigTable: Table helps keep track of current (frame) allocation allowed to winner of recent winner, and the release time.

        - wranIfBsPmConfigurationTable

The configuration of statistics capture and measurement is captured in this table. There is one entry in this table for each BS sector. The entries are defined by wranIfBsPmConfigurationEntry.

          - wranIfBsPmConfigurationEntry

This object is a compound object that represents an entry in wranIfBsPmConfigurationTable.

            - wranIfBsGranularityInterval

Data rate statistics captured in wranIfBsDataRateStatisticsTable are measured over the time interval this object specifies.

            - wranIfBsCountersReportInterval

This MIB determines the interval in which traps in wranIfBsTrapControlRegister related to performance measurement are reported to the NCMS.

            - wranIfBsPmMeasurementBitmap

This MIB object is a bitmap indicating which of the measurement tables are enabled or disabled.

        - wranIfBsRssiCinrMetricsTable

This MIB object contains a table that records BS upstream measurement of a CPE’s transmissions, as well as CPE measurement of BS downstream signal. The data is stored as a histogram. This table is made up of entries defined by wranIfBsRssiCinrMetricsEntry. Each entry is uniquely identified by the CPE’s MAC address and the index of the entry in the histogram.

          - wranIfBsRssiCinrMetricsEntry

This is a compound object made up objects that represent an entry in wranIfBsRssiCinrMetricsTable.

            - wranIfBsCpeMacAddress

MAC Address of the CPE.

            - wranIfBsCpeHistogramIndex

Index in histogram that entry pertains to.

            - wranIfBsChannelDirection

Direction of channel, whether DS or US, that measurement was done on.

            - wranIfBsChannelNumber

Channel number that measurement was done on.

            - wranIfBsStartFrame

Frame number in which measurement was conducted in.

            - wranIfBsMeasurementDuration

Duration of measurement period in units of symbol period.

            - wranIfBsSignalReportType

This object indicates what type of signal was detected during the measurement that is recorded in this entry.

            - wranIfBsMeanCinrReport

Mean CINR report.

            - wranIfBsMeanRssiReport

Mean RSSI report.

            - wranIfBsStdDevCinrReport

Standard Deviation CINR report.

            - wranIfBsStdDevRssiReport

Standard Deviation RSSI report.

        - wranIfBsStartupMetricsTable

This MIB provides a table to track how CPE’s perform during initial network entry and re-entry. This table is made of entries, defined by wranIfBsStartupMetricsEntry. There is one entry for each sector of the BS.

          - wranIfBsStartupMetricsEntry

This object is a compound object that defines entries in wranIfBsStartupMetricsTable.

            - wranIfBsNumAuthAttempt

A counter for the number of CPE authentication attempts.

            - wranIfBsNumAuthSuccess

A counter for the number of successful authentication handhake completions.

            - wranIfBsAuthSuccessRate

Success rate of authentication attempts: wranIfBsAuthSuccessRate = 100 * (wranIfBsNumAuthSuccess/wranIfBsNumAuthAttempt).

            - wranIfBsNumRangingAttempt

Number of CPE ranging requests received.

            - wranIfBsNumRangingSuccess

Number of CPE ranging responses sent.

            - wranIfBsRangingSuccessRate

Success rate of ranging attempts: wranIfBsRangingSuccessRate = 100 * (wranIfBsNumRangingSuccess/wranIfBsNumRangingAttempt).

        - wranIfBsThroughputMetricsTable

This MIB object provides a table to record peak/average data rate. This table is made up of multiple entries, one for each sector of a BS. Each entry is defined by wranIfBsThroughputMetricsEntry.

          - wranIfBsThroughputMetricsEntry

This object is a compound object that contains an entry in wranIfBsThroughputMetricsTable.

            - wranIfBsAvgDsUserThroughput

This records the average user throughput in the DS. This is a function of the number of octets of MAC SDUs transmitted by the BS to CPEs over time.

            - wranIfBsAvgUsUserThroughput

This records the average user throughput in the US. This is a function of the number of octets of MAC SDUs transmitted by CPEs to the BS over time.

            - wranIfBsAvgDsMacThroughput

This records the average MAC throughput in the DS. This is a function of the number of octets of MAC PDUs transmitted by the BS to CPEs over time.

            - wranIfBsAvgUsMacThroughput

This records the average MAC throughput in the US. This is a function of the number of octets of MAC PDUs transmitted by the CPEs to the BS over time.

            - wranIfBsAvgDsPhyThroughput

This records the average PHY throughput in the DS. This is a function of the number of burst octets (e.g. MAC PDU + PHYoverhead) transmitted by the BS to CPEs over time.

            - wranIfBsAvgUsPhyThroughput

This records the average PHY throughput in the US. This is a function of the number of octets of (e.g. MAC PDU + PHYoverhead) transmitted by the CPEs to the BS over time.

            - wranIfBsPeakDsUserThroughput

This records the Peak user throughput in the DS. This is a the maximum of a function of the number of octets of MAC SDUs transmitted by the BS to CPEs over time.

            - wranIfBsPeakUsUserThroughput

This records the peak user throughput in the US. This is a maximum of a function of the number of octets of MAC SDUs transmitted by CPEs to the BS over time.

            - wranIfBsPeakDsMacThroughput

This records the peak MAC throughput in the DS. This is a maximum of a function of the number of octets of MAC PDUs transmitted by the BS to CPEs over time.

            - wranIfBsPeakUsMacThroughput

This records the peak MAC throughput in the US. This is a maximum of a function of the number of octets of MAC PDUs transmitted by the CPEs to the BS over time.

            - wranIfBsPeakDsPhyThroughput

This records the peak PHY throughput in the DS. This is a maximum of the function of the number of burst octets (e.g. MAC PDU + PHYoverhead) transmitted by the BS to CPEs over time.

            - wranIfBsAvgUsPhyThroughput

This records the peak PHY throughput in the US. This is a maximum of the function of the number of octets of (e.g. MAC PDU + PHYoverhead) transmitted by the CPEs to the BS over time.

            - wranIfBsAvgDsCellEdgeThroughput

This records the average MAC throughput in the DS transmitted with the most robust MCS. This is a function of the number of octets of MAC PDUs transmitted by the BS to CPEs over time using QPSK.

            - wranIfBsAvgUsCellEdgeThroughput

This records the average MAC throughput in the US transmitted with the most robust MCS. This is a function of the number of octets of MAC PDUs transmitted by CPEs to the BS over time using QPSK.

            - wranIfBsNumThroughputMeasurements

This tracks the number of throughput measurements.

        - wranIfBsNetworkEntryMetricsTable

This MIB provides a table that contains latency (time) for network entry and network re-entry. Network entry time is measured as the time in between receiving the first RNG-REQ from a CPE until the CPE has received the REG-RSP. Network re-entry process is governed by policies in the Policy Table 234.  This could require execution of the entire network re-entry process if the CPE is forced to tre-initialize itself or it could only require execution of the CPE initialization procedure through the ranging process  (see 7.14.2). There is one entry in the table for each BS sector. Each entry is defined by wranIfBsNetworkEntryMetricsEntry.

          - wranIfBsNetworkEntryMetricsEntry

This object is a compound object that represents an entry in wranIfBsNetworkEntryMetricsTable.

            - wranIfBsAvgNetworkEntryLatency

Average network entry latency, measured in seconds.

            - wranIfBsMaxNetworkEntryLatency

Maximum network entry latency, measured in seconds.

            - wranIfBsAvgNetworkReEntryLatency

Average network re-entry latency, measured in seconds.

            - wranIfBsMaxNetworkReEntryLatency

Maximum network re-entry latency, measured in seconds.

            - wranIfBsNumNetworkEntryAttempts

Number of network entry attempts.

            - wranIfBsNumNetworkReEntryAttempts

Number of network re-entry attempts.

        - wranIfBsPacketErrorRateTable

This MIB object contains information about packet error rate measurements. The table is made up of multiple entries, one for each BS sector. Each entry is defined by wranIfBsPacketErrorRateEntry.

          - wranIfBsPacketErrorRateEntry

This object is a compound object that defines an entry in wranIfBsPacketErrorRateTable.

            - wranIfBsDsPacketsSent

Total number of MAC SDUs that the BS has sent.

            - wranIfBsDsPacketsErrored

Total number of MAC SDUs, including ARQ blocks that have not been acknowledged.

            - wranIfBsDsPacketErrorRate

wranIfBsDsPacketErrorRate = (wranIfBsDsPacketsErrored / wranIfBsDsPacketsSent) * 10000000, in units of 1e-7.

            - wranIfBsUsPacketsReceived

Total number of MAC SDUs that the BS has received.

            - wranIfBsUsPacketsErrored

Total number of MAC SDUs with CRC errors and/or ARQ blocks that required re-transmission that have been received by BS.

            - wranIfBsUsPacketErrorRate

wranIfBsUsPacketErrorRate = (wranIfBsUsPacketsErrored / wranIfBsUsPacketsReceived) * 10000000, in units of 1e-7.

        - wranIfBsUserMetricsTable

This MIB object provides a table to track the current status of users in the cell. There is one row for each BS sector. Each entry is defined by wranIfBsUserMetricsEntry.

          - wranIfBsUserMetricsEntry

This object is a compound object that defines a row in wranIfBsUserMetricsTable.

            - wranIfBsNumActiveUsers

Total number of users that have FIDs with active SFs on them.

            - wranIfBsNumTotalUsers

Total number of users that have completed registration (REG-REQ/RSP) process.

            - wranIfBsNumTimeoutUsers

Total number of users that are in Timeout.

        - wranIfBsServiceFlowMetricsTable

This MIB object provides a table to track metrics related to service flows. This table is made up of multiple entries, one entry for each BS sector. Each entry is defined by wranIfBsServiceFlowMetricsEntry.

          - wranIfBsServiceFlowMetricsEntry

This object is a compound object that represents an entry in wranIfBsServiceFlowMetricsTable.

            - wranIfBsNumDsaReq

Number of DSA-REQ counted during reporting period.

            - wranIfBsNumDsaReqSuccess

Number of successful SF activations counted during reporting period. A successful activation of a SF is noted when a BS receives a DSA-RSP with a successful confirmation in response to a particular DSA-REQ.

            - wranIfBsNumDsaSuccessRate

wranIfBsNumDsaSuccessRate = ( wranIfBsNumDsaReqSuccess / wranIfBsNumDsaReq ) * 100.

            - wranIfBsNumDscReq

Number of DSC-REQ BS sent during reporting period.

            - wranIfBsNumDscReqSuccess

Number of successful SF modifications counted during reporting period. A successful modication of a SF is noted when a BS receives a DSC-RSP with a successful confirmation in response to a particular DSC-REQ.

            - wranIfBsNumDscSuccessRate

wranIfBsNumDscSuccessRate = ( wranIfBsNumDscReqSuccess / wranIfBsNumDscReq ) * 100.

            - wranIfBsNumDsdReq

Number of DSD-REQ BS sent during reporting period.

            - wranIfBsNumDsdReqSuccess

Number of successful SF deletions counted during reporting period. A successful deletion of a SF is noted when a BS receives a DSD-RSP with a successful confirmation in response to a particular DSD-REQ.

            - wranIfBsNumDsdSuccessRate

wranIfBsNumDsdSuccessRate = ( wranIfBsNumDsdReqSuccess / wranIfBsNumDsdReq ) * 100.

            - wranIfBsMaxActiveServiceFlow

Maximum number of service flows active during reporting period.

            - wranIfBsAvgActiveServiceFlow

Average number of servifce flows active during reporting period.

            - wranIfBsMaxProvisionedServiceFlow

Maximum number of pre-provisioned service flows active during reporting period.

            - wranIfBsAvgProvisionedServiceFlow

Average number of pre-provisioned service flows active during reporting period.

            - wranIfBsMaxDsServiceFlow

Maximum number of DS service flows active during reporting period.

            - wranIfBsMaxUsServiceFlow

Maximum number of US service flows active during reporting period.

            - wranIfBsAvgDsServiceFlow

Average number of DS service flows active during reporting period.

            - wranIfBsAvgUsServiceFlow

Average number of US service flows active during reporting period.

            - wranIfBsNumSfidAllocated

Number of SFIDs allocated during reporting period.

        - wranIfBsArqMetricsTable

This MIB provides a table to track performance of ARQ. This table is made up of multiple entries, one for each BS sector. Each entry is defined by wranIfBsArqMetricsEntry.

          - wranIfBsArqMetricsEntry

This object is a compound object that provides a definition of entries in wranIfBsArqMetricsTable.

            - wranIfBsDsNumArqBlocks

Total number of ARQ blocks, including retransmissions, that BS has sent during reporting period.

            - wranIfBsDsNumArqBlocksDropped

Total number of ARQ blocks that were dropped, due to unsuccessful attempts at retransmission.

            - wranIfBsDsArqBlockErrorRate

wranIfBsDsArqBlockErrorRate = ( wranIfBsDsNumArqBlocksDropped / wranIfBsDsNumArqBlocks ) * 10000000, in units of 1e-7.

            - wranIfBsDsNumArqBlockRetransmissions

Total number of ARQ blocks that were retransmitted.

            - wranIfBsDsArqBlockEfficiency

wranIfBsDsArqBlockEfficiency = ( wranIfBsDsNumArqBlockRetransmissions / wranIfBsDsNumArqBlocks ) * 10000000, in units of 1e-7.

            - wranIfBsUsNumArqBlocks

Total number of ARQ blocks, including retransmissions, that BS has received during reporting period.

            - wranIfBsUsNumArqBlockRetransmissions

Total number of ARQ blocks that were retransmitted by CPE during reporting period.

            - wranIfBsDsArqBlockEfficiency

wranIfBsUsArqBlockEfficiency = ( wranIfBsUsNumArqBlockRetransmissions / wranIfBsUsNumArqBlocks ) * 10000000, in units of 1e-7.

        - wranIfBsAuthenticationMetricsTable

This MIB provides a table to track the number of authentication/encryption errors that occur. This information can be used to detect if an active attack on the system is occurring. This table is made up of multiple entries, one for each BS sector. Each entry is defined by wranIfBsAuthenticationMetricsEntry.

          - wranIfBsAuthenticationMetricsEntry

This object is a compound object that defines an entry in wranIfBsAuthenticationMetricsTable.

            - wranIfBsMangmentAuthenticationErrors

A counter that gets incremented everytime a management message from the CPE cannot be properly authenticated.

            - wranIfBsDataAuthenticationErrors

A counter that gets incremented everytime a data message from the CPE cannot be properly authenticated.

            - wranIfBsWiMicAuthenticationErrors

A counter that gets incremented everytime a wireless microphone beacon (MSF1+MSF2+MSF3) cannot be properly authenticated.

            - wranIfBsCbpAuthenticationErrors

A counter that gets incremented everytime a CBP from a neighboring WRAN cannot be properly authenticated.

        - wranIfBsCoexistenceStatusTable

This MIB provides a table to track the ongoing coexistence (Frame Contention) transactions. This table is made up of multiple entries, one for each ongoing transaction. Each entry is defined by wranIfBsCoexistenceStatusEntry. If a Frame Contention Destination receives a FC-REQ from a source on a channel for which it already has a message from, then the existing entry is updated

          - wranIfBsCoexistenceStatusEntry

This object is a compound object that defines an entry in wranIfBsCoexistenceStatusTable.

            - wranIfBsContentionChannel

Channel # of channel that FC-REQ was received on (ie channel that is being contented for).

            - wranIfBsFrameContentionSourceID

BS ID (MAC Address) of the transmitter of a FC-REQ message in a CBP burst. This is pulled from the SCH data in the CBP MAC PDU header pulled from the CBP burst containing the FC-REQ.

            - wranIfBsFrameContentionSeqNum

Sequence # field of received FC-REQ message.

            - wranIfBsFrameConentionNumber

Value of the Frame Contention Number (FCN) in the FC-REQ.

            - wranIfBsContentionReqFrameIndexVector

Bitmap index of data frames w/in a superframe that a Frame Contention Source is requesting, to be scheduled in the next superframe after the current one.

        - wranIfBsCoexistenceSourceTable

This MIB provides a table to track what neighbor BSs are attempting a coexistence (Frame Contention) transaction with a particular BS or have communicated other information (e.g. Backup/Candidate channel list) via CBP. Each entry is defined by wranIfBsCoexistenceSourceEntry.

          - wranIfBsCoexistenceSourceEntry

This object is a compound object that defines an entry in wranIfBsCoexistenceSourceTable. When a CBP is received from a neighboring WRAN that already has an entry in the table, the existing entry is updated.

            - wranIfBsMacAddress

BS ID of BS that sent CBP burst, obtained from SCH data in CBP MAC PDU header (see Table 1 and Table 9).

            - wranIfBsSchDataIndex

SCH Data Index field of CBP MAC PDU header. It indicates the length of and an indication of what SCH fields comprise the wranIfBsSchData in this table.

            - wranIfBsSchData

SCH Data from CBP MAC PDU header. The length of this field is governed by wranIfBsSchDataIndex.

        - wranIfBsCoexistenceResourceListTable

This MIB provides a table to track the resources being used by neighboring WRANs.  This includes the Backup/Candidate Lists being transmitted by neighboring WRANs, as well as the DS/US split. This table is made up of multiple entries, defined by wranIfBsChannelListEntry. A BS may keep an entry in this table for it’s own settings.

          - wranIfBsCoexistenceResourceListEntry

This object is a compound object that defines an entry in wranIfBsChannelListTable. When a CBP is received from a neighboring WRAN that already has an entry in the table, the existing entry is updated.

            - wranIfBsMacAddress

BS ID of BS that sent CBP burst, obtained from SCH data in CBP MAC PDU header (see Table 1 and Table 9).

            - wranIfBsSelfCoexistenceCapcityIndicator

Field within SCH Data field within CBP MAC PDU header that indicates what coexistence capabilities a BS supports. If this field is 0000, the remaining fields of this entry are null.

            - wranIfBsNumBackupCandidateChannels

Number of backup and candidate channels in the wranIfBsBackupCandidateChannelList.

            - wranIfBsNumBackupChannels

Number of backup channels in wranIfBsBackupCandidateChannelList.

            - wranIfBsBackupCandidateChannelList

This object is a vector, of the length = 8bits * wranIfBsNumBackupCandidateChannels, that contains the backup/candidate channel list received in a CBP burst from a neighbor WRAN.

            - wranIfBsCurrentDSUSSplit

Current US/DS split indicated in SCH data of CBP MAC PDU header received from neighbor WRAN.

            - wranIfBsClaimedDSUSSplit

Claimed US/DS split indicated in SCH data of CBP MAC PDU header received from neighbor WRAN.

            - wranIfBsDSUSChangeOffset

DS/US Change Offset indicated in SCH data of CBP MAC PDU header received from neighbor WRAN.

            - wranIfBsFrameAllocationMap

Indicate which frames in a superframe are allocated to the neighbor WRAN BS.

            - wranIfBsScwCycleLength

SCW Cycle Length being applied by neighbor WRAN.

            - wranIfBsScwCycleOffset

SCW Cycle Offset being applied by neighbor WRAN.

            - wranIfBsScwCycleFrameBitmap

DS/US Change Offset indicated in SCH data of CBP MAC PDU header received from neighbor WRAN.

        - wranIfBsCoexistenceCurrentConfigTable

This MIB provides a table to track what frame allocation has been awarded to a frame conention winner, and when the awarded resouce will be realeased. It is made up of multiple entries, one for each channel that frame contention procedure was executed on, as defined in wranIfBsCoexistenceCurrentConfigEntry. When a frame contention winner is selected, the corresponding entries in wranIfCoexistenceStatusTable are removed.

          - wranIfBsCoexistenceCurrentConfigEntry

This object is a compound object that defines an entry in wranIfBsCoexistenceCurrentConfigTable. When a frame contention winner is selected, and entry into this table is filled. When frames are reclaimed the corresponding entry in this table is removed.

            - wranIfBsContentionChannel

Channel # of channel that frame contention was executed on.

            - wranIfBsFrameContentionSourceID

BS ID of frame contention source that has won contention

            - wranIfBsAwardedSeqNum

Sequence number from FC-REQ message of frame contention winner.

            - wranIfBsContentionRspFrameIndexVector

Sequence number from FC-REQ message of frame contention winner.

            - wranIfBsContentionRspFrameReleaseTime

Starting from the next superframe , the number of superframes after which the TV channel shall be released by the frame contention destination .

      - wranIfBsScm

This MIB group has objects related to Security Management. The following tables are included in this MIB object:

- wranIfBsCpeScmAuthStatusTable: Contains information related to the current status of CPE authentication.

- wranIfBsCpeScmSaConfigTable:  Contains information related to attributes of SAs that are configured on CPEs.

- wranIfBsCpeTekRefreshTable:  Contains information related to ongoing SCM Key-Request/Reply transactions.

- wranIfBsCBPAuthCACertTable: Contains CA root certificates used in CBP Authentication

- wranIfBsCBPAuthBsImplicitCertTable: Contains BS implicit certificates used in CBP Authentication

- wranIfBsWiMicAuthCertTable: Contains implicit certificates contained in MSF3 of captured wireless microphone beacons.

        - wranIfBsCpeScmAuthStatusTable

This ojbect provides a table that stores information regarding the current state of the CPE’s authentication state machine. This table is stored on each CPE and made up of one entry, defined by wranIfBsCpeScmAuthStatusEntry.

          - wranIfBsCpeScmAuthStatusEntry

This object is a compound object that defines an entry in wranIfBsCpeScmAuthStatusTable.

            - wranIfBsCpeScmAuthStatus

State of CPE’s authentication state machine (see 8.2.2.1) that CPE is in.

            - wranIfBsCpeScmAuthRecentEvent

Indiciation of most recent event (see 8.2.2.4) that has occurred in the ASM.

            - wranIfBsCpeScmNumAuthAttempts

Current # of EAP authentication attempts.

            - wranIfBsCpeScmAuthRecentMsg

Contents of most recent authentication message, either EAP-Start or EAP-Transfer.

            - wranIfBsCpeScmAuthEapAuthTimerExpiration

Indiciation of when EAP Authentication timer for pending EAP-Start or EAP-Transfer message is to expire.

            - wranIfBsCpeScmAuthGraceTimer1

Indiciation of when Authentication Grace timer for current (active) AK will expire.

            - wranIfBsCpeScmAuthGraceTimer2

Indiciation of when Authentication Grace timer for second generation (non-active) AK will expire.

            - wranIfBsCpeScmAk1Lifetime

Remaining lifetime for current (active) AK will expire.

            - wranIfBsCpeScmAk2Lifetime

Remaining lifetime for second generation (non-active) AK will expire.

            - wranIfBsCpeScmConfigRequest

Contents of SCM configuration request (see Tabe 187) sent by CPE, upon initial authentication or re-authentication, to AAA server.

            - wranIfBsCpeScmConfigReply

Contents of SCM configuration reply (see Tabe 188) sent by AAA server, upon confirmation of initial authentication or re-authentication, to CPE.

        - wranIfBsCpeScmSaConfigTable

This ojbect provides a table that provides the configuration of the SA attributes that are related to SAs configured on each CPE. This table is maintained on each CPE as well as on the BS. On the BS, this table represents the configuration of SAs for all CPEs under its control. This table is made up of one entry for each SA that a CPE supports. Each entry is defined by wrnaIfBsCpeScmSaConfigEntry.

          - wranIfBsCpeScmSaConfigEntry

This object is a compound object that defines an entry in wranIfBsCpeScmSaConfigTable.

            - wranIfBsCpeMacAddress

MAC address of CPE.

            - wranIfBsCpeSaid

SAID of SA that this entry refers to.

            - wranIfBsCpeSaType

Type of SA, either Null, Primary, Secondary, or Group.

            - wranIfBsCpeCryptoSuiteCapability

This MIB provides a bitmap that describes the cryptographic suites that the CPE supports for this particular SA. The complete list of suites is provided in Table 193.

            - wranIfBsCpeTekN-1SequenceNumber

The EKS value for the current (active) generation of the two TEKs that are configured for this SA.

            - wranIfBsCpeTekN-1Lifetime

The lifetime, in units of time (e.g. seconds), for the current (active) generation of the two TEKs that are configured for this SA.

            - wranIfBsCpeTekN-1Pn

Current value of the PN counter for the current (active) generation of the two TEKs that are configured for this SA.

            - wranIfBsCpeTekN-1ExpireTime

Time at which current (active) generation of the two TEKS configured for an SA will expire. This time is calculated as a function of the Reception Time of Key Reply with TEK N-1 + TEK N-1 Lifetime.

            - wranIfBsCpeTekNSequenceNumber

The EKS value for the second (non-active) generation of the two TEKs that are configured for this SA.

            - wranIfBsCpeTekNLifetime

The lifetime, in units of time (e.g. seconds), for the second (non-active) generation of the two TEKs that are configured for this SA.

            - wranIfBsCpeTekNPn

Current value of the PN counter for the second (non-active) generation of the two TEKs that are configured for this SA.

            - wranIfBsCpeTekNExpireTime

Time at which second (non-active) generation of the two TEKS configured for an SA will expire. This time is calculated as a function of the Reception Time of Key Reply with TEK N + TEK N Lifetime.

        - wranIfBsCpeTekRefreshTable

This MIB object provides a table to track information related to ongoing Key-Request transactions. This table has one entry for each current SCM Key-Request transaction. Each entry is defined by wranIfBsCpeTekRefreshEntry.

          - wranIfBsCpeTekRefreshEntry

This object is a compound object that provides a definition for entries in wranIfBsCpeTekRefreshTable.

            - wranIfBsCpeScmReqId

Value of SCM Identifier field of SCM REQ that carried the corresponding Key-Request message.

            - wranIfBsCpeScmKeyReqKeySeqNum

Key Sequence Number of Key-Sequence message.

            - wranIfBsCpeScmKeyReqSaid

SAID that keys are being requested for.

            - wranIfBsCpeScmKeyReqGroupKeyIndicator

Indicator of wheter or not Key-Request was for a GSA or not.

            - wranIfBsCpeScmKeyReqCpeRandom

Random number generated by CPE and sent in the Key-Reqeust.

        - wranIfBsCBPAuthCACertTable

This object provides a table to CA root certificates (see 8.6.2.3) used to validate CBP BS implicit certificates. There will be one entry for each CA that a root certificate is installed for. Each entry is defined by wranIfBsCBPAuthCACertEntry.

          - wranIfBsCBPAuthCACertEntry

This object is a compound object that provides a definition for entries in wranIfBsCBPAuthCACertTable.

            - wranIfBsCBPAuthCACertCAID

CA ID, identifier of CA in CA root certificate.

            - wranIfBsCBPAuthCACertKeyID

Key ID, identifier of assigned to public key reconstruction data in CA root certificate.

            - wranIfBsCBPAuthCACertKeyValidityDate

Key Validity Date (Not Before), date/time at which CA root certificate becomes valid.

            - wranIfBsCBPAuthCACertKeyValidityTimePeriod

Key Validity Time Period, length of time from Key Validity Date (Not Before) in which CA root certificate is valid.

            - wranIfBsCBPAuthCACertVersion

Version of CBP Authentication being applied that CA root certificate supports.

            - wranIfBsCBPAuthCACertECDomainParameters

Elliptic Curve domain parameters that CA uses.

            - wranIfBsCBPAuthCACertCAPubKrd

Public Key Reconstruction Data that can be used to generate the public key associated with CA root certificate.

            - wranIfBsCBPAuthCACertCAPubK

Public Key reconstructed from wranIfBsCBPAuthCACertCAPubKrd associated with CA root certificate.

        - wranIfBsCBPAuthBsImplicitCertTable

This object provides a table to store BS implicit certificates for neighbor WRANs that make use of CBP authentication. There will be one entry for each BS whose implicit certificate is installed (via this object) on the BS or received by CERT-REQ/RSP. Each entry is defined by wranIfBsCBPAuthBsImplicitCertEntry.

          - wranIfBsCBPAuthBsImplicitCertEntry

This object is a compound object that provides a definition for entries in wranIfBsCBPAuthBsImplicitCertTable.

            - wranIfBsCBPAuthBsImplicitCertBsID

BS ID (MAC Address) of BS that implicit certificate belongs to.

            - wranIfBsCBPAuthBsImplicitCertCAID

CA ID, identifier of CA in BS implicit certificate.

            - wranIfBsCBPAuthBsImplicitCertKeyID

Key ID, identifier of assigned to public key reconstruction data in BS implicit certificate.

            - wranIfBsCBPAuthBsImplicitCertKeyValidityDate

Key Validity Date (Not Before), date/time at which BS implicit certificate becomes valid.

            - wranIfBsCBPAuthBsImplictCertKeyValidityTimePeriod

Key Validity Time Period, length of time from Key Validity Date (Not Before) in which BS implicit certificate is valid.

            - wranIfBsCBPAuthBsImplictCertVersion

Version of CBP Authentication being applied that BS implicit certificate supports.

            - wranIfBsCBPAuthBsImplictCertPubKrd

Public Key Reconstruction Data that can be used to generate the public key associated with BS implicit certificate.

            - wranIfBsCBPAuthBsImplictCertPubK

Public Key reconstructed from wranIfBsCBPAuthBsImplicitCertPubKrd hat can be used to generate the public key associated with BS implicit certificate.

            - wranIfBsCBPAuthBsImplicitCertPrKrd

Private Key Reconstruction Data associated with a BSs implicit certificate. This field is only applicable for the entry into 13.1.2.5.5 pertaining a BS's own implicit certificate Private Key Reconstruction Data, and not any other BSs. A BS may keep an entry for its own implicit certificate in this object. In this case,  wranIfBsCBPAuthBsImplicitCertPrKrd and wranIfBsCBPAuthBsImplicitCertPrK are only applicable to a BSs own entry and shall be null for all other BSs implicit certificate entries.

            - wranIfBsCBPAuthBsImplicitCertPrK

Private Key reconstructed from wranIfBsCBPAuthBsImplicitCertPrKrd associated with a BSs implicit certificate. This field is only applicable for the entry into 13.1.2.5.5 pertaining a BS's own implicit certificate Private Key, and not any other BSs.

        - wranIfBsWiMicAuthCertTable

This object provides a table to store wireless microphone implicit certificates contained in MSF3 of decoded wireless microphone beacons. This table is made up of multiple entries, one defined for each unique wireless microphone beacon implicit certificate. Each entry is defined by wranIfBsWiMicAuthCertEntry. Entries are added to this table when a wireless microphone beacon (MSF1+MSF2+MSF3) has been successfully received and decode.

          - wranIfBsWiMicAuthCertEntry

This object is a compound object that provides a definition for entries in wranIfBsWiMicAuthCertTable.

            - wranIfBsWiMicAuthSrcAddress

Source Address from from MSF1 of received wireless microphone beacon. It’s stored as a 48bit IEEE conformant MAC address that identifies the beaconing device associated with the implicit certificate.

            - wranIfBsWiMicAuthImplicitCert

Wireless microphone beacon implicit certificate obtained from MSF3 of received wireless microphone beacon. Format of implicit certificate is defined in Section 7.5.5 of IEEE 802.22.1-2010.

    - wranIfBsSfMgmt

This MIB group provides objects for managing service flows in the network.

Classification rules to be defined by the operator shall be downloadable to the BS and CPEs in a uniform and standardized format. The resulting bevariour of a given classification rule shall be standardized and implementation independent. This shall be done by the MIB.

In this MIB group the following tables are defined:

- wranIfBsProvSfTable:  Contains information about service flows provisioned by the NCMS

- wranIfBsScTable:  Contains the QoS parameter set for service flows defined for the supported service classes.

- wranIfBsSfTable:  Contains information about dynamic service flows that are created/torn-down on-the-fly.

- wranIfBsProvClassifierRuleTable:  Contains information about classifier rules as applied to service flows provisioned by the NCMS.

- wranIfBsClassifierRuleTable:  Contains information about classifier rules as applied to service flows applied to dynamic service flows.

      - wranIfBsProvSfTable

This MIB object defines the profiles for services that are provisioned by the NCMS. Services flow that are provisioned for a particular CPE, are tied to that CPE via that CPE’s MAC Address. This table is made up of multiple entries, each specific to a particular provisioned service flow. Each entry is defined by wranfBsProvSfEntry.

The QoS parameters for provisioned service flows are mapped to information in wranIfBsScTable. Classification rules for provisioned service flows are defined in wranIfBsProvClassifierRuleTable.

        - wranIfBsProvSfEntry

This object is a compound object that defines an entry in wranIfBsProvSfTable.

          - wranIfBsCpeProvMacAddress

MAC address of CPE that service flow is provisioned for.

          - wranIfBsProvSfId

Unique identifier for SF that is provisioned between a BS and a particular CPE.

          - wranIfBsProvSfDirection

Indication of whether or not the SF is an US SF or  a DS SF.

          - wranIfBsProvScIndex

Index into a wranIfBsScTable entry that indicates the QoS parameter set for this service flow.

          - wranIfBsCsSpecification

Indication of which convergence sublayer has has been used to encapsulate the higher-layer SDU.

          - wranIfBsProvSfStatus

Indication of whether or not the provisioned service flow is currently active or not.

          - wranIfBsProvSfProvisioningTime

If currently active (see 13.1.3.1.1.6), the time at which the service flow was provisioned.

          - wranIfBsProvTargetSaid

SAID of SA that service flow is being mapped to. Provisioned service flows can be mapped to the Primary or Secondary SA of the CPE.

          - wranIfBsProvClsRuleList

List of wranIfBsProvClsfrRuleIndex values pointing to entries in the wranIfBsProvClassifierRuleTable that contain packet classification rules assigned to this service flow.

      - wranIfBsScTable

This MIB object provides a table that describes attributes of provisioned and dynamic service flows, such as the QoS Parameter Set. This table is made up of multiple entries, one for each service class. Each entry is defined by wranIfBsScEntry.

        - wranIfBsScEntry

This object is a compound object that defines an entry in wranIfBsScTable.

          - wranIfBsScIndex

Index value to uniquely identify an entry into wranIfBsScTable.

          - wranIfBsQosSfFid

FID that service flow is mapped to.

          - wranIfBsQosServiceClassName

Defines the name of the service class associated with this entry. If service class is not assigned, this will be blank.

          - wranIfBsQosParameterSetType

Indication of whether or not the QoS Parameter set defined by this entry is the Provisioned, Admitted, or Active Parameter Set of the service flow.

          - wranIfBsQosMaxSustainedRate

Peak information/data rate of SDUs carried by the service flow, defined in units of bits per second.

          - wranIfBsQosTrafficSize

If fixed-length SDUs (see 13.1.3.2.1.10) are enabled, this represents the size of SDU assigned to the service flow. If variable-length SDUs (see 13.1.3.2.1.10) are enabled, this represents the average size of SDU assigned the service flow.

          - wranIfBsQosMinReservedRate

Minimum required information/date rate of SDUs carried by the service flow, defined in units of bits per second.

          - wranIfBsQosToleratedJitter

The maximum jitter (variation in delay) that can be suffered by the traffic assigned to the service flow.

          - wranIfBsQosMaxLatency

The maximum delay that can be suffered by traffic assigned to the service flow.

          - wranIfBsQosEnableVariableLengthSdus

Setting this object allows to enable/disable use of variable length SDUs. Default is to allow use of variable length SDUs.

          - wranIfBsQosSchedulingType.

The scheduling type, e.g. BE, nrtPS, rtPS, or UGS. Default is BE.

          - wranIfBsQosArqEnable

Setting this object enables/disables ARQ for a service flow. Default is that ARQ is enabled.

          - wranIfBsQosArqWindowSize

Indication of the maximum number of unacknowledged fragments at any given time. Only valid if wranIfBsQosArqEnable is set.

          - wranIfBsQosArqTxRetryTimeout

Total time before timing out retransmissions of ARQ blocks. For BS, this should include time to compensate for scheduling and the propagation time for transmission.

          - wranIfBsQosArqRxRetryTimeout

Total time before timing out reception of ARQ block retransmission. For BS, this should include time to compensate for scheduling and the propagation time for transmission.

          - wranIfBsQosArqBlockLifetime

The maximum amount of time that an ARQ block can be held in the ARQ state machine before it is dropped.

          - wranIfBsQosArqSyncLossTimeout

Timeout for determing that transmitter and receiver state machines have become unsynchronized.

          - wranIfBsQosArqDeliverInOrderEnable

Disable/enables ability to deliver ARQ blocks to higher layer at receiver in the same order they were transmitted by the transmitter.

          - wranIfBsQosArqRxPurgeTimeout

How much the ARQ window is advanced after an ARQ fragment is received.

          - wranIfBsQosArqBlockSizeReq

This object defines the value of the ARQ block size included in DSA-REQ and REG-REQ messages.

          - wranIfBsQosArqBlockSizeRsp

This object defines the value of the ARQ block size included in DSA-RSP and REG-RSP messages.

          - wranIfBsQosReqTxPolicy

This value is a bitmap that enables/disables the following capabilities for a service flow: Use of broadcast BW request for US, use of multicast BW request for US only, piggyback BW requests on data for US transmissions, enable/disable fragmentation, enable/disable packing, use of CRC for MAC PDU.

          - wranIfBsQosFsnType

Indication of the size of the fragment sequence number (FSN) window that is being used for the connection.

      - wranIfBsActiveSfTable

This MIB object provides a table that is used to manage service flows that are currently active between the BS and CPEs. This table is made up of multiple entries, one for each service flow mapped to a particular CPE. Each entry is defined by wranIfBsSfEntry.

        - wranIfBsActiveSfEntry

This object is a compound object that provides the definition of an entry into wranIfBsSfTable.

          - wranIfBsActiveSfSfid

SFID of service flow that is assigned to a particular CPE.

          - wranIfBsActiveSfDirection

Direction of service flow: BS to CPE (DS), CPE to BS (US).

          - wranIfBsActiveSfStatus

Current state of service flow. Service flow can be in one of four states: inactive, provisioned, admitted (service flow initiated but not received BW yet), or active (service flow initiated and there has been bandwidth assigned to use on service flow).

          - wranIfBsActiveScIndex

Index into a wranIfBsScTable entry that indicates the QoS parameter set for this service flow.

          - wranIfBsActiveCsSpecification

The CS used for encapsulating SDUs for this service flow.

          - wranIfBsActiveTargetSaid

SAID of SA that service flow is being mapped to.

          - wranIfBsActiveClsRuleList

List of wranIfBsClsfrRuleIndex values pointing to entries in the wranIfBsClassifierRuleTable that contain packet classification rules assigned to this service flow.

      - wranIfBsProvClassifierRuleTable

This table contains classifier rules that are to be applied to service flows for CPEs that are provisioned by the NCMS. There are multiple entries in this table, one for each classifier rule. Each entry is defined by wranIfBsProvClassifierRuleEntry.

        - wranIfBsProvClassifierRuleEntry

This object is a compound object the provides the definition of entries in wranIfBsProvClassifierRuleTable.

          - wranIfBsProvClsfrRuleIndex

Index to uniquely identify an entry in wranIfBsProvClassiferRuleTable.

          - wranIfBsProvClsfRulePriority

Priority of the classification rule. This determines the order in which classification rules are applied.

          - wranIfBsProvClsfRuleIpProtocol

Value of IP Protocol field. For IPv6 headers, this refers to the next header entry in the last header of the IP header list. The value of this field follows the "Protocol Numbers" specification defined by IANA.

          - wranIfBsProvClsfRuleIpSrcAddr

Source IP address from IP header.

          - wranIfBsProvClsfRuleIpSrcMask

IP address mask. IP src address (wranIfBsProvClsfRuleIpSrcAddr) is matched when output of applying (bitwise AND) this value to IP src address from IP packet.

          - wranIfBsProvClsfRuleIpDestAddr

Destination IP address from IP header.

          - wranIfBsProvClsfRuleIpDestMask

IP address mask. IP dest address (wranIfBsProvClsfRuleIpDestAddr) is matched when output of applying (bitwise AND) this value to IP dest address from IP packet.

          - wranIfBsProvClsfRuleSrcPortStart

Start (inclusive) of range of source ports that that packet will be compared against.

          - wranIfBsProvClsfRuleSrcPortEnd

End (inclusive) of range of source ports that that packet will be compared against.

          - wranIfBsProvClsfRuleDestPortStart

Start (inclusive) of range of destination ports that that packet will be compared against.

          - wranIfBsProvClsfRuleDestPortEnd

End (inclusive) of range of destination ports that that packet will be compared against.

          - wranIfBsProvClsfRuleDestMacAddr

Destination MAC address to be matched against destination MAC address in Ethernet header.

          - wranIfBsProvClsfRuleDestMacMask

MAC address mask. A destination MAC address (wranIfBsProvClsfRuleDestMacAddr) is matched when the destination MAC address from Ethernet header is applied (bitwise AND) with this mask.

          - wranIfBsProvClsfRuleSrcMacAddr

Source MAC address to be matched against source MAC address in Ethernet header.

          - wranIfBsProvClsfRuleDestMacMask

MAC address mask. A source MAC address (wranIfBsProvClsfRuleSrcMacAddr) is matched when the source MAC address from Ethernet header is applied (bitwise AND) with this mask.

          - wranIfBsProvClsfRuleEnetProtType

Enable/disable use layer 3 protocol type in Ethernet frame, EtherType in DIX/SNAP based frames, DSAP in IEEE 802.3 frames, are used in Ethernet frame classification. If 802.1Q is supported, the EtherType value in the 802.1Q header is used.

          - wranIfBsProvClsfRuleEnetProtocol

Ethernet protocol type value that is used for classification. This value will be processed based on what is set in wranIfBsProvClsfRuleEnetProtType.

          - wranIfBsProvClsfRuleUserPriLow

Low value (inclusive) in range of 3-bit user priority value. This field is part of 16 bit tag of a 802.1Q header. Only valid if 802.1Q is being used.

          - wranIfBsProvClsfRuleUserPriHigh

High value (inclusive) in range of 3-bit user priority value. This field is part of a 16 bit tag of a 802.1Q header. Only valid if 802.1Q is being used.

          - wranIfBsProvClsfRuleVlanId

VLAN Id from Ethernet frame. Only valid if 802.1Q is being used.

          - wranIfBsProvClsfRuleIpv6FlowLabel

Flow label field from IPv6 header.

          - wranIfBsProvClsfRuleIpTypeOfService

The value to match the IP TOS octet from IP header. The 6 MSBs are read in as the DSCP (RFC 2474).

          - wranIfBsProvClsfRuleMap

A bitmap that indicates which classification parameters are included in the classification rule.

      - wranIfBsClassifierRuleTable

This MIB object provides a table to contain classification rules for service flows that are dynamically created/destroyed. There are multiple entries in this table, one for each classifier rule. Each etnry is defined by wranIfBsClassifierRuleEntry.

        - wranIfBsClassifierRuleEntry

This object is a compound object that provides the definition of entires in wranIfBsClassifierRuleTable.

          - wranIfBsClsfrRuleIndex

Index to uniquely identify an entry in wranIfBsClassiferRuleTable.

          - wranIfBsClsfRulePriority

Priority of the classification rule. This determines the order in which classification rules are applied.

          - wranIfBsClsfRuleIpProtocol

Value of IP Protocol field. For IPv6 headers, this refers to the next header entry in the last header of the IP header list. The value of this field follows the "Protocol Numbers" specification defined by IANA.

          - wranIfBsClsfRuleIpSrcAddr

Source IP address from IP header.

          - wranIfBsClsfRuleIpSrcMask

IP address mask. IP src address (wranIfBsClsfRuleIpSrcAddr) is matched when output of applying (bitwise AND) this value to IP src address from IP packet.

          - wranIfBsClsfRuleIpDestAddr

Destination IP address from IP header.

          - wranIfBsClsfRuleIpDestMask

IP address mask. IP dest address (wranIfBsClsfRuleIpDestAddr) is matched when output of applying (bitwise AND) this value to IP dest address from IP packet.

          - wranIfBsClsfRuleSrcPortStart

Start (inclusive) of range of source ports that that packet will be compared against.

          - wranIfBsClsfRuleSrcPortEnd

End (inclusive) of range of source ports that that packet will be compared against.

          - wranIfBsClsfRuleDestPortStart

Start (inclusive) of range of destination ports that that packet will be compared against.

          - wranIfBsClsfRuleDestPortEnd

End (inclusive) of range of destination ports that that packet will be compared against.

          - wranIfBsClsfRuleDestMacAddr

Destination MAC address to be matched against destination MAC address in Ethernet header.

          - wranIfBsClsfRuleDestMacMask

MAC address mask. A destination MAC address (wranIfBsProvClsfRuleDestMacAddr) is matched when the destination MAC address from Ethernet header is applied (bitwise AND) with this mask.

          - wranIfBsClsfRuleSrcMacAddr

Source MAC address to be matched against source MAC address in Ethernet header.

          - wranIfBsClsfRuleDestMacMask

MAC address mask. A source MAC address (wranIfBsProvClsfRuleSrcMacAddr) is matched when the source MAC address from Ethernet header is applied (bitwise AND) with this mask.

          - wranIfBsClsfRuleEnetProtType

Enable/disable use layer 3 protocol type in Ethernet frame, EtherType in DIX/SNAP based frames, DSAP in IEEE 802.3 frames, are used in Ethernet frame classification. If 802.1Q is supported, the EtherType value in the 802.1Q header is used.

          - wranIfBsClsfRuleEnetProtocol

Ethernet protocol type value that is used for classification. This value will be processed based on what is set in wranIfBsClsfRuleEnetProtType.

          - wranIfBsClsfRuleUserPriLow

Low value (inclusive) in range of 3-bit user priority value. This field is part of 16 bit tag of a 802.1Q header. Only valid if 802.1Q is being used.

          - wranIfBsClsfRuleUserPriHigh

High value (inclusive) in range of 3-bit user priority value. This field is part of a 16 bit tag of a 802.1Q header. Only valid if 802.1Q is being used.

          - wranIfBsClsfRuleVlanId

VLAN Id from Ethernet frame. Only valid if 802.1Q is being used.

          - wranIfBsClsfRuleIpv6FlowLabel

Flow label field from IPv6 header.

          - wranIfBsClsfRuleIpTypeOfService

The value to match the IP TOS octet from IP header. The 6 MSBs are read in as the DSCP (RFC 2474).

          - wranIfBsClsfRuleMap

A bitmap that indicates which classification parameters are included in the classification rule.

    - wranIfCpeMib

This MIB defines objects used for managing CPEs. It is broken up into the following sections:

- wranIfCpeConfigurationTable: Definition of system parameters, timers, and constants related to CPE operation.

- wranIfCpeTrapControl: Enabling/disabling of traps concerning events related to CPE operation.

- wranIfCpeThresholdConfigTable: Table containing configuration bounds for recording CPE events

- wranIfCpeTrapDefinitions: Definition of traps and objects that can be reported by traps.

- wranIfCpeNotificatonObjectsTable: Information related to configured traps is stored in this table.

      - wranIfCpeConfigurationTable

This MIB provides a table that provides the values for system parameters. The parameters in this table are applied to all CPEs in the network, hence there is one entry in the table, defined by wranIfCpeConfigurationEntry.

        - wranIfCpeConfigurationEntry

This object is a compound object that provides the definition of the (single) entry into wranIfCpeConfigurationTable.

          - wranIfCpeLostDsMapInterval

Amount of time since reception of last DS-MAP before DS synchronization is considered lost.

          - wranIfCpeLostUsMapInterval

Amount of time since reception of last US-MAP before US synchronization is considered lost.

          - wranIfCpeContentionRangingRetries

Maximum number of retries allowed for contention-based ranging.

          - wranIfCpeContentionBwRetries

Maximum number of retries allowed for contention-based bandwidth requests.

          - wranIfCpeRegReqRetries

Maximum number of retries allowed for registration requests.

          - wranIfCpeTftpBackoffStart

Initial value for TFTP backoff.

          - wranIfCpeTftpBackoffEnd

Last value for TFTP backoff.

          - wranIfCpeTftpReqRetries

Maximum number of retries allowed for attempting TFTP to get CPE configuration.

          - wranIfCpeTftpDownloadRetries

Maximum number of retries allowed for re-attempting TFTP (after a failed/corrupted download) to get CPE configuration.

          - wranIfCpeTftpWait

Time to wait before consecutive attempts to obtain configuration via TFTP.

          - wranIfCpeToDRetries

Maximum number of retries allowed for attempting to establish time of day.

          - wranIfCpeToDRetryPeriod

Amount of time to wait before re-trying establishment of time of day, after failed attempt.

          - wranIfCpeCBCReqRetries

Maximum number of retries allowed for sending CBC request.

          - wranIfCpeTftpCpltRetries

Maximum number of retries allowed for sending TFTP-CPLT message to BS.

          - wranIfCpePowerCtrlProcTime

Maximum CPE is allowed to wait after receiving instruction (??) to adjust power, before power adjustment is executed.

          - wranIfCpeRangCmdProcTime

Maximum time CPE is allowed to wait before applying corrections received in RNG-CMD.

          - wranIfCpeInvitedRangRetries

Maximum number of retries on invited ranging requests.

          - wranIfCpeDSxReqRetries

Maximum number of timeout retries on DSx-REQ.

          - wranIfCpeDSxRspRetries

Maximum number of timeout retries on DSx-RSP.

      - wranIfCpeTrapControl

Defines a control element for traps related to CPE operation. This MIB object is a bitmap that allows the following SNMP traps to be set for CPEs: wranIfCpeDhcpSuccess, wranIfCpeRssiStatusChange, wranIfCpeEirpStatusChange, wranIfCpeScmStateChange.

      - wranIfCpeThresholdConfigTable

This MIB provides a table that allows the setting of thresholds that can be used to detect the crossing of RSSI and EIRP thresholds. Each table is made up of one entry defined by wranIfCpeThresholdConfigEntry.

          - wranIfCpeThresholdConfigEntry

This object is a compound object that provides definition of entries into wranIfCpeThresholdConfigTable.

            - wranIfCpeRssiLowThreshold

Low threshold for generating RSSI alarm.

            - wranIfCpeRssiHighThreshold

High threshold for generating an RSSI alarm.

            - wranIfCpeEirpLowThreshold

Low threshold for generating an EIRP alarm.

            - wranIfCpeEirpHighThreshold

High threshold for generating an EIPR alarm.

      - wranIfCpeTrapDefintion

This MIB group specifies the definition of CPE traps that can be enabled/disable in wranIfCpeTrapControl.

        - wranIfCpeRssiStatusChangeTrap

This trap indicates that the RSSI is outside the range defined by wranIfCpeRssiLowThreshold and wranIfCpeRssiHighThreshold.

        - wranIfCpeEirpStatusChangeTrap

This trap indicates that the EIRP is outside the range defined by wranIfCpeEirpLowThreshold and wranIfCpeEirpHighThreshold.

        - wranIfCpeDhcpSuccessTrap

This trap indicates whether a specific CPE has been successful or not in acquiring an IP address via DHCP.

        - wranIfCpeScmStateChangeTrap

This trap indicates that the CPE has entered the Idle state of the SCM Authentication State Machine.

      - wranIfCpeNotificationObjectsTable

This MIB provides a table to track notification objects that have been reported by traps on a particular CPE. There is one entry in this table, defining the notification objects for that particular CPE. Each entry is defined by wranIfCpeNotificationObjectsEntry.

          - wranIfCpeNotificationObjectsEntry

This object is a compound object that contains the definition of an entry in wranIfCpeNotificationObjectsTable.

            - wranIfCpeNotificationMacAddress

MAC address of CPE generating the trap.

            - wranIfCpeRssiStatus

An RSSI alarm is generated when RSSI is lower than wranIfCpeRssiLowThreshold or higher than wranIfCpeRssiHighThreshold. This alarm is generated when wranIfCpeRssiStatusChange in wranIfCpeTrapControl is set.

            - wranIfCpeEirpStatus

An EIRP alarm is generated when EIRP is lower than wranIfCpeEirpLowThreshold or higher than wranIfCpeEirpHighThreshold. This alarm is generated when wranIfCpeEirpStatusChange in wranIfCpeTrapControl is set.

            - wranIfCpeDhcpStatus

An DHCP alarm is generated when CPE is able to obtain an IP address via DHCP. This alarm is generated when wranIfCpeDhcpSuccess in wranIfCpeTrapControl is set.

            - wranIfCpeScmStatus

An SCM state alarm is generated when the CPE’s Authentication State Machine changes state. This alarm is generated when wranIfCpeScmStateChange in wranIfCpeTrapControl is set.

      - wranIfCpeMibGroups

This objetc helps define which MIB groups are avaliable in this module, and which MIB objects are part of each group. Two groups are defined: wranIfCpeMibConfigGroup and wranIfCpeMibNotificationsGroup.

        - wranIfCpeMibConfigGroup

This group contains configuration objects for the CPE.

        - wranIfCpeMibNotificationsGroup

This group contains CPE event notifications.

      - wranIfCpeMibCompliance

MIB objects that are optional and mandatory for CPE conformance.

    - wranIfSmMib

This MIB group deals with objects related to the configuration, operation, and monitoring of the Spectrum Manager. This MIB group is made up of the the following objects: wranIfSmConfigTable, wranIfSmPendingBlmReqTable, wranIfSsaPendingBlmRepTable, wranIfSmChClassificationStatusTable, wranIfSmChannelSetTable,  , wranIfSmCurrentStatusTable, wranIfSmRegTrackingTable, wranIfSmTrapControl, wranIfSmNotificationObjectsTable, wranIfSmMibGroups, wranIfSmMibCompliance.

      - wranIfSmConfigTable

This MIB object represents a table that tracks what the default configuration of SM timers and constants. There is only one entry in this table, to define the default configuration of the SM. This entry is defined by, wranIfSmConfigEntry.

        - wranIfSmConfigEntry

This object represents the entry in wranIfSmConfigTable.

          - wranIfSmConfigIndex

Index of entry in the table, defaults to 1.

          - wranIfSmT31

Wait for BLM-REP timeout.

          - wranIfSmSsaChAvailabilityCheckTime

Time during which a TV channel shall be checked for the presence of licensed incumbent signals having a level above the incumbent detection threshold prior to commencement of WRAN operation in the channel, and in the case of TV, a related channel at an EIRP level that can affect the measured TV channel.

          - wranIfSmSsaNonOccupancyPeriod

The required period during which WRAN device transmissions SHALL NOT occur in a given TV channel because of the detected presence of an incumbent signal in that channel above the Incumbent detection threshold, or in the case of TV, above a given EIRP level.

          - wranIfSmSsaChannelDetectionTime

Maximum time taken by a WRAN device to detect a licensed incumbent signal above the Incumbent Detection Threshold within a given TV channel during normal WRAN operation.

          - wranIfSmSsaChannelSetupTime

The window of time that may be taken by a WRAN CPE to transmit control information to a WRAN base station in order to establish operation with that base station at the prescribed power or, in the case of TV, at or below the allowable EIRP within a given TV channel.

          - wranIfSmSsaChannelOpeningTxTime

The aggregate duration of control transmissions by WRAN devices during the Channel Setup Time, which starts at the end end of the Channel Availability Check Time..

          - wranIfSmSsaChannelMoveTime

The time taken by WRAN system to cease all interfering transmissions on the current TV channel upon detection of a license incumbent signal above the relevant Incumbent Detection Threshold, or in the case of TV, to alternatively reduces its EIRP to which is allowable within a given TV channel upon detection of a TV signal in the same or a related channel.

          - wranIfSmSsaChannelClosingTxTime

The aggregate duration of conrol transmissions by the WRAN devices during the Channel Move/EIRP Reduction Time, which starts upon detection of a licensed incumbent singal above the relevant Incumbent Detection Threshold.

          - wranIfSmSsaMicProtectionRadius

Radius of contour within which the WRAN system cannot operate due to potential interference with the microphone.

          - wranIfSmSsaT41

Maximum time interval allowed before sensing is performed on the candidate channel to ensure that no incumbents are detected.

          - wranIfSmSsaT42

Maximum time interval allowed before sensing is performed on the backup channel to ensure that no incumbents are detected.

          - wranIfSmSsaT43

Minimum time duration without detection of any incumbent for a candidate channel to transition to the backup channel.

          - wranIfSmSsaT44

Maximum time to ensure that the channel move information is successfully conveyed to all the associated CPEs and BS (self-coexistence mode).

          - wranIfSmSsaT45

Maximum WRAN operation time without access to the incumbent database service.

          - wranIfSmT46

Waiting time before which the BS moves to the first backup channel. This is used to make sure that all the CPEs are ready to move to the backup channel before BS switches operation to this backup channel.

          - wranIfSmSsaT59

Waiting time before which the CPE moves to its backup channels if it no longer hears from its BS. This is used to make sure that the CPE waits long enough after UCS Notification so that BS has had time to move to the backup channel, it it decided to do so.

          - wranIfSmSsaT47

The prescribed time by the WRAN operator to refresh the incumbent database service.

          - wranIfSmSsaT48

Lapse timer keeps track of whether the Operating Channel N has been cleared using spectrum sensing.

          - wranIfSmSsaT49

Lapse timer keeps track of whether the Adjacent Channel N-1 has been cleared using spectrum sensing.

          - wranIfSmSsaT50

Lapse timer keeps track of whether the Adjacent Channel N+1 has been cleared using spectrum sensing.

          - wranIfSmSsaT51

Initiated when SSA loses contact with the SM.

          - wranIfSmSsaT53

The parameter TINSens is used to verify that in-band sensing has been done within the required In-service monitoring period. The TINSens parameter is driven by regulatory domain requirements (Annex A).

          - wranIfSmSsaT54

The parameter TOUTSens is used to verify that out-of-band sensing has been done within the required "Acquiring a channel monitoring period" specified in Annex A. This value would be used to initalize a lapse timer for each channel in the backup candidate channel list at each CPE so that it compared to Tsens.

          - wranIfSmSsaT55

The Tsens parameter corresponds to the maximum length of time required to to carry out the sensing process on a given channe (be it inband or out-of-band). Manufacturers need to specify the sensing time required to detect the specified signals with required accuracy.

          - wranIfSmSsaT60

The T60 parameter corresponds to the maximum length of time required to to carry out the out-of-band sensing process. Manufacturers need to specify the sensing time required to detect the specified signals with required accuracy for out-of-band sensing.

      - wranIfSmPendingBlmReqTable

This MIB object represents a table that tracks the status of the execution of ongoing sesing requests (BLM-REQ). For each BLM-REQ there is a corresponding BLM-RSP to indicate that REQ message was received by the SSA. When an SSA is done with the sensing it will send a BLM-REP to the SM. This table keeps track of any BLM-REP messages that are pending transmission from the SSA. When a report is received a response is acknowledgement is sent to the SSA, and then the entry corresponding to the report and request will be cleared. This entry is defined by, wranIfSmPendingBlmReqEntry.

        - wranIfSmPendingBlmReqEntry

This object represents the entry in wranIfSmPendingBlmReqTable.

          - wranIfSmPendinBlmReqIndex

Index of entry into this table.

          - wranIfSsaPendingBlmReqTransactionId

Transaction ID of BLM-REQ.

          - wranIfSmPendingBlmReqMsg

Contents of BLM-REQ message pending a report.

          - wranIfSmPendingBlmRspReceived

Indication of whether or not BLM-RSP pertaining to BLM-REQ has been received from SSA.

          - wranIfSmPendingBlmRspMulticastReceived

If BLM-REQ was multicasted, indication of whether or not BLM-RSP pertaining to BLM-REQ has been received from each SSA (CPE) in the multicast group.

          - wranIfSmPendingBlmRepTimeout

Indication of current value of T31 set for this BLM-REP.

          - wranIfSmPendingBlmRepReceived

Indication of whether or not BLM-REP pertaining to BLM-REQ has been received from SSA.

          - wranIfSmPendingBlmRepMulticastReceived

If BLM-REQ was multicasted, indication of whether or not BLM-REP pertaining to BLM-REQ has been received from each SSA (CPE) in the multicast group.

          - wranIfSsaPendingBlmRepAck

Indication of whether or not BLM-ACK, used to indicate sent to SSA(s) whether or not BLM-REP was received, is sent to SSA(s).

      - wranIfSmBlmRepTable

This object contains BLM-REP messages received in response to BLM-REQ, It is made up of multiple entries, one for each BLM-REP that pertains to a BLM-REQ. Each entry is defined by wranIfSmBlmRepEntry.

        - wranIfSmBlmRepEntry

This object represents the entry in wranIfSmBlmRepTable.

          - wranIfSmBlmRepIndex

Index of entry in the table.

          - wranIfSmBlmRepSid

SID of CPE that sent the BLM-REP.

          - wranIfSmBlmRepTransactionId

Transaction ID of BLM-REP, it should match a transaction ID of an entry in wranIfSmPendingBlmReqTable.

          - wranIfSmBlmRepMsg

Contents of BLM-REP msg.

      - wranIfSmChClassificationStatusTable

This MIB object represents a table that the status for channels that the SM is managing. It is made up multiple entries, one for each channel, as defined in wranIfSmChClassificationStatusEntry.

        - wranIfSmChClassificationStatusEntry

This object represents the entry in wranIfSmChClassificationStatusTable.

          - wranIfSmChClassificationIndex

Index of entry into thiss table.

          - wranIfSmManagedChannel

Channel number of channel being managed.

          - wranIfSmManagedChannelStatus

The state of the channel as set by the states in the Channel Set Transition Diagram (Figure 162) or disallowed state (if channel is in IPC-UPD).

          - wranIfSmManagedChannelRecentEvent

Most recent event (see 10.2.3.1) that dictated transition to current state.

      - wranIfSmChannelSetTable

This MIB object represents a table that stores information related to the contents of the Occupied, Backup, and Local Priority Channel sets used by the Spectrum Manager (see 10.2.3.2). There is one entry in this table defined by wranIfSmChannelSetEntry.

        - wranIfSmChannelSetEntry

This object defines an entry in wranIfSmChannelSetTable. There is only one entry in this table.

          - wranIfSmChannelSetIndex

Index of entry in the table, defaults to 1.

          - wranIfSmSizeWranOccupiedChannelSet

Number of channels in WRAN Occupied Channel Set

          - wranIfSmWranOccupiedChannelSet

Vector of channels of length 8bits * wranIfSmSizeWranOccupiedChannelSet that indicate the channels that occupy the WRAN Occupied Channel Set used by Spectrum Etiquette procedure (see 10.2.3.2).

          - wranIfSmSizeNghbrWranBackupChannelSet

Number of channels in Neighbor WRAN Backup Channel Set

          - wranIfSmNghbrWranBackupChannelSet

Vector of channels of length 8bits * wranIfSmSizeNghbrWranBackupChannelSet that indicate the channels that occupy the Neighbor WRAN Backup Channel Set used by Spectrum Etiquette procedure (see 10.2.3.2).

          - wranIfSmSizeLocalPrioritySet1

Number of channels in Local Priority Set 1

          - wranIfSmLocalPrioritySet1

Vector of channels of length 8bits * wranIfSmSizeLocalPrioritySet1 that indicate the channels that occupy the Local Prioirty Set 1 used by Spectrum Etiquette procedure (see 10.2.3.2).

          - wranIfSmSizeLocalPrioritySet2

Number of channels in Local Priority Set 2

          - wranIfSmLocalPrioritySet2

Vector of channels of length 8bits * wranIfSmSizeLocalPrioritySet2 that indicate the channels that occupy the Local Prioirty Set 2 used by Spectrum Etiquette procedure (see 10.2.3.2).

          - wranIfSmSizeLocalPrioritySet3

Number of channels in Local Priority Set 3, should be the same size as wranIfSmSizeWranOccupiedChannelSet.

          - wranIfSmLocalPrioritySet3

Vector of channels of length 8bits * wranIfSmSizeLocalPrioritySet3 that indicate the channels that occupy the Local Prioirty Set 3 used by Spectrum Etiquette procedure (see 10.2.3.2). Should contain the same channel set as wranIfSmWranOccupiedChannelSet.

      - wranIfSmCurrentStatusTable

This MIB object represents a table that the records the current status of the SM. This includes the state the SM is in, the event that triggered a move into that state, as well as the current state of any relevant timers. There is one entry in this table, defined in wranIfSmCurrentStatusEntry.

        - wranIfSmCurrentStatusEntry

This object represents the entry in wranIfSmCurrentStatusTable.

          - wranIfSmCurrentState

Index of entry in the table.

          - wranIfSmCurrentState

The state (see Figure 164) that the SM is in.

          - wranIfSmRecentEvent

Recent event that triggered transition into current state as given in 10.2.6.1.

          - wranIfSmRecentAction

Recent action that was taken when transitioning into current state as given in 10.2.6.1.

          - wranIfSmInitiateChannelMove

Current value of ‘Initiate_Channel_Move’ flag.

          - wranIfSmSelfCoexistenceMode

Current value of ‘Self_Coexistence_Mode’ flag.

          - wranIfSmCurrentOperatingChannel

Current operating channel.

          - wranIfSmRecentSignalType

Signal type of signal recently detected (see Table 237).

          - wranIfSmCurrentT47

Current value of T47.

          - wranIfSmCurrentT46

Current value of T46.

      - wranIfSmRegTrackingTable

This MIB object represents a table that the records the SM’s monitoring of CPE’s associated with the BS. It contains the location data string and current value of T30 for each CPE. There are multiple entries in this table (one for each CPE), defined in wranIfSmRegTrackingEntry.

        - wranIfSmRegTrackingEntry

This object represents the entry in wranIfSmRegTrackingTable.

          - wranIfSmRegTrackingIndex

Index of entry in the table.

          - wranIfSmRegTrackingCpeSid

SID of CPE currently associated with BS.

          - wranIfSmRegTrackingCurrentT30

Current value of CPE’s T30, as known by SM.

          - wranIfSmRegTrackingLocStringSize

Size of location string in octets

          - wranIfSmRegTrackingLocString

CPE’s location string.

      - wranIfSmTrapControl

Defines control elements for traps related to the operation of the spectrum manager. This is a 9bit field that enables setting a trap for particular SM events: wranIfSmBlmReqChange, wranIfSmBlmRepChange, wranIfSmOccupiedChannelSetChange, wranIfSmNghbrBackupChannelSetChange, wranIfSmLocalPrioritySet1Change, wranIfSmLocalPriority2Change, wranIfSmLocalPriority3Change, wranIfSmCurrentStatusChange, and wranIfSmRegTrackingChange..

      - wranIfSmTrapDefintion

This MIB group specifies the definition of traps that can be enabled/disable in wranIfSnTrapControl.

        - wranIfSmBlmReqChangeTrap

This trap contains the information related to a BLM-REQ that is pending in the SM.

        - wranIfSmBlmRepChangeTrap

This trap contains the information related to a BLM-REP that is received by the SM.

        - wranIfSmOccupiedChannelSetChangeTrap

This trap contains the information related to the current state of the Occupied Channel Set in the SM.

        - wranIfSmNghbrBackupChannelSetChangeTrap

This trap contains the information related to the current state of the Neighbor WRAN Backup Channel Set in the SM.

        - wranIfSmLocalPrioritySet1ChangeTrap

This trap contains the information related to the current state of the Local Priority Set 1 in the SM.

        - wranIfSmLocalPrioritySet2ChangeTrap

This trap contains the information related to the current state of the Local Priority Set 2 in the SM.

        - wranIfSmLocalPrioritySet3ChangeTrap

This trap contains the information related to the current state of the Local Priority Set 3 in the SM.

        - wranIfSmCurrentStatusChangeTrap

This trap contains the information related to the current state the SM is in.

        - wranIfSmRegTrackingChangeTrap

This trap contains the information related to the tracking of the location of CPE's within the purview of the BS.

      - wranIfSmNotificationObjectsTable

This MIB provides a table to track notification objects that have been reported by traps related to the operation of the SM. It is made up of one entry containing the objects related to the most recent trap/event. The entry is defined by wranIfSmNotificationObjectsEntry.

        - wranIfSmNotificationObjectsEntry

This object .defines an entry in wranIfSmNotificationObjectsTable.

          - wranifSmNotificationSid

SID of station generating trap. wranIfNotificationSid set == 0 is reserved for traps dealing with channel set management (see wranIfSmOccupiedChannelSetChangeTrap, wranIfSmNghbrBackupChannelSetChangeTrap, wranIfSmLocalPrioritySet1ChangeTrap, wranIfSmLocalPrioritySet2ChangeTrap, wranIfSmLocalPrioritySet3ChangeTrap) and the current state of the SM (see wranIfSmCurrentStatusChangeTrap). wranIfNotificationSid set == 1..511 when dealing with traps for bulk measurement (see wranIfSmBlmReqChangeTrap, wranIfSmBlmRepChangeTrap) or CPE registration tracking (see wranIfSmRegTackingChangeTrap).

          - wranIfSmNotificationBlmTransactionId

The transaction ID of the bulk measurement transaction. Only pertinent when wranIfSmBlmReqChangeTrap or wranIfSmBlmRepChangeTrap.

          - wranIfSmNotificationOccupiedChannelSet

The current/new contents of the WRAN Occupied Channel Set. Only pertinent when wranIfSmOccupiedChannelSetChangeTrap is enabled.

          - wranIfSmNotificationBackupChannelSet

The current/new contents of the Neighbor WRAN Backup Channel Set. Only pertinent when wranIfSmNghbrBackupChannelSetChangeTrap is enabled.

          - wranIfSmNotificationLocalPrioritySet1

The current/new contents of the Local Priority Set 1. Only pertinent when wranIfSmLocalPrioritySet1ChangeTrap is enabled.

          - wranIfSmNotificationLocalPrioritySet2

The current/new contents of the Local Priority Set 2. Only pertinent when wranIfSmLocalPrioritySet2ChangeTrap is enabled.

          - wranIfSmNotificationLocalPrioritySet3

The current/new contents of the Local Priority Set 3. Only pertinent when wranIfSmLocalPrioritySet3ChangeTrap is enabled.

          - wranIfSmNotificationRecentAction

Value of wranIfSmRecentAction that triggered transition to current state of SM. Only pertinent when wranIfSmCurrentStatusChangeTrap is enabled.

          - wranIfSmPendingBlmReqStatus

Indicates BLM-RSP pertaining to bulk measurement transaction (defined by wranIfSmNotificationBlmTransactionId) was received from wranIfSmNotificationSid, if wranIfSmNotificationSid is a unicast SID, or if all BLM-RSP received from members of the multicast gourp identified by wranIfSmNotificationSid.

          - wranIfSmBlmRepStatus

Indicates whether or noat BLM-ACK has been sent, pertaining to BLM-REP received in bulk measurement transaction defined by wranIfSmNotificationBlmTransactionId.

          - wranIfSmOccupiedChannelSetStatus

Indication of whether or not channels have been added or removed when the WRAN Occupied Channel Set is modified.

          - wranIfSmNghbrBackupChannelSetStatus

Indication of whether or not channels have been added or removed when the Neighbor WRAN Backup Channel Set is modified.

          - wranIfSmLocalPrioritySet1Status

Indication of whether or not channels have been added or removed when the Local Priority Set 1 is modified.

          - wranIfSmLocalPrioritySet2Status

Indication of whether or not channels have been added or removed when the Local Priority Set 2 is modified.

          - wranIfSmLocalPrioritySet3Status

Indication of whether or not channels have been added or removed when the Local Priority Set 3 is modified.

          - wranIfSmCurrentStateStatus

Bitmap indicating current value of 'Initiate_Channel_Move' flag, value of 'Self_Coexistence_Mode' flag, whether or not operating channel assignment has been changed, whether or not recently detected signal was properly classified (i.e. not found to be == 'undetermined'), whether or not T47 expired, as well as whether or not T46 has expired.

      - wranIfSmMibGroups

This object helps define which MIB groups are available within this module, and which MIB objects are a part of each group.

        - wranIfSmPendingBlmReqGroup

This group contains objects related to tracking pending BLM-REQs.

        - wranIfSmBlmReqGroup

This group contains objects related to tracking received BLM-REPs.

        - wranIfSmChClassificationGroup

This group contains objects used to track how channels the SM is managing have been classified.

        - wranIfSmChannelSetGroup

This group contains objects used to track the various channel sets the SM operates with.

        - wranIfSmCurrentStatusGroup

This group contains objects used to track the current state of the SM.

        - wranIfSmRegTrackingGroup

This group contains objects the SM uses to keep track of the location of registered CPEs.

        - wranIfSmNotificationsGroup

This group contains notification objects related to traps configured for the SM.

      - wranIfSmMibGroups

Defines MIB objects that are optional and mandatory for SM compliance.

    - wranIfSsaMib

This MIB group deals with objects related to the configuration, operation, and monitoring of the Spectrum Automaton.

The automaton needs to be told what needs to be sensed in the given regulatory domain.  This can be done through the MIB referring to the Regulatory Domain information from Annex A. The CPE also need to be told which type of incumbent requires urgent, less urgent and non-urgent reporting to the BS (see Table 282 in Annex A).  The background sensing done by the automaton shall be done under the MIB guidance.

This MIB group is made up of the following objects: wranIfSsaSensingCapTable, wranIfSsaStatusTable, wranIfSsaConfigTable, wranIfSsaPendingBlmRepTable, wranIfSsaSensingRecordTable, wranIfSsaSsfMode0OutputTable, wranIfSsaSsfMode1OutputTable, wranIfSsaSsfMode2OutputTable, wranIfSsaSsfWiMicMSF1Table, wranIfSsaSsfWiMicMSF2Table, wranIfSsaSsfWiMicMSF3Table, and wranIfSsaGeolocationTable.

      - wranIfSsaSensingCapTable

This MIB object represents a table that stores the current sensing capabilities for a SSA under control of the SM. There is one entry in this table for a SSA, defined by wranIfSsaSensingCapEntry. This is MIB is stored at the BS and CPE. These values are also stored in wranIfBsCpeRegCapabilityRspTable (section 13.1.2.2.7) at the BS, in an entry specific to this CPE.

        - wranIfSsaSensingCapEntry

This object represents the entry that stores a CPE’s sensing capabilitites

          - wranIfSsaSensingThreshold

This object is the recommended sensing threshold that the CPE is capable of supporting. It is in units of dBm, encoded in a single, integery byte value that is assumed to be negative (e.g. 0x01 = -1 dBm, 0x72 = -114 dBm).

          - wranIfSsaSensRecContigPeriodDuration

This object is the recommended contiguous sensing period duration that the CPE is capable of supporting. It is in integer, in units of symbols. This value ranges from 0 – 1023, and is encoded in a 2-octet length value.

          - wranIfSsaSensRecNumPeriods

This object is the recommended number of sensing periods that a CPE can support. It is an integer value, encoded in 1-octet length value.

          - wranIfSsaSensRecPeriodInterval

This object is the recommended interval between sensing periods that a CPE can support. It is an integer value, in units of frames. It is encoded in a 2-octet length value.

      - wranIfSsaStatusTable

This MIB object represents a table that tracks what the current state the SSA is in. There is only one entry in this table, to define the current state of the SSA, as well as any parameters of interest for current SSA procedures. This entry is defined by, wranIfSsaStatusEntry.

        - wranIfSsaStatusEntry

This object represents the entry in wranIfSsaStatusTable.

          - wranIfSsaCurrentState

The current state of the SSA is in (see 10.3.1, Figure 173), either "SSA In-band Sensing" or "SSA Out-of-band Sensing".

          - wranIfSsaRecentEvent

The recent event that caused entry into current state in (see 10.3.1, Figure 173).

          - wranIfSsaRecentAction

The recent action, triggered by recent event, that was undertaken while entering into current state in (see 10.3.1, Figure 173).

          - wranIfSsaIncProhibitedChannels

Contents of most recent IPC-UPD message received from SM.

          - wranIfSsaCurrentT48

Current value of T48 at SSA.

          - wranIfSsaCurrentT49

Current value of T49 at SSA.

          - wranIfSsaCurrentT50

Current value of T50 at SSA.

          - wranIfSsaIntraFrameQpCycleLength

Obtained from CHQ-REQ or SCH. Specified in number of superframes, it indicates the spacing between the superframes for which the intra-frame quiet period specification is valid. For example, if this field is set to 1, the Quiet Period Cycle repeats every superframe; if it is set to 2, the Quiet Period Cycle repeats every 2 superframes, etc. When = 0, no intra-frame quiet period is scheduled or the current intra-frame quiet period is canceled

          - wranIfSsaIntraFrameQpCycleOffset

Obtained from CHQ-REQ or SCH. Valid only if intra-frame Sensing Cycle Length > 0. Used for in-band intra-frame sensing.

Specified in number of superframes, it indicates the offset from this SCH transmission to the beginning of the first superframe in the current intra-frame sensing cycle.

          - wranIfSsaIntraFrameQpCycleFrameBitmap

Obtained from CHQ-REQ or SCH. Valid only if Intra-frame Quiet Period Cycle Length > 0. Valid for each superframe identified by the Intra-frame Quiet Period Cycle Length, each bit in the bitmap corresponds to one frame within the superframe. If the bit is set to 0, no intra-frame quiet period shall be scheduled in the corresponding frame. If the bit is set to 1, an intra-frame quiet period shall be scheduled within the corresponding frame for the duration specified by Intra-frame Quiet period Duration.

          - wranIfSsaIntraFrameQpDuration

Obtained from CHQ-REQ or SCH. Valid only if Intra-frame Quiet Period Cycle Length > 0. If this field is set to a value different from 0 (zero): it indicates the number of symbols starting from the end of the frame during which no transmission shall take place.

          - wranIfSsaInterFrameQpDuration

Obtained from CHQ-REQ or SCH. Used for in-band inter-frame sensing, it indicates the duration of the next scheduled quiet period. When > 0 it indicates the number of frames starting from Inter-frame Quiet Period Offset that shall be used to perform inter-frame sensing. When == 0, it cancels the next scheduled quiet period for inter-frame sensing or indicates that no inter-frame sensing are currently scheduled.

          - wranIfSsaInterFrameQpOffset

Obtained from CHQ-REQ or SCH. Used for in-band inter-frame sensing, it indicates the time span between the transmission of this information and the next scheduled quiet period for inter-frame sensing. Bit# 11-4: index the superframe number, Bit# 3-0: index the frame number when the next scheduled quiet period for inter-frame sensing will start.

      - wranIfSsaConfigTable

This MIB object represents a table that tracks what the default configuration of SSA timers and constants. There is only one entry in this table, to define the default configuration of the SSA. This entry is defined by, wranIfSsaConfigEntry.

        - wranIfSsaConfigEntry

This object represents the entry in wranIfSsaConfigTable.

          - wranIfSsaT19

Time DS-channel remains unusable.

          - wranIfSsaT29

Wait for BLM-ACK timeout.

          - wranIfSsaMaxBlmRepRetries

Maximum number of retry attempts allowed for sending BLM-REP.

          - wranIfSmSsaChAvailabilityCheckTime

Time during which a TV channel shall be checked for the presence of licensed incumbent signals having a level above the incumbent detection threshold prior to commencement of WRAN operation in the channel, and in the case of TV, a related channel at an EIRP level that can affect the measured TV channel.

          - wranIfSmSsaNonOccupancyPeriod

The required period during which WRAN device transmissions SHALL NOT occur in a given TV channel because of the detected presence of an incumbent signal in that channel above the Incumbent detection threshold, or in the case of TV, above a given EIRP level.

          - wranIfSmSsaChannelDetectionTime

Maximum time taken by a WRAN device to detect a licensed incumbent signal above the Incumbent Detection Threshold within a given TV channel during normal WRAN operation.

          - wranIfSmSsaChannelSetupTime

The window of time that may be taken by a WRAN CPE to transmit control information to a WRAN base station in order to establish operation with that base station at the prescribed power or, in the case of TV, at or below the allowable EIRP within a given TV channel.

          - wranIfSmSsaChannelOpeningTxTime

The aggregate duration of control transmissions by WRAN devices during the Channel Setup Time, which starts at the end end of the Channel Availability Check Time..

          - wranIfSmSsaChannelMoveTime

The time taken by WRAN system to cease all interfering transmissions on the current TV channel upon detection of a license incumbent signal above the relevant Incumbent Detection Threshold, or in the case of TV, to alternatively reduces its EIRP to which is allowable within a given TV channel upon detection of a TV signal in the same or a related channel.

          - wranIfSmSsaChannelClosingTxTime

The aggregate duration of conrol transmissions by the WRAN devices during the Channel Move/EIRP Reduction Time, which starts upon detection of a licensed incumbent singal above the relevant Incumbent Detection Threshold.

          - wranIfSmSsaMicProtectionRadius

Radius of contour within which the WRAN system cannot operate due to potential interference with the microphone.

          - wranIfSmSsaT41

Maximum time interval allowed before sensing is performed on the candidate channel to ensure that no incumbents are detected.

          - wranIfSmSsaT42

Maximum time interval allowed before sensing is performed on the backup channel to ensure that no incumbents are detected.

          - wranIfSmSsaT43

Minimum time duration without detection of any incumbent for a candidate channel to transition to the backup channel.

          - wranIfSmSsaT44

Maximum time to ensure that the channel move information is successfully conveyed to all the associated CPEs and BS (self-coexistence mode).

          - wranIfSmSsaT45

Maximum WRAN operation time without access to the incumbent database service.

          - wranIfSmSsaT59

Waiting time before which the CPE moves to its backup channels if it no longer hears from its BS. This is used to make sure that the CPE waits long enough after UCS Notification so that BS has had time to move to the backup channel, it it decided to do so.

          - wranIfSmSsaT47

The prescribed time by the WRAN operator to refresh the incumbent database service.

          - wranIfSmSsaT48

Lapse timer keeps track of whether the Operating Channel N has been cleared using spectrum sensing.

          - wranIfSmSsaT49

Lapse timer keeps track of whether the Adjacent Channel N-1 has been cleared using spectrum sensing.

          - wranIfSmSsaT50

Lapse timer keeps track of whether the Adjacent Channel N+1 has been cleared using spectrum sensing.

          - wranIfSmSsaT51

Initiated when SSA loses contact with the SM.

          - wranIfSmSsaT53

The parameter TINSens is used to verify that in-band sensing has been done within the required In-service monitoring period. The TINSens parameter is driven by regulatory domain requirements (Annex A).

          - wranIfSmSsaT54

The parameter TOUTSens is used to verify that out-of-band sensing has been done within the required "Acquiring a channel monitoring period" specified in Annex A. This value would be used to initalize a lapse timer for each channel in the backup candidate channel list at each CPE so that it compared to Tsens.

          - wranIfSmSsaT55

The T55 parameter corresponds to the maximum length of time required to to carry out the in-band sensing process. Manufacturers need to specify the sensing time required to detect the specified signals with required accuracy for in-band sensing.

          - wranIfSmSsaT60

The T60 parameter corresponds to the maximum length of time required to to carry out the out-of-band sensing process. Manufacturers need to specify the sensing time required to detect the specified signals with required accuracy for out-of-band sensing.

      - wranIfSsaPendingBlmRepTable

This MIB object represents a table that tracks the status of the execution of ongoing reporting (BLM-REP) in response to BLM-REQs. For each BLM-REQ there is a corresponding BLM-RSP to indicate that REQ message was received by the SSA. When an SSA is done with the sensing it will send a BLM-REP to the SM. This table keeps track of any BLM-REP messages that are pending acknowledgement from the SM. When a report sent in a response is acknowledged, then the entry corresponding to the report and request will be cleared. This entry is defined by, wranIfSsaPendingBlmRepEntry.

        - wranIfSsaPendingBlmRepEntry

This object represents the entry in wranIfSsaPendingBlmRepTable.

          - wranIfSmPendingBlmReqTransactionId

Transaction ID for pending BLM-REQ.

          - wranIfSsaPendingBlmReqMsg

Contents of pending BLM-REQ message.

          - wranIfSsaPendingBlmRspSent

Indication of whether or not BLM-RSP pertaining to BLM-REQ has been sent.

          - wranIfSsaPendingBlmRepGenerated

Indication of whether or not BLM-REP corresponding to BLM-REQ has been generated, i.e. sensing has been executed.

          - wranIfSsaPendingBlmRepMsg

Contents of BLM-REP message that corresponds to the BLM-REQ.

          - wranIfSsaPendingBlmRepSent

Indication of whether or not BLM-REP pertaining to BLM-REQ has been sent.

          - wranIfSsaPendingBlmRepAck

Indication of whether or not BLM-ACK, has been sent to acknowledge all transmitters of BLM-REP messages pertaining to BLM-REQ that was issued.

          - wranIfSsaPendingBlmRepNumTx

Current number of times that BLM-REP has been resent.

      - wranIfSsaSensingRecordTable

This object contains information the sensing status of each channel. It is made of multiple entries, one for each channel, as defined by wranIfSsaSensingRecordEntry.

        - wranIfSsaSensingRecordEntry

This object represents the entry in wranIfSsaSensingRecordTable.

          - wranIfSsaSensingChannel

Channel that sensing has been conducted on, or not conducted on if in IPC-UPD.

          - wranIfSsaTimeLastSensing

Last time that this channel was sensed.

          - wranIfSsaTimeLastPositive

Last time that signal was positively confirmed on this channel.

          - wranIfSsaSensingPathRssi

RSSI on sensing path.

          - wranIfSsaWranPathRssi

RSSI on WRAN signal path.

          - wranIfSsaSignalType

Type of signal that was sensed on the channel.

          - wranIfSsaWranServiceAdvertisement

If signal type was for WRAN, this indicates the BS ID of the captured SCH from neighbor WRAN.

          - wranIfSsaIdcUpdIndication

Indication if channel is on IPC-UPD.

      - wranIfSsaSsfMode0OutputTable

This object contains the current output of SSF Mode 0 sensing. It is made up of multiple entries, one for each signal type that was sensed. Each entry is defined by wranIfSsaSsfMode0OutputEntry.

        - wranIfSsaSsfMode0OutputEntry

This object represents the entry in wranIfSsaSsfMode0OutputTable.

          - wranIfSsaSsfMode0SignalType

Signal type of signal that was to be sensed for.

          - wranIfSsaSsfMode0SignalPresent

Indication of whether or not a signal of signal type was detected.

      - wranIfSsaSsfMode1OutputTable

This object contains the current output of SSF Mode 1 sensing. It is made up of multiple entries, one for each signal type that was sensed. Each entry is defined by wranIfSsaSsfMode1OutputEntry.

        - wranIfSsaSsfMode1OutputEntry

This object represents the entry in wranIfSsaSsfMode1OutputTable.

          - wranIfSsaSsfMode1SignalType

Signal type of signal that was to be sensed for.

          - wranIfSsaSsfMode1SignalPresent

Indication of whether or not a signal of signal type was detected.

          - wranIfSsaSsfMode1SignalConfidence

Confidence level in signal present decision.

      - wranIfSsaSsfMode2OutputTable

This object contains the current output of SSF Mode 2 sensing. It is made up of multiple entries, one for each signal type that was sensed. Each entry is defined by wranIfSsaSsfMode2OutputEntry.

        - wranIfSsaSsfMode2OutputEntry

This object represents the entry in wranIfSsaSsfMode2OutputTable.

          - wranIfSsaSsfMode2SignalType

Signal type of signal that was to be sensed for.

          - wranIfSsaSsfMode0SignalRssiMean

Mean of RSSI signal measurements.

          - wranIfSsaSsfMode0SignalStdDevRssi

Std Dev of RSSI signal measurements.

      - wranIfSsaSsfWiMicMSF1Table

This object contains the current output the payload of MSF1 of 802.22.1 beacon on recently sensed channels. It is made up of multiple entries for for each channel a MSF1 was sensed on. Each entry is defined by wranIfSsaSsfWiMicMSF1Entry.

        - wranIfSsaSsfWiMicMSF1Entry

This object represents the entry in wranIfSsaSsfWiMicMSF1Table.

          - wranIfSsaSsfWiMicMSF1Channel

Channel number wireless microphone beacon was captured on.

          - wranIfSsaSsfWiMicMSF1Payload

Payload of MSF1, not including the CRC1 field.

          - wranIfSsaSsfWiMicMSF1Crc1Status

Indication of whether or not MSF1 passed verification of CRC1.

      - wranIfSsaSsfWiMicMSF2Table

This object contains the current output the payload of MSF2 of 802.22.1 beacon on recently sensed channels. It is made up of multiple entries for for each channel a MSF2 was sensed on. Each entry is defined by wranIfSsaSsfWiMicMSF1Entry.

        - wranIfSsaSsfWiMicMSF2Entry

This object represents the entry in wranIfSsaSsfWiMicMSF2Table.

          - wranIfSsaSsfWiMicMSF2Channel

Channel number wireless microphone beacon was captured on.

          - wranIfSsaSsfWiMicMSF2Payload

Payload of MSF2, not including the CRC2 field.

          - wranIfSsaSsfWiMicMSF2Crc2Status

Indication of whether or not MSF2 passed verification of CRC2.

      - wranIfSsaSsfWiMicMSF3Table

This object contains the current output the payload of MSF3 of 802.22.1 beacon on recently sensed channels. It is made up of multiple entries for each channel a MSF3 was sensed on. Each entry is defined by wranIfSsaSsfWiMicMSF1Entry.

        - wranIfSsaSsfWiMicMSF3Entry

This object represents the entry in wranIfSsaSsfWiMicMSF3Table.

          - wranIfSsaSsfWiMicMSF3Channel

Channel number wireless microphone beacon was captured on.

          - wranIfSsaSsfWiMicMSF3Payload

Payload of MSF3, not including the CRC3 field.

          - wranIfSsaSsfWiMicMSF3Crc3Status

Indication of whether or not MSF3 passed verification of CRC3.

      - wranIfSsaGeolocationTable

This object contains the current parameters and calculations being used by the Geolocation component of the SSA. It is made up of one entry to contain current values being used/calculated in the BS-to-CPE fine-ranging, CPE-to-CPE fine ranging, geo-location calculation outputs, and final geo-location string. It is made up of one entry, defined by wranIfSsaGeoLocationEntry.

        - wranIfSsaGeolocationEntry

This object represents the entry in wranIfSsaGeolocationTable.

          - wranIfSsaGeolocationVernier1

Vernier1, recorded only at the CPE.

          - wranIfSsaGeolocationVernier2

Vernier2, recorded only at the BS.

          - wranIfSsaGeolocationVernier3

Vernier3, recorded only at the CPE.

          - wranIfSsaGeolocationTRange1

TRange1, also known as T52. Set by BS when downstream bust leaves the BS, i.e. at start of frame preamble.

          - wranIfSsaGeolocationTACbp

TACBP, Timing advance for CBP burst used in ranging calculations..

    - wranIfDatabaseServiceMib

This MIB group deals with objects related to the configuration of access to, as well as the interaction with the Database Service. This group is made up of the following objects: wranIfBsMgmtInfoTable, wranIfBsDeviceEnlistmentTable, wranIfDbsChannelIndicationTable, wranIfDbsAccessTable, wranIfDbsTrapControl, wranIfDbsTrapDefinition, wranIfDbsNotificationObjectsTable, wranIfDbsMibGroups, and wranIfDbsMibCompliance..

      - wranIfBsMgmtInfoTable

This object stores information regarding information on the BS management interface the DBS can access. It is made up of one entry, defined by wranIfBsMgmtInfoEntry.

        - wranIfBsMgmtInfoEntry

This object defines an entry in wranIfBsMgmtInfoTable.

          - wranIfBsMgmtInfoIndex

Index of entry in the table, defaults to 0.

          - wranIfBsSizeMgmtUrl

Size of Base Station Management URL (see 10.7.1.1).

          - wranIfBsMgmtUrl

Base Station Management URL (see 10.7.1.1).

          - wranIfBsSizeMgmtDeviceId

Size of BS FCC Device ID (see 10.7.1.1).

          - wranIfBsMgmtDeviceId

BS FCC Device ID (see 10.7.1.1).

          - wranIfBsSizeMgmtSn

Size of BS serial number (see 10.7.1.1).

          - wranIfBsMgmtSn

BS serial number (see 10.7.1.1).

          - wranIfBsSizeMgmtLocation

Size of Location data string of BS (see 10.7.1.1).

          - wranIfBsMgmtLocation

Location data string of BS (see 10.7.1.1).

          - wranIfBsMgmtAntennaHeight

Antenna height of BS (see 10.7.1.1).

          - wranIfBsSizeMgmtContactName

Size of Contact Name for person(s) who has ownership of the BS (see 10.7.1.1).

          - wranIfBsMgmtContactName

Contact Name for person(s) who has ownership of the BS (see 10.7.1.1).

          - wranIfBsSizeMgmtContactPhysAddress

Size of Physical address for contact who has ownership of the BS (see 10.7.1.1).

          - wranIfBsMgmtContactPhysAddress

Physical address for contact who has ownership of the BS (see 10.7.1.1).

          - wranIfBsSizeMgmtEmailAddress

E-mail address for contact who has ownership of the BS (see 10.7.1.1).

          - wranIfBsMgmtEmailAddress

E-mail address for contact who has ownership of the BS (see 10.7.1.1).

          - wranIfBsSizeMgmtPhoneNumber

Size of Telephone # for contact who has ownership of the BS (see 10.7.1.1).

          - wranIfBsMgmtPhoneNumber

Telephone # for contact who has ownership of the BS (see 10.7.1.1).

          - wranIfBsSizeAccessUrl

Size of Base Station Access Service URL (see 10.7.1.1).

          - wranIfBsAccessUrl

Base Station Access Service URL (see 10.7.1.1).

      - wranIfDbsDeviceEnlistmentTable

This object stores information regarding information on devices entering the network that the BS has attempted to enlist/register with the database service. It is made up of multiple entries, each defined by wranIfBsDeviceEnlistmentEntry.

        - wranIfDbsDeviceEnlistmentEntry

This object defines an entry in wranIfDbsDeviceEnlistmentTable.

          - wranIfDbsDeviceEnlistmentIndex

Index of entry in the table.

          - wranIfDbsDeviceEnlistmentConfirmed

Has this enlistment been confirmed via receipt of M-DEVICE-ENLISTMENT-CONFIRM (see 10.7.1.4) from database service.

          - wranIfDbsConfirmedDeviceType

Type of device being registered, fixed BS, fixed CPE, personal/portable.

          - wranIfDbsSizeConfirmedDeviceId

Size of Device ID of device that is being enlisted/registered with the database service.

          - wranIfDbsConfirmedDeviceId

Device ID of device that is being enlisted/registered with the database service.

          - wranIfDbsSizeConfirmedDeviceSn

Size of Serial number of device that is being enlisted/registered with the database service.

          - wranIfDbsConfirmedDeviceSn

Serial number of device that is being enlisted/registered with the database service.

          - wranIfDbsSizeConfirmedDeviceLocation

SizeLocation string of device requesting enlistment.

          - wranIfDbsConfirmedDeviceLocation

Location string of device requesting enlistment.

          - wranIfDbsSizeProxyDeviceId

Device ID of proxy device BS may use to send queries to the database.

          - wranIfDbsProxyDeviceId

Device ID of proxy device BS may use to send queries to the database.

          - wranIfDbsSizeProxySn

Size of Serial number of proxy device BS may use to send queries to the database.

          - wranIfDbsProxySn

Serial number of proxy device BS may use to send queries to the database.

          - wranIfDbsSizeRespPartyName

Name of party responsible for device enlistment/registration.

          - wranIfDbsRespPartyName

Name of party responsible for device enlistment/registration.

          - wranIfDbsConfirmedDeviceAntennaHeight

Antenna height of of device being enlisted.

          - wranIfDbsSizeConfirmedDeviceContactName

Size of Contact Name for person(s) who has ownership of the device, only pertinent if device type is fixed BS or CPE.

          - wranIfDbsConfirmedDeviceContactName

Contact Name for person(s) who has ownership of the device, only pertinent if device type is fixed BS or CPE.

          - wranIfDbsSizeConfirmedDeviceContactPhysAddress

Size of Physical address for contact who has ownership of the BS, only pertinent of device type is fixed BS or CPE.

          - wranIfDbsConfirmedDeviceContactPhysAddress

Physical address for contact who has ownership of the BS, only pertinent of device type is fixed BS or CPE.

          - wranIfDbsSizeConfirmedDeviceEmailAddress

Size of E-mail address for contact who has ownership of the BS, only pertinent of device type is fixed BS or CPE.

          - wranIfDbsConfirmedDeviceEmailAddress

E-mail address for contact who has ownership of the BS, only pertinent of device type is fixed BS or CPE.

          - wranIfDbsSizeConfirmedDevicePhoneNumber

Size of Telephone # for contact who has ownership of the BS, only pertinent of device type is fixed BS or CPE.

          - wranIfDbsConfirmedDevicePhoneNumber

Telephone # for contact who has ownership of the BS, only pertinent of device type is fixed BS or CPE.

          - wranIfDbsConfirmedDeviceAntennaInformation

Antenna information of device, only applicable if transmission of antenna information is supported by database service.

          - wranIfDbsConfirmedDeviceAntennaAzimuth

Antenna azimuth of device, only applicable if transmission of antenna information is supported by database service.

          - wranIfDbsConfirmedDeviceConfirmationMsgTime

Timestamp of transmission for M-DEVICE-ENLISTMENT-REQUEST.

      - wranIfDbsChannelIndicationTable

This object stores information what channels have been indicated (upon receipt of M-DB-AVAILABLE-CHANNEL-INDICATION) as available and their EIRP limit at a given location for a particular device. It is made up of multiple entries, one each for the tuple of location||channel||EIRP||DeviceID. Each entry is defined by wranIfDbsChannelIndicationEntry.

        - wranIfDbsChannelIndicationEntry

Compound object that defines an entry in wranIfDbsChannelIndicationTable.

          - wranIfDbsChannelIndicationIndex

Index of entry into this table.

          - wranIfDbsSizeDeviceId

Size of Device ID of device that channel is indicated as available for.

          - wranIfDbsDeviceId

Device ID of device that channel is indicated as available for.

          - wranIfDbsSizeDeviceSn

Size of Serial number of device that channel is indicated as available for.

          - wranIfDbsDeviceSn

Serial number of device that channel is indicated as available for.

          - wranIfDbsDeviceChannelNumber

Channel number  that availability is indicated for.

          - wranIfDbsDeviceMaxAllowedEirp

Maximum allowed EIRP on the channel.

          - wranIfDbsSizeDeviceLocation

Size of Location string of device requesting that channel availability is indicated for.

          - wranIfDbsDeviceLocation

Location string of device requesting that channel availability is indicated for.

          - wranIfDbsDeviceDbsIndex

Index of entry in wranIfDbsAccessTable that corresponds to database service that provided this channel indication.

      - wranIfDbsAccessTable

This object stores information regarding access information for  accessing a database service. There is more than one entry in this table, one for each database service that is available. Each entry is defined by wranIfDbsAccessEntry.

        - wranIfDbsAccessEntry

A compound object that stores entries in wranIfDbsAccessTable.

          - wranIfDbsAccessEntryIndex

Index of entry in this table.

          - wranIfDbsSizeAccessUrl

Size of URL used to access database service.

          - wranIfDbsAccessUrl

URL used to access database service.

          - wranIfDbsAccessCredentialType

Indication of the type of credential to be used when authenticating access and transfer of information to database service.

          - wranIfDbsSizeAccessCertCredential

Size of Certificate Credential BS or proxy device uses to authenticate access to the database, only pertinent when wranIfDbsAccessCredentialType is set to certificate.

          - wranIfDbsAccessCertCredential

Certificate Credential BS or proxy device uses to authenticate access to the databas database, only pertinent when wranIfDbsAccessCredentialType is set to certificate.

          - wranIfDbsSizeAccessUserID

Size of login User ID used for accessing the database service, only pertinent when wranIfDbsAccessCredentialType is set to userID and password.

          - wranIfDbsAccessUserID

Login User ID used for accessing the database service, only pertinent when wranIfDbsAccessCredentialType is set to userID and password.

          - wranIfDbsSizeAccessPassword

Size of login password (or hash of password) used for accessing the database service, only pertinent when wranIfDbsAccessCredentialType is set to userID and password.

          - wranIfDbsAccessPassword

Login password (or hash of password) used for accessing the database service, only pertinent when wranIfDbsAccessCredentialType is set to userID and password.

          - wranIfDbsAccessLastTxTime

Time indication of last transmission of a message to the database service.

          - wranIfDbsAccessLastRxTime

Time indication of last reception of a message from the database service.

          - wranIfDbsAccessAntennaInfoRequired

Indication of whether or not antenna info is required to be transmitted to database service.

          - wranIfDbsAccessPriority

Priority of database service. Higher priority means that a particular database service will be preferred when device enlistment and channel indication requests need to be made.

      - wranIfDbsTrapControl

Defines control elements for traps related to interaction with the Database Service. This is 3bit field that enables setting a trap for particular CPE events: wranIfDbsAvalailabilityChange, wranIfDbsEnlistmentChange, and wranIfDbsChannelIndicationChange.

      - wranIfDbsTrapDefintion

This MIB group specifies the definition of traps that can be enabled/disable in wranIfDbsTrapControl.

        - wranIfDbsAvailabilityChangeTrap

This trap contains information related to the last time a particular database service was accessed.

        - wranIfDbsEnlistmentChangeTrap

This traps contains information regarding the status of enlistment confirmation for a particular device in the network.

        - wranIfDbsChannelIndicationChangeTrap

This trap contains information regarding the status of channel indication responses from the database service.

      - wranIfDbsNotificationObjectsTable

This MIB provides a table to track notification objects that have been reported by the traps related to access to the database. It is made up of one entry containing the objects related to the  most recent trap/event. The entry is defined by wranIfDbsNotificationObjectsEntry.

        - wranIfDbsNotificationObjectsEntry

Definition of an entry into wranIfDbsNotificationObjectsTable. Each entry is identified by wranIfNotificationDeviceId.

          - wranIfDbsNotificationSizeDeviceId

Size of Device ID of the device that trap status information is stored for.

          - wranIfDbsNotificationDeviceId

Device ID of the device that trap status information is stored for.

          - wranIfDbsNotificationSizeDeviceSn

Size of Serial Number of the device that trap status information is stored for.

          - wranIfDbsNotificationDeviceSn

Serial Number of the device that trap status information is stored for.

          - wranIfDbsNotificationDbsSizeAccessUrl

Size of Access URL of database service that device enlistment of channel indication query was exchanged with.

          - wranIfDbsNotificationDbsAccessUrl

Access URL of database service that device enlistment of channel indication query was exchanged with.

          - wranIfDbsAccessStatus

Indication of whether or not a message is transmited or received from database service.

          - wranIfDbsConfirmedDeviceStatus

Indication of whether or not the enlistment of the device has been successfully completed or not.

          - wranIfDbsChannelIndicationStatus

Indication of whether channels have been indicated as available or not for the device in question.

      - wranIfDbsMibGroups

This object helps define which MIB groups are available within this module, and which MIB objects are part of each group.

        - wranIfDbsMgmtGroup

This group contains configuration objects related to the managemetn interface on the BS that the database service can access.

        - wranIfDbsEnlistmentGroup

This group contains configuration objects related to enlistment of devices.

        - wranIfDbsChannelIndicationGroup

This group contains configuration objects related to indication of available channels.

        - wranIfDbsAccessGroup

This group contains configuration objects related to current accessibility of the database service.

        - wranIfDbsNotificationsGroup

This group contains notification objects related to traps configured for access to the database service.

      - wranIfDbsMibCompliance

Defines MIB objects that are optional and manadatory for database service.

  - MIB Module Definitions (ASN.1)

    - wranDevMib

IEEE802dot22-WRAN-DEV-MIB	::=	BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION,

RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM SNMPv2-TC

InetAddressType, InetAddress

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPv2-CONF;

wranDevMib	MODULE-IDENTITY

LAST-UPDATED	"201205140000Z" – May 14, 2012

ORGANIZATION	"IEEE 802.22"

CONTACT-INFO

"WG E-mail:  STDS-802-22@LISTSERV.IEEE.ORG

WG Chair:   Apurva N. Mody

E-mail:     apurva.mody@ieee.org

TGa Chair/Editor:  Ranga Reddy

E-mail:		  ranga.reddy@ieee.org"

DESCRIPTION

"This material is from IEEE 802.22a-2012

Copyright (c) 2012. This MIB Module

Defines device related manage objects

for IEEE 802.22-2011 base Customer

Premise Equipment and Base Station, and

is under

iso(1).std(0).iso8802(8802).wran(22).wranDevMib(1)"

REVISION		"201205140000Z"

DESCRIPTION

"The first version of IEEE802dot22-WRAN-DEV-MIB."

::= {iso std(0) iso8802(8802) wran(22) 1}

wranDevBsObjects		OBJECT IDENTIFIER ::= { wranDevMib 1 }

wranDevCpeObjects		OBJECT IDENTIFIER ::= { wranDevMib 2 }

wranDevCommonObjects	OBJECT IDENTIFIER ::= { wranDevMib 3 }

wranDevBsObjects			OBJECT IDENTIFIER ::= { wranDevMib 1 }

wranDevBsSoftwareUpgradeTable	OBJECT IDENTIFIER ::= { wranDevBsObjects 1 }

wranDevBsSoftwareUpgradeTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranDevBsSoftwareUpgradeEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This table defines objects associated with BS software

configuration"

::= { wranDevBsObjects 1 }

wranDevBsSoftwareUpgradeEntry	OBJECT-TYPE

SYNTAX	wranDevBsSoftwareUpgradeEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Definition of an entry into wranDevBsSoftwareUpgradeTable. Each

entry is identified by wranDevBsDeviceIndex."

INDEX	{ wranDevBsDeviceIndex }

::= { wranDevBsSoftwareUpgradeTable 1 }

wranDevBsSoftwareUpgradeEntry ::= SEQUENCE {

wranDevBsDeviceIndex			INTEGER,

wranDevBsVendorId				OCTET STRING,

wranDevBsHwId				OCTET STRING,

wranDevBsCurrentSwVersion		OCTET STRING,

wranDevBsDownloadSwVersion		OCTET STRING,

wranDevBsUpgradeFileName		OCTET STRING,

wranDevBsSoftwareUpdgradeAdminState	INTEGER,

wranDevBsDownloadSwProgress		INTEGER,

wranDevBsSoftwareUpgradeTimeStamp	DateAndTime }

wranDevBsDeviceIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..10)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Index of entry into table."

::= { wranDevBsSoftwareUpgradeEntry 1 }

wranDevBsVendorId		OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(2..256))

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"This value identifiers the managed BS vendor to which SW upgrade

was applied."

::= { wranDevBsSoftwareUpgradeEntry 2 }

wranDevBsHwId		OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(2..256))

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"Version of HW that SW upgrade is applied to."

::= { wranDevBsSoftwareUpgradeEntry 3 }

wranDevBsCurrentSwVersion	OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(2..256))

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"Version of SW that is currently running on the BS. This

value is set by the vendor specified by the Vendor ID. The

SW version and HW ID (wranDevBsHwId) should be a unique

Tuple. After the downloaded softare is activated, the

value in this object shall value of this object shall be

replaced with the version in wranDevBsDownloadSwVersion."

::= { wranDevBsSoftwareUpgradeEntry 4 }

wranDevBsDownloadSwVersion	OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(2..256))

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"Version of the SW to be downloaded. This value is set by the

vendor specified by the Vendor ID. The SW version and  HOW ID

(wranDevBsHwId) should be a unique type. This should be

initialized before software is downloaded or  activated."

::= { wranDevBsSoftwareUpgradeEntry 5 }

wranDevBsUpgradeFileName	OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(2..256))

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"Fully qualified path name that points to the location of the SW

version that is to be downloaded/activated."

::= { wranDevBsSoftwareUpgradeEntry 6 }

wranDevBsSoftwareUpgradeAdminState	OBJECT-TYPE

SYNTAX		INTEGER {null(0), download(1) activate(2)}

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"This value can take on two states. When set to Download, the

software listed by wranDevBsDownloadSwVersion will be

downloaded. When set to Activate, the software recently

downloaded will be activated. The Download and Activate

procedures are vendor specific operations that are not defined

in this standard."

DEFVAL	{ null }

::= { wranDevBsSoftwareUpgradeEntry 7 }

wranDevBsDownloadSwProgress	OBJECT-TYPE

SYNTAX		INTEGER (0..100)

UNITS			"%"

MAC-ACCESS		read-only

STATUS		current

DESCRIPTION

"This value shows the progresss of the SW download

highlighted by

wranDevBsDownloadSwVersion, encoded as the

perecentage of the

download successfully completed."

::= { wranDevBsSoftwareUpgradeEntry 8 }

wranDevBsSoftwareUpgradeTimeStamp	OBJECT-TYPE

SYNTAX		DateAndTime

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"This value is a timestamp to indicate when the last SW

download

or activation took place."

::= { wranDevBsSoftwareUpgradeEntry 9 }

-- wranDevBsNotification: Managed Objects related to SNMP traps on BS

wranDevBsNotification	OBJECT IDENTIFIER ::= { wranDevBsObjects 2 }

wranDevBsTrapControl	OBJECT IDENTIFIER ::= { wranDevBsNotification 1 }

wranDevBsTrapControl	OBJECT-TYPE

SYNTAX	BITS	{ wranDevBsEvent(0),

wranDevBsLogBuffExceedThreshold(1) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Defines control elements for traps. This is a 2bit field that

enables setting a trap when a BS event is logged

(wranDevBsEvent) or to indicate when a event log buffer

size overruns configured threshold

(wranDevBsLogBuffExceedThreshold)."

::= { wranDevBsNotification 1 }

-- wranDevBsTrapDefinition: This object groups all of the notification objects for

-- the BS. It is defined to be compatible with SNMPv1, following section 8.5 and

-- 8.6 of RFC 2758.

wranDevBsTrapDefinition			OBJECT IDENTIFIER

::= { wranDevBsNotification 2 }

wranDevBsEventTrap			OBJECT IDENTIFIER

::= { wranDevBsTrapDefinition 1 }

wranDevBsLogBuffExceedThresholdTrap	OBJECT IDENTIFER

::= { wranDevBsTrapDefinition 2 }

wranDevBsEventTrap	NOTIFICATION-TYPE

OBJECTS	{ wranDevCmnEventId,

wranDevCmnEventLogIndex,

wranDevCmnEventLoggedTime,

wranDevCmnEventDescription,

wranDevCmnEventSeverity }

STATUS	current

DESCRIPTION

"This object is a compound object that contains information the

objects in wranDevCmnEventEntry in wranDevCmnEventLogTable that

describes the event that is logged. This trap is caught when

wranDevBsEvent in wranDevBsTrapControl is set."

::= { wranDevBsTrapDefinition 1 }

wranDevBsLogBuffExceedThresholdTrap	NOTIFICATION-TYPE

OBJECTS	{ wranDevCmnEventId,

wranDevCmnEventLogResidualBuffThreshold }

STATUS	current

DESCRIPTION

"This object is a compound object that indicates the index of an

entry (defined by wranDevCmnEventLogEntry) in

wranDevCmnEventLogConfigTable, and the object in that entry that

defines the ratio (defined by

wranDevCmnEventLogResidualBuffThreshold) of used capacity in the

Event Log vs. total capacity. This trap is caught when

wranDevBsLogBuffExceedThreshold in wranDevBsTrapControl is set."

::= { wranDevBsTrapDefinition 2 }

wranDevCpeObjects				OBJECT IDENTIFIER

::= { wranDevMib 2 }

wranDevCpeConfigFileEncodingTable	OBJECT IDENTIFIER

::= { wranDevCpeObjects 1 }

wranDevCpeConfigFileEncodingTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranDevCpeFileEncodingEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This table defines objects associated with CPE software

configuration. This table may only have one entry, defined by a

compound attribute, wranDevCpeConfigFileEncodingEntry."

::= { wranDevCpeObjects 1 }

wranDevCpeConfigFileEncodingEntry	OBJECT-TYPE

SYNTAX	wranDevCpeConfigFileEncodingEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Definition of an entry into wranDevBsSoftwareUpgradeTable. Each

entry is identified by wranDevBsDeviceIndex."

INDEX	{ wranDevCpeDeviceIndex }

::= { wranDevCpeConfigFileEncodingTable 1 }

wranDevCpeConfigFileEncodingEntry ::= SEQUENCE {

wranDevCpeDeviceIndex			INTEGER,

wranDevCpeMicConfigSetting		OCTET STRING,

wranDevBsVendorId				OCTET STRING,

wranDevBsHwId				OCTET STRING,

wranDevCpeConfigFileVersion		OCTET STRING,

wranDevCpeUpgradeFileName		OCTET STRING,

wranDevCpeSwTftpServer			InetAddress,

wranDevCpeTftpServerTimeStamp		DateAndTime }

wranDevCpeDeviceIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Index of entry into table, defaults to 1."

::= { wranDevCpeConfigFileEncodingEntry 1 }

wranDevCpeMicConfigSetting	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(20))

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This value contains the MIC (Message Integrity Code) calculated

for the CPE configuration file."

::= { wranDevCpeConfigFileEncodingEntry 2 }

wranDevCpeVendorId		OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(2..256))

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"This value identifies the vendor of the managed CPE to which a

configuration file updgrade is to be applied."

::= { wranDevCpeConfigFileEncodingEntry 3 }

wranDevCpeHwId		OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(2..256))

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"This value identifies the hardware version of the CPE which the

configuration file upgrade is to be applied."

::= { wranDevCpeConfigFileEncodingEntry 4 }

wranDevCpeConfigFileVersion	OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(2..256))

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"Version of the configuration file to be downloaded. This value

is set by the vendor specified by the Vendor ID. The SW version

and HW ID (wranDevCpeHwId) should be a unique tuple. This should

be initialized before software is downloaded or activated."

::= { wranDevCpeConfigFileEncodingEntry 5 }

wranDevCpeUpgradeFileName	OBJECT-TYPE

SYNTAX		OCTET STRING (SIZE(2..256))

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"Fully qualified path name that points to the location of the SW

version that is to be downloaded/activated."

::= { wranDevCpeConfigFileEncodingEntry 6 }

wranDevCpeSwTftpServer	OBJECT-TYPE

SYNTAX		InetAddress

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"IP address of the TFTP server on which the new configuration

file resides."

::= { wranDevCpeConfigFileEncodingEntry 7 }

wranDevCpeTftpServerTimeStamp	OBJECT-TYPE

SYNTAX		DateAndTime

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"The time the configuration file was in seconds, as defined by

IETF RFC 868."

::= { wranDevCpeConfigFileEncodingEntry 8 }

-- wranDevCpeNotification: This group of objects relate to SNMP traps on the CPE.

-- There is a control element that enables/disables the traps

-- (wranDevCpeTrapControl) and what event information is logged when a trap is sent

-- (wranDevCpeTrapDefinition).

wranDevCpeNotification	 OBJECT IDENTIFIER ::=  { wranDevCpeObjects 2 }

wranDevCpeTrapControl	 OBJECT IDENTIFIER ::=  { wranDevCpeNotification 1 }

wranDevCpeTrapControl	OBJECT-TYPE

SYNTAX	BITS	{ wranDevCpeEvent(0),

wranDevCpeLogBuffExceedThreshold(1) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Defines control elements for traps. This is a 2bit field that

enables setting a trap when a CPE event is logged

(wranDevBsEvent) or to indicate when a event log buffer

size overruns configured threshold

(wranDevCpeLogBuffExceedThreshold)."

::= { wranDevCpeNotification 1 }

-- wranDevCpeTrapDefinition: This object groups all of the notification objects for

-- the CPE. It is defined to be compatible with SNMPv1, following Section 8.5 and

-- 8.6 RFC 258. It's a compound objet made of the elements of a logged BS event

-- (wranDevCpeEventTrap) and the indication of when the ratio used portion to total

-- size of event log has been exceeded (wranDevCpeLogBuffExceedThresholdTrap).

wranDevCpeTrapDefinition 		OBJECT IDENTIFIER

::= { wranDevCpeNotification 2 }

wranDevCpeEventTrap			OBJECT IDENTIFIER

::= { wranDevCpeTrapDefinition 1 }

wranDevCpeLogBuffExceedThresholdTrap	OBJECT IDENTIFER ::=

{ wranDevCpeTrapDefinition 2 }

wranDevCpeEventTrap	NOTIFICATION-TYPE

OBJECTS	{ wranDevCmnEventId,

wranDevCmnEventLogIndex,

wranDevCmnEventLoggedTime,

wranDevCmnEventDescription,

wranDevCmnEventSeverity }

STATUS	current

DESCRIPTION

"This object is a compound object that contains information the

objects in wranDevCmnEventEntry in wranDevCmnEventLogTable that

describes the event that is logged. This trap is caught when

wranDevCpeEvent in wranDevCpeTrapControl is set."

::= { wranDevCpeTrapDefinition 1 }

wranDevCpeLogBuffExceedThresholdTrap	NOTIFICATION-TYPE

OBJECTS	{ wranDevCmnEventId,

wranDevCmnEventLogResidualBuffThreshold }

STATUS	current

DESCRIPTION

"This object is a compound object that indicates the index of an

entry (defined by wranDevCmnEventLogEntry) in

wranDevCmnEventLogConfigTable, and the object in that entry that

defines the ratio (defined by

wranDevCmnEventLogResidualBuffThreshold) of used capacity in the

Event Log vs. total capacity. This trap is caught when

wranDevCpeLogBuffExceedThreshold in wranDevCpeTrapControl is

set."

::= { wranDevCpeTrapDefinition 2 }

-- wranDevCommonObjects: This object contains managed elements that are common to

-- the CPE and BS.

wranDevCommonObjects	OBJECT IDENTIFIER ::= { wranDevMib 3 }

wranDevCmnEventLog	OBJECT IDENTIFIER ::= { wranDevCommonObjects 1 }

wranDevCmnSnmpAgent	OBJECT IDENTIFIER ::= { wranDevCommonObjects 2 }

wranDevCmnDeviceConfig	OBJECT IDENTIFER ::= { wranDevCommonObjects 3 }

--wranDevCmnEvent log is comprised of three tables that control the configuration

-- of event recording and store recorded events.

wranDevCmnEventLog		OBJECT IDENTIFIER

::= { wranDevCommonObjects 1 }

wranDevCmnEventLogConfigTable OBJECT IDENTIFIER

::= { wranDevCmnEventLog 1 }

wranDevCmnEventTable		OBJECT IDENTIFIER

::= { wranDevCmnEventLog 2 }

wranDevCmnEventLogTable		OBJECT IDENTIFIER

::= { wranDevCmnEventLog 3 }

wranDevCmnEventLogConfigTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranDevCmnEventLogConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Each entry stores the Event Log configuration for a device."

::= { wranDevCmnEventLog 1 }

wranDevCmnEventLogConfigEntry	OBJECT-TYPE

SYNTAX	wranDevCmnEventLogConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Definition of an entry into wranDevCmnEventLogConfigTable. Each

entry is identified by wranDevCmnDeviceIndex."

INDEX	{ wranDevCmnDeviceIndex }

::= { wranDevCmnEventLogConfigTable 1 }

wranDevCmnEventLogConfigEntry::= SEQUENCE {

wranDevCmnDeviceIndex				INTEGER,

wranDevCmnEventLogEntryLimit			INTEGER,

wranDevCmnEventLogLifeTimeLimit		INTEGER,

wranDevCmnEventLogEntryLimitPerEventId	INTEGER,

wranDevCmnEventLogSeverityThreshold		INTEGER,

wranDevCmnEVentLogWrapAroundBuffEnable	TruthEnable,

wranDevCmnEventLogLatestEvent			Unsigned32,

wranDevCmnEventLogPersistenceSupported	TruthEnable,

wranDevCmnEventLogResidualBuffThreshold	INTEGER }

wranDevCmnDeviceIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index value that identifies a BS or CPE entry in the

wranDevCmnEventLogConfigTable."

::= { wranDevCmnEventLogConfigEntry 1 }

wranDevCmnEventLogEntryLimit	OBJECT-TYPE

SYNTAX	INTEGER (1..10000)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximum number of entries in wranDevCmnEventLogConfigTable. If

this value is changed while entries exist in

wranDevCmnEventLogTable, old entries will be discarded until

limit is reached."

DEFVAL	{ 100 }

::= { wranDevCmnEventLogConfigEntry 2 }

wranDevCmnEventLogLifeTimeLimit	OBJECT-TYPE

SYNTAX	INTEGER (1..10000)

UNITS		"minutes"

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"A value of 0 means that an entry is kept indefinitely. Any other

value, it’s the maximum time an entry can exist in

wranDevCmnEventLogTable. If this value is changed while exist in

wranDevCmnEventLogTable, entries older that this limit will be

discarded."

DEFVAL	{ 1440 }

::= { wranDevCmnEventLogConfigEntry 3 }

wranDevCmnEventLogEntryLimitPerEventId	OBJECT-TYPE

SYNTAX	INTEGER (1..100)

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"The number of log entries that can be logged per event."

DEFVAL	{ 10 }

::= { wranDevCmnEventLogConfigEntry 4 }

wranDevCmnEventLogSeverityThreshold	OBJECT-TYPE

SYNTAX		INTEGER (emergency(1),

alert(2),

critical(3),

error(4),

warning(5),

notice(6),

informational(7),

debug(8)}

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"Version of the configuration file to be downloaded. This value

is set by the vendor specified by the Vendor ID. The SW version

and HW ID (wranDevCpeHwId) should be a unique tuple. This should

be initialized before software is downloaded or activated."

DEFVAL	{ warning }

::= { wranDevCpeConfigFileEncodingEntry 5 }

wranDevCmnEventLogWrapAroundBuffEnable	OBJECT-TYPE

SYNTAX	TruthEnable

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"True (1), indicates that the log buffer will be wrapped around

with the buffer is full."

DEFVAL	{ true }

::= { wranDevCpeConfigFileEncodingEntry 6 }

wranDevCmnEventLogLatestEvent	OBJECT-TYPE

SYNTAX	Unsigned32 (1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of latest event in Event Log."

DEFVAL	{ 1 }

::= { wranDevCpeConfigFileEncodingEntry 7 }

wranDevCmnEventLogPersistenceSupported	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"True (1), indicates that the Event Log persisted through power

cycle and reset."

::= { wranDevCpeConfigFileEncodingEntry 8 }

wranDevCmnEventLogResidualBuffThreshold	OBJECT-TYPE

SYNTAX	INTEGER (1..100)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Threshold ratio of used capacity for Event Log to total capacity

of Event Log, that when reached a TRAP is issued."

::= { wranDevCpeConfigFileEncodingEntry 9 }

wranDevCmnEventTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranDevCmnEventEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This compound object defines the types of events that are

supported by a BS or CPE. Each event is defined."

::= { wranDevCmnEventLog 2 }

wranDevCmnEventEntry	OBJECT-TYPE

SYNTAX	wranDevCmnEventEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines the parameters of an event entry into the

wranDevCmnEventTable. Each entry is indexed by

wranDevCmnDeviceIndex and wranDevCmnEventIdentifier."

INDEX	{ wranDevCmnDeviceIndex, wranDevCmnEventIdentifier }

::= { wranDevCmnEventTable 1 }

wranDevCmnEventEntry::= SEQUENCE {

wranDevCmnEventIdentifier			INTEGER,

wranDevCmnEventDescription			SnmpAdminString,

wranDevCmnEventSeverity				INTEGER,

wranDevCmnEventNotification			TruthValue,

wranDevCmnEventNotificationOid		OBJECT IDENTIFIER }

wranDevCmnEventIdentifier	OBJECT-TYPE

SYNTAX	INTEGER (0..100000)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Event Identifier encoded as a numerica value."

::= { wranDevCmnEventEntry 1 }

wranDevCmnEventDescription	OBJECT-TYPE

SYNTAX	SnmpAdminString

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Description of the event in the form of an SnmpAdminString"

::= { wranDevCmnEventEntry 2 }

wranDevCmnEventSeverity	OBJECT-TYPE

SYNTAX		INTEGER {emergency(1),

alert(2),

critical(3),

error(4),

warning(5),

notice(6),

informational(7),

debug(8)}

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"The severity of the event as assigned by the device. The

severity assigned to an event is configurable by the system."

DEFVAL	{ warning }

::= { wranDevCmnEventEntry 3 }

wranDevCmnEventNotification	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"When True (1), an event notification will be reported."

DEFVAL	{ false }

::= { wranDevCmnEventEntry 4 }

wranDevCmnEventNotificationOid	OBJECT-TYPE

SYNTAX		OBJECT IDENTIFIER

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"This is the object identifier of a notification object. If

wranDevCmnEVentNotificate True (1), a trap identified by the OID

will be reported."

::= { wranDevCmnEventEntry 5 }

wranDevCmnEventLogTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranDevCmnEventLogEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This is the log table that stores local events as they happen.

This table shall reside in non-volatile memory that may persist

after power cycle or resent of the device. The maximum number of

entries in this table is determined by the

wranDevCmnEventLogEntryLimit. If it is setup as a wrap-around

log, then the oldest entry will be remove to make room for the

newest entry. If it is not set up as a wrap-around log, then the

log will be flushed. Mulitple entries are stored in the table.

Each entry is defined by wranDevCmnEventLogEntry."

::= { wranDevCmnEventLog 3 }

wranDevCmnEventLogEntry	OBJECT-TYPE

SYNTAX	wranDevCmnEventLogEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object represents an entry in wranDevCmnEventLogTable. It

is indexed by wranDevCmnDeviceIndex, and wranDevCmnEventId."

INDEX	{ wranDevCmnDeviceIndex, wranDevCmnEventId }

::= { wranDevCmnEventLogTable 1 }

wranDevCmnEventLogEntry::= SEQUENCE {

wranDevCmnEventId			Unsigned32,

wranDevCmnEventLoggedTime	TimeStamp,

wranDevCmnEventLogDescription	SnmpAdminString,

wranDevCmnEventLogSeverity	INTEGER }

wranDevCmnEventId	OBJECT-TYPE

SYNTAX	Unsigned32(1..4294967295)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"A counter used to index entires in the Event Log. When it

reaches a maximum value, it will wrap-arounnd if configured to

wrap-around or the log will be flushed if it is not configured

to wrap-around."

::= { wranDevCmnEventLogEntry 1 }

wranDevCmnEventLoggedTime	OBJECT-TYPE

SYNTAX	TimeStamp

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The time that the entry was placed into the Event Log. If this

event happened just before the last initialization of the

management system, then this value is set to 0."

::= { wranDevCmnEventLogEntry 2 }

wranDevCmnEventLogDescription	OBJECT-TYPE

SYNTAX	SnmpAdminString

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"Description of the event, in the form of an SnmpAdminString"

::= { wranDevCmnEventLogEntry 3 }

wranDevCmnEventLogSeverity	OBJECT-TYPE

SYNTAX		INTEGER {emergency(1),

alert(2),

critical(3),

error(4),

warning(5),

notice(6),

informational(7),

debug(8)}

MAX-ACCESS		read-only

STATUS		current

DESCRIPTION

"Severity of the recorded event."

DEFVAL	{ warning }

::= { wranDevCmnEventLogEntry 4 }

-- wranDevCmnSnmpAgent: This object is comprised of one table that deals with SNMP

-- agent configuration.

wranDevCmnSnmpAgent			OBJECT IDENTIFIER

::= { wranDevCommonObjects 2 }

wranDevCmnSnmpV1V2TrapDestTable		OBJECT IDENTIFIER

::=

{ wranDevCmnSnmpAgent 1 }

wranDevCmnSnmpV1V2TrapDestTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranDevCmnSnmpV1V2TrapDestEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This compound object deals with the configuration of items of

the SNMP agent. Each configuration item is represented by

wranDevCmnSnmpV1V2TrapDestEntry."

::= { wranDevCmnSnmpAgent 1 }

wranDevCmnSnmpV1V2TrapDestEntry	OBJECT-TYPE

SYNTAX	wranDevCmnSnmpV1V2TrapDestEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"The compound object contains the parameters that identify the

desitnation of an SNMP trap."

INDEX	{ wranDevCmnSnmpV1V1TrapDestIndex }

::= { wranDevCmnSnmpV1V2TrapDestTable 1 }

wranDevCmnSnmpV1V2TrapDestEntry::= SEQUENCE {

wranDevCmnSnmpV1V1TrapDestIndex		Unsigned32,

wranDevCmnSnmpV1V2TrapDestIpAddrType	InetAddressType,

wranDevCmnSnmpV1V2TrapDestIpAddr		InetAddress,

wranDevCmnSnmpV1V2TrapDestPort		Integer32,

wranDevCmnSnmpV1V2TrapDestRowStatus		RowStatus }

wranDevCmnSnmpV1V1TrapDestIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..8)

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Identifies the trap in the table. This parameter shall have a

maximum value of 8."

::= { wranDevCmnSnmpV1V2TrapDestEntry 1 }

wranDevCmnSnmpV1V2TrapDestIpAddrType	OBJECT-TYPE

SYNTAX	InetAddressType

MAX-ACCESS	read-create

STATUS	current

DESCRIPTION

"Type of IP address stored in wranDevCmnSnmpV1V2TrapDestIpAddr."

::= { wranDevCmnSnmpV1V2TrapDestEntry 2 }

wranDevCmnSnmpV1V2TrapDestIpAddr	OBJECT-TYPE

SYNTAX		InetAddress

MAX-ACCESS		read-create

STATUS		current

DESCRIPTION

"SNMP manager’s IP address that is configured as a destination

for traps."

::= { wranDevCmnSnmpV1V2TrapDestEntry 3 }

wranDevCmnSnmpV1V2TrapDestPort	OBJECT-TYPE

SYNTAX		Integer32 (0..65535)

MAX-ACCESS		read-create

STATUS		current

DESCRIPTION

"Port number of SNMP manager application configured as trap

destination."

::= { wranDevCmnSnmpV1V2TrapDestEntry 4 }

wranDevCmnSnmpV1V2TrapDestRowStatus	OBJECT-TYPE

SYNTAX		RowStatus

MAX-ACCESS		read-create

STATUS		current

DESCRIPTION

"This object is used to make sure that any write operation to

multiple columns is treated as an atomic operation."

::= { wranDevCmnSnmpV1V2TrapDestEntry 5 }

wranDevCmnDeviceConfig	OBJECT IDENTIFER

::= { wranDevCommonObjects 3 }

wranDevCmnResetDevice	OBJECT IDENTIFIER

::=

{ wranDevCmnDeviceConfig 1 }

wranDevMibGroups		OBJECT IDENTIFIER

::=

{ wranDevCmnDeviceConfig 2 }

wranDevMibCompliance	OBJECT IDENTIFIER

::= { wranDevCmnDeviceConfig 3 }

wranDevCmnResetDevice	OBJECT-TYPE

SYNTAX	INTEGER { actionResetDeviceNoAction(0),

actionResetDevice(1) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"There are two actions defiend. When set to

actionResetDeviceNoAction, no action is taken. When set to

actionResetDevice, device wil reset itself."

::= { wranDevCmnDeviceConfig 1 }

-- wranDevMibGroups: This object helps define which MIB groups are necessary to

-- meet conformance and what MIB objects are part of each group.

wranDevMibGroups				OBJECT IDENTIFIER

::= { wranDevCmnDeviceConfig 2 }

wranDevMibBsGroup				OBJECT IDENTIFIER

::= { wranDevMibGroups 1 }

wranDevMibBsSwUpgradeGroup		OBJECT IDENTIFIER

::= { wranDevMibGroups 2 }

wranDevMibCpeGroup			OBJECT IDENTIFIER

::= { wranDevMibGroups 3 }

wranDevMibCmnGroup			OBJECT IDENTIFIER

::= { wranDevMibGroups 4 }

wranDevMibBsNotificationGroup		OBJECT IDENTIFIER

::= { wranDevMibGroups 5 }

wranDevMibCpeNotificationGroup	OBJECT IDENTIFIER

::= { wranDevMibGroups 6 }

wranDevMibBsGroup	OBJECT-GROUP

OBJECTS	{ wranDevBsTrapControl }

STATUS	current

DESCRIPTION

"Group of objects for the BS."

::= { wranDevMibGroups 1 }

wranDevMibBsSwUpgradeGroup	OBJECT-GROUP

OBJECTS	{ wranDevBsVendorId, wranDevBsHwId,

wranDevBsCurrentSwVersion, wranDevBsUpgradeFileName,

wranDevBsSoftwareUpgradeAdminState,

wranDevBsDownloadSwProgress,

wranDevBsSoftwareUpgradeAdminState,

wranDevBsDownloadSwProgress,

wranDevBsSoftwareUpgradeTimeStamp }

STATUS	current

DESCRIPTION

"This group contains the values of the most recent/current entry

in wranDevBsSoftwareUpgradeTable."

::= { wranDevMibGroups 2 }

wranDevMibCpeGroup	OBJECT-GROUP

OBJECTS	{ wranDevCpeMicConfigSetting, wranDevCpeVendorId,

wranDevCpeHwId, wranDevCpeConfigFileVersion,

wranDevCpeUpgradeFileName, wranDevCpeSwTftpServer,

wranDevCpeTftpServertTimeStamp, wranDevCpeTrapControl }

STATUS	current

DESCRIPTION

"This group contains values of the most recent/current entry in

wranDevCpeConfigFileEncodingTable for a particular CPE."

::= { wranDevMibGroups 3 }

wranDevMibCmnGroup	OBJECT-GROUP

OBJECTS	{ wranDevCmnSnmpV1V2TrapDestIpAddrType,

wranDevCmnSnmpV1V2TrapDestIpAddr,

wranDevCmnSnmpV1V2TrapDestPort,

wranDevCmnSnmpV1V2TrapDestRowStatus, wranDevCmnResetDevice,

wranDevCmnDeviceIndex, wranDevCmnEventLogEntryLimit, 						    wranDevCmnEventLogLifeTimeLimit,

wranDevCmnEventLogEntryLimitPerEventId,

wranDevCmnEventLogSeverityThreshold,

wranDevCmnEventLogWrapAroundBuffEnable,

wranDevCmnEventLogLatestEvent,

wranDevCmnEventLogPersistenceSupported,

wranDevCmnEventLogResidualBuffThreshold,

wranDevCmnEventDescription, wranDevCmnEventSeverity,

wranDevCmnEventNotification, wranDevCmnEventNotificationOid,

wranDevCmnEventId, wranDevCmnEventLoggedTime,

wranDevCmnEventLogDescription, wranDevCmnEventLogSeverity }

STATUS	current

DESCRIPTION

"It contains the values of most recent/current entry in

wranDevVMnSnmpV1V2TrapDestTable and wranDevCmnEventLogTable."

::= { wranDevMibGroups 4 }

wranDevMibBsNotificationGroup	OBJECT-GROUP

OBJECTS	{ wranDevBsEventTrap,

wranDevBsLogBuffExceedThresholdTrap }

STATUS	current

DESCRIPTION

"Contains the contents of wranDevBsEventTrap and

wranDevBsLogBuffExceedThresholdTrap."

::= { wranDevMibGroups 5 }

wranDevMibCpeNotificationGroup	OBJECT-GROUP

OBJECTS	{ wranDevCpeEventTrap,

wranDevCpeLogBuffExceedThresholdTrap }

STATUS	current

DESCRIPTION

"Contains the contents of wranDevCpeEventTrap and

wranDevCpeLogBuffExceedThresholdTrap."

::= { wranDevMibGroups 6 }

wranDevMibCompliance	MODULE-COMPLIANCE

STATUS	current

DESCRIPTION

"MIB object groups that are optional and mandatory for

conformance."

MODULE	wranDevMib

MANDATORY-GROUPS	{ wranDevMibBsGroup, wranDevMibBsSwUpgradeGroup,

wranDevMibCpeGroup, wranDevMibCmnGroup }

-- OPTIONAL-GROUPS	{ wranDevMibBsNotificationGroup,

wranDevMibCpeNotificationGroup }

::= { wranDevCmnDeviceConfig 3 }

END

    - wranIfBsMib

    - wranIfBsSfMgmt

    - wranIfCpeMib

IEEE802dot22-WRAN-IF-CPE-MIB	::=	BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64, transmission

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVENTION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPV2-CONF

wranIfCpeMib	MODULE-IDENTITY

LAST-UPDATED	"201205140000Z"	-- May 14, 2012

ORGANIZATION	"IEEE 802.22"

CONTACT-INFO

"WG E-mail:	STDS-802-22@LISTSERV.IEEE.ORG

WG Chair:	Apurva N. Mody

E-mail:	apurva.mody@ieee.org

TGa Chair/Editor:  Ranga Reddy

E-mail:		  ranga.reddy@ieee.org"

DESCRIPTION

"This material is from IEEE 802.22a-2012 Copyright (c) 2012. This

MIB Module Defines managed objects for Customer Premise

Equipment based on IEEE 802.22-2011 and is under

iso(1).std(0).iso8802(8802).wran(22).wranIfCpeMib(4)"

REVISION		"201205140000Z"

DESCRIPTION

"The first version of IEEE802dot22-WRAN-IF-CPE-MIB."

::= {iso std(0) iso8802(8802) wran(22) 4}

wranIfCpeConfigurationTable		OBJECT IDENTIFIER ::= { wranIfCpeMib 1 }

wranIfCpeTrapControl			OBJECT IDENTIFIER ::= { wranIfCpeMib 2 }

wranIfCpeThresholdConfigTable		OBJECT IDENTIFIER ::= { wranIfCpeMib 3 }

wranIfCpeTrapDefinition			OBJECT IDENTIFIER ::= { wranIfCpeMib 4 }

wranIfCpeNotificationObjectsTable	OBJECT IDENTIFIER ::= { wranIfCpeMib 5 }

wranIfCpeMibGroups			OBJECT IDENTIFIER ::= { wranIfCpeMib 6 }

wranIfCpeMibCompliance			OBJECT IDENTIFIER ::= { wranIfCpeMib 7 }

wranIfCpeConfigurationTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfCpeConfigurationEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB provides a table that provides the values for system

parameters. The parameters in this table are applied to all CPEs

in the network, hence there is on entry in the table, defined by

wranIfCpeConfigurationEntry."

::= { wranIfCpeMib 1 }

wranIfCpeConfigurationEntry	OBJECT-TYPE

SYNTAX	wranIfCpeConfigurationEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Definition of an entry into wranIfCpeConfigurationTable. There

is only one entry in each CPE."

::= { wranIfCpeConfigurationTable 1 }

wranIfCpeConfigurationEntry	::= SEQUENCE {

wranIfCpeConfigurationIndex		INTEGER,

wranIfCpeLostDsMapInterval		Integer32,

wranIfCpeLostUsMapInterval		Integer32,

wranIfCpeContentionRangingRetries	Integer32,

wranIfCpeContentionBwRetries		Integer32,

wranIfCpeRegReqRetries			INTEGER,

wranIfCpeTftpBackoffStart		Integer32,

wranIfCpeTftpBackoffEnd			Integer32,

wranIfCpeTftpReqRetries			Integer32,

wranIfCpeTftpDownloadRetries		Integer32,

wranIfCpeTftpWait				Integer32,

wranIfCpeToDRetries			Integer32,

wranIfCpeToDRetryPeriod			Integer32

wranIfCpeCBCReqRetries			INTEGER,

wranIfCpeTftpCpltRetries		INTEGER,

wranIfCpeInvitedRangRetries		Integer32,

wranIfCpeDSxReqRetries			INTEGER,

wranIfCpeDSxRspRetries			INTEGER }

wranIfCpeConfigurationIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry into this table, defaults to 1."

::= { wranIfCpeConfigurationEntry 1 }

wranIfCpeLostDsMapInterval	OBJECT-TYPE

SYNTAX	Integer32 (0..600)

UNITS		"milliseconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Amount of time (in milliseconds) since reception of last DS-MAP

before DS synchronization was lost."

::= { wranIfCpeConfigurationEntry 2 }

wranIfCpeLostUsMapInterval		OBJECT-TYPE

SYNTAX		Integer32 (0..600)

UNITS		"milliseconds"

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"

Amount of time (in milliseconds) since reception of last US-MAP

before US synchronization is considered lost."

::= { wranIfCpeConfigurationEntry 3 }

wranIfCpeContentionRangingRetries			OBJECT-TYPE

SYNTAX		Integer32 (16..65535)

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"Number of retries on contention ranging requests."

::= { wranIfCpeConfigurationEntry 4 }

wranIfCpeContentionBwRetries	OBJECT-TYPE

SYNTAX		Integer32 (16..65535)

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"

Number of retries on contention bandwidth requests."

::= { wranIfCpeConfigurationEntry 5 }

wranIfCpeRegReqRetries	OBJECT-TYPE

SYNTAX		INTEGER (1..255)

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"

Maximum number of retries allowed for registration requests."

::= { wranIfCpeConfigurationEntry 6 }

wranIfCpeTftpBackoffStart	OBJECT-TYPE

SYNTAX		Integer32 (1..65535)

UNITS			"seconds"

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"

Initial backoff for TFTP Start."

::= { wranIfCpeConfigurationEntry 7 }

wranIfCpeTftpBackoffEnd	OBJECT-TYPE

SYNTAX		Integer32 (1..65535)

UNITS		"seconds"

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"Last value for TFTP backoff.This value can take on two states. When set to Download, the

"

::= { wranIfCpeConfigurationEntry 8 }

wranIfCpeTftpReqRetries	OBJECT-TYPE

SYNTAX		Integer32 (16..65535)

""

MAC-ACCESS		read-write

STATUS		current

DESCRIPTION

"Maximum number of retires allowed for attempting

TFTP to get CPE configuration.						"

::= { wranIfCpeConfigurationEntry 9 }

wranIfCpeTftpDownloadRetries	OBJECT-TYPE

SYNTAX		Integer32 (3..65535)

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"			Maximum number of retries allowed for re-attempting TFTP (after

failed/corrupted download) to get CPE configuration."

::= { wranIfCpeConfigurationEntry 10 }

wranIfCpeTftpWait	OBJECT-TYPE

SYNTAX		Integer32 (2..65535)

UNITS			"mintues"

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"Time to wait before consecutive attempts to obtain configuration

via TFTP."

::= { wranIfCpeConfigurationEntry 11 }

wranIfCpeToDRetries	OBJECT-TYPE

SYNTAX		Integer32 (3..65535)

MAX-ACCESS		read-write

STATUS		current

DESCRIPTION

"Number of retries allowed for establishing Time of Day, per

network entry procedures."

::= { wranIfCpeConfigurationEntry 12 }

wranIfCpeToDRetryPeriod	OBJECT-TYPE

SYNTAX	Integer32 (5..65535)

UNITS		"mintues"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Amount of time to wait before retrying establishment of time of

day after failed attempt."

::= { wranIfCpeConfigurationEntry 13 }

wranIfCpeCBCReqRetries	OBJECT-TYPE

SYNTAX	INTEGER (3..16)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximum number of retries for sending CBC request."

DEFVAL { 3 }

::= { wranIfCpeConfigurationEntry 14 }

wranIfCpeTftpCpltRetries	OBJECT-TYPE

SYNTAX	INTEGER (3..16)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximum number of retries allowed for sending TFTP-CPLT messages

to BS."

::= { wranIfCpeConfigurationEntry 15 }

""

"

"

""

"

"

wranIfCpeInvitedRangRetries	OBJECT-TYPE

SYNTAX	Integer32 (16..65535)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximumtime number of retries on invited ranging requets."

::= { wranIfCpeConfigurationEntry 16 }

wranIfCpeDSxReqRetries	OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Number of timeout retries on DSx-REQ."

DEFVAL { 3 }

::= { wranIfCpeConfigurationEntry 17 }

wranIfCpeDSxRspRetries	OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Number of timeout retries on DSx-RSP."

DEFVAL { 3 }

::= { wranIfCpeConfigurationEntry 17 }

wranIfCpeTrapControl	OBJECT-TYPE

SYNTAX	BITS	{ wranIfCpeDhcpSuccess(0),

wranIfCpeRssiStatusChange(1),

wranIfCpeEirpPerScStatusChange(2),

wranIfCpeMaxEirpStatusChange(3),

wranIfCpeScmStateChange(4) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Defines control elements for traps related to CPE operation.

This is a 5bit field that enables setting a trap for paricular

CPE events: wranIfCpeDhcpSuccess, wranIfCpeRssiStatusChange,

wranIfCpeEirpPerScStatusChange, wranIfCpeMaxEirpStatusChange and

wranIfCpeScmIdleStateChange."

::= { wranIfCpeMib 2 }

wranIfCpeThresholdConfigTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfCpeThresholdConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB provides a table that allows the setting fo thresholds

that can be used to detect the crossing of RSSI and EIRP

thresholds. Each table is made of one entry defined by

wranIfCpeThresholdConfigEntry."

::= { wranIfCpeMib 3 }

wranIfCpeThresholdConfigEntry	OBJECT-TYPE

SYNTAX	wranIfCpeThresholdConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Definition of an entry into wranIfCpeConfigurationTable. There

is only one entry in each CPE."

::= { wranIfCpeThresholdConfigTable 1 }

wranIfCpeThresholdConfigEntry	::= SEQUENCE {

wranIfCpeThresholdConfigIndex		INTEGER,

wranIfCpeRssiLowThreshold		INTEGER,

wranIfCpeRssiHighThreshold		INTEGER,

wranIfCpeEirpPerScLowThreshold	INTEGER,

wranIfCpeEirpPerScHighThreshold	INTEGER,

wranIfCpeMaxEirpLowThreshold		INTEGER,

wranIfCpeMaxEirpHighThreshold		INTEGER }

wranIfCpeThresholdConfigIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry into this table, defaults to 1."

::= { wranIfCpeThresholdConfigEntry 1 }

wranIfCpeRssiLowThreshold	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

UNITS		"dBm"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Low threshold for RSSI alarm trap (between -104dBm - +23.5dBm in

0.5 dB steps)."

::= { wranIfCpeThresholdConfigEntry 2 }

wranIfCpeRssiHighThreshold	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

UNITS		"dBm"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"High threshold for RSSI alarm trap (between -104dBm - +23.5dBm

in 0.5 dB steps)."

::= { wranIfCpeThresholdConfigEntry 3 }

wranIfCpeEirpPerScLowThreshold	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

UNITS		"dBm"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Low threshold for EIRP per subcarrier alarm trap (between -

104dBm - +23.5dBm in 0.5 dB steps)."

::= { wranIfCpeThresholdConfigEntry 4 }

wranIfCpeEirpPerScHighThreshold	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

UNITS		"dBm"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"High threshold for EIRP per subcarrier alarm trap (between -

104dBm - +23.5dBm in 0.5 dB steps)."

::= { wranIfCpeThresholdConfigEntry 5 }

wranIfCpeMaxEirpLowThreshold	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

UNITS		"dBm"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Low threshold for maximum EIRP (over all 60 subchannels) _alarm

trap (between -64dBm - +63.5dBm in 0.5 dB steps)."

::= { wranIfCpeThresholdConfigEntry 6 }

wranIfCpeMaxEirpHighThreshold	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

UNITS		"dBm"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"High threshold for maximum EIRP (over all 60 subchannels) _alarm

trap (between -64dBm - +63.5dBm in 0.5 dB steps).

::= { wranIfCpeThresholdConfigEntry 7 }

-- wranIfCpeTrapDefinition: This MIB group specifies the definition of CPE traps

-- that can be enabled/disable in wranIfCpeTrapControl.

wranIfCpeTrapDefinition		OBJECT IDENTIFIER ::= { wranIfCpeMib 4 }

wranIfCpeRssiStatusChangeTrap	OBJECT IDENTIFIER

::= { wranIfCpeTrapDefinition 1 }

wranIfCpeEirpPerScStatusChangeTrap	OBJECT IDENTIFIER

::= { wranIfCpeTrapDefinition 2 }

wranIfCpeDhcpSuccessTrap	OBJECT IDENTIFIER

::= { wranIfCpeTrapDefinition 3 }

wranIfCpeScmStateChangeTrap	OBJECT IDENTIFIER

::= { wranIfCpeTrapDefinition 4 }

wranIfCpeMaxEirpStatusChangeTrap	OBJECT IDENTIFIER

::= { wranIfCpeTrapDefinition 5 }

wranIfCpeRssiStatusChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfCpeMacAddress,

wranIfCpeRssiStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the RSSI alarm

that's contained in wranIfCpeNotificationObjectsTable."

::= { wranIfCpeTrapDefinition 1 }

wranIfCpeEirpPerScStatusChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfCpeMacAddress,

wranIfCpeEirpPerScStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the EIRP per

subcarrier alarm that's contained in

wranIfCpeNotificationObjectsTable."

::= { wranIfCpeTrapDefinition 2 }

wranIfCpeDhcpSuccessTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfCpeMacAddress,

wranIfCpeDhcpStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the DHCP alarm

that's contained in wranIfCpeNotificationObjectsTable."

::= { wranIfCpeTrapDefinition 3 }

wranIfCpeScmStateChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfCpeMacAddress,

wranIfCpeScmStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the SCM state

change alarm that's contained in

wranIfCpeNotificationObjectsTable."

::= { wranIfCpeTrapDefinition 4 }

wranIfCpeMaxEirpStatusChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfCpeMacAddress,

wranIfCpeMaxEirpStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the maximum EIRP

(over all 60 subchannels) subcarrier alarm that's contained in

wranIfCpeNotificationObjectsTable."

::= { wranIfCpeTrapDefinition 5 }

wranIfCpeNotificationObjectsTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfCpeNotificationObjectsEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB provides a table to track notification objects that

have been reported by the traps on a particular CPE. There is

one entry in this table, defining the notification ojbects for

that particular CPE. Each entry is defined by

wranIfCpeNotificationObjectsEntry."

::= { wranIfCpeMib 5 }

wranIfCpeNotificationObjectsEntry	OBJECT-TYPE

SYNTAX	wranIfCpeNotificationObjectsEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Definition of an entry into wranIfCpeNotificationObjectsTable.

There is only one entry in each CPE."

::= { wranIfCpeNotificationObjectsTable 1 }

wranIfCpeNotificationObjectsEntry	::= SEQUENCE {

wranIfCpeNotificationMacAddress	MacAddress,

wranIfCpeRssiStatus			INTEGER,

wranIfCpeEirpPerScStatus		INTEGER,

wranIfCpeDhcpStatus			INTEGER,

wranIfCpeScmIdleStatus			INTEGER,

wranIfCpeMaxEirpStatus			INTEGER }

wranIfCpeNotificationMacAddress	OBJECT-TYPE

SYNTAX	MacAddress

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"MAC Address of the CPE generating the trap."

::= { wranIfCpeNotificationObjectsEntry 2 }

wranIfCpeRssiStatus	OBJECT-TYPE

SYNTAX	INTEGER { lowThresholdAlarm(0),

highThresholdAlarm(1),

noAlarm(2) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication if RSSI alarm was for an RSSI value below

wranIfCpeRssiLowThreshold, for an RSSI value above

wranIfCpeRssiHighThreshold, or the RSSI as returned to within

the bounds set by wranIfCpeRssiLowThreshold and

wranIfCpeRssiHighThreshold."

::= { wranIfCpeThresholdConfigEntry 2 }

wranIfCpeEirpPerScStatus	OBJECT-TYPE

SYNTAX	INTEGER { lowThresholdAlarm(0),

highThresholdAlarm(1),

noAlarm(2) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication if EIRP per subcarrier alarm was for an EIRP value

below wranIfCpeEirpPerScLowThreshold, for an EIRP value above

wranIfCpeEirpPerScHighThreshold, or the EIRP as returned to

within the bounds set by wranIfCpeEirpPerScLowThreshold and

wranIfCpeEirpPerScHighThreshold."

::= { wranIfCpeThresholdConfigEntry 3 }

wranIfCpeDhcpStatus	OBJECT-TYPE

SYNTAX	INTEGER { failure(0),

successful(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"An DHCP alarm is generated when a CPE is successfully able to

obtain an IP address or fails to obtain an IP address."

::= { wranIfCpeThresholdConfigEntry 4 }

wranIfCpeScmStatus	OBJECT-TYPE

SYNTAX	INTEGER { idle(0),

notAuthenticated(1),

authenticated(2),

reAuthWait(3) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"An SCM alarm is generated when a CPE changes to a different

state in the SCM Authentication state machine."

::= { wranIfCpeThresholdConfigEntry 5 }

wranIfCpeMaxEirpStatus	OBJECT-TYPE

SYNTAX	INTEGER { lowThresholdAlarm(0),

highThresholdAlarm(1),

noAlarm(2) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication if maximum EIRP (over all 60 subchannels) was for an

EIRP value below wranIfCpeMaxEirpLowThreshold, for an EIRP value

above wranIfCpeMaxEirpHighThreshold, or the EIRP as returned to

within the bounds set by wranIfCpeMaxEirpLowThreshold and

wranIfCpeMaxEirpHighThreshold."

::= { wranIfCpeThresholdConfigEntry 6 }

-- wranIfCpeMibGroups: This objects helps define which MIB groups are available

-- within this module, and which MIB objects are part of each group.

wranIfCpeMibGroups		OBJECT IDENTIFIER ::= { wranIfCpeMib 6 }

wranIfCpeMibConfigGroup		OBJECT IDENTIFIER ::= { wranIfCpeMibGroups 1 }

wranIfCpeMibNotifcationGroup	OBJECT IDENTIFIER ::= { wranIfCpeMibGroups 2 }

wranIfCpeMibConfigGroup		OBJECT-GROUP

OBJECTS	{ wranIfCpeLostDsMapInterval, wranIfCpeLostUsMapInterval,

wranIfCpeContentionRangingRetries,

wranIfCpeContentionBwRetries, wranIfCpeRegReqRetries,

wranIfCpeTftpBackoffStart, wranIfCpeTftpBackoffEnd,

wranIfCpeTftpReqRetries, wranIfCpeTftpDownloadRetries,

wranIfCpeTftpWait, wranIfCpeToDRetries,

wranIfCpeToDRetryPeriod, wranIfCpeCBCReqRetries,

wranIfCpePowerCtrlProcTime, wranIfCpeRangCmdProcTime,

wranIfCpeRangCmdProcTime, wranIfCpeRangCmdProcTime,

wranIfCpeInvitedRangRetires, wranIfCpeInvitedRangRetires,

wranIfCpeDSxReqRetries, wranIfCpeDSxRspRetries }

STATUS	current

DESCRIPTION

"This group contains configuration objects for the CPE."

::= { wranIfCpeMibGroups 1 }

wranIfCpeMibNotificationsGroup	OBJECT-TYPE

OBJECTS	{ wranIfCpeRssiStatusChangeTrap,

wranIfCpeEirpPerScStatusChangeTrap,

wranIfCpeDhcpSuccessTrap,

wranIfCpeScmStateChangeTrap,

wranIfCpeMaxEirpStatusChangeTrap }

STATUS	current

DESCRIPTION

"This group contains CPE event notifications."

::= {wranIfCpeMibGroups 2 }

wranIfCpeMibCompliance	MODULE-COMPLIANCE

STATUS	current

DESCRIPTION

"MIB objects that are optional and mandatory for CPE

conformance."

MODULE	wranIfCpeMib

MANDATORY-GROUPS	{ wranifCpeMibConfigGroup }

-- OPTIONAL-GROUPS	{ wranIfCpeMibNotificationsGroup }

::= { wranIfCpeMib 7 }

END

    - wranIfSmMib

IEEE802dot22-WRAN-IF-SM-MIB	::=	BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVETION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPV2-CONF

wranIfSmMib	MODULE-IDENTITY

LAST-UPDATED	"201205140000Z"	-- May 14, 2012

ORGANIZATION	"IEEE 802.22"

CONTACT-INFO

"WG E-mail:	STDS-802-22@LISTSERV.IEEE.ORG

WG Chair:	Apurva N. Mody

E-mail:	apurva.mody@ieee.org

TGa Chair/Editor:  Ranga Reddy

E-mail:		  ranga.reddy@ieee.org"

DESCRIPTION

"This material is from IEEE 802.22a-2012 Copyright (c) 2012. This

MIB Module objectes related to configuration, operation and

monitoring of the Spectrum Manager (SM). Objects in this module

are based on IEEE 802.22-2011 and is under

iso(1).std(0).iso8802(8802).wran(22).wranIfSmMib(5)"

REVISION		"201205140000Z"

DESCRIPTION

"The first version of IEEE802dot22-WRAN-IF-SM-MIB."

::= {iso std(0) iso8802(8802) wran(22) 5}

wranIfSmConfigTable			OBJECT IDENTIFIER ::= { wranIfSmMib 1 }

wranIfSmPendingBlmReqTable		OBJECT IDENTIFIER ::= { wranIfSmMib 2 }

wranIfSmBlmRepTable			OBJECT IDENTIFIER ::= { wranIfSmMib 3 }

wranIfSmChClassificationStatusTable	OBJECT IDENTIFIER ::= { wranIfSmMib 4 }

wranIfSmChannelSetTable			OBJECT IDENTIFIER ::= { wranIfSmMib 5 }

wranIfSmCurrentStatusTable		OBJECT IDENTIFIER ::= { wranIfSmMib 6 }

wranIfSmRegTrackingTable		OBJECT IDENTIFIER ::= { wranIfSmMib 7 }

wranIfSmTrapControl			OBJECT IDENTIFIER ::= { wranIfSmMib 8 }

wranIfSmTrapDefinition			OBJECT IDENTIFIER ::= { wranIfSmMib 9 }

wranIfSmNotificationObjectsTable	OBJECT IDENTIFIER ::= { wranIfSmMib 10 }

wranIfSmMibGroups				OBJECT IDENTIFIER ::= { wranIfSmMib 11 }

wranIfSmMibCompliance			OBJECT IDENTIFIER ::= { wranIfSmMib 12 }

wranIfSmConfigTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfSmConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB object represents a table that tracks the default

configuration of SM timers and constants. It is made up of one

entry, defined by wranIfSmConfigEntry."

::= { wranIfSmMib 1 }

wranIfSmConfigEntry	OBJECT-TYPE

SYNTAX	wranIfSmConfigEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfSmConfigTable. Each

entry is identified by wranIfSmConfigIndex."

INDEX	{ wranIfSmConfigIndex }

::= { wranIfSmConfigTable 1 }

wranIfSmConfigEntry	::= SEQUENCE {

wranIfSmConfigIndex			INTEGER,

wranIfSmT31					INTEGER,

wranIfSmSsaChAvailabilityCheckTime	Integer32,

wranIfSmSsaNonOccupancyPeriod		Integer32,

wranIfSmSsaChannelDetectionTime	INTEGER,

wranIfSmSsaChannelSetupTime		INTEGER,

wranIfSmSsaChannelOpeningTxTime	Integer32,

wranIfSmSsaChannelMoveTime		INTEGER,

wranIfSmSsaChannelClosingTxTime	Integer32,

wranIfSmSsaMicProtectionRadius	Integer32,

wranIfSmSsaT41				INTEGER,

wranIfSmSsaT42				INTEGER,

wranIfSmSsaT43				INTEGER,

wranIfSmSsaT44				INTEGER,

wranIfSmSsaT45				Integer32,

wranIfSmT46					Integer32,

wranIfSmSsaT59				Integer32,

wranIfSmSsaT47				Integer32,

wranIfSmSsaT48				Integer32,

wranIfSmSsaT49				Integer32,

wranIfSmSsaT50				Integer32,

wranIfSmSsaT51				Integer32,

wranIfSmSsaT53				Integer32,

wranIfSmSsaT54				Integer32,

wranIfSmSsaT55				INTEGER,

wranIfSmSsaT60				INTEGER }

wranIfSmConfigIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table, defaults to 1."

::= { wranIfSmConfigEntry 1 }

wranIfSmT31			OBJECT-TYPE

SYNTAX	INTEGER (1..64)

UNITS		"frames"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Wait for BLM-REP timeout."

::= { wranIfSmConfigEntry 2 }

wranIfSmSsaChAvailabilityCheckTime	OBJECT-TYPE

SYNTAX	Integer32 (30..3600)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Time during which a TV channel shall be checked for the presence

of licensed incumbent signals having a level above the incumbent

detection threshold prior to commencement of WRAN operation in

the channel, and in the case of TV, a related channel at an EIRP

level that can affect the measured TV channel."

::= { wranIfSmConfigEntry 3 }

wranIfSmSsaNonOccupancyPeriod	OBJECT-TYPE

SYNTAX	Integer32 (10..60)

UNITS		"minutes"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The required period during which WRAN device transmissions SHALL

NOT occur in a given TV channel because of the detected presence

of an incumbent signal in that channel above the Incumbent

detection threshold, or in the case of TV, above a given EIRP

level."

::= { wranIfSmConfigEntry 4 }

wranIfSmSsaChannelDetectionTime	OBJECT-TYPE

SYNTAX	INTEGER (2..128)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximum time taken by a WRAN device to detect a licensed

incumbent signal above the Incumbent Detection Threshold within

a given TV channel during normal WRAN operation."

::= { wranIfSmConfigEntry 5 }

wranIfSmSsaChannelSetupTime	OBJECT-TYPE

SYNTAX	INTEGER (2..128)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The window of time that may be taken by a WRAN CPE to transmit

control information to a WRAN base station in order to establish

operation with that base station at the prescribed power or, in

the case of TV, at or below the allowable EIRP within a given TV

channel."

::= { wranIfSmConfigEntry 6 }

wranIfSmSsaChannelOpeningTxTime	OBJECT-TYPE

SYNTAX	Integer32 (100..480)

UNITS		"milliseconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The aggregate duration of control transmissions by WRAN devices

during the Channel Setup Time that starts at the end of the

Channel Availability Check Time."

::= { wranIfSmConfigEntry 7 }

wranIfSmSsaChannelMoveTime	OBJECT-TYPE

SYNTAX	INTEGER (2..60)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The time taken by WRAN system to cease all interfering

transmissions on the current TV channel upon detection of a

license incumbent signal above the relevant Incumbent Detection

Threshold, or in the case of TV, to alternatively reduces its

EIRP to which is allowable within a given TV channel upon

detection of a TV signal in the same or a related channel."

::= { wranIfSmConfigEntry 8 }

wranIfSmSsaChannelClosingTxTime	OBJECT-TYPE

SYNTAX	Integer32 (100..480)

UNITS		"milliseconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The aggregate duration of control transmissions by the WRAN

devices during the Channel Move/EIRP Reduction Time that starts

upon detection of a licensed incumbent signal above the relevant

Incumbent Detection Threshold."

::= { wranIfSmConfigEntry 9 }

wranIfSmSsaMicProtectionRadius	OBJECT-TYPE

SYNTAX	Integer32 (100..100000)

UNITS		"meters"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Radius of contour within which the WRAN system cannot operate

due to potential interference with the microphone."

::= { wranIfSmConfigEntry 10 }

wranIfSmSsaT41			OBJECT-TYPE

SYNTAX	INTEGER (1..10)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximum time interval allowed before sensing is performed on the

candidate channel to ensure that no incumbents are detected."

::= { wranIfSmConfigEntry 11 }

wranIfSmSsaT42			OBJECT-TYPE

SYNTAX	INTEGER (1..10)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximum time interval allowed before sensing is performed on the

backup channel to ensure that no incumbents are detected."

::= { wranIfSmConfigEntry 12 }

wranIfSmSsaT43			OBJECT-TYPE

SYNTAX	INTEGER (1..100)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Minimum time duration without detection of any incumbent for a

candidate channel to transition to the backup channel."

::= { wranIfSmConfigEntry 13 }

wranIfSmSsaT44			OBJECT-TYPE

SYNTAX	INTEGER (1..100)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximum time to ensure that the channel move information is

successfully conveyed to all the associated CPEs and BS (self-

coexistence mode)."

::= { wranIfSmConfigEntry 14 }

wranIfSmSsaT45			OBJECT-TYPE

SYNTAX	Integer32 (0..720)

UNITS		"hours"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximum WRAN operation time without access to the incumbent

database service, from 0 to 72 hrs in 0.1hr steps."

::= { wranIfSmConfigEntry 15 }

wranIfSmT46			OBJECT-TYPE

SYNTAX	Integer32 (1..4096)

UNITS		"frames"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Waiting time before which the BS moves to the first backup

channel. This is used to make sure that all the CPEs are ready

to move to the backup channel before BS switches operation to

this backup channel."

::= { wranIfSmConfigEntry 16 }

wranIfSmSsaT59			OBJECT-TYPE

SYNTAX	Integer32 (1..4096)

UNITS		"frames"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Waiting time before which the CPE moves to its backup channels

if it no longer hears from its BS. This is used to make sure

that the CPE waits long enough after UCS Notification so that BS

has had time to move to the backup channel, it decided to do

so."

::= { wranIfSmConfigEntry 17 }

wranIfSmSsaT47			OBJECT-TYPE

SYNTAX	Integer32 (0..720)

UNITS		"hours"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The prescribed time by the WRAN operator to refresh the

incumbent database service."

::= { wranIfSmConfigEntry 18 }

wranIfSmSsaT48			OBJECT-TYPE

SYNTAX	Integer32 (0..600)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Lapse timer keeps track of whether the Operating Channel N has

been cleared using spectrum sensing, from 0 to 60 seconds in

0.1s steps."

::= { wranIfSmConfigEntry 19 }

wranIfSmSsaT49			OBJECT-TYPE

SYNTAX	Integer32 (0..600)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Lapse timer keeps track of whether the Adjacent Channel N–1 has

been cleared using spectrum sensing, from 0 to 60 seconds in

0.1s steps."

::= { wranIfSmConfigEntry 20 }

wranIfSmSsaT50			OBJECT-TYPE

SYNTAX	Integer32 (0..600)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Lapse timer keeps track of whether the Adjacent Channel N+1 has

been cleared using spectrum sensing., from 0 to 60 seconds in

0.1s steps."

::= { wranIfSmConfigEntry 21 }

wranIfSmSsaT51			OBJECT-TYPE

SYNTAX	Integer32 (1..600)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Initiated when SSA loses contact with the SM."

::= { wranIfSmConfigEntry 22 }

wranIfSmSsaT53			OBJECT-TYPE

SYNTAX	Integer32 (1..600)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The parameter TINsens is used to verify that in-band sensing has

been done within the required In-service monitoring period. The

TINsens parameter is driven by regulatory domain requirements

(Annex A), from 0 to 60 seconds in 0.1s steps."

::= { wranIfSmConfigEntry 23 }

wranIfSmSsaT54			OBJECT-TYPE

SYNTAX	Integer32 (1..600)

UNITS		"seconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The parameter TOUTsens is used to verify that out-of-band

sensing has been done within the required "Acquiring a channel

monitoring period" specified in  1890HAnnex A. This value would

be used to initialize a lapse timer for each channel in the

backup candidate channel list at each CPE so that it is compared

to Tsensout, from 0 to 60 seconds in 0.1s steps."

::= { wranIfSmConfigEntry 24 }

wranIfSmSsaT55			OBJECT-TYPE

SYNTAX	INTEGER (1..160)

UNITS		"milliseconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The T55 or Tsensin parameter corresponds to the maximum length

of time required to carry out the sensing process on an in-band

channel (see Figure 176). Manufacturers need to specify the

sensing time required to detect the specified signals with

required accuracy, from 0 to 60 seconds in 0.1s steps."

::= { wranIfSmConfigEntry 25 }

wranIfSmSsaT60			OBJECT-TYPE

SYNTAX	INTEGER (20..160)

UNITS		"milliseconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The T60 or Tsensout parameter corresponds to the maximum length

of time required to carry out the out-of- band sensing process to

clear one channel (see Figure 178). Manufacturers need to

specify the sensing time required to detect the specified signals

with required accuracy for out-of-band sensing."

::= { wranIfSmConfigEntry 26 }

wranIfSmPendingBlmReqTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfSmPendingBlmReqEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB object represents a table that tracks the status of the

execution of ongoing sensing requests (BLM-REQ). For each BLM-

REQ there is a corresponding BLM-RSP to indicate that REQ

message was received by the SSA. When an SSA is done with the

sensing it will send a BLM-REP to the SM. This table keeps track

of any BLM-REP messages that are pending transmission from the

SSA. When a report is received a response is acknowledgement is

sent to the SSA, and then the entry corresponding to the report

and request will be cleared. Each entry is defined by

wranIfSmPendingBlmReqEntry."

::= { wranIfSmMib 2 }

wranIfSmPendingBlmReqEntry	OBJECT-TYPE

SYNTAX	wranIfSmPendingBlmReqEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfSmPendingBlmReqTable. Each

entry is identified by wranIfSmPendingBlmReqIndex."

INDEX	{ wranIfSmPendingBlmReqIndex }

::= { wranIfSmPendingBlmReqTable 1 }

wranIfSmPendingBlmReqEntry	::= SEQUENCE {

wranIfSmPendingBlmReqIndex			INTEGER,

wranIfSmPendingBlmReqTransactionId		Integer32,

wranIfSmPendingBlmReqMsg			OCTET STRING,

wranIfSmPendingBlmRspReceived			TruthValue,

wranIfSmPendingBlmRspMulticastReceived	TruthValue,

wranIfSmPendingBlmRepTimeout			Integer32,

wranIfSmPendingBlmRepReceived			TruthValue,

wranIfSmPendingBlmRepMulticastReceived	TruthValue,

wranIfSmPendingBlmRepAck			TruthValue }

wranIfSmPendingBlmReqIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table."

::= { wranIfSmPendingBlmReqEntry 1 }

wranIfSmPendingBlmReqTransactionId	OBJECT-TYPE

SYNTAX	Integer32 (0..65535)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Transaction ID of BLM-REQ."

::= { wranIfSmPendingBlmReqEntry 2 }

wranIfSmPendingBlmReqMsg	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(1..65535))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Contents of BLM-REQ pending a report."

::= { wranIfSmPendingBlmReqEntry 3 }

wranIfSmPendingBlmRspReceived	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Indication of whether (Truth(1)) or not (Truth(0)) BLM-RSP

pertaining to BLM-REQ has been received from SSA."

::= { wranIfSmPendingBlmReqEntry 4 }

wranIfSmPendingBlmRspMulticastReceived	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"If BLM-REQ was multicast, indication of whether (Truth(1)) or

not (Truth(0)) BLM-RSP pertaining to BLM-REQ has been received

from each SSA (CPE) in the multicast group."

::= { wranIfSmPendingBlmReqEntry 5 }

wranIfSmPendingBlmRepTimeout		OBJECT-TYPE

SYNTAX	Integer32 (1..640)

UNITS		"milliseconds"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Indication of current value of T31 set for this BLM-REP."

::= { wranIfSmConfigEntry 6 }

wranIfSmPendingBlmRepReceived	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Indication of whether (Truth(1)) or not (Truth(0)) BLM-REP

pertaining to BLM-REQ has been received from SSA."

::= { wranIfSmPendingBlmReqEntry 7 }

wranIfSmPendingBlmRepMulticastReceived	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"If BLM-REQ was multicast, indication of whether (Truth(1)) or

not (Truth(0)) BLM-REP pertaining to BLM-REQ has been received

from each SSA (CPE) in the multicast group."

::= { wranIfSmPendingBlmReqEntry 8 }

wranIfSsaPendingBlmRepAck	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Indication of whether (Truth(1)) or not (Truth(0)) BLM-ACK, has

been sent to acknowledge all transmitters of BLM-REP messages

pertaining to BLM-REQ that was issued."

::= { wranIfSmPendingBlmReqEntry 9 }

wranIfSmBlmRepTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfSmBlmRepEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object contains BLM-REP messages received in response to

BLM-REQ; it is made up of multiple entries, one for each BLM-REP

that pertains to a BLM-REQ. Once a BLM-REP has been processed

the entry will be removed. Each entry is defined by

wranIfSmBlmRepEntry."

::= { wranIfSmMib 3 }

wranIfSmBlmRepEntry	OBJECT-TYPE

SYNTAX	wranIfSmBlmRepEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfSmBlmRepTable. Each

entry is identified by wranIfSmBlmReqIndex."

INDEX	{ wranIfSmBlmReqIndex }

::= { wranIfSmBlmRepTable 1 }

wranIfSmBlmRepEntry	::= SEQUENCE {

wranIfSmBlmRepIndex			INTEGER,

wranIfSmBlmRepSid				INTEGER,

wranIfSmBlmRepTransactionId		Integer32,

wranIfSmBlmRepMsg				OCTET STRING }

wranIfSmBlmRepIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..100)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table."

::= { wranIfSmBlmRepEntry 1 }

wranIfSmBlmRepSid	OBJECT-TYPE

SYNTAX	INTEGER (1..512)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"SID of CPE that sent the BLM-REP."

::= { wranIfSmBlmRepEntry 2 }

wranIfSmBlmRepTransactionId	OBJECT-TYPE

SYNTAX	Integer32 (0..65535)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Transaction ID of BLM-REP, it should match a transaction ID of

an entry in wranIfSmPendingBlmReqTable."

::= { wranIfSmBlmRepEntry 3 }

wranIfSmBlmRepMsg	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(1..65535))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Contents of BLM-REP msg."

::= { wranIfSmBlmRepEntry 4 }

wranIfSmChClassificationStatusTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfSmChClassificationStatusEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB object represents a table that the status for channels

that the SM is managing. It is made up of multiple entries, one

for each channel, as defined in

wranIfSmChClassificationStatusEntry."

::= { wranIfSmMib 4 }

wranIfSmChClassificationStatusEntry	OBJECT-TYPE

SYNTAX	wranIfSmChClassificationStatusEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in

wranIfSmChClassificationStatusTable. Each entry is identified by

wranIfSmChClassificationStatusIndex."

INDEX	{ wranIfSmChClassificationStatusIndex }

::= { wranIfSmChClassificationStatusTable 1 }

wranIfSmChClassificationStatusEntry	::= SEQUENCE {

wranIfSmChClassificationStatusIndex	INTEGER,

wranIfSmManagedChannel			INTEGER,

wranIfSmManagedChannelStatus		INTEGER,

wranIfSmManagedChannelRecentEvent	INTEGER }

wranIfSmChClassificationStatusIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table."

::= { wranIfSmChClassificationStatusEntry 1 }

wranIfSmManagedChannel	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

" Channel number of channel being managed."

::= { wranIfSmChClassificationStatusEntry 2 }

wranIfSmManagedChannelStatus	OBJECT-TYPE

SYNTAX	INTEGER { unclassified(0),

candidate(1),

protected(2),

operating(3),

backup(4),

disallowed(5) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"The state of the channel as set by the states in the SM Channel

Set Transition Diagram (Figure 162) or disallowed state (if

channel is in IPC-UPD)."

::= { wranIfSmChClassificationStatusEntry 3 }

wranIfSmManagedChannelRecentEvent	OBJECT-TYPE

SYNTAX	INTEGER { disallowed(0),

event1(1),

event2(2),

event3(3),

event4(4),

event5(5),

event6(6),

event7(7),

event8(8) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Most recent event (see 10.2.3.1) that dictated a transition to

the current state. disallowed(0) pertains to reception of IPC-

UPD."

::= { wranIfSmChClassificationStatusEntry 4 }

wranIfSmChannelSetTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfSmChannelSetEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB object represents a table that stores information

related to the contents of the Occupied, Backup Channel, and

Local Priority channel sets used by the Spectrum Manager (see

10.2.3.2)."

::= { wranIfSmMib 5 }

wranIfSmChannelSetEntry	OBJECT-TYPE

SYNTAX	wranIfSmChannelSetEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfSmChannelSetTable. There

is only one entry in this table."

INDEX	{ wranIfSmChannelSetIndex }

::= { wranIfSmChannelSetTable 1 }

wranIfSmChannelSetEntry	::= SEQUENCE {

wranIfSmChannelSetIndex				INTEGER,

wranIfSmSizeWranOccupiedChannelSet		INTEGER,

wranIfSmWranOccupiedChannelSet		OCTET STRING,

wranIfSmSizeNghbrWranBackupChannelSet	INTEGER,

wranIfSmNghbrWranBackupChannelSet		OCTET STRING,

wranIfSmSizeLocalPrioritySet1			INTEGER,

wranIfSmLocalPrioritySet1			OCTET STRING,

wranIfSmSizeLocalPrioritySet2			INTEGER,

wranIfSmLocalPrioritySet2			OCTET STRING,

wranIfSmSizeLocalPrioritySet3			INTEGER,

wranIfSmLocalPrioritySet3			OCTET STRING }

wranIfSmChannelSetIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table."

::= { wranIfSmChannelSetEntry 1 }

wranIfSmSizeWranOccupiedChannelSet	OBJECT-TYPE

SYNTAX	INTEGER (1..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Number of channels in WRAN Occupied Channel Set."

::= { wranIfSmChannelSetEntry 2 }

wranIfSmWranOccupiedChannelSet	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeWranOccupiedChannelSet))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Vector of channels of length 8bits ×

wranIfSmSizeWranOccupiedChannelSet that indicate the channels

that occupy the WRAN Occupied Channel Set used by the Spectrum

Etiquette procedure (see 10.2.3.2)."

::= { wranIfSmChannelSetEntry 3 }

wranIfSmSizeNghbrWranBackupChannelSet	OBJECT-TYPE

SYNTAX	INTEGER (1..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Number of channels in Neighbor WRAN Backup Channel Set."

::= { wranIfSmChannelSetEntry 4 }

wranIfSmNghbrWranBackupChannelSet	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeNghbrWranBackupChannelSet))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Vector of channels of length 8bits ×

wranIfSmSizeNghbrWranBackupChannelSet that indicate the channels

that occupy the Neighbor WRAN Backup Channel Set used by the

Spectrum Etiquette procedure (see 10.2.3.2)."

::= { wranIfSmChannelSetEntry 5 }

wranIfSmSizeLocalPrioritySet1	OBJECT-TYPE

SYNTAX	INTEGER (1..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Number of channels in Local Priority Set 1."

::= { wranIfSmChannelSetEntry 6 }

wranIfSmLocalPrioritySet1	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeLocalPrioritySet1))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Vector of channels of length 8bits ×

wranIfSmSizeLocalPrioritySet1 that indicate the channels

that occupy the Local Priority Set 1 used by the

Spectrum Etiquette procedure (see 10.2.3.2)."

::= { wranIfSmChannelSetEntry 7 }

wranIfSmSizeLocalPrioritySet2	OBJECT-TYPE

SYNTAX	INTEGER (1..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Number of channels in Local Priority Set 2."

::= { wranIfSmChannelSetEntry 8 }

wranIfSmLocalPrioritySet2	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeLocalPrioritySet2))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Vector of channels of length 8bits ×

wranIfSmSizeLocalPrioritySet2 that indicate the channels

that occupy the Local Priority Set 2 used by the

Spectrum Etiquette procedure (see 10.2.3.2)."

::= { wranIfSmChannelSetEntry 9 }

wranIfSmSizeLocalPrioritySet3	OBJECT-TYPE

SYNTAX	INTEGER (1..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Number of channels in Local Priority Set 3, should be the same

size as wranIfSmSizeWranOccupiedChannelSet."

::= { wranIfSmChannelSetEntry 10 }

wranIfSmLocalPrioritySet3	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeLocalPrioritySet3))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Vector of channels of length 8bits ×

wranIfSmSizeLocalPrioritySet3 that indicate the channels

that occupy the Local Priority Set 3 used by the

Spectrum Etiquette procedure (see 10.2.3.2). Should contain the

same channel set as wranIfSmWranOccupiedChannelSet."

::= { wranIfSmChannelSetEntry 11 }

wranIfSmCurrentStatusTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfSmCurrentStatusEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB object represents a table that the records the current

status of the SM. This includes the state the SM is in, the

event that triggered a move into that state, as well as the

current state of any relevant timers. There is one entry in this

table defined in wranIfSmCurrentStatusEntry."

::= { wranIfSmMib 6 }

wranIfSmCurrentStatusEntry	OBJECT-TYPE

SYNTAX	wranIfSmCurrentStatusEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfSmCurrentStatusTable.

There is only one entry in this table."

INDEX	{ wranIfSmCurrentStatusIndex }

::= { wranIfSmCurrentStatusTable 1 }

wranIfSmCurrentStatusEntry	::= SEQUENCE {

wranIfSmCurrentStatusIndex		INTEGER,

wranIfSmCurrentState			INTEGER,

wranIfSmRecentEvent			INTEGER,

wranIfSmRecentAction			INTEGER,

wranIfSmInitiateChannelMove		TruthValue,

wranIfSmSelfCoexistenceMode		TruthValue,

wranIfSmCurrentOperatingChannel	INTEGER,

wranIfSmRecentSignalType		INTEGER,

wranIfSmCurrentT47			Integer32,

wranIfSmCurrentT46			Integer32 }

wranIfSmCurrentStatusIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table."

::= { wranIfSmCurrentStatusEntry 1 }

wranIfSmCurrentState	OBJECT-TYPE

SYNTAX	INTEGER { init(0),

operation(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The state (see Figure 164) that the SM is in. A value of 0

indicates that the SM is in the 'Spectrum Manager at Network

Initialization State'."

::= { wranIfSmCurrentStatusEntry 2 }

wranIfSmRecentEvent	OBJECT-TYPE

SYNTAX	INTEGER { timeT46expires(0),

operatingChannelIdentified(1),

initiateChannelMoveSet(2),

newCpeRegRequest(3),

timerT47expires(4),

signalDetected(5),

selfCoexistenceModeSet(6) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Recent event that triggered a transition into the current state

as given in 10.2.6.1 "

::= { wranIfSmCurrentStatusEntry 3 }

wranIfSmRecentAction	OBJECT-TYPE

SYNTAX	INTEGER { smFindOperatingChannel(0),

smEstablishNetwork(1),

smInitiateChannelMove(2),

smNewCpeVerification(3),

smDatabaseUpdate(4),

smDetermineSignalTypeExecutePolicies(5),

smSelfCoexistenceMode(6) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Recent action that was taken when transitioning into the current

state as given in 10.2.6.1."

::= { wranIfSmCurrentStatusEntry 4 }

wranIfSmInitiateChannelMove	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Current value of ‘Initiate_Channel_Move’ flag. Truth(1) if the

the flag is set to '1', Truth(0) if the flag is set to 0."

::= { wranIfSmCurrentStatusEntry 5 }

wranIfSmSelfCoexistenceMode	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Current value of ‘Self_Coexistence_Mode’ flag. Truth(1) if the

the flag is set to '1', Truth(0) if the flag is set to 0."

::= { wranIfSmCurrentStatusEntry 6 }

wranIfSmCurrentOperatingChannel	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Current operating channel."

::= { wranIfSmCurrentStatusEntry 7 }

wranIfSmRecentSignalType	OBJECT-TYPE

SYNTAX	INTEGER { undetermind(0),

ieee802dot22wran(1),

atsc(2),

dvbt(3),

isdbt(4),

ntsc(5),

pal(6),

secam(7),

wirelessMicrophone(8),

ieee802dot22dot1Sync(9),

ieee802dot22dot1Msf1(10),

ieee802dot22dot1Msf2(11),

ieee802dot22dot1Msf3(12) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Type of signal recently detected (see Table 237)."

::= { wranIfSmCurrentStatusEntry 8 }

wranIfSmCurrentT47	OBJECT-TYPE

SYNTAX	Integer32 (1..720)

UNITS		"hours"

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Current value of T47, in 0.1hr increments from 0 to 72hrs."

::= { wranIfSmCurrentStatusEntry 9 }

wranIfSmCurrentT46	OBJECT-TYPE

SYNTAX	Integer32 (1..4096)

UNITS		"frames"

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Current value of T46."

::= { wranIfSmCurrentStatusEntry 10 }

wranIfSmRegTrackingTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfSmRegTrackingEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB object represents a table that records the SM’s

monitoring of CPE’s associated with the BS. It contains the

location data string and current value of T30 for each CPE.

There are multiple entries in this table (one for each CPE)

defined in wranIfSmRegTrackingEntry."

::= { wranIfSmMib 7 }

wranIfSmRegTrackingEntry	OBJECT-TYPE

SYNTAX	wranIfSmRegTrackingEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfSmRegTrackingTable."

INDEX	{ wranIfSmRegTrackingIndex }

::= { wranIfSmRegTrackingTable 1 }

wranIfSmRegTrackingEntry	::= SEQUENCE {

wranIfSmRegTrackingIndex		INTEGER,

wranIfSmRegTrackingCpeSid		INTEGER,

wranIfSmRegTrackingCurrentT30		Integer32,

wranIfSmRegTrackingLocStringSize	INTEGER,

wranIfSmRegTrackingLocString		OCTET STRING }

wranIfSmRegTrackingIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table."

::= { wranIfSmRegTrackingEntry 1 }

wranIfSmRegTrackingCpeSid	OBJECT-TYPE

SYNTAX	INTEGER (1..511)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"SID of CPE whose location is being tracked."

::= { wranIfSmRegTrackingEntry 2 }

wranIfSmRegTrackingCurrentT30	OBJECT-TYPE

SYNTAX	Integer32 (1..65535)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Current value of CPE’s T30, as known by SM. From 0.16s to

10,485.6s in 0.16s increments."

::= { wranIfSmRegTrackingEntry 3 }

wranIfSmRegTrackingLocStringSize	OBJECT-TYPE

SYNTAX	INTEGER (1..512)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of the location string (wranIfSmRegTrackingLocString) in

octets."

::= { wranIfSmRegTrackingEntry 4 }

wranIfSmRegTrackingLocString	OBJECT-TYPE

SYNTAX	OCTET STRING(SIZE(wranIfSmRegTrackingLocStringSize))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of the location string (wranIfSmRegTrackingLocString) in

octets."

::= { wranIfSmRegTrackingEntry 5 }

wranIfSmTrapControl	OBJECT-TYPE

SYNTAX	BITS	{ wranIfSmBlmReqChange(0),

wranIfSmBlmRepChange(1),

wranIfSmOccupiedChannelSetChange(2),

wranIfSmNghbrBackupChannelSetChange(3),

wranIfSmLocalPrioritySet1Change(4),

wranIfSmLocalPrioritySet2Change(5),

wranIfSmLocalPrioritySet3Change(6),

wranIfSmCurrentStatusChange(7),

wranIfSmRegTrackingChange(8) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Defines control elements for traps related to operation of the

spectrum manager. This is a 9bit field that enables setting a

trap for particular SM events: wranIfSmBlmReqChange,

wranIfSmBlmRepChange, wranIfSmOccupiedChannelSetChange,

wranIfSmNghbrBackupChannelSetChange,

wranIfSmLocalPrioritySet1Change,

wranIfSmLocalPrioritySet2Change,

wranIfSmLocalPrioritySet3Change, wranIfSmCurrentStatusChange,

and wranIfSmRegTrackingChange."

::= { wranIfSmMib 8 }

wranIfSmTrapDefinition			 OBJECT IDENTIFIER

::= { wranIfSmMib 9 }

wranIfSmBlmReqChangeTrap		 OBJECT IDENTIFIER

::= { wranIfSmTrapDefinition 1 }

wranIfSmBlmRepChangeTrap		 OBJECT IDENTIFIER

::= { wranIfSmTrapDefinition 2 }

wranIfSmOccupiedChannelSetChangeTrap OBJECT IDENTIFIER

::= { wranIfSmTrapDefinition 3 }

wranIfSmNghbrBackupChannelSetChangeTrap	OBJECT IDENTIFIER

::= { wranIfSmTrapDefinition 4 }

wranIfSmLocalPrioritySet1ChangeTrap	 OBJECT IDENTIFIER

::= { wranIfSmTrapDefinition 5 }

wranIfSmLocalPrioritySet2ChangeTrap	 OBJECT IDENTIFIER

::= { wranIfSmTrapDefinition 6 }

wranIfSmLocalPrioritySet3ChangeTrap	 OBJECT IDENTIFIER

::= { wranIfSmTrapDefinition 7 }

wranIfSmCurrentStatusChangeTrap	 OBJECT IDENTIFIER

::= { wranIfSmTrapDefinition 8 }

wranIfSmRegTrackingChangeTrap		 OBJECT IDENTIFIER

::= { wranIfSmTrapDefinition 9 }

wranIfSmBlmReqChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfSmPendingBlmReqTransactionId,

wranIfSmPendingBlmReqStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to a BLM-REQ that is

pending in the SM."

::= { wranIfSmTrapDefinition 1 }

wranIfSmBlmRepChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfSmBlmRepTransactionId,

wranIfSmBlmRepStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to a BLM-REP that is

received by the SM."

::= { wranIfSmTrapDefinition 2 }

wranIfSmOccupiedChannelSetChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfSmWranOccupiedChannelSet,

wranIfSmOccupiedChannelSetStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the current state

of the Occupied Channel Set in the SM."

::= { wranIfSmTrapDefinition 3 }

wranIfSmNghbrBackupChannelSetChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfSmNghbrWranBackupChannelSet,

wranIfSmNghbrBackupChannelSetStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the current state

of the Neighbor WRAN Backup Channel Set in the SM."

::= { wranIfSmTrapDefinition 4 }

wranIfSmLocalPrioritySet1ChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfSmLocalPrioritySet1,

wranIfSmLocalPrioritySet1Status }

STATUS	current

DESCRIPTION

"This trap contains the information related to the current state

of the Local Priority Set 1 in the SM."

::= { wranIfSmTrapDefinition 5 }

wranIfSmLocalPrioritySet2ChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfSmLocalPrioritySet2,

wranIfSmLocalPrioritySet2Status }

STATUS	current

DESCRIPTION

"This trap contains the information related to the current state

of the Local Priority Set 2 in the SM."

::= { wranIfSmTrapDefinition 6 }

wranIfSmLocalPrioritySet3ChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfSmLocalPrioritySet3,

wranIfSmLocalPrioritySet3Status }

STATUS	current

DESCRIPTION

"This trap contains the information related to the current state

of the Local Priority Set 3 in the SM."

::= { wranIfSmTrapDefinition 7 }

wranIfSmCurrentStatusChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfSmRecentAction,

wranIfSmCurrentStateStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the current state

the SM is in."

::= { wranIfSmTrapDefinition 8 }

wranIfSmRegTrackingChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfSmRegTrackingCpeSid,

wranIfSmRegTrackingStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the tracking of

the location of CPE's within the purview of the BS."

::= { wranIfSmTrapDefinition 9 }

wranIfSmNotificationObjectsTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfSmNotificationObjectsEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB provides a table to track notification objects that

have been reproted by the traps related to the operation of the

SM. It is made up of one entry containing objects related to the

most recent trap/event. The entry is defined by

wranIfSmNotificationObjectsEntry."

::= { wranIfSmMib 10 }

wranIfSmNotificationObjectsEntry	OBJECT-TYPE

SYNTAX	wranIfSmNotificationObjectsEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in

wranIfSmNotificationObjectsTable."

INDEX	{ wranIfSmNotificationSid }

::= { wranIfSmNotificationObjectsTable 1 }

wranIfSmNotificationObjectsEntry	::= SEQUENCE {

wranIfSmNotificationSid				INTEGER,

wranIfSmNotificationBlmTransactionId	Integer32,

wranIfSmNotificationOccupiedChannelSet	OCTET STRING,

wranIfSmNotificationBackupChannelSet	OCTET STRING,

wranIfSmNotificationLocalPrioritySet1	OCTET STRING,

wranIfSmNotificationLocalPrioritySet2	OCTET STRING,

wranIfSmNotificationLocalPrioritySet3	OCTET STRING,

wranIfSmNotificationRecentAction		INTEGER,

wranIfSmPendingBlmReqStatus			INTEGER,

wranIfSmBlmRepStatus				INTEGER,

wranIfSmOccupiedChannelSetStatus		INTEGER,

wranIfSmNghbrBackupChannelSetStatus		INTEGER,

wranIfSmLocalPrioritySet1Status		INTEGER,

wranIfSmLocalPrioritySet2Status		Integer32,

wranIfSmLocalPrioritySet3Status		Integer32,

wranIfSmCurrentStateStatus			Integer32,

wranIfSmRegTrackingStatus			Integer32 }

wranIfSmNotificationSid	OBJECT-TYPE

SYNTAX	INTEGER (0..511)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"SID of station generating trap. wranIfSmNotificationSid set == 0

is reserved for traps dealing with channel set management (see

wranIfSmOccupiedChannelSetChangeTrap,

wranIfSmNghbrBackupChannelSetChangeTrap,

wranIfSmLocalPrioritySet1ChangeTrap,

wranIfSmLocalPrioritySet2ChangeTrap,

wranIfSmLocalPrioritySet3ChangeTrap) and the current state of

the SM (see wranIfSmCurentStatusChangeTrap).

wranIfSmNotificationSid set == 1..511 when dealing with traps

for bulk measurement (see wranIfSmBlmReqChangeTrap,

wranIfSmBlmRepChangeTrap) or CPE registration tracking (see

wranIfSmRegTrackingChangeTrap)."

::= { wranIfSmNotificationObjectsEntry 1 }

wranIfSmNotificationBlmTransactionId	OBJECT-TYPE

SYNTAX	Integer32 (0..65535)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The transaction ID of the bulk measurement transaction. Only

pertinent when wranIfSmBlmReqChangeTrap or

wranIfSmBlmRepChangeTrap is enabled."

::= { wranIfSmNotificationObjectsEntry 2 }

wranIfSmNotificationOccupiedChannelSet	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeWranOccupiedChannelSet))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The current/new contents of the WRAN Occupied Channel Set (only

pertinent when wranIfSmOccupiedChannelSetChangeTrap is

enabled)."

::= { wranIfSmNotificationObjectsEntry 3 }

wranIfSmNotificationBackupChannelSet	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeNghbrWranBackupChannelSet))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The current/new contents of the Neighbor WRAN Backup Channel

Set (only pertinent when wranIfSmNghbrBackupChannelSetChangeTrap

is enabled)."

::= { wranIfSmNotificationObjectsEntry 4 }

wranIfSmNotificationLocalPrioritySet1	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeLocalPrioritySet1))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The current/new contents of the Local Priority Set 1 (only

pertinent when wranIfSmLocalPrioritySet1ChangeTrap is enabled)."

::= { wranIfSmNotificationObjectsEntry 5 }

wranIfSmNotificationLocalPrioritySet2	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeLocalPrioritySet2))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The current/new contents of the Local Priority Set 2 (only

pertinent when wranIfSmLocalPrioritySet2ChangeTrap is enabled)."

::= { wranIfSmNotificationObjectsEntry 6 }

wranIfSmNotificationLocalPrioritySet3	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfSmSizeLocalPrioritySet3))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"The current/new contents of the Local Priority Set 3 (only

pertinent when wranIfSmLocalPrioritySet3ChangeTrap is enabled)."

::= { wranIfSmNotificationObjectsEntry 7 }

wranIfSmNotificationRecentAction	OBJECT-TYPE

SYNTAX	INTEGER (0..6)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Value of wranIfSmRecentAction that triggered transition to

current state of SM (only pertinent when

wranIfSmCurentStatusChangeTrap is enabled)."

::= { wranIfSmNotificationObjectsEntry 8 }

wranIfSmPendingBlmReqStatus	OBJECT-TYPE

SYNTAX	INTEGER { blmRspUnicastReceived(0),

blmRspMulticastReceived(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indicates BLM-RSP pertaining to bulk measurement transaction

(defined by wranIfSmNotificationBlmTransactionId) was received

from wranIfSmNotificationSid, if wranIfSmNotificationSid is a

unicast SID, or if all BLM-RSP received from members of

multicast group identified by wranIfSmNotificationSid."

::= { wranIfSmNotificationObjectsEntry 9 }

wranIfSmBlmRepStatus	OBJECT-TYPE

SYNTAX	INTEGER { blmRepAcknowledged(0),

blmRepUnAcknowledged(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indicates whether or not BLM-ACK has been sent, pertaining to

BLM-REP received in bulk measurement transaction defined by

wranIfSmNotificationBlmTransactionId."

::= { wranIfSmNotificationObjectsEntry 10 }

wranIfSmOccupiedChannelSetStatus	OBJECT-TYPE

SYNTAX	INTEGER { channelsAdded(0),

channelsRemoved(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication of whether or not channels have been added or removed

when the WRAN Occupied Channel Set is modified."

::= { wranIfSmNotificationObjectsEntry 11 }

wranIfSmNghbrBackupChannelSetStatus	OBJECT-TYPE

SYNTAX	INTEGER { channelsAdded(0),

channelsRemoved(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication of whether or not channels have been added or removed

when the Neighbor WRAN Backup Channel Set is modified."

::= { wranIfSmNotificationObjectsEntry 12 }

wranIfSmLocalPrioritySet1Status	OBJECT-TYPE

SYNTAX	INTEGER { channelsAdded(0),

channelsRemoved(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication of whether or not channels have been added or removed

when the Local Priority Set 1 is modified."

::= { wranIfSmNotificationObjectsEntry 13 }

wranIfSmLocalPrioritySet2Status	OBJECT-TYPE

SYNTAX	INTEGER { channelsAdded(0),

channelsRemoved(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication of whether or not channels have been added or removed

when the Local Priority Set 2 is modified."

::= { wranIfSmNotificationObjectsEntry 14 }

wranIfSmLocalPrioritySet3Status	OBJECT-TYPE

SYNTAX	INTEGER { channelsAdded(0),

channelsRemoved(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication of whether or not channels have been added or removed

when the Local Priority Set 3 is modified."

::= { wranIfSmNotificationObjectsEntry 15 }

wranIfSmCurrentStateStatus	OBJECT-TYPE

SYNTAX	BITS { initiateChannelMoveSet(0),

coexistenceModeSet(1),

currentOperatingChannelChange(2),

signalDetectionClassificationSuccess(3),

expirationT47(4),

expirationT46(5) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Bitmap indicating current value of 'Initiate_Channel_Move' has

bit set (bit 0); whether or not 'Self_Coexistence_Mode' flag is

set (bit 1); whether or not the operating channel assignment has

changed (bit 2); whether recently detected signal was properly

classified, e.g. signal type == 'undetermined' or not (bit 3);

whether or not T47 has expired (bit 4); whether or not T46 has

expired (bit 5). Setting the bit to 0 makes the condition false,

setting the bit to 1 makes the condition true."

::= { wranIfSmNotificationObjectsEntry 16 }

wranIfSmRegTrackingStatus	OBJECT-TYPE

SYNTAX	INTEGER { locationChanged(0),

locationNotChanged(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication of whether or not CPE location, pertaining to CPE SID

identified by wranIfSmNotificationSid, has changed."

::= { wranIfSmNotificationObjectsEntry 17 }

-- wranIfSmMibGroups: this object helps define which MIB groups are available

-- within this module, and which MIB objects are a part of each group

wranIfSmMibGroups				OBJECT IDENTIFER

::= { wranIfSmMib 11 }

wranIfSmPendingBlmReqGroup		OBJECT IDENTIFIER

::= { wranIfSmMibGroups 1 }

wranIfSmBlmRepGroup			OBJECT IDENTIFIER

::= { wranIfSmMibGroups 2 }

wranIfSmChClassificationGroup		OBJECT IDENTIFIER

::= { wranIfSmMibGroups 3 }

wranIfSmChannelSetGroup			OBJECT IDENTIFIER

::= { wranIfSmMibGroups 4 }

wranIfSmCurrentStatusGroup		OBJECT IDENTIFIER

::= { wranIfSmMibGroups 5 }

wranIfSmRegTrackingGroup		OBJECT IDENTIFIER

::= { wranIfSmMibGroups 6 }

wranIfSmNotificationsGroup		OBJECT IDENTIFIER

::= { wranIfSmMibGroups 7 }

wranIfSmPendingBlmReqGroup		OBJECT-GROUP

OBJECTS	{ wranIfSmPendingBlmReqIndex,

wranIfSmPendingBlmReqTransactionId,

wranIfSmPendingBlmReqMsg, wranIfSmPendingBlmRspReceived,

wranIfSmPendingBlmRspMulticastReceived,

wranIfSmPendingBlmRepTimeout,

wranIfSmPendingBlmRepReceived,

wranIfSmPendingBlmRepMulticastReceived,

wranIfSmPendingBlmRepAck }

STATUS	current

DESCRIPTION

"This group contains objects related to tracking pending

BLM-REQs."

::= { wranIfSmMibGroups 1 }

wranIfSmBlmRepGroup		OBJECT-GROUP

OBJECTS	{ wranIfSmBlmRepIndex, wranIfSmBlmRepSid,

wranIfSmBlmRepTransactionId,

wranIfSmPendingBlmRspReceived, wranIfSmBlmRepMsg }

STATUS	current

DESCRIPTION

"This group contains objects related to tracking received

BLM-REPs."

::= { wranIfSmMibGroups 2 }

wranIfSmChClassificationGroup		OBJECT-GROUP

OBJECTS	{ wranIfSmChClassificationStatusIndex,

wranIfSmManagedChannel, wranIfSmManagedChannelStatus,

wranIfSmManagedChannelRecentEvent }

STATUS	current

DESCRIPTION

"This group contains objects used to track how channels the

SM is managing have been classified."

::= { wranIfSmMibGroups 3 }

wranIfSmChannelSetGroup		OBJECT-GROUP

OBJECTS	{ wranIfSmChannelSetIndex,

wranIfSmSizeWranOccupiedChannelSet,

wranIfSmWranOccupiedChannelSet,

wranIfSmSizeNghbrWranBackupChannelSet,

wranIfSmNghbrWranBackupChannelSet,

wranIfSmSizeLocalPrioritySet1, wranIfSmLocalPrioritySet1,

wranIfSmSizeLocalPrioritySet2, wranIfSmLocalPrioritySet2,

wranIfSmSizeLocalPrioritySet3,

wranIfSmLocalPrioritySet3 }

STATUS	current

DESCRIPTION

"This group contains objects used to track the various

channel sets the SM operates with."

::= { wranIfSmMibGroups 4 }

wranIfSmCurrentStatusGroup		OBJECT-GROUP

OBJECTS	{ wranIfSmCurrentStatusIndex, wranIfSmCurrentState,

wranIfSmRecentEvent, wranIfSmRecentAction,

wranIfSmInitiateChannelMove, wranIfSmSelfCoexistenceMode,

wranIfSmCurrentOperatingChannel,

wranIfSmRecentSignalType, wranIfSmCurrentT47,

wranIfSmCurrentT46 }

STATUS	current

DESCRIPTION

"This group contains objects used to track the current

state of the SM."

::= { wranIfSmMibGroups 5 }

wranIfSmRegTrackingGroup		OBJECT-GROUP

OBJECTS	{ wranIfSmRegTrackingIndex, wranIfSmRegTrackingCpeSid,

wranIfSmRegTrackingCurrentT30,

wranIfSmRegTrackingLocStringSize,

wranIfSmRegTrackingLocString }

STATUS	current

DESCRIPTION

"This group contains objects the SM uses to keep track of

the location of registered CPEs."

::= { wranIfSmMibGroups 6 }

wranIfSmNotificationsGroup		OBJECT-GROUP

OBJECTS	{ wranIfSmBlmReqChangeTrap, wranIfSmBlmRepChangeTrap,

wranIfSmOccupiedChannelSetChangeTrap,

wranIfSmNghbrBackupChannelSetChangeTrap,

wranIfSmLocalPrioritySet1ChangeTrap,

wranIfSmLocalPrioritySet2ChangeTrap,

wranIfSmLocalPrioritySet3ChangeTrap,

wranIfSmCurrentStatusChangeTrap,

wranIfSmRegTrackingChangeTrap }

STATUS	current

DESCRIPTION

"This group contains notification objects related to

traps configured for the SM."

::= { wranIfSmMibGroups 7 }

wranIfSmMibCompliance	MODULE-COMPLIANCE

STATUS	current

DESCRIPTION

"MIB objects that are optional and mandatory for SM compliance."

MODULE	wranIfSmMib

MANDATORY-GROUPS	{ wranIfSmPendingBlmReqGroup, wranIfSmBlmRepGroup,

wranIfSmChClassificationGroup,

wranIfSmChannelSetGroup,

wranIfSmCurrentStatusGroup,

wranIfSmRegTrackingGroup }

-- OPTIONAL-GROUPS	{ wranIfSmNotificationsGroup }

::= { wranIfSmMib 12 }

END

    - wranIfSsaMib

    - wranIfDatabaseServiceMib

IEEE802dot22-WRAN-IF-DATABASESERVICE-MIB	::=	BEGIN

IMPORTS

MODULE-IDENTITY,

OBJECT-TYPE,

NOTIFICATION-TYPE,

Unsigned32, Integer32, Counter32,

Counter64

FROM SNMPv2-SMI

SnmpAdminString

FROM SNMP-FRAMEWORK-MIB

TEXTUAL-CONVETION,

MacAddress, RowStatus, TruthValue,

TimeStamp, DateAndTime

FROM INET-ADDRESS-MIB

OBJECT-GROUP,

MODULE-COMPLIANCE,

NOTIFICATION-GROUP

FROM SNMPV2-CONF

wranIfDatabaseServiceMib	MODULE-IDENTITY

LAST-UPDATED	"201205140000Z"	-- May 14, 2012

ORGANIZATION	"IEEE 802.22"

CONTACT-INFO

"WG E-mail:	STDS-802-22@LISTSERV.IEEE.ORG

WG Chair:	Apurva N. Mody

E-mail:	apurva.mody@ieee.org

TGa Chair/Editor:  Ranga Reddy

E-mail:		  ranga.reddy@ieee.org"

DESCRIPTION

"This material is from IEEE 802.22a-2012 Copyright (c) 2012. This

MIB Module defines managed objects related to the configuration

of access to, as well as interaction with the database service.

Objects in this module are based on IEEE 802.22-2011 and is

under

iso(1).std(0).iso8802(8802).wran(22).wranIfDatabaseServiceMib(7)"

REVISION		"201205140000Z"

DESCRIPTION

"The first version of IEEE802dot22-WRAN-IF-DATABASESERVICE-MIB."

::= {iso std(0) iso8802(8802) wran(22) 7}

wranIfBsMgmtInfoTable			OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 1 }

wranIfBsDeviceEnlistmentTable		OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 2 }

wranIfDbsChannelIndicationTable	OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 3 }

wranIfDbsAccessTable			OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 4 }

wranIfDbsTrapControl			OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 5 }

wranIfDbsTrapDefintion			OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 6 }

wranIfDbsNotificationObjectsTable	OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 7 }

wranIfDbsMibGroups			OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 8 }

wranIfDbsMibCompliance			OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 9 }

wranIfBsMgmtInfoTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfBsMgmtInfoEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object stores information regarding information on the BS

management interface the DBS can access. It is made up of one

entry, defined by wranIfBsMgmtInfoEntry."

::= { wranIfDatabaseServiceMib 1 }

wranIfBsMgmtInfoEntry	OBJECT-TYPE

SYNTAX	wranIfBsMgmtInfoEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfBsMgmtInfoTable. Each

entry is identified by wranIfBsMgmtInfoIndex."

INDEX	{ wranIfBsMgmtInfoIndex }

::= { wranIfBsMgmtInfoTable 1 }

wranIfBsMgmtInfoEntry	::= SEQUENCE {

wranIfBsMgmtInfoIndex			INTEGER,

wranIfBsSizeMgmtUrl			INTEGER,

wranIfBsMgmtUrl				OCTET STRING,

wranIfBsSizeMgmtDeviceId		INTEGER,

wranIfBsMgmtDeviceId			OCTET STRING,

wranIfBsSizeMgmtSn			INTEGER,

wranIfBsMgmtSn				OCTET STRING,

wranIfBsSizeMgmtLocation		INTEGER,

wranIfBsMgmtLocation			OCTET STRING,

wranIfBsMgmtAntennaHeight		INTEGER,

wranIfBsSizeMgmtContactName		INTEGER,

wranIfBsMgmtContactName			OCTET STRING,

wranIfBsSizeMgmtContactPhysAddress	INTEGER,

wranIfBsMgmtContactPhysAddress	OCTET STRING,

wranIfBsSizeMgmtEmailAddress		INTEGER,

wranIfBsMgmtEmailAddress		OCTET STRING,

wranIfBsSizeMgmtPhoneNumber		INTEGER,

wranIfBsMgmtPhoneNumber			OCTET STRING,

wranIfBsSizeAccessUrl			INTEGER,

wranIfBsAccessUrl				OCTET STRING }

wranIfBsMgmtInfoIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..1)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table, defaults to 1."

::= { wranIfBsMgmtInfoEntry 1 }

wranIfBsSizeMgmtUrl	OBJECT-TYPE

SYNTAX	INTEGER (2..512)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Base Station Management URL, see 10.7.1.1"

::= { wranIfBsMgmtInfoEntry 2 }

wranIfBsMgmtUrl	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfBsSizeMgmtUrl))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Base Station Management URL, see 10.7.1.1"

::= { wranIfBsMgmtInfoEntry 3 }

wranIfBsSizeMgmtDeviceId	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of BS FCC Device ID (see 10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 4 }

wranIfBsMgmtDeviceId	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfBsSizeMgmtDeviceId))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"BS FCC Device ID (see 10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 5 }

wranIfBsSizeMgmtSn	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of BS serial number (see 10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 5 }

wranIfBsMgmtSn	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfBsSizeMgmtSn))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"BS serial number (see 10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 6 }

wranIfBsSizeMgmtLocation	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"BS Location data string of BS (see 10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 7 }

wranIfBsMgmtLocation	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfBsSizeMgmtLocation))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"BS Location data string of BS (see 10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 8 }

wranIfBsMgmtAntennaHeight	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

UNITS		"meters"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

" Antenna height at the BS (see 10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 9 }

wranIfBsSizeMgmtContactName	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Contact Name for person(s) who has ownership of the BS

(see 10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 10 }

wranIfBsMgmtContactName	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfBsSizeMgmtContactName))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Contact Name for person(s) who has ownership of the BS (see

10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 11 }

wranIfBsSizeMgmtContactPhysAddress	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Physical address for contacting the owner of the BS (see

10.7.1.1). "

::= { wranIfBsMgmtInfoEntry 12 }

wranIfBsMgmtContactPhysAddress	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfBsSizeMgmtContactPhysAddress))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Physical address for contacting the owner of the BS (see

10.7.1.1). "

::= { wranIfBsMgmtInfoEntry 13 }

wranIfBsSizeMgmtEmailAddress	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of E-mail address for contacting the owner of the BS (see

10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 14 }

wranIfBsMgmtEmailAddress	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfBsSizeMgmtEmailAddress))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"E-mail address for contacting the owner of the BS (see

10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 15 }

wranIfBsSizeMgmtPhoneNumber	OBJECT-TYPE

SYNTAX	INTEGER (2..128)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Telephone # for contacting the owner of the BS (see

10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 16 }

wranIfBsMgmtPhoneNumber	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfBsSizeMgmtPhoneNumber))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Telephone # for contacting the owner of the BS (see

10.7.1.1)."

::= { wranIfBsMgmtInfoEntry 17 }

wranIfBsSizeAccessUrl	OBJECT-TYPE

SYNTAX	INTEGER (2..512)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Base Station Database Access Service URL, see 10.7.1.1"

::= { wranIfBsMgmtInfoEntry 18 }

wranIfBsAccessUrl	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfBsSizeAccessUrl))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Base Station Database Access Service URL, see 10.7.1.1"

::= { wranIfBsMgmtInfoEntry 19 }

wranIfDbsDeviceEnlistmentTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfDbsDeviceEnlistmentEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object stores information regarding information on devices

entering the network that the BS has attempted to

enlist/register with the database service. It is made up of

multiple values, defined by wranIfDbsDeviceEnlistmentEntry"

::= { wranIfDatabaseServiceMib 2 }

wranIfDbsDeviceEnlistmentEntry	OBJECT-TYPE

SYNTAX	wranIfDbsDeviceEnlistmentEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfDbsDeviceEnlistmentTable.

Each entry is identified by wranIfDbsDeviceEnlistmentIndex."

INDEX	{ wranIfDbsDeviceEnlistmentIndex }

::= { wranIfDbsDeviceEnlistmentTable 1 }

wranIfBsDeviceEnlistmentEntry	::= SEQUENCE {

wranIfDbsDeviceEnlistmentIndex			INTEGER,

wranIfDbsDeviceEnlistmentConfirmed			TruthValue,

wranIfDbsConfirmedDeviceType				INTEGER,

wranIfDbsSizeConfirmedDevceID				INTEGER,

wranIfDbsConfirmedDeviceId				OCTET STRING,

wranIfDbsSizeConfirmedDeviceSn			INTEGER,

wranIfDbsConfirmedDeviceSn				OCTET STRING,

wranIfDbsSizeConfirmedDeviceLocation		INTEGER,

wranIfDbsConfirmedDeviceLocation			OCTET STRING,

wranIfDbsSizeProxyDeviceId				INTEGER,

wranIfDbsProxyDeviceId					OCTET STRING,

wranIfDbsSizeProxySn					INTEGER,

wranIfDbsProxySn						OCTET STRING,

wranIfDbsSizeRespPartyName				INTEGER,

wranIfDbsRespPartyName					OCTET STRING,

wranIfDbsConfirmedDeviceAntennaHeight		INTEGER,

wranIfDbsSizeConfirmedDeviceContactName		INTEGER,

wranIfDbsConfirmedDeviceContactName			OCTET STRING,

wranIfDbsSizeConfirmedDeviceContactPhysAddress	INTEGER,

wranIfDbsConfirmedDeviceContactPhysAddress	OCTET STRING,

wranIfDbsSizeConfirmedDeviceEmailAddress		INTEGER,

wranIfDbsConfirmedDeviceEmailAddress		OCTET STRING,

wranIfDbsSizeConfirmedDevicePhoneNumber		INTEGER,

wranIfDbsConfirmedDevicePhoneNumber			OCTET STRING,

wranIfDbsConfirmedDeviceAntennaInformation	OCTET STRING,

wranIfDbsConfirmedDeviceAntennaAzimuth		INTEGER,

wranIfDbsDeviceConfirmationMsgTime			DateAndTime }

wranIfDbsDeviceEnlistmentIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..255)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table."

::= { wranIfDbsDeviceEnlistmentEntry 1 }

wranIfDbsDeviceEnlistmentConfirmed	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Truth(1) indicates this enlistment been confirmed via receipt of

M-DEVICE-ENLISTMENT-CONFIRM (see  10.7.1.4) from the database

service."

::= { wranIfDbsDeviceEnlistmentEntry 2 }

wranIfDbsConfirmedDeviceType	OBJECT-TYPE

SYNTAX	INTEGER { fixedBS(0),

fixeCpe(1),

personal-portable(2) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Type of device being registered, fixed BS(0), fixed CPE(1),

personal/portable(2) (see 10.7.1.4). 3-255 are reserved."

::= { wranIfDbsDeviceEnlistmentEntry 3 }

wranIfDbsSizeConfirmedDeviceId	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Device ID for the device that is being

enlisted/registered with the database service."

::= { wranIfDbsDeviceEnlistmentEntry 4 }

wranIfDbsConfirmedDeviceId	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeConfirmedDeviceId))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Device ID for the device that is being enlisted/registered with

the database service."

::= { wranIfDbsDeviceEnlistmentEntry 5 }

wranIfDbsSizeConfirmedDeviceSn	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Serial number of the device that is being

enlisted/registered with the database service. "

::= { wranIfDbsDeviceEnlistmentEntry 6 }

wranIfDbsConfirmedDeviceSn	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeConfirmedDeviceSn))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Serial number of the device that is being enlisted/registered

with the database service. "

::= { wranIfDbsDeviceEnlistmentEntry 7 }

wranIfDbsSizeConfirmedDeviceLocation	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Location string of the device requesting enlistment."

::= { wranIfDbsDeviceEnlistmentEntry 8 }

wranIfDbsConfirmedDeviceLocation	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeConfirmedDeviceLocation))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Location string of the device requesting enlistment."

::= { wranIfDbsDeviceEnlistmentEntry 9 }

wranIfDbsSizeProxyDeviceId	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Device ID of proxy device BS may use to send queries to

the database. "

::= { wranIfDbsDeviceEnlistmentEntry 10 }

wranIfDbsProxyDeviceId	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeProxyDeviceId))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Device ID of proxy device BS may use to send queries to the

database. "

::= { wranIfDbsDeviceEnlistmentEntry 11 }

wranIfDbsSizeProxySn	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Serial number of proxy device BS may use to send queries

to the database "

::= { wranIfDbsDeviceEnlistmentEntry 12 }

wranIfDbsProxySn	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(2..256))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Serial number of proxy device BS may use to send queries to the

database "

::= { wranIfDbsDeviceEnlistmentEntry 13 }

wranIfDbsSizeRespPartyName	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Name of the party responsible for the device

enlistment/registration"

::= { wranIfDbsDeviceEnlistmentEntry 14 }

wranIfDbsRespPartyName	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeRespPartyName))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Name of the party responsible for the device

enlistment/registration"

::= { wranIfDbsDeviceEnlistmentEntry 15 }

wranIfDbsConfirmedDeviceAntennaHeight	OBJECT-TYPE

SYNTAX	INTEGER (0..100)

UNITS		"meters"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

" Antenna height of the device being enlisted."

::= { wranIfDbsDeviceEnlistmentEntry 16 }

wranIfDbsSizeConfirmedDeviceContactName	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Contact Name for person(s) who has ownership of device,

only pertinent if device type is fixed BS or CPE."

::= { wranIfDbsDeviceEnlistmentEntry 17 }

wranIfDbsConfirmedDeviceContactName	OBJECT-TYPE

SYNTAX	OCTET STRING

(SIZE(wranIfDbsSizeConfirmedDeviceContactName))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Contact Name for person(s) who has ownership of device, only

pertinent if device type is fixed BS or CPE."

::= { wranIfDbsDeviceEnlistmentEntry 18 }

wranIfDbsSizeConfirmedDeviceContactPhysAddress	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Physical address for contacting the owner of the BS; the

only pertinent device types are fixed BS and fixed CPE."

::= { wranIfDbsDeviceEnlistmentEntry 19 }

wranIfDbsConfirmedDeviceContactPhysAddress	OBJECT-TYPE

SYNTAX 	OCTET STRING

(SIZE(wranIfDbsSizeConfirmedDeviceContactPhysAddress))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Physical address for contacting the owner of the BS; the only

pertinent device types are fixed BS and fixed CPE."

::= { wranIfDbsDeviceEnlistmentEntry 20 }

wranIfDbsSizeConfirmedDeviceEmailAddress	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of E-mail address for contacting the owner of the BS; the

only pertinent device types are fixed BS and fixed CPE."

::= { wranIfDbsDeviceEnlistmentEntry 21 }

wranIfDbsConfirmedDeviceEmailAddress	OBJECT-TYPE

SYNTAX	OCTET STRING

(SIZE(wranIfDbsSizeConfirmedDeviceEmailAddress))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"E-mail address for contacting the owner of the BS; the only

pertinent device types are fixed BS and fixed CPE."

::= { wranIfDbsDeviceEnlistmentEntry 22 }

wranIfDbsSizeConfirmedDevicePhoneNumber	OBJECT-TYPE

SYNTAX	INTEGER (2..128)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Telephone number for contacting the owner of the BS; the

only pertinent device types are fixed BS and fixed CPE."

::= { wranIfDbsDeviceEnlistmentEntry 23 }

wranIfDbsConfirmedDevicePhoneNumber	OBJECT-TYPE

SYNTAX	OCTET STRING

(SIZE(wranIfDbsSizeConfirmedDevicePhoneNumber))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Telephone number for contacting the owner of the BS; the only

pertinent device types are fixed BS and fixed CPE."

::= { wranIfDbsDeviceEnlistmentEntry 24 }

wranIfDbsConfirmedDeviceAntennaInformation	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(72))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Antenna information of device; only applicable if transmission

of antenna information is supported by the database service."

::= { wranIfDbsDeviceEnlistmentEntry 25 }

wranIfDbsConfirmedDeviceAntennaAzimuth	OBJECT-TYPE

SYNTAX	INTEGER (0..360)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Antenna azimuth of device; only applicable if transmission of

antenna information is supported by the database service."

::= { wranIfDbsDeviceEnlistmentEntry 26 }

wranIfDbsDeviceConfirmationMsgTime	OBJECT-TYPE

SYNTAX	DateAndTime

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

" Timestamp of transmission for M-DEVICE-ENLISTMENT-REQUEST"

::= { wranIfDbsDeviceEnlistmentEntry 27 }

wranIfDbsChannelIndicationTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfDbsChannelIndicationEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object stores information what channels have been indicated

(upon receipt of M-DB-AVAILABLE-CHANNEL-INDICATION) as available

and their EIRP limit at a given location for a particular device.

It is made up of multiple entries, one each for the tuple of

location||channel||EIRP||DeviceID. Each entry is defined by

wranIfDbsChannelIndicationEntry."

::= { wranIfDatabaseServiceMib 3 }

wranIfDbsChannelIndicationEntry	OBJECT-TYPE

SYNTAX	wranIfDbsChannelIndicationEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfDbsChannelIndicationTable.

Each entry is identified by wranIfDbsChannelIndicationIndex."

INDEX	{ wranIfDbsChannelIndicationIndex }

::= { wranIfDbsChannelIndicationTable 1 }

wranIfDbsChannelIndicationEntry	::= SEQUENCE {

wranIfDbsChannelIndicationIndex	INTEGER,

wranIfDbsSizeDeviceId			INTEGER,

wranIfDbsDeviceId				OCTET STRING,

wranIfDbsSizeDeviceSn			INTEGER,

wranIfDbsDeviceSn				OCTET STRING,

wranIfDbsDeviceChannelNumber		INTEGER,

wranIfDbsDeviceMaxAllowedEirp		INTEGER,

wranIfDbsSizeDeviceLocation		INTEGER,

wranIfDbsDeviceLocation			OCTET STRING,

wranIfDbsDeviceDbsIndex			INTEGER }

wranIfDbsChannelIndicationIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..10000)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table."

::= { wranIfDbsChannelIndicationEntry 1 }

wranIfDbsSizeDeviceId	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Device ID of device for which the channel is indicated

as available."

::= { wranIfDbsChannelIndicationEntry 2 }

wranIfDbsDeviceId	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeDeviceId))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Device ID of device for which the channel is indicated as

available."

::= { wranIfDbsChannelIndicationEntry 3 }

wranIfDbsSizeDeviceSn	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Serial number of device for which the channel is indicated as

available."

::= { wranIfDbsChannelIndicationEntry 4 }

wranIfDbsDeviceSn	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeDeviceSn))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Serial number of device for which the channel is indicated as

available."

::= { wranIfDbsChannelIndicationEntry 5 }

wranIfDbsDeviceChannelNumber	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Channel number for which availability is indicated."

::= { wranIfDbsChannelIndicationEntry 6 }

wranIfDbsDeviceMaxAllowedEirp	OBJECT-TYPE

SYNTAX	INTEGER (0..255)

UNITS		"dBm"

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Maximum allowed EIRP on the channel. (between -64dBm - +63.5 dBm

in 0.5 dB steps)."

::= { wranIfDbsChannelIndicationEntry 7 }

wranIfDbsSizeDeviceLocation	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Location string of the device requesting enlistment."

::= { wranIfBsDeviceEnlistmentEntry 8 }

wranIfDbsDeviceLocation	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeDeviceLocation))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Location string of the device requesting enlistment."

::= { wranIfBsDeviceEnlistmentEntry 9 }

wranIfDbsDeviceDbsIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..20)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Index of entry in wranIfDbsAccessTable (denoted by

wranIfDbsAccessEntryIndex) that corresponds to the database

service that provided this channel indication."

::= { wranIfDbsChannelIndicationEntry 10 }

wranIfDbsAccessTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfDbsAccessEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object stores information regarding access information for

accessing a database service. There is more than one entry in

this table, one for each database service that is available.

Each entry is defined by wranIfDbsAccessEntry."

::= { wranIfDatabaseServiceMib 4 }

wranIfDbsAccessEntry	OBJECT-TYPE

SYNTAX	wranIfDbsAccessEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This object defines an entry in wranIfDbsAccessTable.

Each entry is identified by wranIfDbsAccessEntryIndex."

INDEX	{ wranIfDbsAccessEntryIndex }

::= { wranIfDbsAccessTable 1 }

wranIfDbsAccessEntry		::= SEQUENCE {

wranIfDbsAccessEntryIndex		INTEGER,

wranIfDbsSizeAccessUrl			INTEGER,

wranIfDbsAccessUrl			OCTET STRING,

wranIfDbsAccessCredentialType		INTEGER,

wranIfDbsSizeAccessCertCredential	INTEGER,

wranIfDbsAccessCertCredential		OCTET STRING,

wranIfDbsSizeAccessUserID		INTEGER,

wranIfDbsAccessUserID			OCTET STRING,

wranIfDbsSizeAccessPassword		INTEGER,

wranIfDbsAccessPassword			OCTET STRING,

wranIfDbsAccessLastTxTime		DateAndTime,

wranIfDbsAccessLastRxTime		DateAndTime,

wranIfDbsAccessAntennaInfoRequired	TruthValue,

wranIfDbsAccessPriority			INTEGER }

wranIfDbsAccessEntryIndex	OBJECT-TYPE

SYNTAX	INTEGER (1..20)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Index of entry in the table."

::= { wranIfDbsAccessEntry 1 }

wranIfDbsSizeAccessUrl	OBJECT-TYPE

SYNTAX	INTEGER (2..512)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"URL used to access database service. "

::= { wranIfDbsAccessEntry 2 }

wranIfDbsAccessUrl	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeAccessUrl))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"URL used to access database service."

::= { wranIfDbsAccessEntry 3 }

wranIfDbsAccessCredentialType	OBJECT-TYPE

SYNTAX	INTEGER { useridAndpassword(0),

certificate(1) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Indication of the type of credential to be used when

authenticating access and transfer of information to the

database service (0x00=userIdAndpassword, 0x01=certificate,

0x02-0xFF=reserved"

::= { wranIfDbsAccessEntry 4 }

wranIfDbsSizeAccessCertCredential	OBJECT-TYPE

SYNTAX	INTEGER (2..10000)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of Certificate Credential BS or proxy device used to

authenticate access to the database service, e.g., password,

certificate. Only pertinent when wranIfDbsAccessCredentialType

set to 0x01."

::= { wranIfDbsAccessEntry 5 }

wranIfDbsAccessCertCredential	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeAccessCertCredential))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Certificate Credential BS or proxy device used to authenticate

access to the database service, e.g., password, certificate.

Only pertinent when wranIfDbsAccessCredentialType set to 0x01."

::= { wranIfDbsAccessEntry 6 }

wranIfDbsSizeAccessUserID	OBJECT-TYPE

SYNTAX	INTEGER (2..100)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of login User ID used for accessing database service. Only

pertinent when wranIfDbsAccessCredentialType set to 0x00."

::= { wranIfDbsAccessEntry 7 }

wranIfDbsAccessUserID	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeAccessUserID))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Login User ID used for accessing database service. Only

pertinent when wranIfDbsAccessCredentialType set to 0x00."

::= { wranIfDbsAccessEntry 8 }

wranIfDbsSizeAccessPassword	OBJECT-TYPE

SYNTAX	INTEGER (6..512)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Size of login password (or hash of password) used for accessing

database service. Only pertinent when

wranIfDbsAccessCredentialType set to 0x00."

::= { wranIfDbsAccessEntry 9 }

wranIfDbsAccessPassword	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfDbsSizeAccessPassword))

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Login password (or hash of password) used for accessing database

service. Only pertinent when wranIfDbsAccessCredentialType set

to 0x00."

::= { wranIfDbsAccessEntry 10 }

wranIfDbsAccessLastTxTime	OBJECT-TYPE

SYNTAX	DateAndTime

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Time indication of the last transmission of a message to the

database service."

::= { wranIfDbsAccessEntry 11 }

wranIfDbsAccessLastRxTime	OBJECT-TYPE

SYNTAX	DateAndTime

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Time indication of the last reception of a message from the

database service."

::= { wranIfDbsAccessEntry 12 }

wranIfDbsAccessAntennaInfoRequired	OBJECT-TYPE

SYNTAX	TruthValue

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Truth(1), indication of whether or not antenna info is required

to be transmitted to database service. "

::= { wranIfDbsAccessEntry 13 }

wranIfDbsAccessPriority	OBJECT-TYPE

SYNTAX	INTEGER (1..100)

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Priority of database service. Higher priority means that a

particular database service will be preferred when device

enlistment and channel indication requests need to be made."

::= { wranIfDbsAccessEntry 14 }

wranIfDbsTrapControl	OBJECT-TYPE

SYNTAX	BITS	{ wranIfDbsAvailibilityChange(0),

wranIfDbsEnlistmentChange(1),

wranIfDbsChannelIndicationChange(2) }

MAX-ACCESS	read-write

STATUS	current

DESCRIPTION

"Defines control elements for traps related to interaction with

the Database Service. This is a 3bit field that enables setting

a trap for particular CPE events: wranIfDbsAvailabilityChange,

wranIfDbsEnlistmentChange, and

wranIfDbsChannelIndicationChange."

::= { wranIfDatabaseServiceMib 5 }

wranIfDbsTrapDefinition			 OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 6 }

wranIfDbsAvailabilityChangeTrap	 OBJECT IDENTIFIER

::= { wranIfDbsTrapDefinition 1 }

wranIfDbsEnlistmentChangeTrap		 OBJECT IDENTIFIER

::= { wranIfDbsTrapDefinition 2 }

wranIfDbsChannelIndicationChangeTrap OBJECT IDENTIFIER

::= { wranIfDbsTrapDefinition 3 }

wranIfDbsAvailabilityChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfDbsSizeAccessUrl, wranIfDbsAccessUrl,

wranIfDbsAccessStatus }

STATUS	current

DESCRIPTION

"This trap contains the information related to the last time a

particular database service was accessed."

::= { wranIfDbsTrapDefinition 1 }

wranIfDbsEnlistmentChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfDbSizeConfirmedDeviceId,

wranIfDbsConfirmedDeviceId,

wranIfDbsSizeConfirmedDeviceSn,

wranIfDbsConfirmedDeviceSn,

wranIfDbsConfirmedDeviceStatus }

STATUS	current

DESCRIPTION

"This trap contains information regarding the status of

enlistment confirmation for a particular device in the network."

::= { wranIfDbsTrapDefinition 2 }

wranIfDbsChannelIndicationChangeTrap	NOTIFICATION-TYPE

OBJECTS	{ wranIfDbsSizeDeviceId, wranIfDbsDeviceId,

wranIfDbsSizeDeviceSn, wranIfDbsSizeDeviceSn,

wranIfDbsChannelIndicationStatus }

STATUS	current

DESCRIPTION

"This trap contains information regarding the status of channel

indication responses from the database service."

::= { wranIfDbsTrapDefinition 3 }

wranIfDbsNotificationObjectsTable	OBJECT-TYPE

SYNTAX	SEQUENCE OF wranIfDbsNotificationObjectsEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"This MIB provides a table to track notification objects that

have been reported by the traps related to access to the

database. It is made up of one entry, containing the objects

related to the most recent trap/event. The entry is defined by

wranIfDbsNotificationObjectsEntry."

::= { wranIfDatabaseServiceMib 7 }

wranIfDbsNotificationObjectsEntry	OBJECT-TYPE

SYNTAX	wranIfDbsNotificationObjectsEntry

MAX-ACCESS	not-accessible

STATUS	current

DESCRIPTION

"Definition of an entry into wranIfDbsNotificationObjectsTable.

Each entry is identified by wranIfNotificationDeviceId."

INDEX { wranIfNotificationDeviceId }

::= { wranIfDbsNotificationObjectsTable 1 }

wranIfDbsNotificationObjectsEntry	::= SEQUENCE {

wranIfNotificationSizeDeviceId	INTEGER,

wranIfNotificationDeviceId		OCTET STRING,

wranIfNotificationSizeDeviceSn	INTEGER,

wranIfNotificationDeviceSn		OCTET STRING,

wranIfNotificationDbsSizeAccessUrl	INTEGER,

wranIfNotificationDbsAccessUrl	OCTEG STRING,

wranIfDbsAccessStatus			INTEGER,

wranIfDbsConfirmedDeviceStatus	INTEGER,

wranIfDbsChannelIndicationStatus	INTEGER }

wranIfNotificationSizeDeviceId	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Size of Device ID of device generating the trap."

::= { wranIfDbsNotificationObjectsEntry 1 }

wranIfNotificationDeviceId	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfNotificationSizeDeviceId))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Device ID of device generating the trap."

::= { wranIfDbsNotificationObjectsEntry 2 }

wranIfNotificationSizeDeviceSn	OBJECT-TYPE

SYNTAX	INTEGER (2..256)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Size of Serial number of the device generating the trap."

::= { wranIfDbsNotificationObjectsEntry 3 }

wranIfNotificationDeviceSn	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfNotificationSizeDeviceSn))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Serial number of the device generating the trap."

::= { wranIfDbsNotificationObjectsEntry 4 }

wranIfNotificationDbsSizeAccessUrl	OBJECT-TYPE

SYNTAX	INTEGER (2..512)

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"URL of database service whose connectivity update was captured

by the trap. "

::= { wranIfDbsNotificationObjectsEntry 5 }

wranIfNotificationDbsAccessUrl	OBJECT-TYPE

SYNTAX	OCTET STRING (SIZE(wranIfNotificationDbsSizeAccessUrl))

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"URL of database service whose connectivity update was captured

by the trap. "

::= { wranIfDbsNotificationObjectsEntry 6 }

wranIfDbsAccessStatus	OBJECT-TYPE

SYNTAX	INTEGER { messageRxFromDatabase(0),

messageTxToDatabase (1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication of whether or not a message is transmitted or

received from database service."

::= { wranIfDbsNotificationObjectsEntry 7 }

wranIfDbsConfirmedDeviceStatus	OBJECT-TYPE

SYNTAX	INTEGER {  deviceNotConfirmed(0),

deviceConfirmed(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication of whether or not the enlistment of the device has

been successfully completed or not."

::= { wranIfDbsNotificationObjectsEntry 8 }

wranIfDbsChannelIndicationStatus	OBJECT-TYPE

SYNTAX	INTEGER { channelsNotAvailable(0),

channelsAvailable(1) }

MAX-ACCESS	read-only

STATUS	current

DESCRIPTION

"Indication of whether channels have been indicated as available

or not for the device in question."

::= { wranIfDbsNotificationObjectsEntry 9 }

-- wranIfDbsMibGroups: this object helps define which MIB groups are available

-- within this module, and which MIB objects are part of each group.

wranIfDbsMibGroups			OBJECT IDENTIFIER

::= { wranIfDatabaseServiceMib 8 }

wranIfDbsMgmtGroup			OBJECT IDENTIFIER

::= { wranIfDbsMibGroups 1 }

wranIfDbsEnlistmentGroup		OBJECT IDENTIFIER

::= { wranIfDbsMibGroups 2 }

wranIfDbsChannelIndicationGroup	OBJECT IDENTIFIER

::= { wranIfDbsMibGroups 3 }

wranIfDbsAccessGroup			OBJECT IDENTIFIER

::= { wranIfDbsMibGroups 4 }

wranIfDbsNotificationsGroup		OBJECT IDENTIFIER

::= { wranIfDbsMibGroups 5 }

wranIfDbsMgmtGroup		OBJECT-GROUP

OBJECTS	{ wranIfBsMgmtInfoIndex, wranIfBsSizeMgmtUrl,

wranIfBsMgmtUrl, wranIfBsSizeMgmtDeviceId,

wranIfBsMgmtDeviceId, wranIfBsSizeMgmtSn, wranIfBsMgmtSn,

wranIfBsSizeMgmtLocation, wranIfBsMgmtLocation,

wranIfBsMgmtAntennaHeight, wranIfBsSizeMgmtContactName,

wranIfBsMgmtContactName,

wranIfBsSizeMgmtContactPhysAddress,

wranIfBsMgmtContactPhysAddress,

wranIfBsSizeMgmtEmailAddress, wranIfBsMgmtEmailAddress,

wranIfBsSizeMgmtPhoneNumber, wranIfBsMgmtPhoneNumber,

wranIfBsSizeAccessUrl, wranIfBsAccessUrl }

STATUS	current

DESCRIPTION

"This group contains configuration objects related to the

management interface on the BS that DBS can access."

::= { wranIfDbsMibGroups 1 }

wranIfDbsEnlistmentGroup		OBJECT-GROUP

OBJECTS	{ wranIfDbsDeviceEnlistmentIndex,

wranIfDbsDeviceEnlistmentConfirmed,

wranIfDbsConfirmedDeviceType,

wranIfDbsSizeConfirmedDevceID,

wranIfDbsConfirmedDeviceId,

wranIfDbsSizeConfirmedDeviceSn,

wranIfDbsConfirmedDeviceSn,

wranIfDbsSizeConfirmedDeviceLocation,

wranIfDbsConfirmedDeviceLocation,

wranIfDbsSizeProxyDeviceId, wranIfDbsProxyDeviceId,

wranIfDbsSizeProxySn, wranIfDbsProxySn,

wranIfDbsSizeRespPartyName, wranIfDbsRespPartyName,

wranIfDbsConfirmedDeviceAntennaHeight,

wranIfDbsSizeConfirmedDeviceContactName,

wranIfDbsConfirmedDeviceContactName,

wranIfDbsSizeConfirmedDeviceContactPhysAddress,

wranIfDbsConfirmedDeviceContactPhysAddress,

wranIfDbsSizeConfirmedDeviceEmailAddress,

wranIfDbsConfirmedDeviceEmailAddress,

wranIfDbsSizeConfirmedDevicePhoneNumber,

wranIfDbsConfirmedDevicePhoneNumber,

wranIfDbsConfirmedDeviceAntennaInformation,

wranIfDbsConfirmedDeviceAntennaAzimuth,

wranIfDbsDeviceConfirmationMsgTime }

STATUS	current

DESCRIPTION

"This group contains configuration objects related to

enlistment of devices."

::= { wranIfDbsMibGroups 2 }

wranIfDbsChannelIndicationGroup		OBJECT-GROUP

OBJECTS	{ wranIfDbsChannelIndicationIndex, wranIfDbsSizeDeviceId,

wranIfDbsDeviceId, wranIfDbsSizeDeviceSn,

wranIfDbsDeviceSn, wranIfDbsDeviceChannelNumber,

wranIfDbsDeviceMaxAllowedEirp,

wranIfDbsSizeDeviceLocation, wranIfDbsDeviceLocation,

wranIfDbsDeviceDbsIndex }

STATUS	current

DESCRIPTION

"This group contains configuration objects related to

indication of available channels."

::= { wranIfDbsMibGroups 3 }

wranIfDbsAccessGroup		OBJECT-GROUP

OBJECTS	{ wranIfDbsAccessEntryIndex, wranIfDbsSizeAccessUrl,

wranIfDbsAccessUrl, wranIfDbsAccessCredentialType,

wranIfDbsSizeAccessCertCredential,

wranIfDbsAccessCertCredential, wranIfDbsSizeAccessUserID,

wranIfDbsAccessUserID, wranIfDbsSizeAccessPassword,

wranIfDbsAccessPassword, wranIfDbsAccessLastTxTime,

wranIfDbsAccessLastRxTime,

wranIfDbsAccessAntennaInfoRequired,

wranIfDbsAccessPriority }

STATUS	current

DESCRIPTION

"This group contains configuration objects related to

current accessibility of the database service."

::= { wranIfDbsMibGroups 4 }

wranIfDbsNotificationsGroup		OBJECT-GROUP

OBJECTS	{ wranIfDbsAvailabilityChangeTrap,

wranIfDbsEnlistmentChangeTrap,

wranIfDbsChannelIndicationChangeTrap }

STATUS	current

DESCRIPTION

"This group contains notification objects related to

traps configured for access to the database service."

::= { wranIfDbsMibGroups 5 }

wranIfDbsMibCompliance	MODULE-COMPLIANCE

STATUS	current

DESCRIPTION

"MIB objects that are optional and mandatory for database service

access compliance."

MODULE	wranIfDatabaseServiceMib

MANDATORY-GROUPS	{ wranIfDbsMgmtGroup, wranIfDbsEnlistmentGroup,

wranIfDbsChannelIndicationGroup,

wranIfDbsAccessGroup }

-- OPTIONAL-GROUPS	{ wranIfDbsNotificationsGroup }

::= { wranIfDatabaseServiceMib 9 }

END

References: