IEEE P802.22
Wireless RANs

| IEEE 802.22 Requirements for IETF PAWS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-10-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Charles W. Einolf, Jr. | NABA | 3007 Argentina Place Mitchellville, MD 20716 USA | +1-301-806-1544 | c.einolf@ieee.org |
| Victor Tawil | MSTV | 4100 Wisconsin Avenue Washington, DC 20016 | +1-202-966-1956 | vtawil@mstv.org |

Abstract

IEEE 802.22 Requirements for IETF PAWS

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Requirements

D. Data Model Requirements:

D.1: The Data Model MUST support specifying the

- Antenna Parameters

- Height Parameters (From device to the database)

  - mandatory AGL in meters: Unsigned INT, 1 Byte (0 – 255 m)

  - Our assumption is that the database will compute the HAAT in meters from the AGL and (lat, long)

- Gain Parameters (From the device to the database)

  - Antenna directionality information of the device in dB: relative to the main lobe maximum gain for every 5 degree azimuth clockwise starting from the direction of the maximum antenna gain expressed in unit of 0.25 dB over the range –63.75 dB (encoded 0x00) to 0 dB (0xFF). Character String	72 bytes

  - Antenna azimuth	Integer	2 bytes	Antenna azimuth in degrees, clockwise from true North.

- RF Mask Parameters (From the device to the database)

  - Regulatory Domain (3 ASCII Letters), Device Type (Regulatory Class e. g. Fixed, Personal Portable, Mobile etc) (INT), Mask Number Index (Long INT) (where Mask Number Index corresponds to a particular RF Mask of an equipment that is stored in the database and that has passed the regulatory certification)

D.2: The data model MUST support specifying an ID of the subject. This ID would be the ID of the device used to be certified by a regulatory

body for a regulatory domain

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device at the geo-location registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 0x03-0xFF = Reserved |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |

D.3: The Data Model MUST support specifying the location of the subject

and accuracy of location determination

| Name | Type |  | Length |  |  | Description |
| --- | --- | --- | --- | --- | --- | --- |
| Location Data String Length | Integer |  | 2 bytes |  |  | Length of Location Data String field (# of characters) |
| Location Data String | Character String |  | NMEA 0183 Character String |  |  | The value identifies the location of the device (latitude, longitude).20F |
| Timestamp Length | Integer |  | 2 bytes |  |  | Length of Timestamp field (# of characters) |
| Timestamp |  | Character String |  | NMEA 0183 $ZDA string | Timestamp of the present request at time of transmission and as encoded in the $ZDA substring of the NMEA 0183 string. |  |

NMEA GPGGA String is the subset of the NMEA 0183

D.4: The Data Model MUST support specifying a list of available channel

list and time constrains for the channel list availability.

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device- ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial number | Character String | Variable |  |
| Number of Channels Available | Integer | 1 byte |  |
| { If( Number of Channels Available > 0) |  |  | If the number of channels is equal to 0, this means that the device cannot operate. |
| For (i=1; i≤ Number of Channels Available; i++) { Channel_Number Max_Allowed_EIRP (dBm) Availability schedule } | Vector of 2xN bytes and a number of pairs of NMEA 0183 $ZDA strings | Variable | List of available channel numbers and corresponding maximum allowed EIRP expressed in dBm over the range –64 dBm (encoded 0x00) to +63.5 dBm (encoded 0xFF) as well as the availability schedule (start and stop date/time) for each channel in Universal date and time system. |
| } |  |  |  |
| Status Message | Character String | Variable | Various status messages coming from the database service (e.g., unapproved device flag) |
| Timestamp Length | Integer | 2 bytes | Length of Timestamp field (# of characters) |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-AVAILABLE-CHANNEL-REQUEST. |

D.5: The Data Model MUST support specifying the maximum output power of

the subject.

Please see above. We need the Maximum EIRP.

D.6: The Data Model MUST support specifying channel availability information for multiple locations.

802.22 BS shall act as a proxy and query the database for each device. If a query is to be grouped or made in a batch mode, all the information related to each device shall be provided to the BS (i. e. the database is not to perform the intersection for all these devices and locations).

D.7: The Data Model MUST support specifying channel availability

information for an area around a specified location.

Does not apply to the 802.22

P. Protocol Requirements:

P.1: The protocol MUST provide a mechanism for the subject to discover

the WS Database it has to use at a given location.

P.2: The protocol MUST support regulatory domain discovery.

P.3: The protocol between the master device and the WS Database MUST

support pushing updates in channel availability changes to subjects

P.4: The protocol between the master device and the WS Database MUST

support mutual authentication and authorization.

P.5: The protocol between the master device and the WS Database MUST

support integrity and confidentiality protection.

P.6: The protocol MUST support both username/password and digital

certificates based authentication.

O. Operational Requirements:

O.1: A master device MUST query the WS Database for the available

channels as often as required by the regulation (eg, FCC requires once

per day) to verify that the operating channels continue to remain available.

O.2: A master device MUST determine its location with the accuracy

required by the regulation (eg, FCC requires +/- 100m) before placing a

query to the DB

O.3: A master device which changes its location during its operation,

MUST query the WS Database for available operating channels each time

it moves more than the distance specified by the regulation (eg FCC specifies 100m) from the location it previously made the query from.

O.4: The WS Database MUST provide the available channel list when

requested and MAY also provide time constraints for the channel list

and maximum output power to the master device.

O.5: A master device MUST query the WS Database and include the FCC ID

of the slave device in the query before allowing the slave device to

use the available channel.

O.6: A master device MUST be capable to validate the digital

certificate of the WS Database.

O.7: A master device MUST be capable to check the validity of the WS

Database certificate and whether it has been revoked or not.

See footnote 3.