IEEE P802.22
Wireless RANs

| IEEE 802.22 Requirements for IETF PAWS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-10-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody |  |  |  |  |
| Gerald Chouinard |  |  |  |  |
| Ranga Reddy |  |  |  |  |

Abstract

IEEE 802.22 Requirements for IETF PAWS

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Requirements

D. Data Model Requirements:

D.1: The Data Model MUST support specifying the

- Antenna Parameters (WSD => database)

- Height Parameters

  - mandatory AGL in meters: Unsigned INT, 2 Bytes (0 – 655.36 cm)
Example: 802.22 primitive:

| Antenna height | Integer | 1 byte | Antenna height above ground level in meters. |
| --- | --- | --- | --- |

[Note: portable is defined as 1.5 m and base stations in Canada can be at up to 500 m HAAT which could be completely contributed by the antenna (e.g., mounted on a broadcast tower.)]

  - Our assumption is that the database will compute the HAAT in meters from the AGL and ground elevation at the specified (lat, long) obtained from a topographic database by the database server.

- Gain Parameters

  - Antenna directionality information in dB: relative to the main lobe maximum gain for every 5 degree azimuth clockwise starting from the direction of the maximum antenna gain expressed in unit of 0.25 dB over the range –63.75 dB (encoded 0x00) to 0 dB (0xFF):
Character String	72 bytes

  - Antenna azimuth	 in degrees, clockwise from true North: Integer	2 bytes

Example: 802.22 primitive:

| Antenna information | Character String | 72 bytes | Antenna directionality information of the device in dB relative to the main lobe maximum gain for every 5 degree azimuth clockwise starting from the direction of the maximum antenna gain expressed in unit of 0.25 dB over the range –63.75 dB (encoded 0x00) to 0 dB (0xFF). (to allow the database calculation of the channel availability and the maximum allowed EIRP values at the registering location19) |
| --- | --- | --- | --- |
| Antenna azimuth | Integer | 2 bytes | Antenna azimuth in degrees, clockwise from true North |

- RF Mask Parameters (WSD => database)

  - Regulatory Domain (3 ASCII Letters), Device Type (Regulatory Class e. g. Fixed, Personal Portable, Mobile, etc.) (INT), Mask Number Index (Long INT) (where Mask Number Index corresponds to a particular RF Mask of an equipment that is stored in the database and that has passed the regulatory certification)

Example: 802.22 primitive:

| Device Type | Integer | 1 byte | The value identifies the type of device at the geolocation registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 0x03–0xFF = Reserved |
| --- | --- | --- | --- |

D.2: The data model MUST support specifying an ID of the subject. This ID would be the ID of the device used to be certified by a regulatory body for a regulatory domain (WSD => database)

Example: 802.22 primitives:

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device at the geo-location registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 0x03-0xFF = Reserved |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |

Note: The device-ID and Serial Number could be replaced by a single universal identifier or multiple universal device identifiers which would represent parsing of a single identifier for practical purpose.

D.3: The Data Model MUST support specifying the location of the WSD, the uncertainty in meters and confidence in percentagefor the location determination. (WSD => database)

Example: 802.22 primitives:

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Location Data String Length | Integer | 2 bytes | Length of Location Data String field (# of characters) |
| Location Data String | Character String | NMEA 0183 Character String | The value identifies the location of the device (latitude, longitude).20 |

Note: NMEA 0183 $GPGGA or $GPGLL String can carry latitude and longitude information. This NMEA string would need to be augmented to include the uncertainty (m) and the confidence level (%).  The uncertainty could also be artificially increased to take into account the size of the area around the WSD where other WSD’s not requiring geolocation (e.g., Mode I devices in the USA).

D.4: The Data Model MUST support specifying a list of available channel along with the maximum EIRP (dBm) that can be accommodatedand time constraints for the channel list availability. (Database => WSD)

Example: 802.22 primitives:

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Number of Channels Available | Integer | 1 byte |  |
| { If( Number of Channels Available > 0) |  |  | If the number of channels is equal to 0, this means that the device cannot operate. |
| For (i=1; i≤ Number of Channels Available; i++) { Channel_Number Max_Allowed_EIRP (dBm) Availability schedule } | Vector of 2xN bytes and a number of pairs of NMEA 0183 $ZDA strings | Variable | List of available channel numbers and corresponding maximum allowed EIRP expressed in dBm over the range –64 dBm (encoded 0x00) to +63.5 dBm (encoded 0xFF) as well as the availability schedule (start and stop date/time) for each channel in Universal date and time system. |
| } |  |  |  |

D.5: The Data Model MUST support specifying the maximum output power of

the subject. (See D.4 above. Also specifying the “maximum output power” is not sufficient since in may or may not include the antenna gain. Specifying the EIRP is needed.)

D.6: The Data Model may support specifying channel availability information for multiple locations. (Database => WSD)

802.22 BS shall act as a proxy and query the database for each device. If a query is to be grouped or made in a batch mode, all the information related to each device shall be provided to the BS (i.e., the database is not to perform the intersection for all these devices and locations).

(This option may be useful to ‘batch’ the database query process but it is not clear whether this would really increase the data transfer efficiency. In the case of the 802.22 WRAN systems, the base station will need to acquire all the information about the available channels for each of these CPEs (and optionally the related maximum EIRP’s) so that the operating channel and the backup channels (to which the WRAN cell will need to move to if the current operating channel becomes occupied by an incumbent) can be determined locally from the best ‘intersection’ of these channels for all the associated CPEs to ensure a transparent channel move.  This will allow database queries for only new CPEs coming on board or being moved and constraints to be added locally at the base station to execute the ‘intersection’ process to produce the updated list of operational and backup channels that is to be transmitted to all CPEs for refreshing.

D.7: The Data Model MUST support specifying channel availability

information for an area around a specified location. (Database => WSD)

This is an unnecessary option. This can be done by any WSD that can query the database on behalf of other WSDs. Such master WSD would define the coordinates of the apexes of a polygon that would represent the area to be verified for channel availability and query the database for each of these coordinates.  The database does not need to know that this is a query for an area.  It just needs to respond to the queries as usual.

P. Protocol Requirements:

P.1: The protocol MUST provide a mechanism for the subject to discover

the WS Database it has to use at a given location.

P.2: The protocol MUST support regulatory domain discovery.

P.3: The protocol between the master device and the WS Database MUST

support the database pushing updates in channel availability changes to subjects

P.4: The protocol between the master device and the WS Database MUST

support mutual authentication and authorization.

P.5: The protocol between the master device and the WS Database MUST

support integrity and confidentiality protection.

P.6: The protocol MUST support both username/password and digital

certificates based authentication.

O. Operational Requirements:

O.1: A master device MUST query the WS Database for the available

channels as often as required by the regulation (e.g., FCC requires once

per day) to verify that the operating channels (and backup channels in the case of transparent switch-over) continue to remain available.

O.2: A master device MUST determine its location along with its uncertainty and confidence level and send it to the database so that the proper buffer distance around the device can be added to make sure that the worst case situation required by the regulation (e.g., FCC requires +/- 50m) is considered in the distance calculations taking place at the DB.

O.3: A master device which changes its location during its operation,

MUST query the WS Database for available operating channels each time

it moves more than the distance specified by the regulation (e.g., FCC specifies 100m) from the location it occupied when it previously made the query.

O.4: The WS Database MUST provide the available channel list and the maximum EIRP associated with each channel when requested and MAY also provide time constraints for each channel in the list to the master device.

O.5: A master device MUST be able to query the WS Database for itself as well as for its  slave devices and compile the channel availability (and maximum EIRP thereof) so that a common channel (i.e., intersection) can be selected for use by all these WSD to form a network, and optionally further common channels can be selected as backup channels to allow for network channel switch that would be transparent to the user

.

O.6: A master device MUST be capable of validating the digital

certificate of the WS Database and whether it has been revoked or not.

Antenna directionality will represent the antenna gain pattern in the horizontal plane in dB referred to the gain of its main lobe and it is assumed that the database service will use its knowledge of the geolocation of the base station and the device being enlisted to calculate the azimuth of the device antenna main lobe for interference calculations in the case of base station and CPE operation. Omnidirectional antennas shall be assumed as the default.