IEEE P802.22
Wireless RANs

| IEEE 802.22 Requirements for IETF PAWS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-10-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Apurva Mody | BAE Systems | Merrimack, NH | 603-809-0459 | apurva.mody@baesystems.com |
| Gerald Chouinard | CRC | Ottawa, Ontario, Canada | 613-998-2500 | gerald.chouinard@crc.ca |
| Ranga Reddy | Self | 400 Deal Lake Drive, Apt 2B, Asbury Park, NJ 07712 | 732-693-5812 | ranga.reddy@me.com |
| Jerome J. Kalke | CBS | 15252 Golf View Drive, Haymarket, VA 20169 USA |  | jkalkesr@att.net |
| Nancy Bravin | Self |  |  | nbravin@earthlink.net |

Abstract

IEEE 802.22 Requirements for IETF PAWS.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Requirements

D. Data Model Requirements:

D.1: The Data Model MUST support specifying the

- Antenna Parameters (WSD => database)

- Height Parameters

  - mandatory AGL in meters: Unsigned INT, 2 Bytes (0 – 65,535 cm)
Example: 802.22 primitive:

| Antenna height | Integer | 1 byte | Antenna height above ground level in meters. |
| --- | --- | --- | --- |

[Note: portable is defined as 1.5 m and base stations in Canada can be at up to 500 m HAAT which could be completely contributed by the antenna (e.g., mounted on a broadcast tower.)]

  - Our assumption is that the database will compute the HAAT in meters from the AGL and ground elevation at the specified location (lat, long) obtained from a topographic database by the database server.

- Gain Parameters

  - Antenna directionality information in dB: relative to the main lobe maximum gain for every 5 degree azimuth clockwise starting from the direction of the maximum antenna gain expressed in unit of 0.25 dB over the range –63.75 dB (encoded 0x00) to 0 dB (0xFF):
Character String	72 bytes

  - Antenna azimuth	 in degrees, clockwise from true North: Integer	2 bytes

Example: 802.22 primitive:

| Antenna information | Character String | 72 bytes | Antenna directionality information of the device in dB relative to the main lobe maximum gain for every 5 degree azimuth clockwise starting from the direction of the maximum antenna gain expressed in unit of 0.25 dB over the range –63.75 dB (encoded 0x00) to 0 dB (0xFF). (to allow the database calculation of the channel availability and the maximum allowed EIRP values at the registering location19) |
| --- | --- | --- | --- |
| Antenna azimuth | Integer | 2 bytes | Antenna azimuth in degrees, clockwise from true North |

- RF Mask Parameters (WSD => database)

  - Regulatory Domain (3 ASCII Letters), Device Type (Regulatory Class e. g. Fixed, Personal Portable, Mobile, etc.) (INT), Mask Number Index (Long INT) (where Mask Number Index corresponds to a particular RF Mask of an equipment that is stored in the database and that has passed the regulatory certification)

Example: 802.22 primitive:

| Device Type | Integer | 1 byte | The value identifies the type of device at the geolocation registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 0x03–0xFF = Reserved |
| --- | --- | --- | --- |

D.2: The data model MUST support specifying an ID of the subject. This ID would be the ID of the device used to be certified by a regulatory body for a regulatory domain (WSD => database)

Example: 802.22 primitives:

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device at the geo-location registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 0x03-0xFF = Reserved |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |

Note: The device-ID and Serial Number could be replaced by a universal identifier made of one or many parts  for practical purpose.

D.3: The Data Model MUST support specifying the location of the WSD, the uncertainty in meters and confidence in percentagefor the location determination. (WSD => database)

Example: 802.22 primitives:

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Location Data String Length | Integer | 2 bytes | Length of Location Data String field (# of characters) |
| Location Data String | Character String | NMEA 0183 Character String | The value identifies the location of the device (latitude, longitude).20 |

Note: NMEA 0183 $GPGGA or $GPGLL String can carry latitude and longitude information. This NMEA string would need to be augmented to include the uncertainty (m) and the confidence level (%).  The uncertainty could also be artificially increased to take into account the size of the area around the WSD where other WSD’s not requiring geolocation (e.g., Mode I devices in the USA) would operate.

D.4: The Data Model MUST support specifying a list of available channel along with the maximum EIRP (dBm) that can be accommodatedand time constraints for the channel list availability. (Database => WSD)

Example: 802.22 primitives:

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Number of Channels Available | Integer | 1 byte |  |
| { If( Number of Channels Available > 0) |  |  | If the number of channels is equal to 0, this means that the device cannot operate. |
| For (i=1; i≤ Number of Channels Available; i++) { Channel_Number Max_Allowed_EIRP (dBm) Availability schedule } | Vector of 2xN bytes and a number of pairs of NMEA 0183 $ZDA strings | Variable | List of available channel numbers and corresponding maximum allowed EIRP expressed in dBm over the range –64 dBm (encoded 0x00) to +63.5 dBm (encoded 0xFF) as well as the availability schedule (start and stop date/time) for each channel in Universal date and time system. |
| } |  |  |  |

D.5: The Data Model MUST support specifying the maximum output power of

the subject. (See D.4 above. Also, specifying the “maximum output power” is not sufficient since it may or may not include the antenna gain. Specifying the EIRP is needed. D.5 should be deleted since it is proposed to be covered in D.4.)

D.6: The Data Model MUST support specifying channel availability information for single and multiple locations. The database MUST also allow a master device to act as a proxy for other WSD and query on their behalf. (Database => WSD)

In case of 802.22 systems which are to provide point-to-multipoint broadband access service primarily to rural areas, the BS acts as a proxy for all its associated CPEs and queries the database for each device. If a query is to be grouped or made in a batch mode, all the information related to each device shall be provided to the BS (i.e., the database is not to perform the intersection for all these devices and locations).

Note: This option may be useful to ‘batch’ the database query process but it is not clear whether this would really increase the data transfer efficiency. In the case of the 802.22 WRAN systems, the base station will need to acquire all the information about the available channels for each of these CPEs (and optionally the related maximum EIRP’s) so that the operating channel and the backup channels (to which the WRAN cell will need to move to if the current operating channel becomes unavailable) can be determined locally from the best ‘intersection’ of these channels for all the associated CPEs to ensure a transparent channel move.  This will allow database queries for only new CPEs coming on board or being moved and constraints to be added locally at the base station to execute the ‘intersection’ process to produce the updated list of operational and backup channels that is to be transmitted to all CPEs for refreshing.

D.7: The Data Model MUST support specifying channel availability

information for an area around a specified location. (Database => WSD)

In our opinion, this is can be done if a query to the database can be done with dummy device IDs for example for planning purposes and hence, we feel that this is not really needed. If a query is done to the database with dummy device ID, the databes should not register these new devices and only provide the list of available channels. There should therefore be a need to identify whether the included device ID is a real one or not.

D.8: The Data Model should support reporting to the database the channels that the master WSD has selected as the operating channel and the backup channels.

P. Protocol Requirements:

P.1: The protocol MUST provide a mechanism for the subject to discover

the WS Database it has to use at a given location. [Agree]

P.2: The protocol MUST support regulatory domain discovery. [Agree]

P.3: The protocol between the master device and the WS Database MUST

support the ability for the database to push updates on channel availability changes to subjects

P.4: The protocol between the master device and the WS Database MUST

support mutual authentication and authorization. [Agree]

P.5: The protocol between the master device and the WS Database MUST

support integrity and confidentiality protection. [Agree]

P.6: The protocol MUST support both username/password and digital

certificates based authentication. [Agree]

P.7: The protocol MUST require the master WSD to maintain contact with the database as specified by the local regulator and re-register its operating and backup channels with at least the same periodicity.

O. Operational Requirements:

O.1: A master device MUST query the WS Database for the available

channels as often as required by the regulation (e.g., FCC requires once

per day) to verify that the operating channels (and backup channels in the case of transparent switch-over) continue to remain available.

O.2: A master device MUST determine its location along with its uncertainty (e.g., FCC requires +/- 50m) and confidence level (e.g., 95%) and send it to the database so that the proper WSD position and buffer distance around the device can be added to make sure that the worst case situation required by the regulation  is considered in the distance calculations taking place at the DB.

O.3: A master device which changes its location during its operation,

MUST query the WS Database for available operating channels each time

it moves more than the distance specified by the regulation (e.g., FCC specifies 100 m) from the location it occupied when it previously made the query.

O.4: The WS Database MUST provide the available channel list and the maximum EIRP corresponding to each channel when requested and MAY also provide time constraints for each channel in the list to the master device.

O.5: A master device MUST be able to query the WS Database for itself as well as for its  associated devices and compile the channel availability (and maximum EIRP thereof) so that a common channel (i.e., intersection) can be selected for use by all these WSDs to form a network. Furthermore, common channels may also need to be selected in a similar way  to become backup channels to allow for network channel switch that would be transparent to the user

.

O.6: A master device MUST be capable of validating the digital

certificate of the WS Database and whether it has been revoked or not.

[Repeat, see O.6]

O.7: The database must be capable of keeping track of the channels that are being utilized as current operating channel by the master devices and remove those channels from the list of available channels in those locations for queries by other WSDs unless the given master WSD fails to re-register within the specified contact period. Accordingly, the protocol should provide the master WSDs with a means to release their operating channel when not needed.

Note that without an active channel management mechanism (e.g., 802.22 spectrum manager), it is unlikely that having the DB just specifying which channels are available to protect incumbent services will be sufficient to allow for proper operation of multiple WSDs in an area without interference being caused among themselves and to result in the spectrum being used efficiently.

Antenna directionality will represent the antenna gain pattern in the horizontal plane in dB referred to the gain of its main lobe and it is assumed that the database service will use its knowledge of the geolocation of the base station and the device being enlisted to calculate the azimuth of the device antenna main lobe for interference calculations in the case of base station and CPE operation. Omnidirectional antennas shall be assumed as the default.