IEEE P802.22.2
Wireless RANs Recommended Practice

| New Annex A on the ATSC A/74 D/U’s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2011-11-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Gerald Chouinard | CRC | 3701 Carling Ave., Ottawa, Ontario, Canada K2H8S2 | 1-613-998-2500 | gerald.chouinard@crc.ca |

Abstract

This document contains the proposal for the new Annex A that is to be added to the Recommended Practice as per the resolution of comment #110.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Apurva Mody> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Add the following Annex at the end of the Recommended Practice:

Annex A

(Informative)

Desired-to-undesired ratios to be used for the protection of the ATSC DTV systems

The level of susceptibility of DTV receivers to co-channel and adjacent channels for various channel separations was established by the ATSC in its reference document A/74 “ATSC Recommended Practice: Receiver Performance Guidelines” which DTV receiver manufacturers consider in their receiver design. This ATSC DTV receiver interference susceptibility is provided in Table A1.

| ATSC A/74 DTV RX Performance Guidelines | Measured D/U at 3 desired signal levels |  |  | D/U at the noise-limited contour (at -84 dBm) |
| --- | --- | --- | --- | --- |
|  | Strong | Moderate | Weak |  |
|  | -28 dBm | -53 dBm | -68 dBm |  |
| N (continuous) | 15.5 dB | 15.5 dB | 15.5 dB | 23 dB |
| N (impulsive) | 5 dB | 5 dB | 5 dB | 5 dB |
| N+/-1 | -33 dB | -33 dB | -33 dB | -33 dB |
| N+/-2 | -20 dB | -40 dB | -44 dB | -48.2 dB |
| N+/-3 | -20 dB | -40 dB | -48 dB | -56.4 dB |
| N+/-4 | -20 dB | -40 dB | -52 dB | -64.7 dB |
| N+/-5 | -20 dB | -42 dB | -56 dB | -70.8 dB |
| N+/-6 to N+/-13 | -20 dB | -45 dB | -57 dB | -69.7 dB |
| N+/-14 and 15 | -20 dB | -45 dB | -50 dB | -55.3 dB |
| RF front-end overload |  |  |  | -8 dBm |

Table A.1  ATSC DTV receiver susceptibility to interference (D/U requirements)

These performance guidelines were developed based on laboratory measurements carried out during the development of the ATSC Standard. The measurements of the required desired-to-undesired ratios (D/U) were carried out for 3 levels of desired signal presented to the receivers: strong, moderate and weak levels. In order to determine the D/U’s at the edge of the noise-limited coverage area which represent the worst case situation, a linear extrapolation (in dB values) needed to be done from the ‘moderate’ and ‘weak’ cases to establish the D/U’s corresponding to a desired signal level of -84 dBm. The resulting D/U’s are presented in the last column on the right of Table A.1.

It should be noted that in the case of co-channel interference, even though the D/U was measured at 15.5 dB, it was decided by the FCC that this D/U ratio is only valid at locations where the signal-to-noise ratio is strong = (S/N= 28 dB or greater). At the edge of the noise-limited coverage area, where the signal-to-noise (S/N) ratio is 15.5 dB, the co-channel D/U ratio was increased to 23 dB in the FCC OET Bulletin 69 to limit the contribution of co-channel interference at the edge of coverage to correspond to a DTV receiver desensitization of 0.7 dB due to co-channel interference.

_________________________