IEEE P802.22
Wireless RANs

| Functional Requirements for IEEE 802.22b amendment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-2-1 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mingtuo Zhou | NICT | 20 Science Park Road, #01-09A/10 TeleTech Park, Singapore Science Park II | +65- 67711007 | mingtuo@nict.com.sg |
| Xin Zhang | NICT |  |  |  |
| Chunyi Song | NICT | Hikari no Oka 3-4, Yokosuka, Japan |  |  |
| Chang Woo Pyo | NICT |  |  |  |
| M. Azizur Rahman | NICT |  |  |  |
| Hiroshi Harada | NICT |  |  |  |

Abstract

This document presents the functional requirements for IEEE 802.22b amendment.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Overview

The 802.22b amendment shall be developed in accordance with the P802.22b Project Authorization Request (PAR) [1] and its Five Criteria Statement (5C) [2].

The standard shall be developed as an amendment to 802.22. The resulting standard shall fit within the following scope:

“This amendment specifies alternate Physical Layer (PHY) and necessary Medium Access Control Layer (MAC) enhancements to IEEE std. 802.22-2011 for operation in Very High Frequency (VHF)/ Ultra High Frequency (UHF) TV broadcast bands between 54 MHz and 862 MHz to support enhanced broadband services and monitoring applications. The standard supports aggregate data rates greater than the maximum data rate supported by the IEEE Std. 802.22-2011. This standard defines new classes of 802.22 devices to address these applications and supports more than 512 devices in a network. This standard also specifies techniques to enhance communications among the devices and makes necessary amendments to the cognitive, security & parameters and connection management clauses. This amendment supports mechanisms to enable coexistence with other 802 systems in the same band.”

The functional requirements are presented as following.

2. Definition of Words

Shall: These words or the adjective "REQUIRED" means that the item is an absolute requirement.

Should: This word or the adjective “RECOMMENDED” means that there may exist valid reasons in particular circumstances to ignore this item, but the full implications should be understood and the case carefully weighed before choosing a different course.

May: This word or the adjective “OPTIONAL” means that this item is truly optional. One implementation may include the item because the target marketplace requires it or because it enhances the product, for example; another implementation may omit the same item

3. Functional Requirements

This amendment supports enhanced broadband services (e.g., emergency broadband, remote medical services, and marine broadband, etc) and monitoring applications (smart metering, critical infrastructer monitoring, homeland security/mornitoring, etc) in TVWS, which require high data throughput.

- Throughput

[Req01] This amendment shall provide a means of achieving aggregated throughput at least 2 times higher than the maximum throughput supported by the IEEE Std. 802.22-2011, e.g., Higher Modulation and Coding Scheme (MCS), MIMO, MISO, channel aggregation, etc.

- Device classes and complexity

[Req02] This amendment shall define at least two classes of CPEs to effectively support different service applications of broadband services and monitoring applications. One class of CPE shall be designed with low complexity to fit for mornitoring applications.

Note: 	-Clarification of classes of CPE

-Low complexity of CPE

: for sensing, metering, …

- High complexity of CPE

: for monitoring, supporting mesh, MIMO, channel bonding …

[Req03] This amendment should provide mechanisms of energy efficient operations, eg. Battery operated CPEs for monitoring applications.

Note:	 -Combine Req02 and Req03 after  clarifying classes of CPEs

- Number of devices

[Req04] This amendment shall support at least 2048 CPEs to cover a regional area network in some monitoring applications. For example, IEEE 802.16p supports as many as 16000 connection IDs.

- Network topology

[Req05] This amendment should support multi-hop connections among CPEs.

Note: 	-Number of hops TBD

-More than 2 hops for high complexity CPEs

-Low complexity CPEs ? :

-Define the number of hops among CPEs after defining Req02

[Req06] This amendment should support peer-to-peer communications between CPEs.

Note: 	-Going to support mesh

-Under controlled by BS, peer-to-peer communication may be included in Req05

- Real-time applications

[Req07] This amendment should support QoS mechanisms for real-time monitoring applications.

Note: 	-Looks like present 802.22 is good enough, but need to check

-Extend QoS methods?

- Coexistence

[Req08] This amendment should support mechanisms to coexist with IEEE 802 other systems in the same band.

- Cognitive capability

[Req09] This amendment should provide enhanced cognitive management to meet up-to-date regulation rules of TVWS.

- Security

[Req10] This amendment may provide enhancement to exsiting or alternate security mechanisms.

- Backward compatibility

[Req11] IEEE 802.22b BS should contain all the capabilities of the IEEE 802.22 BS, i.e., IEEE 802.22-2011 CPEs are able to communicate to IEEE 802.22b BS.

- Channelization

[Req12] This amendment should provide a means of channelization for cost-effective compliance with regulatory spectral mask.

4. Reference

1. IEEE P802.22b PAR in doc.: IEEE 802.22-yy/xxxxr004-rasg

2. IEEE P802.22b 5C in doc.: IEEE 802.22-yy/xxxxr00000-04-rasg

3. IEEE Std.802.22-2011

4. doc.: IEEE 802.22-yy/xxxxr0