IEEE P802.22
Wireless RANs

| Selection Criteria for IEEE 802.22b amendment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-03-xx |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-woo Pyo | NICT | 3-4, Hikarino-oka, Yokosuka, Japan | +81-46-847-5120 | cwpyo@nict.go.jp |

Abstract

This document presents selection criteria for evaluating proposals of IEEE 802.22b amendament.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Contributors

(This will grow to reflect those providing explicit contributions / review comments of this document. Please let Chang-woo Pyo know if any name is conspicuously missing from this list.)

| Name | Company | Address | Phone | Email |
| --- | --- | --- | --- | --- |
| Chang-woo Pyo | NICT | 3-4, Hikarino-oka, Yokosuka, Japan | +81-46-847-5120 | cwpyo@nict.go.jp |
| Xin Zhang | NICT |  |  | Zhang@nict.com.sg |
| Chunyi Song | NICT |  |  | songe@nict.go.jp |
| Mingtuo Zhou | NICT |  |  | mingtuo@nict.com.sg |
| Hiroshi Harada | NICT |  |  | harada@nict.go.jp |
| Apurva N. Mody | BAE systems |  |  | apurva.mody@ieee.org |
| Sunghyun Hwang | ETRI |  |  | sunghyun.hwang@whitespacealliance.org |
| Shigenobu Sasaki | Niigata University |  |  | shinsasaki@ieee.org |
| Bingxuan Zhao | Niigata University |  |  | zhaobx@telecom0.eng.niigata-u.ac.jp |
| Kalke Jerome J. | CBS |  |  |  |
| Flynn Peter | Texas Instruments |  |  |  |

# Revision History

| Revision | Comments | Date |
| --- | --- | --- |
| R0 | Initial contribution | 16 March 2012 |
| R1 | Revise contents | 15 May 2012 |
| R4 | Editorial revision, Add proposal defined model | 05 June 2012 |

- Introduction

This selection criteria document provides technical criteria for evaluating proposals of IEEE 802.22 Task Group “b”, IEEE 802.22b, which specifies alternate Physical Layer (PHY) and necessary Medium Access Control Layer (MAC) enhancements to support enhanced broadband services and monitoring applications and regulatory requirements in UHF/VHF TV broadcast bands.

- Usage Cases

Ten (10) usage cases are considered to enhance broadband services and monitoring applications. These usage cases are used to clarify network configuration of the IEEE 802.22b.

| Category | Usage Cases | Properties |
| --- | --- | --- |
| A) Smart Grid & Monitoring | A1) Regional Area Smart Grid/Metering | -Low capacity/complexity CPEs -Very large number of monitoring CPEs -Fixed and Potable CPEs -Real time monitoring -Low duty cycle -High reliability and security -Large coverage area -Infrastructure connection |
|  | A2) Agriculture/Farm House Monitoring |  |
|  | A3) Critical Infrastructure/Hazard Monitoring |  |
|  | A4) Environment Monitoring |  |
|  | A5) Homeland Security/Monitoring |  |
|  | A6) Smart Traffic Management and Communication |  |
| B) Broadband Service Extension | B1) Temporary Broadband Infrastructure (e.g., emergency broadband infrastructure) | -Fixed and Portable CPEs -Higher capacity CPEs than Category A) -High QoS, reliability and security -Higher data rate than Category A) -Easy network setup -Infrastructure and Ad hoc connection |
|  | B2) Remote Medical Service |  |
|  | B3) Archipelago/Marine Broadband Service |  |
| C) Combined Service | C1) Combined Smart Grid, Monitoring and Broadband Service | - Category A) and B) |

- Criteria Overview for Performance Study

| Network Model | Smart Metering & Monitoring Model | Broadband Service Extension Model | Proposal Defined Model |
| --- | --- | --- | --- |
| Entities | BS H-CPEs L-CPEs | BS H-CPEs | Each proposal shall define entities |
| Topology | Tree topology (Infrastructure) | Multi-hop connection | Each proposal shall define topology |
| Applications | Metering applications Video monitoring applications | Internet (Web browsing), Downlink video transmission | Each proposal shall define applications |
| Channel Model | Basic channel model (802.22std-2011) | Basic channel model (802.22std-2011) | Each proposal shall define channel model |
| Performance criteria | Throughput Delay Packet Loss | Throughput Delay Packet Loss | Throughput Delay Packet Loss |
| Reference Packet Error Rate (PER) | 10e-3 for metering, 10e-4 for monitoring | 10e-3 | Each proposal shall define BER |
| Mandatory / Option | Mandatory model for performance study | Optional model for performance study | Optional model for performance study |

- Smart Metering Model (SMM)

  - Introduction

This Model is used for studying the performance of smart grid and metering indicated in Usage Case Category A1 [4], which has the properties of a very large number of metering CPEs, low duty cycle, infrastructure connections, etc.

  - Network Entities

    - Base Station (BS): 802.22b BS is generalized equipment set providing connectivity, management and control of the customer premise equipment (CPE).

    - High-capability CPE (H-CPE): 802.22b H-CPE is enhanced 802.22-2011 CPE that is able to manage a local network consisting of a number of L-CPEs and to provide multi-hop connections between L-CPEs and 802.22b BS.

    - Low-capability CPE (L-CPE): 802.22b L-CPE is a device mainly for metering and monitoring, which will be installed at home, office, or on infrastructure, etc.

  - SMM Network Installation

A BS covers a certain regional area, which calls a 802.22b WRAN, where includes H-CPEs and L-CPEs as shown in Figure 1. Moreover, each H-CPE may serve a large number of L-CPEs to gather the metering information from the grouped metering L-CPEs, which typically are installed in a specific location at home, buildings, offices etc.

      - Base Station (BS): A BS locates a center of a 802.22b WRAN, where the maximum radius of BS coverage (a) shown in Figure 1 is 10Km. (Case 1 in link margin example shown in Table 1)

      - High-capability CPE (H-CPE): H-CPEs locate randomly within a 802.22b WRAN, where the average distance (b) from BS to H-CPEs shown in Figure 1 is 5Km.

      - Low-capability CPE (L-CPE): A group of L-CPEs served by an H-CPE locates randomly within the H-CPE coverage area, where the H-CPE coverage area is determined by the wireless access availability of the L-CPEs to communicate with the H-CPE. The maximum radius of H-CPE coverage (c) is 2Km (TBD), which also means the maximum communication range of L-CPE on 6 MHz bandwidth. (Case 3 in link margin example shown in Table 1). Note that H-CPE may have different wireless access availabilities for the communication with BS and L-CPEs.

Figure 1 An example of network installation for metering services

  - Device Capabilities

    - Base Station (BS)

      - Transmission power: Fixed mode (4W EIRP)

      - Antenna gain: 6 dbi

      - Antenna height: 30m (250m height above average terrain (HAAT) + 30ms meters above ground level (AGL))

    - High-capability CPE (H-CPE)

      - Transmission power:Fixed mode (4W EIRP)

      - Antenna gain: 11 dbi

      - Antenna height: 10m HAAT

    - Low-capability CPE (L-CPE)

      - Transmission power: Portable mode (100mW EIRP)

      - Antenna gain: 0 dbi

      - Antenna height: 2m HAAT

  - Maximum available hops between BS to end devices (L-CPEs)

2 (All H-CPEs shall communicate with BS directly, and all L-CPEs shall communicate with BS through H-CPE providing intermediate connection for L-CPEs. Note that this assumption is only available for the performance study of the proposals, then there is no limitation for the connection patterns of the proposals.)

  - Number of Devices

    - Base Station (BS): 1

    - High-capability CPE (H-CPE): 25

    - Low-capability CPE (L-CPE): 2500  (100 L-CPEs per H-CPE) - TBD

  - Traffic

| Traffic Model A |  |
| --- | --- |
| Traffic Type | Metering information |
| Protocol | Constant bit rate (CBR) with ARQ |
| Data Traffic Direction | Uplink from CPE to BS (e.g., L-CPE -> H-CPE -> BS) |
| Maximum Packet (MSDU) Length | 1500 Bytes |
| One Metering Packet Length | 1500 bytes (One metering packet length may be smaller than maximum packet length, but maximum packet length (1500 Bytes) is used for one metering packet length to transmit from L-CPE to H-CPE for simplicity) |
| Packet Arrival | Constant packet arrival shown in Figure 2 (1 packet per second, 1 packet per 2 seconds, 1 packet per 5 seconds): Each L-CPE initiates packet transmission in each second after the first packet being randomly started. |
| Packet Traffic | 12Kbit/sec (1 packet per second) 6Kbit/sec (1 packet per 2 seconds) 2.4Kbit/sec (1 packet per 5 seconds) |
| ARQ Algorithm | Depends on proposals. (Note that each proposal presents the ARQ algorithm used in performance study.) |
| Simultaneous Communications | - 2500 L-CPEs per BS assuming 1 packet per second from L-CPE (12Kbit/sec) - 2500 L-CPEs per BS assuming 1 packet per 2 seconds from L-CPE (6Kbit/sec) - 2500 L-CPEs per BS assuming 1 packet per 5 seconds from L-CPE (2.4Kbit/sec) |
| Buffer | H-CPE (Infinite-Buffer, Finite Buffer) (Note that each proposal presents the buffer size if the finite buffer is used) |
| Conditions | For simplicity, ignore the process network management frame exchanges such as registration, ranging, bandwidth request etc. Only consider data transmission/retransmission and ARQ |
| DL Frame Allocation | For simplicity, we ignore the management frame exchanging process, but 10% DL frame allocation (1ms) is considered for the management frame (frame preamble, header, management frames, etc) overhead except for ARQ. |

Figure 2 An example of metering packet arrival and transmission pattern per user

Figure 3 An example of metering packet transmissions in a network

  - Reference Channel Model

    - Basic Channel Model (802.22 Channel Model)

      - Link margin calculation shows in Table 1 on the conditions shown in Table 2

Table 1 Link Margin Examples (12-55/r1)

Case 1: Broadband service downlink (BS to H-CPE)

Case 2: broadband service uplink (H-CPE to BS)

Case 3: L-CPE to H-CPE

Case 4: H-CPE to L-CPE

Case 5: H-CPE to H-CPE

Table 2 Conditions for Link Budget Calucation (12-55/r0)

  - Performance Criteria

    - Network Throughput

      - The network throughput is defined as the number of information bits per second that a network can successfully deliver or receive using the scheduling algorithms.

    - Delay (Maximum delay and average delay)

        - The maximum packet delay is defined as the maximum interval between packets originated at the source station (L-CPE) and received at the destination station (BS). The average packet delay is defined as the average interval between packets originated at the source station (L-CPE) and received at the destination station (BS).

    - Packet Loss Radio

      - The packet loss ratio in a network is defined as:

- Packet Loss Ratio = Total Number of Successfully Received Packets / Total Number of Successfully Transmitted Packets

    - Reference Packet Error Rate (PER) on 1500bytes : 10e-3

- Real-time/Near real-time Video Monitoring Model (RMM)

  - Introduction

This Model is used for studying the performance of monitoring services indicated in Usage Case Category A2-A5 [4], which has the properties of a large number of monitoring CPEs, real-time monitoring, infrastructure connections, etc.

  - Network Entities

    - Base Station (BS): 802.22b BS is generalized equipment set providing connectivity, management and control of the customer premise equipment (CPE).

    - High-capability CPE (H-CPE): 802.22b H-CPE is enhanced 802.22-2011 CPE that is able to manage a local network consisting of a number of L-CPEs and to provide multi-hop connections between L-CPEs and 802.22b BS.

    - Low-capability CPE (L-CPE): 802.22b L-CPE is a device mainly for metering and monitoring, which will be installed by end users at home, office, or on infrastructure, etc.

  - RMM Network Installation

A BS covers a certain regional area, which calls a 802.22b WRAN, where includes H-CPEs and L-CPEs as shown in Figure 4. Morover, one H-CPE may serve a small number of monitoring L-CPEs as shown in Figure 4 since a monitoring L-CPE may covers a certain area to detect events, where the area could be wide.

      - Base Station (BS): A BS locates a center of a 802.22b WRAN, where the maximum radius of BS coverage (a) shown in Figure 4 is 10Km. (Case 1 in link margin example shown in Table 1).

      - High-capability CPE (H-CPE): H-CPEs locate randomly within a 802.22b WRAN, where the average distance (b) from BS to H-CPEs shown in Figure 1 is 5Km.

      - Low-capability CPE (L-CPE): L-CPEs served by an H-CPE locate randomly within the H-CPE coverage area, where the H-CPE coverage area is determined by the wireless access availability of the L-CPEs to communicate with the H-CPE. The maximum radius of H-CPE coverage (c) is 2Km (TBD), which also means the maximum communication range of L-CPE. (Case 3 in link margin example shown in Table 1). Note that H-CPE may have different wireless access availabilities for the communication with BS and L-CPEs.

Figure 4 An example of network installation for monitoring services

  - Device Capabilities

    - Base Station (BS)

      - Transmission power: Fixed mode (4W EIRP)

      - Antenna gain: 6 dbi

      - Antenna height: 30m (250m height above average terrain (HAAT) + 30ms meters above ground level (AGL))

    - High-capability CPE (H-CPE)

      - Transmission power:Fixed mode (4W EIRP)

      - Antenna gain: 11 dbi

      - Antenna height: 10m HAAT

    - Low-capability CPE (L-CPE)

      - Transmission power: Portable mode (100mW EIRP)

      - Antenna gain: 0 dbi

      - Antenna height: 2m HAAT

  - Maximum available hops between BS to end devices (L-CPEs)

2 (All H-CPEs shall communicate with BS directly, and all L-CPEs shall communicate with BS through H-CPE providing intermediate connection for L-CPEs. Note that this assumption is only available for the performance study of the proposals, then there is no limitation for the connection patterns of the proposals.)

  - Number of Devices

    - Base Station (BS): 1

    - High-capability CPE (H-CPE): 250

    - Low-capability CPE (L-CPE): 2500  (10 L-CPEs per H-CPE)

  - Traffic

| Traffic Model B |  |
| --- | --- |
| Traffic Type | Real-time/Near real-time Video Monitoring |
| Protocol | Constant bit rate (CBR) with No-ARQ |
| Data Traffic Direction | Uplink from CPE to BS (L-CPE -> H-CPE -> BS) |
| Maximum Packet (MSDU) Length | 1500 Bytes |
| One Monitoring Packet Length | 1500 Bytes (One monitoring packet length may be smaller than maximum packet length, but maximum packet length (1500 Bytes) is used for one monitoring packet length to transmit from L-CPE to H-CPE for simplicity) |
| Packet Traffic | 400Kbit per sec (Low resolution), 800Kbit per sec (High resolution) |
| Monitoring Duration | Average 10s on Uniform Distribution |
| Packet Arrival from L-CPE to H-CPE | Each H-CPE is randomly selected to monitoring an area served by the H-CPE, and the selected H-CPE selects one L-CPE randomly or round-robin way, which initiates packet transmission and continues packet transmissions on monitoring duration. |
| ARQ Algorithm | No ARQ |
| Simultaneous Communications | 25 ~ 100 H-CPEs |
| Buffer | H-CPE (Infinite-Buffer, Finite Buffer) (Note that each proposal presents the buffer size if the finite buffer is used) |
| Conditions | For simplicity, ignore the process network management frame exchanges such as registration, ranging, bandwidth request etc. Only consider data transmission. |
| DL Frame Allocation | For simplicity, we ignore the management frame exchanging process, but 10% DL frame allocation (1ms) is considered for the management frame (frame preamble, header, management frames, etc) overhead. |

Figure 5 An example of monitoring packet arrival and transmission pattern per user

Figure 6 An example of monitoring packet transmissions in a network

  - Reference Channel Model

    - Basic Channel Model (802.22 Channel Model)

      - See Table 1 and Table 2

  - Performance Criteria

    - Network Throughput

      - The network throughput is defined as the number of information bits per second that a network can successfully deliver or receive using the scheduling algorithms.

    - Delay (Maximum delay and average delay)

      - The maximum packet delay is defined as the maximum interval between packets originated at the source station (L-CPE) and received at the destination station (BS). The average packet delay is defined as the average interval between packets originated at the source station (L-CPE) and received at the destination station (BS).Packet Loss Radio

    - Packet Loss Ratio

      - The packet loss ratio in a network is defined as:

- Packet Loss Ratio = Total Number of Successfully Received Packets / Total Number of Successfully Transmitted Packets

    - Reference Packet Error Rate (PER) on 1500bytes : 10e-4 (TBD)

- Broadband Service Extension Model (BSEM) on Multihop

  - Introduction

This Model is used for studying the performance of broadband service extension indicated in Usage Case Category B1 [4], which has the properties of higher data transmission for internet access,  ad hoc connections, etc.

  - Network Entities

    - Base Station (BS): 802.22b BS is generalized equipment set providing connectivity, management and control of the customer premise equipment (CPE).

    - High-capability CPE (H-CPE): 802.22b H-CPE is enhanced 802.22-2011 CPE that is able to communicate with other H-CPE.

  - BSEM Network Installation

A BS covers a certain regional area, which calls a 802.22b WRAN, where includes H-CPEs as shown in Figure 7. Each H-CPE can communicate with BS directly or indirectly through other H-CPEs.

    - Base Station (BS): A BS locates a center of a 802.22b WRAN, where the maximum radius of BS coverage (a) shown in Figure 1 is 10Km. (Case 1 in link margin example shown in Table 1)

    - High-capability CPE (H-CPE): H-CPEs locate randomly within a 802.22b WRAN, where the average coverage is 5Km (Case 5 in link margin example shown in Table 1).

Figure 7 An example of network installation for extended broadband services

  - Device Capabilities

    - Base Station (BS)

      - Transmission power: Fixed mode (4W EIRP)

      - Antenna gain: 6 dbi

      - Antenna height: 280ms (250m height above average terrain (HAAT) + 30ms meters above ground level (AGL))

    - High-capability CPE (H-CPE)

      - Transmission power:Fixed mode (1W EIRP)

      - Antenna gain: 11 dbi

      - Antenna height: 10m HAAT

  - Maximum available hops between BS to end devices (H-CPEs)

At least 1 (H-CPEs may communicate with BS directly, as well as through other H-CPEs.)

  - Number of Devices

    - Base Station (BS): 1

    - High-capability CPE (H-CPE): 250

  - Traffic

| Traffic Model C |  |
| --- | --- |
| Protocol | Internet Web-browsing (HTTP) |
| Data Traffic Direction | Downlink from BS to H-CPE (BS -> H-CPE … -> H-CPE) |
| Packet Length | 350 bytes for HTTP requests and 1500 bytes for payload data |
| Packet Traffic | See the traffic pattern in Figure 8 |
| Packet Arrival from L-CPE to H-CPE | Each H-CPE randomly initiates HTTP request …. |
| Simultaneous Communications | 50 H-CPEs ~ 250 H-CPEs |
| Buffer | H-CPE (Infinite-Buffer, Finite Buffer) (Note that each proposal presents the buffer size if the finite buffer is used) |
| Conditions | For simplicity, ignore the process network management frame exchanges such as registration, ranging, bandwidth request etc. Only consider data transmission/retransmission and ARQ |
| DL and UL Frame Allocation for management | For simplicity, we ignore the management frame exchanging process, but overall 10% DL and UL frame allocation (1ms) is considered for the management frame (frame preamble, header, management frames, etc) overhead and HTTP management except for data. |

Algorithm: After each reading time the new requests for pages are generated by the H-CPE (mean of 31 seconds), generate a HTTP request with the following parameters enlisted below. The parsing time is the time taken by the HTTP page to fill in all subpage requests which appear from the master page. After going through few of the subpages the H-CPE quits the session which is indicated by the last packet of the session. This is shown in 8.

Reading Time

First Packet of Session

Last Packet of Session

Reading Time

Parsing Time

Figure 8: HTTP traffic pattern and parameters

  - Reference Channel Model

    - Basic Channel Model (802.22 Channel Model)

      - See Table 1 and Table 2

  - Performance Criteria

    - Network Throughput

      - The network throughput is defined as the number of information bits per second that a network can successfully deliver or receive using the scheduling algorithms.

    - Delay (Maximum delay and average delay)

      - The maximum packet delay is defined as the maximum interval between packets originated at the source station (L-CPE) and received at the destination station (BS). The average packet delay is defined as the average interval between packets originated at the source station (L-CPE) and received at the destination station (BS).Packet Loss Radio

    - Packet Loss Ratio

      - The packet loss ratio in a network is defined as:

Packet Loss Ratio = Total Number of Successfully Received Packets / Total Number of Successfully Transmitted Packets

    - Reference Packet Error Rate (PER) on 1500bytes : 10e-3

- Proposal Defined Model (PDM)

  - Introduction

This Model is used for studying the performance of smart metering/monitoring and broadband service extension upon the user-defined configurations. This model enables to provide a flexible study for the perfomance.

  - Network Entities

    - Proposal shall define network entities of BS, H-CPEs and/or L-CPEs used in the PDM.

  - BSEM Network Installation

    - Proposal shall define network installation used in the PDM.

  - Device Capabilities

    - Proposal shall define device capabilities used in the PDM.

  - Maximum available hops between BS to end devices (H-CPEs)

    - Proposal shall define maximum available hops between BS to end devices used in the PDM.

  - Number of Devices

    - Proposal shall define number of devices used in the PDM.

  - Traffic

    - Proposal shall define traffic model used in the PDM.

  - Reference Channel Model

    - Proposal shall define channel model used in the PDM.

  - Performance Criteria

    - Network Throughput

      - The network throughput is defined as the number of information bits per second that a network can successfully deliver or receive using the scheduling algorithms.

    - Delay (Maximum delay and average delay)

      - The maximum packet delay is defined as the maximum interval between packets originated at the source station (L-CPE) and received at the destination station (BS). The average packet delay is defined as the average interval between packets originated at the source station (L-CPE) and received at the destination station (BS).Packet Loss Radio

    - Packet Loss Radio

      - The packet loss ratio in a network is defined as:

Packet Loss Ratio = Total Number of Successfully Received Packets / Total Number of Successfully Transmitted Packets

    - Reference Packet Error Rate (PER) : Proposal shall define BER used in the PDM.

- Reference

- IEEE P802.22b PAR in doc.: IEEE 802.22-yy/xxxxr004-rasg

- IEEE P802.22b 5C in doc.: IEEE 802.22-yy/xxxxr00000-04-rasg

- IEEE Std.802.22-2011

- Usage Cases in 802.22 Smart Grid and Critical Infrastructure Monitoring : IEEE 802.22-11/73r2

- Functional Requirements for IEEE 802.22b amendment: doc.: IEEE 802.22-yy/xxxxr02-000b

- 802.22b CPE Clarifications and Capabilities: 22-12-0018-00-000b

- Preliminary Link Budget Analysis for 802.22b: 22-12-0055-00-000b

- Link Budget Analysis for IEEE 802.22b: 22-12-0054-00-000b