IEEE P802.22
Wireless RANs

| Prediscussion of Selection Criteria for IEEE 802.22b amendment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-04-10 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Chang-woo Pyo | NICT | 3-4, Hikarino-oka, Yokosuka, Japan | +81-46-847-5120 | cwpyo@nict.go.jp |
| Xin Zhang | NICT |  |  |  |
| Chunyi Song | NICT |  |  |  |
| Mingtuo Zhou | NICT |  |  |  |
| Hiroshi Harada | NICT |  |  |  |

Abstract

This document presents itmes for selection criteria for evaluating proposals of IEEE 802.22b amendament.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Criteria Overview for Performance Study

| Network Model | Smart Grid & Monitoring Model | Broadband Service Extension Model |
| --- | --- | --- |
| Entities | BS H-CPEs L-CPEs | BS H-CPEs |
| Topology | Tree topology | Tree topology or Mesh topology |
| Applications | Metering applications Video monitoring application | Internet (Web browsing) VoIP |
| Channel Model | Basic channel model (22) Proposed channel model (new) | Basic channel model (22) |
| Performance criteria | Throughput Delay Packet Loss | Throughput Packet Loss |

- Smart Grid & Monitoring Model

  - Introduction

- This Model is used for studying the performance of Smart Grid and Monitoring indicated in Usage Case Category A [1], which has the properties of very large number of monitoring or metering CPEs, real-time monitoring, low duty cycle, infrastructure connections, etc.

  - Network Entities

    - Base Station (BS): Generalized equipment set providing connectivity, management and control of the customer premise equipment (CPE).

      - Number of BSs : 1

    - High-capability CPE (H-CPE): 802.22b H-CPE is enhanced 802.22-2011 CPE that is able to manage a local network consisting of a number of L-CPEs and to provide multi-hop connections between L-CPEs and 802.22b BS.

      - Number of H-CPEs : 30

    - Low-capability CPE (L-CPE): 802.22b L-CPE is a terminal station mainly for metering and monitoring, which will be installed by end users at home, office, or on infrastructure, etc., therefore the quantity of this L-CPE in a single 802.22b network will be huge. For this reason, it is featured by low complexity, easy installation, and relatively short communication distance.

      - Number of L-CPEs: 70 per H-CPE (Overall 2100 L-CPEs in a network)

  - Network Topology

    - Tree Topology

      - Supported hops: 2 hops from BS to L-CPE via H-CPE

      - Grid location of entities

        - Center of grid for BS

        - Equally covered the BS coverage area by H-CPE

        - Equally covered the H-CPE area by L-CPEs

  - Applications

    - Metering Applications

      - Protocol : TCP (Reno)

      - Packet Size: 64 MSDU bytes for TCP connection establishment (3-way handshake) and 1500 MSDU bytes for metering payload data from Source to Destination

        - Source: L-CPE

        - Destination: BS

      - Traffic: randomly initiated by L-CPEs, uniformly distributed packet arrival rate (e.g.,1 time per day or 1 time per hour)

      - Algorithm: at the start of simulation, generate a TCP connection establishment with the following TCP parameter configuration (as appropriate for the simulation platform):

| TCP Model Parameters |  |
| --- | --- |
| MSS | Ethernet (1500) |
| Receive Buffer (bytes) | 65535 |
| Receive Buffer Adjustment | None |
| Delayed ACK Mechanism | Segment/Clock based |
| Maximum ACK Delay (sec) | 0.05 |
| Slow-Start Initial Count (MSS) | 1 |
| Fast Retransmit | Enabled |
| Duplicate ACK Threshold | 3 |
| Fast Recovery | Reno |
| Window Scaling | Enabled |
| Selective ACK (SACK) | Disabled |
| ECN Capability | Disabled |
| Segment Send Threshold | Byte Boundary |
| Active Connection Threshold | Unlimited |
| Karn's Algorithm | Enabled |
| Nagle Algorithm | Disabled |
| Initial Sequence Number | Auto Complete |
| Initial RTO (sec) | 3.0 |
| Min RTO (sec) | 1.0 |
| Max RTO (sec) | 64.0 |
| RTT Gain | 0.125 |
| Deviation gain | 0.25 |
| RTT Deviation Coefficient | 4.0 |
| Timer Granularity | 0.5 |

    - Compressed Video Applications

      - Protocol: UDP (Video Stremaing)

      - Traffic: Constant Bit Rate (CBR), randomly initiated by one L-CPE in a group (1 time per hour), which is managed by H-CPE. Video monitoring duriation by a L-CPE is 60 seconds

        - Source: L-CPEs

        - Destination: BS

        - Destination sends one “video streaming request” to Source at time “StartTime” and receiveds stream from Source

      - Packet Length, Packet Arriving Interval: 300Kbps (Low Resolution) or 1Mbps (High Resolution)

  - Reference Channel Model

    - Basic Channel Model

      - 802.22 Channel Model

  - Performance Criteria

    - Network Throughput

    - Delay

    - Packet Loss

- Broadband Service Extension Model

  - Introduction

    - This Model is used for studying the performance of Broadband Service Extension indicated in Usage Case Category B [1], which has the properties of higher data transmission for internet acces, infrastructure or Ad hoc connections, etc.

  - Network Entities

    - Base Station (BS)

      - Number of BSs : 1

    - High-capability CPE (H-CPE)

      - Number of H-CPEs: 100

  - Network Topology

    - Tree Topology

      - Supported hops: more than 2 hops for maximum from BS to H-CPE

      - Random location of H-CPEs

    - Mesh Topology

      - Supported hops: more than 2 hops for maximum from BS to H-CPE

      - Random location of H-CPEs

  - Applications

    - Internet Web-browsing

      - Protocol: HTTP (version 1.0 or above)

      - MSDU sizes: 350 bytes for HTTP requests and 1500 bytes for payload data

        - Source: BS

        - Destination: H-CPE

      - Algorithm: After each reading time the new requests for pages are generated by the H-CPE (mean of 31 seconds), generate a HTTP request with the following parameters enlisted below. The parsing time is the time taken by the HTTP page to fill in all subpage requests which appear from the master page. After going through few of the subpages the H-CPE quits the session which is indicated by the last packet of the session. This is shown in Figure 1.

Reading Time

First Packet of Session

Last Packet of Session

Reading Time

Parsing Time

Figure 1: HTTP traffic pattern

| Component | Distribution | Parameters | PDF |
| --- | --- | --- | --- |
| Main object size (SM) | Truncated Lognormal | Mean = 10710 bytes SD = 25032 bytes Min = 100 bytes Max = 2 Mbytes (before truncation) | σ=1.37,μ=8.37 if x>max or x<min, discard and generate a new value for x |
| Reading time (Dpc) | Exponential | Mean = 30 sec |  = 0.033 |
| Parsing time (Tp) | Exponential | Mean = 0.13 sec | fx=λe−λx,x≥0 λ=7.69 |

    - Voice over IP (VoIP)

      - Protocol: VoIP

      - Traffic

        - Source: H-CPE/BS

        - Destination: BS/H-CPE

        - 96 kbps per call (Voice Codec G.711)

      - Call Arrivals from/to H-CPE

        - Poisson distribution

          - Mean: 1 hour

      - Call holding times

        - Exponential distribution

          - Mean: 3 minutes

  - Reference Channel Model

    - Basic Channel Model

  - Performance Criteria

    - Network Throughput

    - Delay

    - Packet Loss

- References

[1] Usage Cases in 802.22 Smart Grid and Critical Infrastructure Monitoring : IEEE 802.22-11/73r2