IEEE P802.22
Wireless RANs

| Management Plane Interfaces & Procedure Primitive Format |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | Self | 400 Deal Lake Drive, Apt 2B Asbury Park, NJ 07712 | 732-693-5812 | ranga.reddy@me.com |

Abstract

This contribution provides text to describe format of M-SAP and C-SAP primitives in Clause 14, as well as begins to layout how text in Clause 14 will be ordered.

R0: Initial Version

R1: Adds specification of SAP type to format of SAP

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[add a new clause, Clause 14, titled “Management Plane Interfaces & Procedures”]

[add a new sub-clause, Clause 14.1, titled “Primitive Format”, and add the following text into Section 14.1]

  - Primitve Format

In this section we define the components that make up each primitive:

    - Purpose

Text describing the function that this primitive services.

    - SAP Type

Type of SAP, either M-SAP or C-SAP this primitive operates over.

    - Operation Type

Specification of the Operation Type of the primitive, i.e. what kind of transaction the primitive is executing. The Operation Type can be an:

- Information Request (Info-REQ)

- Information Response (Info-RSP)

- Event Indication

- Acknowledgement

    - Destination

Entity that is receiving the primitive, e.g. BS, CPE, NCMS/AAA.

    - Data

A set of one for more information elements that carry information that is pertinent to the servicing of the primitive. For each information element data type and size of data is provided.

    - When Generated

Description of one or more, events/instances that cause primitive to be generated.

    - Effect of Receipt

Description of actions that are undertaken when primitive is received, one effect is provided for each item in “When Generated” (see 14.1.5).

[add a new sub-clause, Clause 14.2, titled “Primitive Defintions”, and add the following text to section 14.2]

  - Primitive Definitions

Primitives for both SAPs are defined in this Clause. Definitions for Mangement SAP (M-SAP) and Control SAP (C-SAP) primitives are provided for in Section 14.2.1 and Section 14.2.2, respectively. M-SAP primitives cover System Configuration, Monitoring Stastics, Notifications/Triggers, Sensing/Geo-location Reporting, and Communication with the Database Service. C-SAP primitives cover CPE Management, Session Management, Security Context Management, Radio Resource Management, and AAA service signaling.

    - Management SAP (M-SAP) Primitive Defintions

    - Control SAP (C-SAP) Primitive Defintions

References:

[1] IEEE 802.22 DCN: 22-12/27r1, “TGa Agenda Items for March 2012 Plenary”, https://mentor.ieee.org/802.22/dcn/12/22-12-0027-01-0mib-tga-agenda-items-for-march-2012-plenary.ppt