IEEE P802.22
Wireless RANs

| Accounting Primitives |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-05-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | Self | 400 Deal Lake Drive, Apt 2B Asbury Park, NJ 07712 | 732-693-5812 | ranga.reddy@me.com |

Abstract

This contribution provides text to provide the definition of primitives related to Accounting Management (wranIfBsMib->wranIfBsAm).

R0: Initial Version of this document

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

    - Management SAP (M-SAP) Primitive Defintions

[add a subsection to 14.1.1, 14.1.1.1 titled “Accounting Mangement Primitives”]

      - Accouting Management Primitives

[Insert the following text into 14.1.1.1]

Accounting management pertains to monitoring and managing information regarding CPE data transmission usage. Accounting records get updated whenever the CPE starts registration (REG-REQ) upon entry, is provisioned/configured for a new service flow (DSA-REQ/RSP), has a service flow configuration altered (DSC-REQ/RSP), and/or a CPE requests de-registration (DREG-REQ).

There are three Accounting Management Primitives: M-Accounting-REQUEST, M-Accounting-CONFIRMATION, and M-Accounting-INDICATION.

        - M-ACCOUNTING-REQUEST

          - Purpose

To requst accounting records for CPE’s from BS.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

BS

          - Data

- Record Index: Index into wranIfBsOtaUsageDataRecordTable that contains record for CPE

- Station ID: 9-bit Station ID assigned to CPE

- Flow ID: 3-Bit Flow ID that traffic is mapped to

- CPE MAC Address: IEEE 48-bit MAC Address identifying CPE

- Session ID: ID of session that current pertains to

See wranIfBsOtaUsageDataRecordTable (in wranIfBsAm) for further explanation on accounting record info.

          - When Generated

NCMS generates this primitive when it requires accounting record data from the BS.

          - Effect of Receipt

BS pulls relevant data from accouting record table (wranIfBsOtaUsageDataRecordTable) and sends it to the NCMS in the M-ACCOUNTING-CONFIRMATION primitive.

        - M-ACCOUNTING-CONFIRMATION

          - Purpose

BS uses this primitive to provide accounting data requested in M-ACCOUNTING-REQUEST.

          - SAP-Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

NCMS

          - Data

- Record Index: Index into wranIfBsOtaUsageDataRecordTable that contains record for CPE

- Station ID: 9-bit Station ID assigned to CPE

- Flow ID: 3-bit Flow ID that traffic is mapped to

- CPE MAC Address: IEEE 48-bit MAC Address identifying CPE

- Session ID: ID of session that current pertains to

- MAC SDU Count: Count of # of MAC SDUs transmitted on given Flow ID

- MAC PDU Count: Count of # of PDUs transmitted

- Start Time: Date/Time for start of session

- End Time: Date/Time for end of session

- # of service flows: # of service flow IDs (SFID) that are active on this Flow ID for the session

- List of service flow IDs: List of service flow IDs that are active on this Flow ID for the session

- # of QoS Parameter Sets: # of QoS Paramter sets used by service flows active on Flow ID for the session

- List of QoS Parameter Sets: List of QoS Parameter Sets used by service flows active on Flow ID for the session

See wranIfBsOtaUsageDataRecordTable (in wranIfBsAm) for further explanation on accounting record info.

          - When Generated

Sent by BS when BS receives a M-ACCOUNTING-REQUEST from NCMS

          - Effect on Receipt

NCMS will unpackage the Data item of received primitive and submit it whatever process NCMS/Operator uses for accounting purposes.

        - M-ACCOUNTING-INDICATION

          - Purpose

BS uses this primitive to inform that an accounting record has been updated. Accounting records get started/manipulated/deleted/processed during the following events: when a BS receives a REG-REQ from a CPE during registration, when a CPE requests de-registration (DREG-REQ), a new service flow has been activated on CPE via DSA-REQ/RSP, an existing service flow has been modified on the CPE via DSC-REQ/RSP, or a service flow has been deleted via DSD-REQ/RSP.

          - SAP-Type

M-SAP

          - Operation Type

Event Indication

          - Destination

NCMS

          - Data

- CPE MAC Address: IEEE 48-bit MAC Address identifying CPE

- # of service flows: # of service flow IDs (SFID) that are active on this Flow ID for the session. Only valid for Accouting Event Type = DSA, DSC, DSD

- List of service flow IDs: List of service flow IDs that are active on this Flow ID for the session. Only valid for Accouting Event Type = DSA, DSC, DSD

- Accounting Event Type: Indicates events that trigger manipulation of accounting records. Can take on the folloing values: REG, DSA, DSC, DSD, DREG

          - When Generated

BS generates this event when one of the following events has occurred: when a BS receives a REG-REQ from a CPE during registration, when a CPE requests de-registration (DREG-REQ), a new service flow has been activated on CPE via DSA-REQ/RSP, an existing service flow has been modified on the CPE via DSC-REQ/RSP, or a service flow has been deleted via DSD-REQ/RSP.

          - Effect on Receipt

NCMS will take information in Data item and generate the appropriate M-ACCOUNTING-REQUEST primitive to send to the BS to retrieve accounting records.

References:

[1] IEEE 802.22 DCN: 22-12/27r1, “TGa Agenda Items for March 2012 Plenary”, https://mentor.ieee.org/802.22/dcn/12/22-12-0027-01-0mib-tga-agenda-items-for-march-2012-plenary.ppt