IEEE P802.22
Wireless RANs

| Suggested Text for Table 273 - PHY parameters and timers |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sung-Hyun Hwang | ETRI | Korea | +82-42-860-1133 | shwang@etri.re.kr |
| Gwang-Zeen Ko | ETRI | Korea | +82-42-860-4862 | gogogo@etri.re.kr |
| Byung-Jang Jeong | ETRI | Korea | +82-42-860-6765 | bjjeong@etri.re.kr |
| Myung-Sun Song | ETRI | Korea | +82-42-860-5046 | mssong@etri.re.kr |
| Jae-Ick Choi | ETRI | Korea | +82-42-860-6160 | jichoi@etri.re.kr |

Abstract

As a resolution of comments ID #10 for IEEE 802.22a draft v2.0, this document proposes the suggested changes for Table 273 in Section 12.1.2.

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 1. Related comments

As the resolution of comments ID #10 for IEEE 802.22a draft v2.0, I submit the suggested changes for Table 273 in Section 12.1.2. The relating comment is as follows:

Table 273 defines various PHY parameters and timers that are used throughout the draft.

2. Suggested text

It does not need to define all min/default/max values for every PHY parameters. In some cases, Max value alone (or Min value) is good enough to define the parameter.

The reasons for suggested changes are as follows in detail. Regarding the TTG, the gap is always 210 us to absorb the propagation delay for up to 30 km and a CPE turnaround time of 10 us. From ‘Bandwidth Request Backoff Start’ to ‘Start of CDMA codes group’, the start value is ‘0’. Lastly, ‘EIRP per subcarrier’ is duplicated with ‘EIRP Density Level’. It should be deleted.

I propose the text changes as follows:

- 12.1.2 PHY (Initialization, Operation, and DS/US synchronization)

Table 273— PHY Parameters, Timers, Message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| BS | DCD Interval | Time between transmission of DCD messages | - | - | 10 s |
| BS | UCD Interval | Time between transmission of UCD messages | - | - | 10 s |
| BS | UCD Transition | The time the BS shall wait after repeating a UCD message with an incremented Configuration Change Count before issuing a US-MAP message referring to Upstream_Burst_Profiles defined in that UCD message | 2 MAC frames | - | - |
| BS | DCD Transition | The time the BS shall wait after repeating a DCD message with an incremented Configuration Change Count before issuing a DS-MAP message referring to Downstream_Burst_Profiles defined in that DCD message | 2 MAC frames | - | - |
| BS | Initial Ranging Interval | Time between Initial Ranging opportunities assigned by the BS | - | - | 2 s |
| BS | CLK-CMP Interval | Time between the clock compare measurements used for the generation of CLK-CMP messages. | 50 ms | 50 ms | 50 ms |
| CPE | Lost DS-MAP Interval (T56) | Time since last received DS-MAP message before downstream synchronization is considered lost | - | - | 600 ms |
| CPE | Lost US-MAP Interval (T57) | Time since last received US-MAP message before upstream synchronization is considered lost | - | - | 600 ms |
| CPE | Lost SCH (T58) | Number of SCH that can be lost until synchronization is considered lost | - | - | 15 |
| CPE | CDMA Ranging Retries | Number of retries on CDMA Ranging Requests | 1 | - | 4 |
| CPE, BS | Invited Ranging Retries | Number of retries on inviting Ranging Requests | 16 | - | - |
| BS | US-MAP Process Time | Time provided between arrival of the last bit of a US-MAP at a CPE and effectiveness of that map | 5 symbols | - | - |
| BS | CPE Ranging Response Processing Time | Time allowed for a CPE following receipt of a ranging response before it is expected to reply to an invited ranging request | 10 ms | - | - |
| CPE | T1 | Wait for DCD timeout | - | - | 5 * DCD interval maximum value |
| CPE | T2 | Wait for broadcast ranging timeout | - | - | 5 * ranging interval |
| CPE | T3 | Ranging Response reception timeout following the transmission of a Ranging Request | - | 200 ms | 200 ms |
| CPE | T4 | Wait for unicast ranging opportunity. If the pending-until-complete field was used earlier by this CPE, then the value of that field shall be added to this interval. | 1 s | 30 min. (fixed) 10 min. (portable | 30 min. |
| BS | T5 | Wait for Upstream Channel Change response | - | - | 2 s |
| CPE | T12 | Wait for UCD descriptor | - | - | 5 * UCD Interval maximum value |
| CPE | T20 | Time the CPE searches for preambles on a given channel | 2 MAC frames | - | - |
| CPE | T21 | Time the CPE searches for DS-MAP on a given channel | - | - | 10 s |
| BS | EIRPBS | EIRP of BS (DS) | -64 dBm | - | 63.5 dBm |
| BS | TTG | Transmit/Receive Transition Gap | - | 210 µs | - |
| BS | DIUC Mandatory Exit Threshold | CINR at or below which this DIUC can no longer be used and where change to a more robust DIUC is required. | -64 dB | - | +63.5 dB |
| BS | DIUC Mandatory Entry Threshold | The minimum CINR required to start using this DIUC when changing from a more robust DIUC is required | -64 dB | - | +63.5 dB |
| BS | Boosting | Boosting applied to a DS allocation | -12 dB | 0 dB | +9 dB |
| BS, CPE | BW Request Backoff Start | Initial size of BW Request opportunity used by CPEs to contend to send BW requests to BS | 0 | - | 15 |
| BS, CPE | BW Request Backoff End | Final size of BW Request opportunity used by CPEs to contend to send BW requests to BS | 0 | - | 15 |
| BS, CPE | UCS Notification Backoff Start | Initial backoff window size in units of UCS Notification opportunity used by CPEs to contend to send UCS notifications to BS. | 0 | - | 15 |
| BS, CPE | UCS Notification Backoff End | Final size of UCS Notification opportunity used by CPEs to contend to send UCS Notification to BS | 0 | - | 15 |
| BS, CPE | Contention-based reservation Timeout | Number of US-MAPs to receive before contention-based reservation is attempted again for the same connection | 0 | - | 255 |
| BS, CPE | BW Request opportunity size | Size (in OFDM slots) of PHY bursts, that a CPE may use to format and transmit a bandwidth request message in a contention request opportunity. | 0 | - | 255 |
| BS, CPE | UCS Notification request opportunity size | Size (in OFDM slots) of PHY bursts that a CPE may use to transmit a UCS Notification. | 0 | - | 255 |
| BS, CPE | # of initial ranging codes | Number of initial ranging CDMA codes (N) | 0 | - | 255 |
| BS, CPE | # of periodic ranging codes | Number of periodic ranging CDMA codes (M) | 0 | - | 255 |
| BS, CPE | # of bandwidth request codes | Number of bandwidth request CDMA codes (L) | 0 | - | 255 |
| BS, CPE | # of UCS notification codes | Number of UCS notification CDMA codes (I) | 0 | - | 255 |
| BS, CPE | Start of CDMA codes group | Indicates the starting number, S, of the group of codes used for the US. See 9.9.3 | 0 | - | 255 |
| BS,CPE | EIRP Density Level | EIRP Transmitted per subcarrier | -104 dBm | - | +23.5 dBm |
| BS, CPE | EIRP Control | EIRP per subcarrier that the CPE should apply to correct its’ current transmission EIRP | -104 dBm | - | +23.5 dBm |