IEEE P802.22
Wireless RANs

| Suggested MAC, Coexistence, and Cognitive Radio Parameter Updates |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-12-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | Self | 2501 Farmcrest Dr, Apt 1133 Herndon, VA 20171 | +011-732-693-5812 | ranga.reddy@me.com |

Abstract

This contribution proposes a resolution to comments 9, 11, & 13 in TGa LB2 comment database (DCN: 22-12/85r0).

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures

<http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <Carl R. Stevenson> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Introduction

Comments 9, 11, and 13 in the comment database for LB2 of TGa (DCN: 22-12/85r0), indicate that Tables 272 (MAC), 274 (Coexistence), and 276 (Cognitive Radio) are incomplete and don’t have minimum, default, and/or maximum values specified for all parameters and timers.

2. Suggested text

We propose that it is not necessary to define minimum, default, and/or maximum values for all parameters in Tables 272, 274, and 276.  The following updates are then suggested for these tables:

- In the case when a min/default/max value is not specified, the entry will be filled in with a ‘-‘

- When a min/default/max value is specified, the entry will reflect the values this parameter/timer is defined with elsewhere in the main standard.

In Table 272, the following considerations are made:

- For “Max # of multicast groups” parameter, we must note that Station ID = 0x0 is reserved, so with 9bits of Station ID that leaves 511 maximum # of Station IDs. “Max # of multicast groups” is given by maximum # of station IDs - # of CPEs in the cell. If we have at most only 1 CPE, the “Max # of multicast groups” is 510.

In Table 274, the following considerations are made:

- For “SCWBackoff_Max” parameter, we set the minimum # of superframes to 0 and maximum # of superframes to 15. We do not set the default. These selections are made to align the inputs to operate in a manner similar backoff procedures elsewhere in the standard (e.g. BW request, UCS notification).

In Table 276, the following considerations are made:

- The “T19” parameter will remain unspecified for now. In fact there doesn’t seem to be a reference to use of a T19 anywhere in the draft. Recommendation is (in a future revision/maintenance PAR) to remove the T19 timer or re-assign it for some other purpose.

================Start of modifications for Table 272 =================

Table 272— MAC Parameters, Timers, Message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| CPE, BS | DSx Request Retries | Number of Timeout Retries on DSA/DSC/DSD Requests | - | 3 | - |
| CPE, BS | DSx Response Retries | Number of Timeout Retries on DSA/DSC/DSD Responses | - | 3 | - |
| CPE | T6 | Wait for registration response | - | - | 3 s |
| CPE, BS | T7 | Wait for DSA/DSC/DSD Response timeout | - | - | 1 s |
| CPE, BS | T8 | Wait for DSA/DSC Acknowledge timeout | - | - | 300 ms |
| BS | T9 | Registration Timeout, the time allowed between the BS sending a RNG-CMD (success) to a CPE, and receiving a CBC-REQ from that same CPE | 300 ms | 300 ms | - |
| CPE, BS | T10 | Wait for Transaction End timeout | - | - | 3 s |
| BS | T13 | The time allowed for an CPE, following receipt of a REG-RSP message to send a TFTP-CPLT message to the BS | 15 min | 15 min | - |
| CPE | T14 | Wait for DSx-RSP/DSX-RVD Timeout | - | - | 200 ms |
| BS | T15 | Wait for MCA-RSP | 20 ms | 20 ms | - |
| CPE | T16 | Wait for bandwidth request grant | 10 ms | -- | Service QoS dependent |
| CPE | T18 | Wait for CBC-RSP timeout | - | 50 ms | << T9 |
| CPE, BS | T22 | Wait for ARQ-Reset |  |  | 0.5 s |
| CPE | T26 | Wait for TFTP-RSP | 10 ms | 10 ms | 200 ms |
| BS | T27 as idle timer | Maximum time between unicast grants to CPE when BS believes CPE upstream transmission quality is good enough | CPE Ranging Response Processing Time | - | - |
| BS | T27 as active timer | Maximum time between unicast grants to CPE when BS believes CPE upstream transmission quality is not good enough | CPE Ranging Response Processing Time | - | - |
| BS | T28 | Time allowed for the BS to complete the transmission of the backup/candidate channel list to its CPEs after initial registration by a new CPE, including the database service query | - | 60 s | << T45 |
| CPE | CBC Request Retries | Number of retries on CBC Request | - | 3 | - |
|  | DSx Flow Control | Maximum # of ongoing dynamic service flow (DSx) transactions that are ongoing | 1 | - | Infinite |
|  | MCA Flow Control | Maximum # of ongoing multicast group assignment (MCA-REQ/RSP) transactions | 1 | 0 | Infinite |
|  | Max # of multicast groups | Maximum # of multicast groups the BS supports in a cell | - | - | 510 |
| BS, CPE | T30 | CPE registration Timer (see 7.7.7.3.4 and 7.14.2.11) | 160 ms | 40.8 s | 10,485.6 s |
| BS, CPE | ARQ_BSN_MODULUS | Number of unique BSN values | - | 2^10 | - |
| BS, CPE | ARQ_WINDOW_SIZE | Max # of un-acknowledged ARQ blocks at a given time | -- | -- | ≤ (ARQ_BSN_MODULUS)/2 |
| BS, CPE | ARQ_BLOCK_LIFETIME | Max time interval an ARQ block shall be managed by the Tx ARQ state machine | 10 µs | - | 655.36ms |
| BS, CPE | ARQ_RETRY_TIMEOUT (TRANSMITTER_DELAY/RECEIVER_DELAY) | Minimum time interval a transmitter shall wait before retransmission of a unacknowledged block | 10 µs | - | 655.36ms |
| BS, CPE | ARQ_SYNC_LOSS_TIMEOUT | Max amount of time ARQ_TX_WINDOW_START or ARQ_RX_WINDOW_START shall be allowed to remain at the same value before declaring a loss of synchronization of the sender and receiver state machines for an ongoing transfer. | 10 µs | - | 655.36ms |
| BS, CPE | ARQ_RX_PURGE_TIMEOUT | Time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START | 10 µs | - | 655.36ms |
| BS, CPE | ARQ_BLOCK_SIZE | Size of ARQ block that SDU is fragmented into | 1 octet | - | 2040 octet |
| BS | Max CPE Transmit EIRP | Maximum CPE Transmit EIRP as negotiated during registration | -64 dBm | - | +63.5 dBm |
| CPE | Registration Request Retries | Number of retries on registration requests | 3 | - | - |
| CPE | Request Retries | Number of retries on bandwidth allocation requests | 16 | - | - |

================End of modifications for Table 272=================

================Start of modifications for Table 274 =================

Table 274 - Coexistence Parameters, Timers, Message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| BS | Tse (T33) | Time between transmission of the broadcast message of the operating backup and candidate channel sets for the purpose of spectrum etiquette. | - | - | 60 s |
| BS | T32 | Wait for the Frame Contention Response message | 1 superframe | - | 32 superframes |
| BS | FCW | Frame Contention Window: number of superframes during which a BS (FC_DST) accumulates the frame contention requests before reacting to it and responding to the FC_SRC’s. | 0 superframe | - | 16 superframes |
| BS | SCWBackoff_Max | Integer denoting the maximum superframes for the SCW backoff window | 0 | - | 15 |
| BS | Frame_Contention_Min | Number of frames not available for contention at a BS in a coexistence situation | 0 | 2 | 8 |
| BS | FCN_range | Exponent in base 2 defining the extent of the range of the random number FCN | 4 | - | 16 |
| BS | SF_release | Number of super-frame after which the BS releases the frames wan by another BS through frame contention | - | 5 | - |
| CPE | TCBP (T34) | The minimum time between transmissions of a CBP packet carrying its MAC address for identification by nearby CPEs and BSs for coexistence purpose, as well as by spectrum monitoring systems to identify potential interference situations. Timing value may depend on the regulatory domain where the WRAN system operates (see Annex A) | 8 s |  | 15 min |
| BS | SCWBackoff_Timer (T35) | Backoff timer that controls exiting or continuation of Frame contention procedure | - | - | SCWBackoff_Max |

================End of modifications for Table 274=================

================Start of modifications for Table 276 =================

Table 276 - Cognitive radio capability parameters, Timers, Message IEs

| Entity/Scope | Name | Reference | Min value | Default value | Max value |
| --- | --- | --- | --- | --- | --- |
| CPE | T19 | Time DS-channel remains unusable | - | - | - |
| CPE | T29 | Wait for BLM-ACK timeout | 10 ms | - | 300 ms |
| BS | T31 | Wait for BLM-REP timeout | 1 MAC Frame | - | - |
| CPE | BLM-REP Retries | Number of retries allowed for sending BLM-REP | - | 3 | - |
| BS, CPE | Channel Availability Check Time | The time during which a channel SHALL be checked for the presence of licensed incumbent signals having a level above the Incumbent Detection Threshold prior to the commencement of WRAN operation in that channel, and in the case of TV, a related channel at an EIRP level that can affect the measured channel. | - | 30 s | - |
| BS, CPE | Non-Occupancy Period | The required period during which WRAN device transmissions SHALL NOT occur in a given channel because of the detected presence of an incumbent signal in that channel above the Incumbent Detection Threshold, or in the case of TV, above a given EIRP level. | 10 min | - | - |
| BS, CPE | Channel Detection Time | The maximum time taken by a WRAN device to detect a licensed incumbent signal above the Incumbent Detection Threshold within a given channel during normal WRAN operation. | - | <=2 s to >=90% Probability of Detection with a False Alarm rate of <=10% | - |
| BS, CPE | Channel Setup Time | The window of time that may be taken by a WRAN CPE to transmit control information to a WRAN base station in order to establish operation with that base station at the prescribed power or, in the case of TV, at or below the allowable EIRP within a given channel. | - | 2 s | - |
| BS, CPE | Channel Opening Transmission Time (Aggregate transmission time) | The aggregate duration of control transmissions by WRAN devices during the Channel Setup Time, which starts at the end of the Channel Availability Check Time | - | 100 ms | - |
| BS, CPE | Channel Move Time (In-service monitoring) | The time taken by a WRAN system to cease all interfering transmissions on the current channel upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold, or in the case TV, to alternatively reduce its EIRP to that which is allowable within a given channel upon detection of a TV signal in the same or a related channel | - | 2 s | - |
| BS, CPE | Channel Closing Transmission Time (Aggregate transmission time) | The aggregate duration of control transmissions by the WRAN devices during the Channel Move/EIRP Reduction Time, which starts upon detection of a licensed incumbent signal above the relevant Incumbent Detection Threshold | - | 100 ms | - |
| BS, CPE | Channel Number | The channel number which is to be sensed by the SSF (SSF Subclause 10.4) | 0 | - | 255 |
| BS, CPE | Channel Bandwidth | The bandwidth of the channel to be sensed by the SSF (SSF Subclause 10.4) | - | 6/7/8 MHz (depending on regulatory domain) | - |
| BS, CPE | Maximum Probability of false alarm | In sensing modes 0 and 1 this value specifies the maximum probability of false alarm for each sensing mode decision in the signal present array | 0.0 | - | 0.255 |
| BS, CPE | NumSensingPeriods | Number of sensing periods field in a Sensing Window Specification Array entry | 0 | 1 | 127 |
| BS, CPE | SensingPeriodDuration | Duration of sensing period field (in units of OFDM symbols) in a Sensing Window Specification Array entry | 0 | 16 | 1023 |
| BS, CPE | SensingPeriodInterval | Periodicity of Sensing period field in a Sensing Window Specification Array entry | 0 | 200 | 2047 |
| BS, CPE | Sensing Window Specification Array | Array containing Sensing Window specification for each signal type enumerated in Signal Type Array | 1*sizeof(NumSensingPeriods+SensingPeriodDuration+SensingPeriodInterval) | - | 32*sizeof(NumSensingPeriods+SensingPeriodDuration+SensingPeriodInterval) |
| BS, CPE | Signal Type Array | Bitmap that indicates which signal types are to be sensed for in a given regulatory domain | - | Set on a regulatory domain-by-domain basis (see Annex A, Table 292) | - |
| BS, CPE | Sensing Mode | The sensing mode a CPE supports. Negotiated during CPE initialization. Various modes are described in Table 238. | - | No Sensing | - |
| BS, CPE | Signal Present Decision | Indication of whether or not a signal of a specific type has been detected. All unused values are reserved | 0x00 (Absent) | 0x7F (No Decision could be made) | 0xFF (Signal Present) |
| BS, CPE | Confidence Metric | Confidence with which sensing can determine the signal type | 0x00 (No Confidence) | - | 0xFF (Full confidence) |
| BS, CPE | Mean RSSI | Mean of M RSSI measurements | -104 dBm | - | +23.5 dBm |
| BS, CPE | Standard Deviation of RSSI | Standard deviation of the M RSSI measurements | +0.0 dB | - | +25.5 dB |
| BS, CPE | RSSI_detection_threshold | Energy detection threshold indicating the presence of an incumbent, other WRAN system, or interference | -120 dBm | - | -10 dBm |
| BS, CPE | Microphone Protection Radius | Radius of the contour within which the WRAN system cannot operate due to potential interference with the microphone | 0.1 km | - | 100 km |
| BS, CPE | TCandidate_Channel_Refresh (T41) | Maximum time interval allowed before sensing is performed on the candidate channel to ensure that no incumbents are detected. | 1 s | 6 s | 10 s |
| BS, CPE | TBackup_Channel_Refresh (T42) | Maximum time interval allowed before sensing is performed on the backup channel to ensure that no incumbents are detected. | 1 s | 6 s | 10 s |
| BS, CPE | TCandidate_to_Backup_Transition (T43) | Minimum time duration without detection of any incumbent for a candidate channel to transition to the backup channel. | 1 s | 30 s | 100 s |
| BS, CPE | TCh_Move (T44) | Maximum time to ensure that the channel move information is successfully conveyed to all the associated CPEs and BS (self coexistence mode). | 1 s | - | 10 s |
| BS, CPE | TNo_DB (T45) | Maximum WRAN operation time without access to the incumbent database service | 0.1 hr | - | 72 hrs |
| BS | TWait_Before_Channel_Move (T46) | Waiting time before which the BS moves to the first backup channel. This is used to make sure that all the CPEs are ready to move to the backup channel before BS switches operation to this backup channel. | 1 frame | - | 256x16 frames |
| CPE | TWait_Before_Channel_Move (T59) | Waiting time before which the CPE moves to its backup channels if it no longer hears from its BS. This is used to make sure that the CPE waits long enough after its UCS Notification so that the BS has had time to move to a backup channel, if it decided to do so. | 1 frame | - | 256x16 frames |
| BS, CPE | TRefresh_Database_Info (T47) | The prescribed time by the WRAN operator to refresh the incumbent database service. | 0.1 hr | - | 72 hrs |
| BS, CPE | TClear_N (T48) | Lapser Timer keeps track of whether the Operating Channel N has been cleared using spectrum sensing | 0.1 s | - | 60 s |
| BS, CPE | TClear_N-1 (T49) | Lapser Timer keeps track of whether the Adjacent Channel N-1 has been cleared using spectrum sensing | 0.1 s | - | 60 s |
| BS, CPE | TClear_N+1 (T50) | Lapser Timer keeps track of whether the Adjacent Channel N+1 has been cleared using spectrum sensing | 0.1 s | - | 60 s |
| BS, CPE | TLoss_of_BS_Contact (T51) | Initiated when the CPE loses contact with the BS | 1 s | - | 600 s |
| BS, CPE | TRange1 (T52) | Used for terrestrial geolocation. Initiated when the downstream burst leaves the BS (i.e., at the start of the frame preamble). | 1 TU | - | 1000 TU |
| BS, CPE | TINsens (T53) | The parameter TINsens is used to verify that in-band sensing has been done within the required In-service monitoring period. The TINsens parameter is driven by the regulatory domain requirements. See Annex A. | 0.1 s | - | 60 s |
| BS, CPE | TOUTsens (T54) | The parameter TOUTsens is used to verify that out-of-band sensing has been done within the required "Acquiring a channel monitoring period" specified in Annex A, Table 294 (30 s in the US). This value would be used to either initialize a ''lapse timer'' for each channel in the backup/candidate list at each CPE so that it is compared to Tsens | 0.1 s | - | 60 s |
| BS, CPE | Tsensin (T55) | Tsensin parameter corresponds to the maximum length of time required to carry out the sensing process on an in-band channel (N, N-1 or N+1). Manufacturers need to specify the sensing time required to detect the specified signals with required accuracy (see Figure 176). | 1 ms | - | 160 ms |
| CPE | Tsensout (T60) | Tsensout parameter corresponds to the maximum length of time required to carry out an out-of-band sensing process for a specified channel N (i.e., N, N-1 and N+1). Manufacturers need to specify the sensing time required to detect the specified signals out-of-band with required accuracy (see Figure 178). | 20 ms | - | 160 ms |
| BS, CPE | IETF Country code | 2 character, ASCII string denoted the regulatory domain of operation (e.g. “US” is for United States” | - | 2 characters | - |
| BS, CPE | TACBP | Timing Advance of the CBP burst (see step 2 of 10.5.2.3) (Note that the geolocation process will have to pre-adjust TACBP depending on the distance between the two CPE to be geolocated and the reference CPEs so that the delays measured by Vernier3 fall within the symbol cyclic prefix (e.g., 74.68 µs corresponding to 22.4 km). | -1024 TU | - | +1024 TU |

================End of modifications for Table 276=================