IEEE P802.22
Wireless RANs

| M-SAP Definition |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-02-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Ranga Reddy | Self | 2501 Farmcrest Drive, Apt 1133, Herndon, VA 20171 | 732-693-5812 | ranga.reddy@me.com |

Notice: This document has been prepared to assist IEEE 802.22. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).   The material in this document is subject to change in form and content after further study.  The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.22.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http://standards.ieee.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair Apurva Mody <apurva.mody@ieee.org> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.22 Working Group.  If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This contribution includes that text that was developed to address CID 5 & 8 in the LB2 database (DCN: 22-12/85r1). The following text defines M-SAP primitives.

Introduction

In during LB2 balloting it was determined that M-SAP definition was incomplete. Comment 5 and 8 in the comment database (DCN: 22-12/85r1: https://mentor.ieee.org/802.22/dcn/12/22-12-0085-01-0mib-tga-lb2-comment-database.xls). Comment 5 requests that the M-SAP defintion be completed. Comment 8 requests that the cognitive radio related primitives in 10.7 be relocated to the appropriate sections within Clause 14. The primitives within section 10.7.1-10.7.3 and 10.7.6 have been relocated to the M-SAP. This contribution includes text to be included in the draft amendment that satisfies this comment.

Editorial Instruction

- Change name of section from “Management SAP (C-SAP) Primitive Definitions” to “Management SAP (M-SAP)”

- Replace the text that currently exists in 14.2.1 with the new text below

[============================================Start of new text========================================]

      - Accounting Management Primitives

Accounting management pertains to monitoring and managing information regarding CPE data transmission usage. Accounting records get updated whenever the CPE starts registration (REG-REQ) upon entry, is provisioned/configured for a new service flow (DSA-REQ/RSP), has a service flow configuration altered (DSC-REQ/RSP), and/or a CPE requests de-registration (DREG-REQ).

There are three Accounting Management Primitives: M-Accounting-REQUEST, M-Accounting-CONFIRMATION, and M-Accounting-INDICATION.

        - M-ACCOUNTING-REQUEST

          - Purpose

To request accounting records for CPE’s from BS.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

BS

          - Data

- M-ACCOUNTING-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Record Index | Integer | 11 bits | Index into wranIfBsOtaUsageDataRecordTable that contains record for CPE |
| Station ID | Integer | 9 bits | Station ID assigned to CPE |
| Flow ID | Integer | 3 bits | Flow ID that traffic is mapped to |
| CPE MAC Address | MAC Address | 48 bits | CPE’s 48bit IEEE MAC address |
| Session ID | Integer | 32 bits | Session ID of session that record pertains to |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-ACCOUNTING-INDICATION. Time format defined in Section 14.1.5 |
| Padding |  | 1 bit | Set to ‘0’ |

See wranIfBsOtaUsageDataRecordTable (in wranIfBsAm) for further explanation on accounting record info.

          - When Generated

NCMS generates this primitive when it requires an accounting record data from the BS.

          - Effect of Receipt

BS pulls relevant data from accouting record table (wranIfBsOtaUsageDataRecordTable) and sends the data in the M-ACCOUNTING-CONFIRMATION primitive.

        - M-ACCOUNTING-CONFIRMATION

          - Purpose

BS uses this primitive to provide accounting data requested in M-ACCOUNTING-REQUEST.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

NCMS

          - Data

- M-ACCOUNTING-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Record Index | Integer | 11 bits | Index into wranIfBsOtaUsageDataRecordTable that contains record for CPE |
| Station ID | Integer | 9 bits | Station ID assigned to CPE |
| Flow ID | Integer | 3 bits | Flow ID that traffic is mapped to |
| CPE MAC Address | MAC Address | 48 bits | CPE’s 48bit IEEE MAC address |
| Session ID | Integer | 32 bits | Session ID of session that record pertains to |
| MAC SDU Count | Integer | 32 bits | Count of # of MAC SDUs transmitted on given Flow ID |
| MAC PDU Count | Integer | 32 bits | Count of # of PDUs transmitted |
| Start Time | Character String | 20 octets | String denoting Start Time of session. Time format defined in Section 14.1.5 |
| End Time | Character String | 20 octets | String denoting End Time of session |
| # of Service Flows | Integer | 8 bits | Number of service flows listed |
| Service Flow List | Octet String | # of Service Flows * 32 bits | String of octets, where by each set of 4 octets is a Service Flow ID |
| # of QoS Profile Sets | Integer | 8 bits | Number of QoS Profile Sets listed |
| QoS Profile List | Octet String | # of QoS Profile Sets * 16 bits | List of indexes into wranIfBsScTable, that point to the definiton of the QoS Parameter set of each service flow listed in Service Flow List |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-ACCOUNTING-REQUEST. Time format defined in 14.1.5 |
| Padding |  | 1 bit | Set to ‘0’ |

See wranIfBsOtaUsageDataRecordTable (in wranIfBsAm) for further explanation on accounting record info.

          - When Generated

Sent by BS when BS receives a M-ACCOUNTING-REQUEST from NCMS.

          - Effect of Receipt

NCMS will unpackage the Data item of received primitive and submit it whatever process NCMS/Operator uses for accounting purposes.

        - M-ACCOUNTING-INDICATION

          - Purpose

BS uses this primitive to inidicate to the NCMS that accounting records have been updated at the BS. Accounting records get started/manipulated/deleted/processed during the following events: when a BS receives a REG-REQ from a CPE during registration, when a CPE requests de-registration (DREG-REQ), a new service flow has been activated on CPE via DSA-REQ/RSP, an existing service flow has been modified on the CPE via DSC-REQ/RSP, or a service flow has been deleted via DSD-REQ/RSP.

          - SAP Type

M-SAP

          - Operation Type

Event Indication

          - Destination

NCMS

          - Data

- M-ACCOUNTING-INDICATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| CPE MAC Address | MAC Address | 48 bits | IEEE 48 bit MAC address identifying CPE |
| # of Service Flows | Integer | 8 bits | Number of service flows listed |
| Service Flow List | Octet String | # of Service Flows * 32 bits | String of octets, where by each set of 4 octets is a Service Flow ID |
| Account Event | Integer | 3 bits | Indicates events that trigger manipulation of accounting records. Can take on the following values: 000: REG 001: DSA 010: DSC 011: DREG 100-111: Reserved |
| Timestamp | Character String | 20 characters | Timestamp of the present request at time of transmission. Time format defined in Section 14.1.5. |
| Padding |  | 5 bits | Set to ‘00000’ |

          - When Generated

BS generates this event when one of the following events has occurred: when a BS receives a REG-REQ from a CPE during registration, when a CPE requests de-registration (DREG-REQ), a new service flow has been activated on CPE via DSA-REQ/RSP, an existing service flow has been modified on the CPE via DSC-REQ/RSP, or a service flow has been deleted via DSD-REQ/RSP.

          - Effect of Receipt

NCMS will take information in Data item and generate the appropriate M-ACCOUNTING-REQUEST primitive to send to the BS to retrieve accounting records.

      - Internet Protocol (IP) Management Primitives

IP management pertains to executing primitives related to establishing IP connections using the secondary management connection during CPE intialization (see Section 7.14.2.13).

There are three Accounting Management Primitives: M-DHCP-DISCOVER-REQUEST, M-DHCP-OFFER-CONFIRMATION, M-DHCP-SETUP-REQUEST, and M-DHCP-SETUP-CONFIRMATION.

        - M-DHCP-DISCOVER-REQUEST

          - Purpose

Encapsulate DHCPDISCOVER message (see IETF RFC 2131), created by CPE DHCP client, to NCMS to discover a DHCP server can provide IP management/setup.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

NCMS/DHCP

          - Data

- M-DHCP-DISCOVER-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| CPE MAC Address | MAC Address | 48 bits | CPE’s 48bit IEEE MAC address |
| DHCPDISCOVER message size | Integer | 16 bits | Size of DHCPDISCOVER message in octets |
| DHCPDISCOVER message | Octet String | DHCPDISCOVER message size * 8 bits | Contents of DHCPDISCOVER message |
| Timestamp | Character String | 20 characters | Timestamp of the present request at time of transmission. Time format defined in Section 14.1.5. |

          - When Generated

CPE generates this primitive during the initialization procedure if it requires allocation of an IP address.

          - Effect of Receipt

NCMS forwards DHCPDISCOVER message to DHCP server (or servers) in the operator network.

        - M-DHCP-OFFER-CONFIRMATION

          - Purpose

NCMS/DHCP server uses this primitive to provide DHCP server information to a CPE.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

CPE

          - Data

- M-DHCP-OFFER-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| CPE MAC Address | MAC Address | 48 bits | CPE’s 48bit IEEE MAC address |
| DHCPOFFER message size | Integer | 16 bits | Size of DHCPOFFER message in octets |
| DHCPOFFER message | Octet String | DHCPOFFER message size * 8 bits | Contents of DHCPOFFER message |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-DHCP-DISCOVER-REQUEST. Time format defined in Section 14.1.5. |

          - When Generated

Sent by NCMS/DHCP server when NCMS/DHCP server receives a M-DHCP-DISCOVER-REQUEST from CPE. If more than once DHCP server exists within NCMS, than one copy of this primitive is generated and sent for each DHCP server.

          - Effect of Receipt

CPE will unpackage the DHCPOFFER message and pass it to DHCP client on CPE.

        - M-DHCP-SETUP-REQUEST

          - Purpose

Encapsulate DHCPREQUEST message (see IETF RFC 2131), created by CPE DHCP client, to NCMS to request parameters and/or IP address offered in DHCPOFFER message sent by DHCP server.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

NCMS/DHCP

          - Data

- M-DHCP-SETUP-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| CPE MAC Address | MAC Address | 48 bits | CPE’s 48bit IEEE MAC address |
| DHCPREQUEST message size | Integer | 16 bits | Size of DHCPREQUEST message in octets |
| DHCPREQUEST message | Octet String | DHCPREQUEST message size * 8 bits | Contents of DHCPREQUEST message |
| Timestamp | Character String | 20 characters | Timestamp of the present request at time of transmission. Time format defined 14.1.5. |

          - When Generated

CPE generates this primitive during the initialization procedure once it has received the M-DHCP-OFFER-CONFIRMATION primitive containing a DHCPOFFER message from a DHCP server.

          - Effect of Receipt

NCMS forwards DHCPDISCOVER message to DHCP server the CPE has selected within the operator network.

        - M-DHCP-SETUP-CONFIRMATION

          - Purpose

NCMS/DHCP server uses this primitive to provide DHCP configuration parameters and length of DHCP lease to CPE.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

CPE

          - Data

- M-DHCP-SETUP-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| CPE MAC Address | MAC Address | 48 bits | CPE’s 48bit IEEE MAC address |
| DHCPACK message size | Integer | 16 bits | Size of DHCPACK message in octets |
| DHCPACK message | Octet String | DHCPACK message size * 8 bits | Contents of DHCPACK message |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-DHCP-SETUP-REQUEST. Time format defined in Section 14.1.5. |

          - When Generated

Sent by NCMS/DHCP server when NCMS/DHCP server receives a M-DHCP-STEUP-REQUEST from CPE.

          - Effect of Receipt

CPE will unpackage the DHCPACK message and used the contained information to configure IP connection on the CPE.

      - Database Service Primitives

The following list of messages, present in the IEEE P802.22 Standard, defines the necessary messaging to support access to the database service by the BS.  The format described below shall be used for the messages sent directly to the database service as well as those received directly from the database service. Some parameters in the following primitives are variable length character strings. The length of these parameters is given in terms of the number of characters in those strings. The total size of those parameters is the number of characters (the length) multiplied by the size of each character. For ASCII character sets, each character is 1 octet. For Unicode character sets, each character is 2 octets. Note that all variable length character strings shall be null terminated.

        - M-DB-AVAILABLE-REQUEST

          - Purpose

Allow the BS to verify that it is connected to the database service in order to receive channel availability and maximum allowed EIRP updates.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

Database Service Provider

          - Data

- M-DB-AVAILABLE-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Base station-ID Length | Integer | 2 bytes | Length of Base station-ID field (# of characters) |
| Base station-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |
| Database Service URL Length | Integer | 2 bytes | Length of database service URL field (# of characters). This is used to set the Locator for the Database service. |
| Database Service URL | Character String | Variable | A fully qualified URL starting with, http:// or https:// |
| Base Station Database Service Access URL Length | Integer | 2 bytes | Length of Base Station Database Service URL filed (# of characters) |
| Base Station Database Service Access URL | Character String | Variable | A fully qualified URL. This is used to set the Locator for the Base Station Access by the Database Service. |
| Base Station Management URL Length | Integer | 2 bytes | Length of Base Station Management URL field (# of characters). |
| Base Station Management URL | Character String | Variable | A fully qualified URL. This is used to set the Locator for the BS Management Service. |
| Timestamp | Character String | 20 characters | Timestamp of the present request at time of transmission. Time format defined 14.1.5. |

          - When Generated

When BS needs to confirm connectivity to a database service provider (see SM_Database_Update 10.2.6.5).

          - Effect of Receipt

Database Service receives primitive, processes it, and responds with M-DB-AVAILABLE-CONFIRMATION to confirm that the BS that sent the primitive has connectivity with it.

        - M-DB-AVAILABLE-CONFIRMATION

          - Purpose

Allows the database service to confirm that the BS is still connected to the database service.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

BS

          - Data

- M-DB-AVAILABLE-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Base station-ID Length | Integer | 2 bytes | Length of Base station-ID field (# of characters) |
| Base station-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |
| Status/Error Code | Integer | 2 bytes | Error/status code returned by database service. Values defined in Section 5.13 in IETF PAWS protocol. |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-DB-AVAILABLE-REQUEST. Time format defined in 14.1.5. |

          - When Generated

When Database Service receives M-DB-AVAILABLE-REQUEST from a BS.

          - Effect of Receipt

BS will process received primitive, and proceded to “Update location specific list of available channels from the database service – Reset Timer T45” stage of the SM_Database_Update procedure as defined in 10.2.6.5. If BS doesn’t receive primitive before T45 expires, execute Policy 1e (see Table 234).

        - M-DB-ENLISTMENT-REQUEST

          - Purpose

Allow the BS to enlist with the database service a device that has joined its WRAN network30.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

Database Service Provider

          - Data

- M-DB-ENLISTMENT-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device obtained as part of its process to associate 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 0x03-0xFF = Reserved |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |
| Technology Identifier Length | Integer | 2 bytes | Length of Technology Identifier field (# of characters) |
| Technology Identifier | Character String | Variable | Technology Identifier string that identifies the type of technology being used (e.g. “IEEE_802.22”) |
| Proxy Device-ID Length | Integer | 2 bytes | Length of Proxy Device-ID field (# of characters) |
| Proxy Device-ID | Character String | Variable | This element is the device ID for the device (most likely the controlling BS) that is acting as the proxy to the database service. (In US, this is the FCC-ID.) |
| Proxy Serial Number Length | Integer | 2 bytes | Length of Proxy Serial Number field (# of characters) |
| Proxy Serial Number | Character String | Variable | This element is the serial number for the device (most likely the controlling BS) that is acting as the proxy to the database service. |
| Location Data String Length | Integer | 2 bytes | Length of Location Data String field (# of characters) |
| Location Data String | Character String | NMEA 0183 | The value identifies the location of the device (latitude, longitude). |
| Responsible Party Name Length | Integer | 2 bytes | Length of Responsible Party Name field (# of characters) |
| Responsible Party Name | Character String | Variable |  |
| Antenna height | Integer | 1 byte | Antenna height above ground level in meters. |
| Antenna Operation Type | Integer | 1 bytes | Indicate where (e.g. the environment) this antenna is positioned: 0x00 = outdoor 0x01 = indoor 0x02-0xFF = Reserved |
| If (Device Type = 0x00 or 0x01) { |  |  |  |
| Contact Name Length | Integer | 2 bytes | Length of Contact Name field (# of characters) |
| Contact Name | Character String | Variable |  |
| Contact Physical Address Length | Integer | 2 bytes | Length of Contact Physical Address field (# of characters) |
| Contact Physical Address | Character String | Variable |  |
| Contact Email Address Length | Integer | 2 bytes | Length of Contact Email Address field (# of characters) |
| Contact Email Address | Character String | Variable |  |
| Contact Telephone Number Length | Integer | 2 bytes | Length of Contact Telephone Number field (# of characters) |
| Contact Telephone Number | Character String | Variable |  |
| } |  |  |  |
| If (Device Type = 0x00) { |  |  |  |
| Access Type | Integer | 1 byte | The value identifies the type of access being used for communication to the database service. 0x00 = URL 0x01 = IPv4 IP address 0x02 = IPv6 IP address 0x03-0xFF = Reserved |
| Base Station Network Address { |  |  |  |
| If (Access Type = 0x00) |  |  |  |
| Database Service URL Length | Integer | 2 bytes | Length of Database Service URL field (# of characters) |
| Database Service URL | Character String | Variable | A fully qualified URL starting with, http:// or https:// |
| Else If (Access Type = 0x01) |  |  |  |
| Database Service IPv4 Address | Integer | 4 bytes | IPv4 network address for database service |
| Else if (Access Type = 0x02) |  |  |  |
| Database Service IPv6 Address | Integer | 16 bytes | IPv6 network address for database service |
| } |  |  |  |
| Base station port number | Integer | 2 bytes | Port number of the internet connection |
| If (wranIfDatabaseServiceBSAntennaInformationSupportedMib) { |  |  |  |
| Antenna information | Character String | 72 bytes | Antenna directionality information of the device in dB relative to the main lobe maximum gain for every 5 degree azimuth clockwise starting from the direction of the maximum antenna gain expressed in unit of 0.25 dB over the range –63.75 dB (encoded 0x00) to 0 dB (0xFF). (to allow the database calculation of the channel availability and the maximum allowed EIRP values at the registering location31 |
| Antenna azimuth | Integer | 2 bytes | Antenna azimuth in degrees, clockwise from true North. |
| Antenna Polarization | Integer | 1 bytes | Antenna Polarization type: 0x00 = Elliptical 0x01 = Horizontal 0x02 = Left-hand circular 0x03 = Right-hand circular 0x04 = Vertical circular 0x05-0xFF = Reserved |
| } |  |  |  |
| } |  |  |  |
| Timestamp | Character String | 20 characters | Timestamp of the present request at time of transmission. Time format defined in Section 14.1.5. |

          - When Generated

When BS needs to enlist or register a device under its’ control (including itself) with the database service provider.

          - Effect of Receipt

Database Service receives primitive, processes it, and responds with M-DEVCE-ENLISTMENT-CONFIRM to confirm that the requested device was successfully enlisted with the database service provider.

        - M-DB-ENLISTMENT-CONFIRMATION

          - Purpose

Allows the database service to confirm to the BS that the new device has been successfully registered.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

BS

          - Data

- M-DB-ENLISTMENT-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |
| Technology Identifier Length | Integer | 2 bytes | Length of Technology Identifier field (# of characters) |
| Technology Identifier | Character String | Variable | Technology Identifier string that identifies the type of technology being used (e.g. “IEEE_802.22”) |
| Status/Error Code | Integer | 2 bytes | Error/status code returned by database service. Values defined in Section 5.13 in IETF PAWS protocol. |
| Timestamp | Character String | NMEA 0183 $ZDA string | Copied from the timestamp in the M-DB-ENLISTMENT-REQUEST. Time format defined in Section 14.1.5. |

          - When Generated

When Database Service receives M-DB-ENLISTMENT-REQUEST from a BS.

          - Effect of Receipt

BS will process received primitive, and continue to enlist and delist (see M-DB-DELIST-REQUST/CONFIRMATION) as devices enter and/or leave the network.

        - M-DB-AVAILABLE-CHANNEL-REQUEST

          - Purpose

Allow the BS to request a list of available channels and maximum EIRP per channel from the database service for the specified type of device at a particular location.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

Database Service Provider

          - Data

- M-DB-AVAILABLE-CHANNEL-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device Type | Integer | 1 byte | The value identifies the type of device at the geo-location registering 0x00 = Fixed base station 0x01 = Fixed CPE 0x02 = Personal/portable mode 0x03-0xFF = Reserved |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial Number | Character String | Variable |  |
| Technology Identifier Length | Integer | 2 bytes | Length of Technology Identifier field (# of characters) |
| Technology Identifier | Character String | Variable | Technology Identifier string that identifies the type of technology being used (e.g. “IEEE_802.22”) |
| Location Data Type | Integer | 1 byte | Location data type identifier: 0x00 – Single Point Location 0x01 – Polygon for Geo-graphic area 0x02 – Batch request for multiple points |
| if(Location Data Type == 0x00) { |  |  |  |
| Single Point Location Data Length | Integer | 2 bytes | length of Single Point Location Data String in octets |
| Single Point Location Data String | Character String | NMEA 0183 Character String | String that identifies location of the device (latitude, longitude). |
| } |  |  |  |
| elseif(Location Data Type == 0x01 or 0x02) { |  |  |  |
| Number of points | Integer | 1 byte | # of location data points that represents either vertices of a polygon representing the area being queried for (Location Data Type == 0x01) or a set of points representing a batch request (Location Data Type == 0x02) |
| for(i=0;i<Number of vertices;i++) { |  |  |  |
| Point String Length | Integer | 2 bytes | length of Point Data String in octets |
| Point Data String | Character String | NMEA 0183 Character String | String that identifies location of a vertex of a polygon that defines the area being queried |
| } |  |  |  |
| } |  |  |  |
| Timestamp | Character String | 20 octets | String representing timestamp for when message was sent. Time format is defined in Section 14.1.5 |

          - When Generated

When BS needs request a list of available channels for devices operating under its control. If Location Data Type == 0x01 or 0x02, then the DeviceID and Serial Number fields shall be those of the BS/SM making the query.

          - Effect of Receipt

Database Service receives primitive, processes it, and responds with M-DB-AVAILABLE-CHANNEL-CONFIRMATION to confirm that the BS that sent the primitive has connectivity with it.

        - M-DB-AVAILABLE-CHANNEL-CONFIRMATION

          - Purpose

Allows the database service to confirm that the BS is still connected to the database service.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

BS

          - Data

- M-DB-AVAILABLE-CHANNEL-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device- ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial number | Character String | Variable |  |
| Technology Identifier Length | Integer | 2 bytes | Length of Technology Identifier field (# of characters) |
| Technology Identifier | Character String | Variable | Technology Identifier string that identifies the type of technology being used (e.g. “IEEE_802.22”) |
| Location Data Type | Integer | 1 byte | Location data type identifier: 0x00 – Single Point Location 0x01 – Polygon for Geo-graphic area 0x02 – Batch request for multiple points |
| if (Location Data Type == 0x00 or 0x01) { |  |  |  |
| Number of Location Data Type 0/1 Channels Available | Integer | 1 byte |  |
| if (Number of Location Data Type 0/1 Channels Available > 0) { |  |  | If the number of channels is equal to 0, this means that the device cannot operate. |
| for (i=0;i< Number of Location Data Type 0/1 Channels Available; i++) { |  |  |  |
| Channel Start Frequency | Integer | 8 bytes | Frequency for start of channel in Hz |
| Channel End Frequency | Integer | 8 bytes | Frequency for end of channel in Hz |
| Max_Allowed_EIRP | Integer | 1 bytes | Maximum allowed EIRP expressed in dBm over the range -64 dBm (encoded 0x00) to +63.5 dBm (encoded 0xFF) |
| Channel Availability Start Time | Character String | 20 octets | String representing timestamp for when channel availability starts. Time format defined in Section 14.1.5 |
| Channel Availability End Time | Character String | 20 octets | String representing timestamp for when channel availability ends. Time format defiend in Section 14.1.5. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| if (Location Data Type == 0x02) { |  |  |  |
| Number of Location in Batch Request |  |  |  |
| for(x=0;x<Number of Location in Batch Request; x++) { |  |  |  |
| Batch Location String Length | Integer | 2 bytes | Length of location string |
| Batch Location String | Character String | NMEA 0183 Character String | String that identifies location in a set of batch locations |
| Number of Location Data Type 2 Channels available |  |  |  |
| if (Number of Location Data Type 2 Channels Available > 0) { |  |  | If the number of channels is equal to 0, this means that the device cannot operate. |
| for (y=0;y< Number of Location Data Type 0/1 Channels Available; y++) { |  |  |  |
| Channel Start Frequency | Integer | 8 bytes | Frequency for start of channel in Hz |
| Channel End Frequency | Integer | 8 bytes | Frequency for end of channel in Hz |
| Max_Allowed_EIRP | Integer | 1 bytes | Maximum allowed EIRP expressed in dBm over the range -64 dBm (encoded 0x00) to +63.5 dBm (encoded 0xFF) |
| Channel Availability Start Time | Character String | 20 octets | String representing timestamp for when channel availability starts. Time format defined in Section 14.1.5 |
| Channel Availability End Time | Character String | 20 octets | String representing timestamp for when channel availability ends. Time format defiend in Section 14.1.5. |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| Status/Error Code | Integer | 2 bytes | Error code as defined by IETF PAWS protocol, section 5.13 |
| Timestamp | Character String | 20 octets | Timestamp as copied from the M-DB-AVAILABLE-CHANNEL-REQUEST. Time format is defined in Section 14.1.5. |

          - When Generated

When Database Service receives M-DB-AVAILABLE-CHANNEL-REQUEST from a BS.

          - Effect of Receipt

BS will process received primitive, then procede to execute the SM_Database_Update procedure as defined in 10.2.6.5, following the “Is the current operating channel available for the BS and all the associated CPEs?”.

        - M-DB-DELIST-REQUEST

          - Purpose

Allow the BS to request the database service to remove the enlistment of a device that was associated with that base station.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

Database Service Provider

          - Data

- M-DB-DELIST-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device-ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial Number | Characters String | Variable |  |
| Technology Identifier Length | Integer | 2 bytes | Length of Technology Identifier field (# of characters) |
| Technology Identifier | Character String | Variable | Technology Identifier string that identifies the type of technology being used (e.g. “IEEE_802.22”) |
| Responsible Party Name Length | Integer | 2 bytes | Length of Responsible Party Name field (# of characters) |
| Responsible Party Name | Character String | Variable |  |
| Location Data String Length | Integer | 2 bytes | Length of Location Data String |
| Location Data String | Character String | NMEA 0183 Character string | The value identifies the location of the device (latitude, longitude).33 |
| Timestamp | Character String | 20 octets | String representing timestamp for when message was sent. Time format is defined in Section 14.1.5 |

          - When Generated

When BS needs delist/deregister a device under its control that has been previously (successfully) enlisted with the database service provider.

          - Effect of Receipt

Database Service receives primitive, processes it, and responds with M-DB-DELIST-CONFIRMATION to confirm that the BS that requested device was successfully delisted.

        - M-DB-DELIST-CONFIRMATION

          - Purpose

Allows the database service to inform the BS whether its request to remove the enlistment of a device that was associated with that base station was successfully received and executed by the database service.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

BS

          - Data

- M-DB-DELIST-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device- ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial number | Character String | Variable |  |
| Technology Identifier Length | Integer | 2 bytes | Length of Technology Identifier field (# of characters) |
| Technology Identifier | Character String | Variable | Technology Identifier string that identifies the type of technology being used (e.g. “IEEE_802.22”) |
| Responsible Party Name Length | Integer | 2 bytes | Length of Responsible Party Name field (# of characters) |
| Responsible Party Name | Character String | Variable |  |
| Location Data String Length | Integer | 2 bytes | Length of Location Data String |
| Location Data String | Char | NMEA 0183 Character string | The value identifies the location of the device (latitude, longitude).33 |
| Status/Error Code | Integer | 2 bytes | Error code as defined by IETF PAWS protocol, section 5.13 |
| Timestamp | Character String | 20 octets | Timestamp as copied from the M- DB-DELIST-REQUEST. Time format is defined in Section 14.1.5 |

          - When Generated

When Database Service receives M-DB-DELIST-REQUEST from a BS.

          - Effect of Receipt

BS will process received primitive, then may delete the enlistment information for the device that has been recently delist or may keep that information in case the device needs to be re-enlisted in the future.

        - M-DB-OPERATING-TV-CH-SELECTION-CONFIRMATION

          - Purpose

Allows the SM to inform the database service when it has selected a new operating channel.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

Database Service

          - Data

- M-DB-OPERATING-TV-CH-SELECTION-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Device-ID Length | Integer | 2 bytes | Length of Device-ID field (# of characters) |
| Device- ID | Character String | Variable | In US, this is FCC-ID |
| Serial Number Length | Integer | 2 bytes | Length of Serial Number field (# of characters) |
| Serial number | Character String | Variable |  |
| Technology Identifier Length | Integer | 2 bytes | Length of Technology Identifier field (# of characters) |
| Technology Identifier | Character String | Variable | Technology Identifier string that identifies the type of technology being used (e.g. “IEEE_802.22”) |
| Channel Frequency Start (Hz) | Integer | 8 bytes | Start of channel, frequency in Hz |
| Channel Frequency End (Hz) | Integer | 8 bytes | End of channel, frequency |
| Location Data String Length | Integer | 2 bytes | Length of Location Data String |
| Location Data String | Char | NMEA 0183 Character string | The value identifies the location of the device (latitude, longitude). |
| Timestamp | Character String | 20 octets | String representing timestamp for when message was sent. Time format is defined in Section 14.1.5 |

          - When Generated

When SM selects a new operating channel.

          - Effect of Receipt

Database service will process received primitive, update it’s records regarding operating TV channel usage. If successfully received the database service will respond with M-DB-OPERATING-TV-CH-SELECTION-INDICATION to inform the SM that the M-DB-OPEARTING-TV-CH-SELECTION-CONFIRMATION primitive was received. If any error codes come up as a result, e.g. device making the request is not registered, it’s operating outside of an area where channels are available; the database service will indicate the error in M-DB-OPERATING-TV-CH-SELECTION-INDICATION

        - M-DB-OPERATING-TV-CH-SELECTION-INDICATION

          - Purpose

Allows the database service to inform SM that it successfully received a operating channel update.

          - SAP Type

M-SAP

          - Operation Type

Event Indication

          - Destination

SM

          - Data

- M-DB-OPERATING-TV-CH-SELECTION-INDICATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Status/Error Code | Integer | 2 bytes | Error code as defined by IETF PAWS protocol, section 5.13 |
| Timestamp | Character String | 20 octets | Timestamp as copied from the M-DB-OPERATING_TV-CH-SELECTION-CONFIRMATION that was received by database service. Time format is defined in Section 14.1.5 |

          - When Generated

When SM selects a new operating channel.

          - Effect of Receipt

Database service will process received primitive, update it’s records regarding operating TV channel usage. If successfully received the database service will respond with M-DB-OPERATING-TV-CH-SELECTION-INDICATION to inform the SM that the M-DB-OPEARTING-TV-CH-SELECTION-CONFIRMATION primitive was received.

      - BS Configuration and Monitoring Primitives

The BS SM occasionally sends the available channel list to its higher layers for additional channel classification.  The available channel list can be presented to its higher layers to have channels classified as disallowed.  The classification of an operating channel by the BS is also performed by its higher layers.  The M-SAP is an interface that provides a means of exchange information between the SM and the higher layers in the BS.  Table 265 summarizes the primitives supported by the SM to pass the available channel list and to receive disallowed channel classifications and the selected operating channel through the M-SAP interface. The primitives are discussed in the subclauses referenced in the table.

- — BS Configuration and Monitoring Primitives supported by the M-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| M-AVAIL-TV-CH-REPORT | 14.2.1.4.1 | 14.2.1.4.2 |  |
| M-DISALLOWED-TV-CHS |  |  | 14.2.1.4.3 |
| M-OPERATING-TV-CH |  |  | 14.2.1.4.4 |

        - M-AVAIL-TV-CH-REPORT-REQUEST

          - Purpose

Sent by the BS SM to request the higher layers for a selection of an operating channel based on the available channel list information provided this primitive. Table 272 specifies the parameters for the M-AVAIL-TV-CH-REPORT-REQUEST primitive.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

BS Higher Layer (see Section 5.1, Figure 3)

          - Data

- M-AVAIL-TV-CH-REPORT-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Number of Available Channels | Integer | 1 byte | Number of channels provided |
| For(i=0; i<Number of Available Channels; i++) { |  |  |  |
| Channel Start Frequency | Integer | 8 bytes | Channel start frequency in Hz |
| Channel End Frequency | Integer | 8 bytes | Channel end frequency in Hz |
| Maximum Allowed EIRP | Integer | 1 byte | Maximum allowed EIRP on channel ‘TV Channel Number’, defined on the range -64 dBm to +63.5 dBm in 0.5 dB steps. |
| } |  |  |  |
| Mode | Integer | 1 byte | The expected response from the higher layers: 0x00 = Test 0x01 = Request for disallowed channel classification 0x02 = Request for selection of operating channel 0x03 – 0xFF: Reserved |
| Timestamp | Character String | 20 characters | Timestamp of the present request at time of transmission. Time format defined in 14.1.5. |

          - When Generated

The M-AVAIL-TV-CH-REPORT-REQUEST primitive is generated by the BS SM and issued to the higher layers either (depending on the mode) to request disallowed channel classification or selection of an operating channel during BS initialization as described in 7.14.1.

          - Effect of Receipt

When the higher layers receive the M-AVAIL-TV-CH-REPORT-REQUEST primitive, they generate an M-AVAIL-TV-CH-REPORT-INDICATION primitive to notify the SM if the request was successfully received.

        - M-AVAIL-TV-CH-REPORT-INDICATION

          - Purpose

The M-WRAN-SERVICE-REPORT-INDICATION primitive allows the higher layers to inform the SM if the receipt of the available channel list was successful. Table 267 specifies the parameters for the M-AVAIL-TV-CH-REPORT.confirm primitive.

          - SAP Type

M-SAP

          - Operation Type

Event Indication

          - Destination

BS SM

          - Data

- M-AVAIL-TV-CH-REPORT-INDICATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Status | Integer | 1 byte | The value indicates whether the request to select operating channel or disallowed channel classification was successfully generated: 0x00 = SUCCESS 0x01 = INVALID_REQUEST 0x02-0xFF = Reserved |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-AVAIL-TV-CH-REPORT-REQUEST. Time format defined in Section 14.1.5. |

          - When Generated

The M-AVAIL-TV-CH-REPORT-INDICATION primitive is generated by the higher layers and issued to its MIB when an M-AVAIL-TV-CH-REPORT-REQUEST primitive is received to indicate whether the available channel list was successfully generated.

          - Effect of Receipt

When the SM of a CPE receives the M-AVAIL-TV-CH-REPORT-INDICATION primitive and depending on the mode, it expects the higher layers either to return nothing, an M-DISALLOWED-TV-CHS-CONFIRMATION primitive with classified disallowed channels, or an M-OPERATING-TV-CH-CONFIRMATION with the selected channel.

        - M-DISALLOWED-TV-CHS-CONFIRMATION

          - Purpose

The M-DISALLOWED-TV-CHS-CONFIRMATION primitive is used by the higher layers to return the disallowed channels on the available channel list to the SM per its request.  Table 268 specifies the parameters for the M-DISALLOWED-TV-CHS.indication primitive.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

BS SM

          - Data

- M-DISALLOWED-TV-CHS-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Number of Disallowed Channels | Integer | 1 byte | Number of disallowed channels provided |
| For(i=0; i<Number of Disallowed Channels; i++) { |  |  |  |
| Channel Start Frequency | Integer | 8 bytes | Channel start frequency in Hz |
| Channel End Frequency | Integer | 8 bytes | Channel end frequency in Hz |
| } |  |  |  |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-AVAIL-TV-CH-REPORT-REQUEST. Time format defined in Section 14.1.5 |

          - When Generated

The M-DISALLOWED-TV-CHS-CONFIRMATION primitive is generated by the higher layers and issued to the MIB to indicate the disallowed channels from the available channel list.

          - Effect of Receipt

When the SM receives the M-DISALLOWED-TV-CHS-CONFIRMATION it will identify whether the response to its request for the higher layers to classify channels as disallowed from the available channel list was successfully received by the higher layers, in which case, the SM will obtain the classified disallowed channels and the BS will continue to the following steps of initialization and perform detection, described in 7.14.1.6. If the response is not successful the SM may decide to issue another request.

        - M-OPERATING-TV-CH-CONFIRMATION

          - Purpose

The M-OPERATING-TV-CH-CONFIRMATION primitive is used by the higher layers to return the selected operating channel on the available channel list to the SM per its request.  Table 269 specifies the parameters for the M-OPERATING-TV-CH-CONFIRMATION primitive.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

BS SM

          - Data

- M-OPERATING-TV-CH-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Channel Start Frequency | Integer | 8 bytes | Channel start frequency in Hz |
| Channel End Frequency | Integer | 8 bytes | Channel end frequency in Hz |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-AVAIL-TV-CH-REPORT-REQUEST. Time format defined in Section 14.1.5. |

          - When Generated

The M-OPERATING-TV-CH-CONFIRMATION primitive is generated by the higher layers and issued to the BS SM to indicate the selected operating channel from the available channel list.

          - Effect of Receipt

When the SM receives the M-OPERATING-TV-CH-CONFIRMATION it will identify whether the response to its request for the higher layers to select the operating channel from the available channel list was successfully received by the higher layers, in which case, the SM will obtain the selected operating channel and the BS will continue to commence operation on the selected channel. If the response is not successful the SM may decide to issue another request.

      - CPE Reports the Resulting Available WRAN Services List

The selection of WRAN service by the CPE is performed by its higher layers.  The M-SAP is an interface that provides a means of exchange information between the SA and the higher layers.  Table 276 summarizes the primitives supported by the SM to pass the available WRAN services list and the selected WRAN service through the M-SAP interface. The primitives are discussed in the subclauses referenced in the table.

- — Available WRAN Services List Primitives supported by the M-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| M-WRAN-SERVICE-REPORT | 14.2.1.5.1 |  | 14.2.1.5.3 |
| M-WRAN-SERVICE-INDICATION |  | 14.2.1.5.2 |  |

        - M-WRAN-SERVICE-REPORT-REQUEST

          - Purpose

The M-WRAN-SERVICE-REPORT-REQUEST primitive is sent by the CPE SA to request the higher layers for a selection of a WRAN service based on the available WRAN service list information provided this primitive.  Table 277 specifies the parameters for the M-WRAN-SERVICE-REPORT.request primitive.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

CPE Higher Layer (see Section 5.1, Figure 4)

          - Data

- M-WRAN-SERVICE-REPORT-REQUEST Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Number of Available Channels | Integer | 1 byte | Number of channels provided |
| For(i=0; i<Number of Available Channels; i++) { |  |  |  |
| Channel Start Frequency | Integer | 8 bytes | Channel start frequency in Hz |
| Channel End Frequency | Integer | 8 bytes | Channel end frequency in Hz |
| RSSI on Channel | Integer | 1 byte | Received signal strength on channel ‘TV Channel Number’, defined on the range -104 dBm to +23.5 dBm in 0.5 dB steps. |
| } |  |  |  |
| Timestamp | Character String | 20 characters | Timestamp of the present request at time of transmission. Time format defined in Section 14.1.5. |

          - When Generated

The M-WRAN-SERVICE-REPORT-REQUEST primitive is generated by the CPE SA and issued to the higher layers to request a selection of a WRAN service during CPE initialization as described in 7.14.2.5.

          - Effect of Receipt

When the higher layers receive the M-WRAN-SERVICE-REPORT-REQUEST primitive, it generates a M-WRAN-SERVICE-INDICATION primitive to notify the CPE SA if the request was successfully received. If successful, the higher layers will then send a M-WRAN-SERVICE-REPORT-RESPONSE primitive containing a selected channel.

        - M-WRAN-SERVICE-INDICATION

          - Purpose

The M-WRAN-SERVICE-INDICATION primitive allows the higher layers to inform the SA if the request to select an available WRAN service was successful. Table 278 specifies the parameters for the M-WRAN-SERVICE-INDICATION primitive.

          - SAP Type

M-SAP

          - Operation Type

Event Indication

          - Destination

CPE SA

          - Data

- M-WRAN-SERVICE-INDICATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Status | Integer | 1 byte | The value indicates whether the request to select operating channel or disallowed channel classification was successfully generated: 0x00 = SUCCESS 0x01 = INVALID_REQUEST 0x02-0xFF = Reserved |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-WRAN-SERVICE-REPORT-REQUEST. Time format |

          - When Generated

The M-WRAN-SERVICE-INDICATION primitive is generated by the higher layers and issued to the CPE SA when an M-WRAN-SERVICE-REPORT-REQUEST primitive is received to indicate whether the request to select a WRAN service was successfully generated.

          - Effect of Receipt

When the CPE SA receives the M-WRAN-SERVICE-INDICATION primitive with a successful status, then it shall await a M-WRAN-SERVICE-REPORT-CONFIRMATION containing a selected channel. If the CPE receives the M-WRAN-SERVICE-REPORT-CONFIRMATION with a failure status, it may stop or send another M-WRAN_SERVICE-REPORT REQEUST to request service of a new channel.

        - M-WRAN-SERVICE-REPORT-CONFIRMATION

          - Purpose

The M-WRAN-SERVICE-REPORT-CONFIRMATION primitive is used by the higher layers to return a selected WRAN channel from the available WRAN service list to the SA per its request.  Table 279 specifies the parameters for the M-WRAN-SERVICE-RESPONSE.indication primitive.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

CPE SA

          - Data

- M-WRAN-SERVICE-REPORT-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Channel Start Frequency | Integer | 8 bytes | Channel start frequency in Hz |
| Channel End Frequency | Integer | 8 bytes | Channel end frequency in Hz |
| Timestamp | Character String | 20 characters | Copied from the timestamp in the M-WRAN-SERVICE-REPORT-REQUEST. Time format defined in Section 14.1.5 |

          - When Generated

The M-WRAN-SERVICE-REPORT-CONFIRMATION primitive is generated by the higher layers and issued to the CPE SA to indicate the selected channel from the available WRAN service list.

          - Effect of Receipt

When the SA receives the M-WRAN-SERVICE-REPORT-CONFIRMATION it will identify whether the response to its request for the higher layers to select a channel from the available WRAN service list was successfully received by the higher layers, in which case, the SA will obtain the selected channel and CPE will continue to the following steps of initialization. If the response is not successful the SA may decide to issue another query.

      - Antenna Primitives

Essential antenna information is provided to the MAC by the antenna through the M-SAP. The M-SAP is an interface that provides a means of exchanging information between the SM at the BS MAC and the SSA at the CPE MAC and their respective antenna. Table 280 summarizes the primitives supported by the MAC to access antenna information through the M-SAP interface. The primitives are discussed in the subclauses referenced in the Table.

- — Available WRAN Services List Primitives supported by the M-SAP

| Name | Request | Indication | Confirm |
| --- | --- | --- | --- |
| M-ANTENNA-INTEGRATED | 14.2.1.6.1 |  | 14.2.1.6.2 |
| M-ANTENNA-INFOMRATION | 14.2.1.6.3 |  | 14.2.1.6.4 |

        - M-ANTENNA-INTEGRATED-REQUEST

          - Purpose

The M-ANTENNA-INTEGRATED-REQUEST primitive allows the MAC to identify whether the device’s antenna is integrated or non-integrated through the M-SAP in order to know whether it is required to get antenna gain information for calculation of EIRP. The M-ANTENNA-INTEGRATED-REQUEST primitive has no attributes.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

Antenna Interface (see 9.12.2)

          - Data

There are no attributes for this primitive.

          - When Generated

The M-ANTENNA-INTEGRATED-REQUEST primitive shall be generated by the MAC and issued to its antenna to identify the whether its antenna is integrated or non-integrated.

          - Effect of Receipt

When the antenna receives the M-ANTENNA-INTEGRATED-REQUEST primitive, the antenna shall generate an M-ANTENNA-INTEGRATED-CONFIRMATION primitive to indicate whether the antenna is integrated or non-integrated.

        - M-ANTENNA-INTEGRATED-CONFIRMATION

          - Purpose

The M-ANTENNA-INTEGRATED-CONFIRMATION primitive allows the antenna to inform the MAC whether it is integrated or non-integrated through the M-SAP. Table 281 specifies the parameters for the M-ANTENNA-INTEGRATED-CONFIRMATION primitive.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

CPE, BS

          - Data

- M-ANENNA-INTEGRATED-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Antenna Type | Integer | 1 byte | The value indicates whether the antenna is integrated or non-integrated: 0x00 = integrated antenna 0x01 = non-integrated antenna 0x02-0xFF - Reserved |

          - When Generated

The M-ANTENNA-INTEGRATED-CONFIRMATION primitive shall be generated by the antenna and issued to its MAC when an M-ANTENNA-INTEGRATED-REQUEST primitive is received to indicate whether the antenna is integrated or non-integrated through the M-SAP.

          - Effect of Receipt

When the MAC receives the M-ANTENNA-INTEGRATED-CONFIRMATION primitive, the SM at the BS and the SSA at the CPE shall identify whether the antenna is integrated or non-integrated.

        - M-ANTENNA-INFORMATION-REQUEST

          - Purpose

The M-ANTENNA-INFORMATION-REQUEST primitive allows the MAC to request antenna information from the antenna. The M-ANTENNA-INFORMATION-REQUEST primitive has no attributes.

          - SAP Type

M-SAP

          - Operation Type

Information Request

          - Destination

Antenna Interface (see 9.12.2)

          - Data

This primitive has no attributes

          - When Generated

The M-ANTENNA-INFORMATION-REQUEST primitive shall be generated by the SM of a BS or the SSA of the CPE and issued to their respective antenna for antenna information.

          - Effect of Receipt

When the antenna receives the M-ANTENNA-INFORMATION-REQUEST primitive, the antenna shall generate an M-ANTENNA-INFORMATION-CONFIRMATION containing information that describes the attributes of the antenna.

        - M-ANTENNA-INFORMATION-CONFIRMATION

          - Purpose

The M-ANTENNA-INFORMATION-CONFIRMATION primitive is used to respond to the MAC request with antenna information. Table 282 specifies the parameters for the M-ANTENNA-INFORMATION-CONFIRMATION primitive.

          - SAP Type

M-SAP

          - Operation Type

Information Confirmation

          - Destination

CPE, BS

          - Data

- M-ANTENNA-INFORMATION-CONFIRMATION Parameters

| Name | Type | Length | Description |
| --- | --- | --- | --- |
| Number of channels | Integer | 1 byte | This indicates the number of channels that gain information is provided for. |
| For(i=0; i<Number of Available Channels; i++) { |  |  |  |
| Channel_Number | Integer | 1 byte | TV Channel Number |
| Maximum Gain | Integer | 1 byte | Maximum Gain on ‘TV Channel Number’, defined on the range -63.75 dBi to 0 dBi in 0.25 dB steps. |
| } |  |  |  |

          - When Generated

The M-ANTENNA-INFORMATION-CONFIRMATION primitive shall be generated by the antenna and issued to the MAC to respond with information about the antenna.

          - Effect of Receipt

When the MAC receives the M-ANTENNA-INFORMATION.response, MAC shall store the maximum gain (dBi) for each channel so that the device can convert from transmit power to EIRP.

[============================================End of new text=========================================]

- Add the following text to the end of section 14.1.5:

[============================================Start of new text=========================================]

      - Handling Timestamp Fields in Primitives

Primitives with data fields that represent a time value or timestamp, shall be defined in terms of UTC time. The format of the string is pulled from IETF RFC 3339. The following format shall be used: “YYYY-MM-DDThh:mm:ssZ”; where YYYY is the 4 digit year, MM is the 2 digit month (1..12), DD is the 2 digit day (01..31), hh is the 2 digit hour (00..23), mm is the 2 digit minute (00..59), and ss is the 2 digit minute (00..59).

      - Handling Channel Number fields in primitives

Primitives with data fields that represent a TV channel, define that channel in terms of the start frequency and end frequency in Hz. The start and end frequencies represent the frequencies of the lower and upper band edges. All primitives, except Antenna Primitives (see Section 14.1.5.6) shall use this formatting of TV channel numbers. When forming MAC management messages from C-SAP, SM-SSF-SAP, and SM-GL-SAP, the start frequency and end frequency shall be translated to an 8bit channel number using the tables in Section A.5 of Annex A.

[============================================End of new text=========================================]

References:

[1] “TGa Comment Database”, https://mentor.ieee.org/802.22/dcn/12/22-12-0085-01-0mib-tga-lb2-comment-database.xls

30 Note that this interface allows enlistment of TVBD devices (beyond IEEE 802.22 BS and CPE) that may not need to be formally “registered” as required by the FCC R&O 08-260 for potential broader capability and applicability of the database service.

31 Antenna directionality will represent the antenna gain pattern in the horizontal plan in dB referred to the gain of its main lobe and it is assumed that the database service will use its knowledge of the geolocation of the base station and the device being enlisted to calculated the azimuth of the device antenna main lobe for interference calculations in the case of base station and CEP operation. Omni directional antennas shall be assumed as the default.

33 See Footnote 3

33 See Footnote 3